3GPP TSG-RAN WG1 Meeting #119	R1-2410309

Orlando, United States of America, November 18th – 22nd, 2024

Agenda Item:	5

Source:	Ericsson

Title:	Discussion LS on OCC for CB-msg3 NPUSCH

Document for:	Discussion

# 1	Introduction

During RAN2 #127bis, there were Rel-18 discussions on the potential support of Orthogonal Cover Codes (OCC) for Contention Based (CB)-msg3 NPUSCH, which resulted in a liaison Statement (LS) entitled “LS on OCC for CB-msg3 NPUSCH” [1].

The overall description in the LS states the following:

RAN2 is working on CB-msg3 EDT in IDLE state and would like to ask RAN1 whether the OCC solution for NPUSCH on which RAN1 is working can be applied to CB-msg3 NPUSCH or not.

The LS contains the following action for RAN1:

RAN2 respectfully asks RAN1 to take the information above into consideration and provide the feedback

In this contribution we provide our views on the received LS accounting for the overall description and the action for RAN1.

# 2	Discussion

Below we describe the steps followed by legacy Early Data Transmission (EDT) as to compared with what is intended to be achieved with “CB-msg3 NPUSCH” EDT:

| Legacy EDT | Rel-19 CB-msg3 NPUSCH EDT |
| --- | --- |
| - By transmitting an NPRACH preamble from the configured set of EDT preambles, the NB-IoT UE indicates to the eNodeB that it has data to be transmitted via Msg3. - Upon detecting the EDT preamble, the eNodeB responds with Msg2 (NPDCCH [DCI Format N1] + NPDSCH). - Then, upon receiving the response from the eNodeB as per the previous step, the NB-IoT UE will transmit data through Msg3 (NPUSCH). - The small amount of data to be transmitted via Msg3 consists of TBSs < 1000 bits. - Thereafter, using Msg4 the eNodeB responds to the NB-IoT UE (NPDCCH [DCI Format N1] + NPDSCH). | - Skipped step. - Skipped step. - The NB-IoT UE will transmit data through Msg3 (NPUSCH) on time-frequency resources randomly selected from a pool of NPUSCH resources. - The small amount of data to be transmitted via Msg3 consists of TBSs < 1000 bits. - Thereafter, using Msg4 the eNodeB responds to the NB-IoT UE . |

In our understanding, the “Rel-19 CB-msg3 NPUSCH EDT” framework is still under development in RAN2 according with the following RAN2 agreements:

| Agreements RAN2# 127-bis |  |
| --- | --- |
| At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details. CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1 |  |

Due to the work in progress, what the “CB-msg3 EDT” configuration will provide in terms of “PUSCH resources for Msg3 transmission,” is not yet decided. On this matter, without knowing the time-frequency resource framework of “CB-msg3 EDT” (e.g., window/periodicity, starting point within the window/periodicity, frequency resources available, etc) is not possible to say with certainty whether using OCC for “CB-msg3 EDT” is feasible or not.

Nonetheless, it should be possible to provide a set of consideration under which OCC for “CB-msg3 EDT” could be feasible to be supported:

- Using NPUSCH Format 1 single-tone to perform OCC-based transmissions in “CB-msg3 EDT”. This because RAN1 has only agreed OCC schemes for NPUSCH Format 1 single-tone.

- Using an OCC length 2. This for avoiding creating a potential bottleneck for Msg4.

- The benefits/Gains of using OCC for “CB-msg3 EDT” depend on the collision rate (and successful decoding of the OCC’d UEs), which in turn depends on the design aspects mentioned in the previous bullets and the time-frequency resource framework of “CB-msg3 EDT”.

Overall, it should be technically feasible to support OCC for CB-msg3 NPUSCH assuming that there are well defined time-frequency resources, and that the UE will randomly select both the time-frequency resource and the OCC sequence.

Below we provide a high-level illustration of OCC for “CB-msg3 EDT”:

Figure 1: Example of applying OCC-based transmissions on a “CB-msg3 EDT” framework.

Due to the work in progress, what the “CB-msg3 EDT” configuration will provide in terms of “PUSCH resources for Msg3 transmission,” is not yet decided.

In relation with the previous observation, knowing the time-frequency resource framework of “CB-msg3 EDT” (e.g., window/periodicity, starting point within the window/periodicity, frequency resources available, etc) is not possible to say with certainty whether using OCC for “CB-msg3 EDT” is feasible or not.

RAN1 informs RAN2 the following set of considerations under which OCC for “CB-msg3 EDT” could be feasible to be supported:

- Using NPUSCH Format 1 single-tone to perform OCC-based transmissions in “CB-msg3 EDT”. This because RAN1 has only agreed OCC schemes for NPUSCH Format 1 single-tone.

- Using an OCC length 2. This for avoiding creating a potential bottleneck for Msg4.

- The benefits/Gains of using OCC for “CB-msg3 EDT” depend on the collision rate (and successful decoding of the OCC’d UEs), which in turn depends on the design aspects mentioned in the previous bullets and the time-frequency resource framework of “CB-msg3 EDT”.

- Overall, it should be technically feasible to support OCC for CB-msg3 NPUSCH assuming that there are well defined time-frequency resources, and that the UE will randomly select both the time-frequency resource and the OCC sequence.

# 3	Conclusion

Based on the discussion in the previous section we made the following observations:

Observation 1	Due to the work in progress, what the “CB-msg3 EDT” configuration will provide in terms of “PUSCH resources for Msg3 transmission,” is not yet decided.

Observation 2	In relation with the previous observation, knowing the time-frequency resource framework of “CB-msg3 EDT” (e.g., window/periodicity, starting point within the window/periodicity, frequency resources available, etc) is not possible to say with certainty whether using OCC for “CB-msg3 EDT” is feasible or not.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 informs RAN2 the following set of considerations under which OCC for “CB-msg3 EDT” could be feasible to be supported:

	Using NPUSCH Format 1 single-tone to perform OCC-based transmissions in “CB-msg3 EDT”. This because RAN1 has only agreed OCC schemes for NPUSCH Format 1 single-tone.

	Using an OCC length 2. This for avoiding creating a potential bottleneck for Msg4.

	The benefits/Gains of using OCC for “CB-msg3 EDT” depend on the collision rate (and successful decoding of the OCC’d UEs), which in turn depends on the design aspects mentioned in the previous bullets and the time-frequency resource framework of “CB-msg3EDT”.

Overall, it should be technically feasible to support OCC for CB-msg3 NPUSCH assuming that there are well defined time-frequency resources, and that the UE will randomly select both the time-frequency resource and the OCC sequence.

# 4	References

R2-2409242, “LS on OCC for CB-msg3 NPUSCH”, 3GPP TSG-RAN WG2 Meeting #127bis, Hefei, Anhui, China, 14 - 18 October, 2024.