3GPP TSG RAN WG1 #119			R1-2410366

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.11.5

Source:	Lenovo

Title:	Discussion on IoT-NTN TDD mode

Document for:	Discussion and decision

- Introduction

In the RAN#105 plenary meeting, a work item for Rel.19 IoT NTN was approved. One of the objectives is to specify the following to IoT NTN.

- Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4]

  - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2]

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]

The following agreements on uplink capacity enhancement in IoT NTN were achieved last meeting:

Agreement

The target operating DL CNR of NB-IoT NTN TDD is obtained following the parameters in TR 36.763 and modifying the carrier frequency to 1.6GHz:

- For LEO-600, the operating DL CNR is 4.91dB (i.e. with 0dBi UE antenna gain)

- For LEO-1200, the operating DL CNR is 5.51dB (i.e. with 0dBi UE antenna gain)

- When reporting link level simulation results, companies are encouraged to report the CNR margin with respect to the above operating DL CNRs.

- Companies may report values for other scenarios (e.g. UE antenna gain of -5.5 dBi, LEO-800) with the corresponding assumptions.

Agreement

For the study phase, RAN1 assumes as a baseline for evaluations (which includes DL synchronization as per the WID) that downlink PHY channels and signals are transmitted on the anchor carrier following subframe index/location and SFN mapping of NB-IoT NTN FDD.

- For the purpose of study, the DL subframes of the Rel-19 TDD pattern may not fully cover all the instances of the PHY channels and signals and eNB will not transmit the corresponding instances.

  - FFS: What is the TDD pattern

Agreement

RAN1 evaluates via link level simulations the following:

- NPSS/NSSS detection

- NPBCH decoding

Agreement

The impact of the TDD pattern at least on the following channels / signals are evaluated at least qualitatively (e.g. specification impact, number of available instances, etc.):

- SIB1-NB

- Other SIBs needed for basic NB-IoT NTN operation (e.g. SIB2, SIB31)

- Paging

- NPRACH / RAR

- NPSS/NSSS/NPBCH

- NPDCCH, NPUSCH, NPDSCH

- Note: Study for the above channels / signals may continue during the normative phase.

- It is up to companies to perform link level evaluations for some of the aspects above

Agreement

For link level simulations of NPSS / NSSS / NPBCH, RAN1 uses the following assumptions:

| Parameter | Value |
| --- | --- |
| Carrier frequency | 1.6GHz |
| Channel model | NTN TDL-C rural, 30 degrees |
| Frequency error / timing drift (including XO error) | 34 ppm for NPSS / NSSS (24ppm Doppler + 10ppm XO error) 0.1ppm for NPBCH |
| Variation of frequency error / variation of timing drift | 0.27ppm/s for NPSS / NSSS 0 for NPBCH |
| UE velocity | 3km/h |
| Target performance | For NPSS/NSSS: - 99% detection probability with 0.1% false alarm rate - FFS: How to define correct detection, including successful timing / frequency estimation / cell ID detection. For NPBCH: - 1% BLER For the channels / signals above, acquisition delay shall be reported for the corresponding detection probability |
| Other assumptions (e.g. combining length, frequency error hypothesis, etc.) | To be reported and justified by companies |

Agreement

For the study phase, RAN1 evaluates the following combination of N (in radio frames) and D (D = number of consecutive downlink subframes):

- N = 9:

  - D = 8

  - D = 20

  - D = 30

- N = 8:

  - D = 20

- Companies to report which subframe indices are included in the set of consecutive downlink subframes.

- Other combinations are not precluded to be reported by companies

NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase.

Agreement

For the study phase, RAN1 evaluates the following combination of N (in radio frames) and U (U = number of consecutive uplink subframes):

- N = 9:

  - U = 8

  - U = 20

  - U = 30

- N = 8:

  - U = 20

- Companies to report which subframe indices are included in the set of consecutive uplink subframes.

- Other combinations are not precluded to be reported by companies

- For evaluations including both uplink and downlink (if any), companies are encouraged to prioritize the case of D=U

NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase.

Agreement

For the study phase, RAN1 assumes as a baseline for evaluations that uplink PHY channels are transmitted on the anchor carrier following subframe index/location and SFN mapping of NB-IoT NTN FDD.

- For the purpose of study, the UL subframes of the Rel-19 TDD pattern may not fully cover all the instances of the PHY channels and eNB will not receive the corresponding instances.

  - FFS: What is the TDD pattern

In this contribution, detailed discussion of support on IoT NTN TDD mode are presented.

- Discussion

Legacy IRIDIUM TDMA Frame Structure

In the legacy IRIDIUM system, the fundamental unit of the TDMA channel is a time-slot as shown in Figure 1. Time slots are organized into frames.

- The frame consists of a 20.32 millisecond downlink simplex time-slot, followed by four 8.28 millisecond uplink time-slots and four downlink time-slots, which provide the duplex channel capability.

- The TDMA frame also includes various guard times to allow hardware set up and to provide tolerance for uplink channel operations.

- The simplex time-slot supports the downlink-only, ring and messaging channels. The Acquisition, Synchronization, and Traffic channels use the uplink time-slots. The Broadcast, Synchronization, and Traffic channels use the downlink duplex time-slots.

Figure 1: TDD frame structure used in the 1616-1626.5 MHz MSS band

TDD pattern for IoT NTN

As per the WID new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames (RF). The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline.

In order to fully be compatible to adopt NB-IoT deployed in the same frequency band of 1616-1626.5 MHz, RAN1 should focus on the frame structure with the case of N=9 (90ms periodicity) for the study and specification work.

Proposal 1: RAN1 should focus on the frame structure with N=9 for the 1616-1626.5 MHz band for the study and specification work.

Consecutive DL subframe in TDD pattern

In Figure 2 we show an example arrangement of the 90ms pattern for IoT NTN TDD mode, assuming an example consecutive DL subframes of 8 subframe for initial access. The consecutive DL subframe of 8ms is aligned with the legacy slot duration where the time duration of time slot for each one is 8.28ms for legacy IRIDIUM system. In the example, not all basic DL signals would be received by the UE within the 8ms consecutive DL subframes, so at least two period of TDD pattern is needed for DL synchronization (e.g., PSS/SSS/PBCH) at least for the initial access.

Figure 2: The number of consecutive DL subframes of 8 in TDD pattern

Proposal 2: The number of consecutive DL subframes of 8 in TDD pattern is the baseline and is assumed by UE in initial access.

Regarding the time relationship between the NBIoT frame and the TDD pattern, a time offset between the TDD pattern and the NBIoT frame boundary should be introduced, the time offset can be fixed as 3 or 4 subframes, which can be selected by the RAN1 based on the considering of scheduling delay for NPDSCH in the limited consecutive DL subframes.

Figure 3: A time offset between the TDD pattern and the NBIoT frame boundary is 3 or 4 subframes.

Proposal 3: A time offset between the TDD pattern and the NBIoT frame boundary should be introduced, the time offset can be fixed as 3 or 4 subframes.

In case more than one legacy DL time slots (e.g., 8.28ms) of Iridium systems can be used for NBIoT NTN, the consecutive DL subframes can be further broadcast in system information. The number of the set of DL subframes is indicated to be 24 as shown in Figure 4, corresponding 3 legacy DL time slots of Iridium systems.

Figure 4: The number of consecutive DL subframes indicated by system information.

Proposal 4: The number of consecutive DL subframes in TDD pattern is further indicated by system information.

Consecutive UL subframe in TDD pattern

Similar as the DL subframe indication, the number of consecutive UL subframes of 8 in TDD pattern is the baseline and can be further indicated by system information.

Proposal 5: The number of consecutive UL subframes of 8 in TDD pattern is the baseline and can be further indicated by system information.

Valid subframe/invalid subframe indication for DL/UL

Following the NBIoT valid/invalid subframe indication mechanism to avoid the potential collision with LTE in-band resource, TDD pattern in IoT NTN can introduce the valid/invalid subframe for DL and UL with bitmap manner separately, the bit length is determined by the consecutive DL/UL subframe in TDD pattern. In particular, in order to reuse the legacy NBIoT valid/invalid subframe indication parameter (e.g., subframeBitmap with length of 40bits), the consecutive DL subframe in TDD pattern can be assumed as 40, and the guard periods can be further extended to set the end part of consecutive DL subframe as invalid subframe.

Figure Valid subframe/invalid subframe indication for DL

Proposal 6: TDD pattern in IoT NTN can introduce the valid/invalid subframe for DL and UL with bitmap manner separately, the bit length is determined by the consecutive DL/UL subframe in TDD pattern.

SI / Paging Cycle

For legacy NB-IoT, the default paging cycle is {rf128, rf256, rf512, rf1024}, and the paging message may be transmitted every X={128, 256,…} radio frames. However, in the TDD pattern designed above, there will be the only several DL subframes for each N=9 radio frames. If the IoT NTN TDD mode follows the legacy paging cycle configuration, eNB may skip the paging transmission when the corresponding frame is for GP or UL, only transmit paging messages in the radio frame index satisfying nf mod X=0, where nf is the radio frame index and the corresponding subframes are valid subframes. The paging resources are reduced, and the performance of paging can’t be guaranteed. Therefore, the existing pre-configured resources for paging need to further study. Similarly, system information for legacy NBIoT is repeatedly transmitted in specific subframes of frames indicated by the si-RepetitionPattern within the SI window where the period of the SI window is configured as {rf64, rf128, rf256, rf512, rf1024, rf2048, rf4096}, and the corresponding potential new configuration for SI window needs further study.

Proposal 7: The existing pre-configured resources for SI/paging need to further study.

RACH

For legacy NB-IoT, NPRACH resource for FDD is configured with the period of 40ms, 80ms, 160ms, 240m, …, 2560ms, and the start time within a NPRACH periodicity is 8ms, 16ms, 32ms, …,1024ms. Similar to the issue identified for paging/SI, the repetitions of NPRACH transmission are dropped when it collides with subframes for GP or DL subframes. The coverage of NPRACH will be significantly affected due to the lengthy contiguous GP and DL resources if legacy RACH configuration is followed. Therefore, new NRACH resources configuration need to further study, at least to align with the new TDD pattern.

Proposal 8: New NRACH resources configuration need to further study, at least to align with the new TDD pattern.

- Conclusions

In this contribution, considerations of IoT NTN TDD mode are provided. The following proposals are present:

Proposal 1: RAN1 should focus on the frame structure with N=9 for the 1616-1626.5 MHz band for the study and specification work.

Proposal 2: The number of consecutive DL subframes of 8 in TDD pattern is the baseline and is assumed by UE in initial access.

Proposal 3: A time offset between the TDD pattern and the NBIoT frame boundary should be introduced, the time offset can be fixed as 3 or 4 subframes.

Proposal 4: The number of consecutive DL subframes in TDD pattern is further indicated by system information.

Proposal 5: The number of consecutive UL subframes of 8 in TDD pattern is the baseline and can be further indicated by system information.

Proposal 6: TDD pattern in IoT NTN can introduce the valid/invalid subframe for DL and UL with bitmap manner separately, the bit length is determined by the consecutive DL/UL subframe in TDD pattern.

Proposal 7: The existing pre-configured resources for SI/paging need to further study.

Proposal 8: New NRACH resources configuration need to further study, at least to align with the new TDD pattern.

- References

3GPP TR 36.763 V1.0.0 (2021-06)