3GPP TSG RAN WG1 #119			R1-2410396

Orland, US, November 18th – 22nd, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on adaptation of common signal/channel transmissions

Agenda Item:	9.5.3

Document for: 	Discussion and Decision

- Introduction

At the RAN#102 meeting, the new work item of “Enhancement of network energy savings for NR” was agreed [1]. In this contribution, we provide our views on the third objective on adaptation of common signal/channel transmissions, and the scope of the objective is shown as below.

- Discussion

For NES purpose, we discuss the candidates for adaptations and applicable scenarios for each common signal/channel. First, we’d like to clarify how to obtain NES gain by adaptation over the legacy operation. One approach is to introduce new properties/parameters to reduce occasion/duration in time domain at gNB. The other approach is to introduce more efficient/dynamic mechanism to change property of common signal/channel following a change in condition/situation (e.g., when configured PRACH capacity is lacking, NW decides to provide additional PRACH resources more dynamically by less overhead signaling to satisfy the situation). Any techniques in this objective should be evaluated from these perspectives for NES operation.

Observation 1:

- For adaptation of common signal/channel in NES operation, the following two approaches should be considered.

  - Introduce new properties/parameters to reduce the gNB TX/RX occasion/duration in time domain.

  - Introduce more efficient/dynamic mechanism to change properties of common signals/channels.

2.1 SSB adaptation in time domain

Legacy configuration/behavior

For initial cell selection, a UE assumes SSB periodicity of 20 ms, and after cell selection, the UE acquires the SSB configuration of the serving cell from SIB1 mainly for rate matching purpose. For neighboring cell measurement purpose, the UE would acquire neighboring cell’s SSB configurations. After RRC connected, the UE can acquire these parameters from RRC dedicated signaling (RRCReconfiguration).

These parameters included in SIB1/RRCReconfiguration can be updated (i.e., adapted) even by using the current specification. For idle mode UEs deriving SSB/PRACH related parameters from SIB1, the SIB1 update is indicated by DCI format 1_0 as a “Short message”. It can be sent every default paging cycle*{2, 4, 8, 16}. That implies that adaptation of SSB and PRACH configurations in SIB1 can be done by the current specification with relatively long-term adaptation.

2.1.1	PCell adaptation

Conclusion@118bis There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 Agreement @118 For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3)

For idle/inactive mode UEs

In the previous meeting, no consensus was made for the support of adaptation of SSB for idle mode UEs in Rel-19.

For RRC connected UEs

In the RAN1#118 meeting, it was agreed that adaptation of CD-SSB on sync-raster would not be supported in Rel-19 NES. Our original preference was to support CD-SSB on syn-raster because NCD-SSB is an additional transmission on top of CD-SSB in a cell used as PCell, and NCD-SSB can only be used for specific UEs or purpose, e.g., RedCap UEs or dedicated BWP operation. In a typical PCell operation, there is only a CD-SSB. Through the discussion in the previous meeting, we understood how to achieve NES gain by introduction of adaptive NCD-SSB in Rel-19 is to operate both of sparser CD-SSB (e.g., 160 ms periodicity) and adaptive NCD-SSB with maintaining performance of communication according to the condition where relatively denser CD-SSB (e.g., 40 ms periodicity) is needed following the legacy spec. to achieve same level of performance.

Specifically speaking, if there are only low mobility UEs and/or non-critical services, Rel-19 SSB can be adapted to be sparser, while if there are high mobility UEs and/or critical services, Rel-19 SSB should be adapted to be denser. Thus, in our understanding, Rel-19 adaptive SSB is a technique to facilitate practical NES operation according to a situation (similar to Rel-19 adaptation of additional PRACH) rather than to improve NES gain over what can be implemented by the legacy spec.

Observation 2:

- By using SSB adaptation on PCell Rel-19, the only way how to achieve NES gain would be to operate both of sparser CD-SSB (e.g., 160 ms periodicity) and adaptive NCD-SSB, and adapt the NCD-SSB’s periodicity such that certain level of system performance can be maintained according to the condition.

  - Note: Relatively denser CD-SSB (e.g., 40 ms periodicity) is needed to achieve same level of performance in the legacy operation.

  - Note: Rel-19 NES capable UEs may use the adaptive NCD-SSB for BM/RRM/synchronization/QCL assumption.

On adaptation mechanism, it is important to consider in what condition/when adaptation can be practically beneficial for NW (i.e., gNB can decide to apply). Based on the above discussion, NW will adapt SSB periodicity property (e.g., to sparse SSB transmission) to obtain NES gain when there are only some particular types of UEs, e.g., only low mobility UEs and/or non-critical service UEs in an area covered by the cell with SSB adaptation. This operation would be done based on NW’s information and prediction with a historical tendency. We think the change of tendency (i.e., what types of UE are in the cell) could be handled by semi-static manner (i.e., not necessary to be handled by dynamic manner). In this sense, RAN1 should carefully evaluate what type of indication to be supported for adaptation of NCD-SSB considering the practical use-cases.

Proposal 1:

- Discuss and evaluate carefully in what condition/when gNB can decide to apply adaptation of SSB burst periodicity, and necessity of dynamic signaling for SSB adaptation on PCell.

2.1.2	SCell adaptation

Agreement @118 For adaptation mechanism(s) of SSB in time-domain, - … - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

On the FFS part about SCell SSB adaptation, we support CD-SSB adaptation (B1) in SCell. Regarding UE impact compared to legacy operation where the SSB is configured with periodicity > 20 ms for SCell, the impact on UE is same between the legacy SSB operation and potential adaptive SSB in Rel-19 in SCell. The main difference between CD-SSB and NCD-SSB is whether UEs in (initial) cell selection might find the SSB or not. Considering that it is not restricted in the legacy spec to operate CD-SSB with periodicity > 20 ms, which means some UE in initial cell selection may detect the SSB in some time while others may overlook it in some time. The problem that is not restricted in the spec now can be avoided or minimized by NW implementation, e.g., by setting the cell as cell barred. This would be a typical SCell operation where the SSB regardless of cell-defining or not is configured with periodicity > 20 ms in the real deployment. Therefore, there is no reason to prevent from supporting adaptation of CD-SSB on sync-raster on SCell.

Proposal 2:

- Support adaptation of CD-SSB on sync-raster in SCell (Option B1).

2.2 PRACH adaptation in time domain

Agreement@116bis For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211

Motivation/use-case of additional PRACH in our view

On adaptation of PRACH in time domain, it was discussed and agreed that at least PRACH adaptation should be performed based on additional PRACH resources. We believe that Rel-19 additional PRACH is to improve RACH capacity and/or RACH latency for better communication performance during NES operation (while gNB is infrequent wake-up to monitor legacy PRACH resources by configuring less ROs or longer periodicity).

The applicable case would be that NW may configure smaller number of ROs and/or longer RACH periodicity as a default for NES operation by reducing RO monitoring. Generally, NW may be able to expect the rough number of IDLE UEs, while cannot expect the probability of traffic occurrence for each of IDLE UEs. Configuring many ROs for those IDLE UEs will cause inefficient NW energy usage due to unused ROs, but if only a limited number of ROs is configured while traffic occurs for many idle UEs, a lot of PRACH transmissions will occur within a cell, that exceeds with the RO capacity. In order to address this, additional PRACH resources can be indicated to new UEs which try to access a NES cell.

2.2.1	Additional PRACH resources collision handling

@117 Agreement For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed - Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources Agreement At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule. - Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes. - Note: This mapping is not impacted by time domain PRACH adaptation - Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs. @118 Agreement Extend the RAN1#117 agreement on SSB-RO mapping rule for additional PRACH resources to Case 1 - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs Agreement For SSB-RO mapping rule for additional PRACH resources for Case 2. - Extend the RAN1#117 and RAN1#118 agreements on SSB-RO mapping

For a potential overlapping between additional PRACH resources and legacy PRACH resources, we support to introduce a mechanism of UE handling the overlap. In our view, when additional PRACH resources (for Rel-19 adaptation) and legacy PRACH resource overlap, it might result in that in a single RO, gNB has to expect PRACH which corresponds to multiple different SSB beams. To avoid the overlap, there seems to be two approaches: one is that NW configuration ensures no overlapping by its implementation, and the other is to let UE handle overlaps based on particular rules, e.g., similar to RO validation based on SSB and TDD configuration. It is not desirable from NW operator’s perspective that gNB always has to configure additional PRACH resource such that it does not overlap with any legacy PRACH resource at all, i.e., UE-side handling for overlap is preferred. A possible argument against UE-side handling may be that it may result in smaller number of valid ROs for additional PRACH resources, however we do not see this problematic. Even for the legacy PRACH resources, not all configured resources will be used or will be regarded as valid resources. For example, after SSB-RO mapping, there can be remaining PRACH resources that are not mapped to any SSB(s), or in TDD operation, some PRACH resources cannot be regarded as valid due to a lack of enough gap duration between SSB and PRACH resources.

In addition, legacy PRACH resources should not be impacted by additional resources, and hence it is a natural way for Rel-19 UE to regard additional PRACH resources overlapping with legacy PRACH resources as invalid.

Proposal 3:

- At least, not support specifying some configuration restriction on any overlap in both time and frequency, the following can be considered,

  - The overlapped RO(s) of additional PRACH resources with legacy PRACH resources in both time and frequency domain are regarded as invalid RO which is not used for SSB-RO mapping for the additional PRACH resource.

2.2.2	Frequency domain parameters of additional PRACH

Agreement@118bis For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different. - FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement). - Discuss further following options for signaling - Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources. - msg1-FrequencyStart at least for 4-step RACH - FFS: other applicable legacy frequency domain parameter(s) - Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources - Note: Offset to legacy frequency domain parameter(s) is a new parameter - FFS: applicable legacy frequency domain parameter(s) Agreement@117 For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed - Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources

In the last meeting, an agreement (the first one above) was made regarding the frequency domain resource for additional PRACH resources, while several points still remain FFS.

In terms of applicable cases, we interpreted the agreement as we should discuss the for which case(s) it is applicable to configure frequency domain resources for additional PRACH different from that of legacy PRACH. In our understanding, this FFS related to applicable cases intended that frequency setting of additional RACH could be used only for resolving overlap between legacy and additional in time domain. For example, if there is no overlap between legacy and “original” additional PRACH (which means additional PRACH resources have same frequency as legacy) (e.g., Case 1), frequency setting of additional RACH is not necessarily required.

In our current opinion, the above restriction would make unnecessary specification complexity, so we would not like to introduce any restrictions of frequency setting for additional PRACH, i.e., frequency parameters are applicable to additional PRACH resources in any cases.

In terms of options for signaling, we prefer option 1. The difference of each option is that option 1 can set parameters independently whereas frequency domain of additional resources can be set by how much gap they require compared to the location of legacy resources. In terms of specification perspectives, it might be straightforward to use the same way as the legacy resource determination in the frequency domain of additional PRACH resources.

Proposal 4:

- For options for signalling of additional PRACH resources, we prefer option 1.

  - Frequency domain parameters for additional PRACH resources should be applicable in any cases.

2.2.3	Time domain parameters of additional PRACH

Agreement@118bis For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources. Agreement@118bis For adaptation of PRACH in time-domain, support both of the following - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources - FFS: Additional details Agreement@118bis For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2(>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211) - For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2(>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211) Agreement@118 For adaptation of PRACH in time-domain, select at least one from the following alternatives for configuration of the additional PRACH resources - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources and - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g. - Opt 1-1: Scaled/adjusted PRACH configuration period - Opt 1-2: Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration - Opt 1-3: Muting/masking ROs - Opt 1-4: additional timing offset(s) - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources, - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g. - Opt 2-1: Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources) - Opt 2-2: Additional timing offset(s) - FFS: Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed) FFS: Additional frequency domain parameter(s) (e.g., freq. starting offset)

It was discussed whether PRACH configuration index is same and/or different from the one for the legacy PRACH resources and as a result, both cases (alt. 1 and alt. 2) are supported. Our understanding is that the goal of this functionality would be to maximize the duration in which gNB can turn into RX sleep. For that purpose, we believe it would be important that additional PRACH resources are confined with legacy ones in time domain, i.e., additional and legacy PRACH resources can be close to each other as much as possible in time domain so that a single RX sleep duration can be longer. To achieve such a condensed resource mapping by combining two PRACH resource from same of different configuration indexes, timing offset and muting/masking should be considered among the captured options in our view.

For timing offset, it was discussed that slot-level and frame-level offset can be considered. Our understanding is that using only frame-level timing offset may not be enough for dense resource mapping, so we support both frame-level and slot-level offsets.  In terms of frame-level timing offset, we think either changing the value of (x, y) or y, or introducing new parameters is fine. Therefore, we support Opt. 1-2 and Opt. 1-4. Note that we believe this direction should apply to both alternatives (alt. 1, and alt. 2).

For muting/masking of the resources, we think both alt. 1 and alt. 2 can be used. In our view, the configurable muting/masking ROs can offer more flexible PRACH pattern to achieve enough additional PRACH capacity and enough confined NW RX occasions (with legacy in time domain).

Proposal 5:

- For configurations of the additional PRACH resources for both alt. 1 and alt. 2, we support the following additional parameters:

  - Timing offset (slot-level, frame-level)

  - Muting/masking subset of ROs

- Conclusion

In this contribution, we provided the following observations and proposals of adaptation of common signal/channel transmissions for network energy saving.

Observation 1:

- For adaptation of common signal/channel in NES operation, the following two approaches should be considered.

  - Introduce new properties/parameters to reduce the gNB TX/RX occasion/duration in time domain.

  - Introduce more efficient/dynamic mechanism to change properties of common signals/channels.

Observation 2:

- By using SSB adaptation on PCell Rel-19, the only way how to achieve NES gain would be to operate both of sparser CD-SSB (e.g., 160 ms periodicity) and adaptive NCD-SSB, and adapt the NCD-SSB’s periodicity such that certain level of system performance can be maintained according to the condition.

  - Note: Relatively denser CD-SSB (e.g., 40 ms periodicity) is needed to achieve same level of performance in the legacy operation.

  - Note: Rel-19 NES capable UEs may use the adaptive NCD-SSB for BM/RRM/synchronization/QCL assumption.

Proposal 1:

- Discuss and evaluate carefully in what condition/when gNB can decide to apply adaptation of SSB burst periodicity, and necessity of dynamic signaling for SSB adaptation on PCell.

Proposal 2:

- Support adaptation of CD-SSB on sync-raster in SCell (Option B1).

Proposal 3:

- At least, not support specifying some configuration restriction on any overlap in both time and frequency, the following can be considered,

  - The overlapped RO(s) of additional PRACH resources with legacy PRACH resources in both time and frequency domain are regarded as invalid RO which is not used for SSB-RO mapping for the additional PRACH resource.

Proposal 4:

- For options for signalling of additional PRACH resources, we prefer option 1.

  - Frequency domain parameters for additional PRACH resources should be applicable in any cases.

Proposal 5:

- For configurations of the additional PRACH resources for both alt. 1 and alt. 2, we support the following additional parameters:

  - Timing offset (slot-level, frame-level)

  - Muting/masking subset of ROs

References

- RP-234065, New WID: Enhancements of network energy savings for NR, RAN Meeting #102, December 11th-15th, 2023.

- 3GPP, RAN1#118 meeting, Chairman’s notes