3GPP TSG-RAN WG2 Meeting #128															R2-2409583

# Orlando, US 18th Nov. 2024 – 22nd Nov. 2024                       revision of R2-2407954

Agenda Item:	8.2.3

Source: 	Xiaomi

Title:  	Discussion on A-IOT paging procedure

Document for:	Discussion and Decision

1	Introduction

In R19, a new SI Ambient IoT (Internet of Things) in NR is setup.

In RAN2#125bis meeting, RAN2 reached following agreements on paging.

Agreements

- Legacy paging message for device will not be supported.

- Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

- RAN2 assumes that the device will not support tracking/RAN area update procedure.

- For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message.    FFS pending the details from SA2

In RAN2#126 meeting, RAN2 reached following agreements on paging.

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

In RAN2#127 meeting, RAN2 reached following agreements on paging.

Agreements:

- Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

=>	wait for further RAN1 progress on indication of the start of access occasion.

In RAN2#127bis meeting, RAN2 reached following agreements on paging.

Agreements

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

- Based on this information the device determines whether to skip sending the response to paging.

- From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs.    RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

- If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

- For all use cases, device determines the random access type from the paging message.   FFS whether it is explicitly or implicitly.   FFS whether we will down selection for all RA types (2step, 3step, CFRA) or try to unify the design for 2step, 3step

In this paper, we would like to discuss the following issues in paging procedure.

- How to handle paging duplication from device side.

- Content of A-IOT paging message from CN and to device;

- Energy harvest;

2	Discussion

2.1 Paging duplication detection

In RAN2#127 meeting, to solve the device availability issue and paging reception failure issue, RAN2 greed that “Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN.”

From reader perspective, there are three options to implement the multiple/subsequent AIoT paging.

Option 1: It is up to reader implementation to when/how to trigger subsequent AIoT paging (i.e., repetition paging) and times of paging repetition.

Option 2: It is up to reader implementation to when/how to trigger subsequent AIoT paging (i.e., repetition paging) based on the times or paging interval provided by upstream node, e.g., CN.

Option 3: It is up to upstream node to trigger the multiple/subsequent AIoT paging one by one and reader will perform this transmission upon receiving the paging trigger from upstream node.

Proposal 1: RAN2 is kindly asked to discuss which option is WF for multiple/subsequent AIoT paging from reader perspective and paging repetition mechanism should be captured in TR.

- Option 1: It is up to reader implementation to when/how to trigger subsequent AIoT paging (i.e., repetition paging) and times of paging repetition.

- Option 2: It is up to reader implementation to when/how to trigger subsequent AIoT paging (i.e., repetition paging) based on the times or paging interval provided by upstream node, e.g., CN.

- Option 3: It is up to upstream node to trigger the multiple/subsequent AIoT paging one by one and reader will perform this transmission upon receiving the paging trigger from upstream node.

For paging trigger CF access, the target device id will be included in paging message, and the reader will also know the target device complete the service or not, then network can send paging only include the device id who did not complete the service, no need to repeat the paging message to include the device id who had complete the service.

Proposal 2: Multiple/subsequent AIoT paging is only used for paging to trigger CB access trigger, i.e., paging trigger CF access will not support multiple/subsequent AIoT paging.

If multiple/subsequent AIoT paging is supported, how to identify the paging duplication and how to handle the duplicated paging message from device perspective? In RAN2#127bis meeting, RAN2 agreed that paging message can include information to avoid duplicate response and device determines whether to skip sending the response to paging based on this information. But the details are FFS.

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

- Based on this information the device determines whether to skip sending the response to paging.

In NR, there are several mechanisms for changing/duplication detection, E.g.,

- SIB valueTag (0..31) is used to identify whether the SIB is changed or not if the UE comes from a coverage hole.

- NDI (1 bit) in DCI is used to indicated the current transmission is new transmission or retransmission.

- SN number in PDCP or RLC header is used for duplication detection.

Some additional information for paging duplication detection was mentioned in last RAN2 meeting, e.g.,

- A-IOT Session id

- Timer based solution;

These mechanisms can also be considered for ambient IOT paging duplication detection. We think NDI is only 1 bit and it is too short. SIB value tag or SN are similar for ambient IOT paging duplication and it can be considered. Some companies also think the UE can store initial paging message and compare the paging message content. However, I think the paging message content should be stored in the memory and it will cost more memory and it is complex to compare the paging message content. Some companies also think the ambient IOT task id is included in paging message and this task id is used for paging duplication detection. However, it is not clear this kind of information is visible to the device or not. Furthermore, we think the task id is too long, and it will increase the size of paging and also waste the memory to store it in device. Some companies also propose timer-based solution, but we think timer-based solution does not work. It is hard for device to maintain a timer because the device may power off and it is also complex for device to maintain a timer because the device should update the timer frequent. Furthermore, we are not sure whether it is possible for device to maintain a timer for a long time.

Proposal 3: Paging value tag like SIB valueTag is included in paging message for paging duplication detection. FFS on size of paging value tag.

2.2 Content of A-IOT paging message from CN and to device

RAN2 agreed that AIOT paging may contain (1) an ID of a single A-IoT device; (2) a group ID that maps to multiple A-IoT devices; (3) all devices; (4) multiple IDs of A-IoT devices;

1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

According to RAN1 agreements, the TDM/FDM resource will be used for initial access and the concerned devices can access the network on different TDM/FDM resource and it is common understanding that CB/CF access resource should be indicated to the devices before the device initial the initial access procedure. If CF access is triggered, the reader will also allocate specific resource for each device. So, the reader should know some information to decide CB or CF access is triggered and corresponding resource allocation.

In RAN2#127 meeting, RAN2 agreed that some information from CN is visible to the reader. For (1) and (4), CF access can be triggered. For (2) and (3), CB access is triggered. So reader should know the target ID information and determine CF or CB access is triggered. If CB access is triggered, the reader will send R2D command to define to slot and corresponding access occasions for devices to perform CB access in this slot. For (4), it is up to reader to decide to trigger CB access or CF access.

Agreements - At least the following information are considered useful to be visible to the reader from CN - The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful - targeted for one or more than one devices; - approximate number of target devices (if available). FFS on mandatory/optional 2 RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

Proposal 4a: The target ID related information in paging from CN should be visible to reader and reader can decide to trigger CB or CF access, i.e., single device ID from CN will trigger CF access, Group ID or “all” from CN will trigger CB access

Proposal 4b: It is up to reader to decide to trigger CB access or CF access when multiple device IDs are from CN, i.e., access type is not indicated from CN.

From device perspective, the device can know the CF or CB access according to the device id related information and resource allocation for access.

Proposal 4c: The device will know CF access or CB access according to the device related ID information and resource allocation in paging message, i.e., CF or CB access indication will not be included to device from reader in paging message.

In R19, only DT and DO-DTT will be triggered, the device will response based on network R2D command trigger step by step. So, we cannot see the necessary to make the device know the service type, i.e., inventory only or command only or inventory + command.

Proposal 5: The service type of A-IoT (e.g., inventory only, command only, inventory + command) is not indicated to device from reader.

The key point of contention-free access is there is no collision of RN16 and access occasion for the access and followed communication. So, the per device access occasion should be configured for the target device in paging message, the same reason for RN16 if needed in CF access.

Proposal 6: For CF access trigger, per device access occasion and RN 16 if needed is indicated per device in paging message, FFS implicit or explicit.

For CB access, we assume “slot” like slot in RFID is still used in ambient IOT, so there is R2D trigger command to define slot beginning and the previous slot ending. So, the radio resource configuration can be configured in:

Option 1: paging message;

Option 2: R2D command message.

For option 2, the reader can change the access occasion in one slot according to the access load in previous slot.

Proposal 7: The radio resource configuration for CB access is included in (1) R2D command message, i.e., the command indicates the new slot beginning or (2) paging message, i.e, access occasion configuration in each slot is same.

Some companies may also want to include 2/3 step CB access indication in paging message and device can know which CB access type can be performed. However, it is not clear whether both 2 step CB access and 3 step CB access will be supported or not. We can wait for agreement on this in RACH session.

2.3 Energy harvesting impact

From device perspective, there are 4 phases to monitor R2D command or communication with reader.

Phase 1: paging monitor.

Phase 2: after paging until its own slot comes.

Phase 3: CB/CF access phase including device id reporting.

Phase 4: command procedure.

In each phase, the device may fail to receive the R2D command due to limited energy. So RAN2 can decide which phase should be considered for energy harvest impact study.

Proposal 8: RAN2 is kindly asked to discuss which phase will be considered for energy harvest impact study, i.e., (1) paging monitor. (2) after paging until its own slot comes. (3) CB/CF access phase including device id reporting. (4) command procedure.

During the RAN1#118 meeting, the following agreement has been achieved for energy harvesting. The potential solutions are under RAN1 discussion, RAN2 can wait for progress in RAN1.

Agreement For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection.

Proposal 9: RAN2 waits for RAN1 progress on potential solutions of energy harvest.

3	Conclusion

In this contribution, we discussed the procedures and signalling for the initial access in A-IOT and have corresponding proposals.

Paging duplication detection:

Proposal 1: RAN2 is kindly asked to discuss which option is WF for multiple/subsequent AIoT paging from reader perspective and paging repetition mechanism should be captured in TR.

- Option 1: It is up to reader implementation to when/how to trigger subsequent AIoT paging (i.e., repetition paging) and times of paging repetition.

- Option 2: It is up to reader implementation to when/how to trigger subsequent AIoT paging (i.e., repetition paging) based on the times or paging interval provided by upstream node, e.g., CN.

- Option 3: It is up to upstream node to trigger the multiple/subsequent AIoT paging one by one and reader will perform this transmission upon receiving the paging trigger from upstream node.

Proposal 2: Multiple/subsequent AIoT paging is only used for paging to trigger CB access trigger, i.e., paging trigger CF access will not support multiple/subsequent AIoT paging.

Proposal 3: Paging value tag like SIB valueTag is included in paging message for paging duplication detection. FFS on size of paging value tag.

Content of A-IOT paging message from CN and to device:

Proposal 4a: The target ID related information in paging from CN should be visible to reader and reader can decide to trigger CB or CF access, i.e., single device ID from CN will trigger CF access, Group ID or “all” from CN will trigger CB access

Proposal 4b: It is up to reader to decide to trigger CB access or CF access when multiple device IDs are from CN, i.e., access type is not indicated from CN.

Proposal 4c: The device will know CF access or CB access according to the device related ID information and resource allocation in paging message, i.e., CF or CB access indication will not be included to device from reader in paging message.

Proposal 5: The service type of A-IoT (e.g., inventory only, command only, inventory + command) is not indicated to device from reader.

Proposal 6: For CF access trigger, per device access occasion and RN 16 if needed is indicated per device in paging message, FFS implicit or explicit.

Proposal 7: The radio resource configuration for CB access is included in (1) R2D command message, i.e., the command indicates the new slot beginning or (2) paging message, i.e, access occasion configuration in each slot is same.

Energy harvesting impact:

Proposal 8: RAN2 is kindly asked to discuss which phase will be considered for energy harvest impact study, i.e., (1) paging monitor. (2) after paging until its own slot comes. (3) CB/CF access phase including device id reporting. (4) command procedure.

Proposal 9: RAN2 waits for RAN1 progress on potential solutions of energy harvest.