3GPP TSG-RAN WG2 Meeting #128	R2-2409591

Orlando, USA, 18th – 22th November, 2024

Agenda Item:	8.9.3

Source:	Huawei, HiSilicon

Title:	Further consideration on UL capacity enhancement

Document for: Discussion and Decision

# 1	Introduction

According to the RAN plenary #105 meeting, the following objectives on IoT-NTN [1] were agreed:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In the last meeting, RAN2 has made the following agreements:

Agreements:

- RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

- RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed.

- At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

- CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

- RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

- At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission

- The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

- CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

In this contribution, we will discuss the details of SA and DSA mechanisms.

# 2	Discussion

## 2.1 SA

### 2.1.1 RSRP-based validation for CB-msg3 EDT

In the last meeting, RAN2 has agreed that the running TAT is not needed to initiate a CB-msg3 EDT transmission for 3.75kHz SCS NB-IoT and eMTC CE mode A. So obviously the RSRP-based validation parameter is not needed for these two cases.

Besides, RAN2 should assume TA validation mechanism may still be needed for 15kHz SCS NB-IoT and eMTC CE mode B, i.e., at least TAT is needed. However, whether the RSRP-based validation parameter is needed or not for these two cases is further discussed below.

Although we are calling it CB-msg3 EDT now, it is similar as legacy PUR if TA needs to be validated. For TA validation, the following criteria as specified in TS 36.331 for TN.

#### 5.3.3.19 Timing alignment validation for transmission using PUR The UE shall consider the timing alignment value for transmission using PUR to be valid when the following conditions are fulfilled: 1> either pur-TimeAlignmentTimer is not configured or pur-TimeAlignmentTimer is running as confirmed by lower layers; and 1> either pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT) is not configured or the following conditions are fulfilled: 2> compared to the stored serving cell reference (N)RSRP value, the serving cell (N)RSRP has not increased by more than increaseThresh; and 2> compared to the stored serving cell reference (N)RSRP value, the serving cell (N)RSRP has not decreased by more than decreaseThresh;

In R17 IoT NTN, RAN4 has agreed that RSRP-based validation requirements is not needed for TA validation in RAN4#105 since the RSRP doesn’t change much in NTN:

Issue 2-3-1: PUR, RSRP-based TA validation - Agreement - Do not define requirements for the legacy RSRP-based TA validation for PUR for both GEO and LEO

So the RSRP-based validation parameter is not needed for 15kHz SCS NB-IoT and eMTC CE mode B either.

Observation 1: There is no RAN4 requirement for configuring RSRP-based validation parameter in IoT NTN.

Proposal 1: The RSRP-based validation parameter is not needed for CB-msg3 EDT in IoT NTN.

### 2.1.2 How to provide the CB-msg3 EDT Configuration

During the last meeting, it was agreed at least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. However, this is only applied to 3.75kHz SCS NB-IoT and eMTC CE mode A, which don’t need TA validation.

For 15kHz SCS NB-IoT and eMTC CE mode B, UE still needs to be configured with CB-msg3 EDT Configuration by RRC dedicated signalling, e.g., RRCRelease. Or alternatively these UE can share the resource configured in the system information and only acquires TA validation parameters via RRCRelease.

Proposal 2: For 15kHz SCS NB-IoT and eMTC CE mode B, at least TA validation parameters need to be acquired via RRC dedicated signalling.

### 2.1.3 Contention resolution of CB-msg3 EDT

Since the contention-based MSG3 resource is shared by multiple UEs, when different UEs choose the same resource, contention occurs just like what may happen during the RACH procedure. Therefore, contention resolution is also needed via Msg4 for the contention-based MSG3 transmission. To realize this, similar mechanism as contention resolution during RACH can be used. Specifically, the network obtains the UE ID in Msg3 and identifies the winner of the contention. And in order to indicate the contention resolution result to the UE, the NW can send a MAC CE including information of the UE ID via Msg4 for UE to check.

Proposal 3: A MAC CE including UE ID information is used for contention resolution for CB-msg3 EDT. FFS on the detailed design of the MAC CE.

Msg3 transmission may fail again if different UEs select the same resource again. In the RACH procedure, Backoff indicator is introduced to solve the issue. UE selects a random backoff time according to a Backoff Indicator included in Random Access Response and delays the subsequent Random Access transmission by the backoff time. The similar mechanism can be introduced for contention based Msg3 EDT transmission to handle Msg3 transmission failures. Specifically the backoff parameters can be configured to UE via system information and UE randomly selects a backoff time based on the backoff parameters.

Proposal 4: The backoff parameters are provided to UE via system information and UE randomly selects a backoff time based on the backoff parameters before another attempt.

Another issue is whether UE can fall back to 4-step random access EDT after a number of CB-msg3 EDT attempts. Generally, it is beneficial to have this backup mechanism in case it is difficult for CB-msg3 EDT attempts to be successful.

Proposal 5: After a number of failed CB-msg3 EDT attempts, the UE falls back to 4-step random access EDT.

### 2.1.4 RNTI used for Msg4 transmission

To complete the contention resolution, the UE needs to have the RNTI to monitor the PDCCH for MSG4 scheduling. In the last meeting, RAN2 has agreed that the RNTI used for Msg4 transmission is derived based on the resource associated to the PUSCH occasion for CB-Msg3 EDT transmission. The detail is FFS.

In legacy RACH procedure, UE determines RA-RNTI according to selected PRACH resources.

36.321 5.1.4 Random Access Response reception The RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI= 1 + t_id + 10*f_id where t_id is the index of the first subframe of the specified PRACH (0≤ t_id <10), and f_id is the index of the specified PRACH within that subframe, in ascending order of frequency domain (0≤ f_id< 6) except for NB-IoT UEs, BL UEs or UEs in enhanced coverage. If the PRACH resource is on a TDD carrier, the f_id is set to , where is defined in clause 5.7.1 of TS 36.211 [7]. For BL UEs and UEs in enhanced coverage, RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI=1+t_id + 10*f_id + 60*(SFN_id mod (Wmax/10)) where t_id is the index of the first subframe of the specified PRACH (0≤ t_id <10), f_id is the index of the specified PRACH within that subframe, in ascending order of frequency domain (0≤ f_id< 6), SFN_id is the index of the first radio frame of the specified PRACH, and Wmax is 400, maximum possible RAR window size in subframes for BL UEs or UEs in enhanced coverage. If the PRACH resource is on a TDD carrier, the f_id is set to , where is defined in clause 5.7.1 of TS 36.211 [7]. For NB-IoT UEs, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI=1 + floor(SFN_id/4) + 256*carrier_id where SFN_id is the index of the first radio frame of the specified PRACH and carrier_id is the index of the UL carrier associated with the specified PRACH. The carrier_id of the anchor carrier is 0. For NB-IoT UEs operating in TDD mode, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI = 1 + floor(SFN_id/4) + 256*(H-SFN mod 2)

RNTI used for Msg4 in CB-MSG3 EDT can use the similar mechanism as RA-RNTI.

Proposal 6: RNTI used for Msg4 in CB-MSG3 EDT can take the formula for RA-RNTI calculation as baseline. FFS on the exact parameters.

During legacy RACH procedure, Msg2 contains Temporary C-RNTI. When UE receives Msg4 and considers this Contention Resolution successful, UE sets the C-RNTI to the value of the Temporary C-RNTI. However, Msg2 is omitted for CB-msg3 EDT. Which RNTI is used by the UE for further scheduling after MSG4 needs to be decided by RAN2. A straightforward way is that NW allocates a C-RNTI in Msg4 to the UE.

Proposal 7: NW should allocate one C-RNTI in Msg4 for the UE to receive the scheduling after MSG4.

## 2.2 DSA

### 2.2.1 DSA Configuration

In the last meeting, RAN2 has agreed to support DSA with the intention to improve the UL capacity. The basic ideas of DSA mechanisms are:

- UE selects multiple random slots to send multiple duplicated packets

- NW goes through the slots and decodes the packet with no collision with any other packet. The ones that have collision with other packet(s) will not be successfully decoded.

From our perspective, PUSCH resource for CB-msg3 EDT can be reused for DSA. And in order for the UE to know how many copies to be sent, NW should indicate UE this in the system information. Besides, based on the simulation performed by companies, although the exact threshold is different, it is common understanding that DSA performance is good only when the system load is below a threshold. Thus it is better that NW indicates whether DSA is enabled to make sure UEs don’t use it when the system load is high.

Proposal 8: PUSCH resource for CB-msg3 EDT can be reused for DSA.

Proposal 9: The network indicates how many copies the UE sends for DSA and whether DSA is enabled or not.

2.2.2 RNTI monitoring for DSA

In DSA, UE sends multiple copies of the Msg3 via different resource and the network may successfully receive any of these copies if collision doesn’t happen. As agreed for SA, the RNTI used for Msg4 transmission is derived based on the resource associated to the PUSCH occasion for CB-Msg3 EDT transmission. If the same mechanism is applied for DSA, since UE uses different PUSCH resources to send multiple copies of the Msg3, UE will use different RNTIs to monitor PDCCH scheduling Msg4 for different copies. Correspondingly, the UE will start multiple MAC-contentionResolutionTimers after sending Msg3. If there is overlapping among the multiple MAC-contentionResolutionTimers running time which is highly possible, the IoT UE will have to monitor multiple RNTIs at the same time. However, IoT NTN UE may not have the capability to monitor multiple RNTIs at the same time. So it is better to have a common RNTI for all the copies in DSA to avoid adding extra complexity to IoT UEs.

Proposal 10: In DSA, one common RNTI is used for monitoring multiple MSG4s which correspond to multiple MSG3 copies. FFS on how the common RNTI is decided.

## 2.3 OCC

From the agreements made by RAN1 for OCC enhancements in the Annex, it can be observed that some simulation and evaluation were made, and further study are needed. In last meeting, we have sent an LS to RAN1 to ask whether the OCC solution for NPUSCH on which RAN1 is working can be applied to CB-msg3 NPUSCH or not. RAN2 can wait for the reply from RAN1 before further discussing OCC to save effort as RAN1 is discussing this in parallel.

Proposal 11: Postpone RAN2 discussion on OCC before receiving feedback from RAN1.

# 3	Conclusion

This contribution analyses the TA issue for regenerative payload, and proposes the following:

SA

Observation 1: There is no RAN4 requirement for configuring RSRP-based validation parameter in IoT NTN.

Proposal 1: The RSRP-based validation parameter is not needed for CB-msg3 EDT in IoT NTN.

Proposal 2: For 15kHz SCS NB-IoT and eMTC CE mode B, at least TA validation parameters need to be acquired via RRC dedicated signalling.

Proposal 3: A MAC CE including UE ID information is used for contention resolution for CB-msg3 EDT. FFS on the detailed design of the MAC CE.

Proposal 4: The backoff parameters are provided to UE via system information and UE randomly selects a backoff time based on the backoff parameters before another attempt.

Proposal 5: After a number of failed CB-msg3 EDT attempts, the UE falls back to 4-step random access EDT.

Proposal 6: RNTI used for Msg4 in CB-MSG3 EDT can take the formula for RA-RNTI calculation as baseline. FFS on the exact parameters.

Proposal 7: NW should allocate one C-RNTI in Msg4 for the UE to receive the scheduling after MSG4.

DSA

Proposal 8: PUSCH resource for CB-msg3 EDT can be reused for DSA.

Proposal 9: The network indicates how many copies the UE sends for DSA and whether DSA is enabled or not.

Proposal 10: In DSA, one common RNTI is used for monitoring multiple MSG4s which correspond to multiple MSG3 copies. FFS on how the common RNTI is decided.

OCC

Proposal 11: Postpone RAN2 discussion on OCC before receiving feedback from RAN1.

# 4	References

- RP-242397, WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

- Annex

- 5.1 RAN1 made the following agreements for OCC enhancements:

Agreement in RAN1 #116 For single-tone NPUSCH format 1 transmissions with both 3.75kHz and 15kHz SCS, the following OCC schemes are considered by RAN1 for further study: - Time domain OCC where OCC spreads across: - Symbol-level - Slot-level - Repetition-level - RV-level For multi-tone NPUSCH format 1 transmissions, the following OCC schemes are considered by RAN1 for further study: - Time domain OCC where OCC spreads across: - Symbol-level - Slot-level (including N slot level) - Repetition-level - RV-level - Intra-symbol pre-DFT spreading OCC Agreement in RAN1 #116bis OCC multiplexing is not supported between a UE using NPUSCH format 1 with 3.75kHz SCS and another UE using NPUSCH format 1 with 15kHz SCS. For OCC of NPUSCH format 1, RAN1 will not consider multiplexing more than 4 UEs. For single-tone DMRS when OCC is applied to NPUSCH format 1, RAN1 considers at least the following for further study: - TDM of DMRS. The time domain locations of DMRS for different UEs are different. No OCC is applied for the DMRS of different UEs. - FFS: Detailed mapping - CDM of DMRS. The time domain locations of DMRS for different UEs are the same. Different OCCs are applied for the DMRS of different UEs. - FFS: Detailed mapping - Other schemes are not precluded, including combinations of the above Agreement in RAN1 #117 For 3.75kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued. For 15kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued. Inter-repetition OCC for NPRACH is not studied further in RAN1. For the time-domain DMRS pattern (including blanked DMRS, if any): - For 15kHz single-tone, RAN1 strives to reuse the Rel-17 DMRS pattern - For 3.75kHz single-tone - RAN1 studies - Rel-17 DMRS pattern - A new DMRS pattern - The DMRS overhead (including blanked DMRS, if any) for OCC is the same as for Rel-17 The Rel-17 guard period locations and length for NB-IoT 3.75kHz UL slot are preserved when OCC is applied to NPUSCH format 1. Agreement in RAN1 #118 RAN1 studies whether the following types of UL transmission gap will impact the design of OCC for IoT-NTN when considering e.g. phase continuity - UL gaps for synchronization (from Rel-13) - Gaps around NPRACH occasions - UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17) - TDM DMRS that are muted - Guard periods for 3.75kHz UL transmissions The following combinations are considered for further simulation in RAN1 for 3.75kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 2: OCC2, Symbol-level, CDM DMRS with new pattern - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 6: OCC4, Symbol-level, CDM DMRS with new pattern The following combinations are considered for further simulation in RAN1 for 15kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 5: OCC4, Symbol-level, TDM DMRS - Option 7: OCC4, Slot -level, TDM DMRS - Option 8: OCC4, Slot-level, CDM DMRS with legacy pattern Note 1: For TDM, the legacy DMRS pattern, with DMRS symbols appropriately muted/blanked is used. Companies to report their assumption on whether spreading is applied to the legacy DMRS pattern for 15 kHz SCS. Note 2: Companies to report DMRS sequence applied. For 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB Agreement in RAN1 #118bis At least the following schemes are supported for single-tone: - For 3.75kHz SCS OCC for NPUSCH format 1: - OCC length 2, Symbol-level - FFS: DMRS pattern(s) - For 15kHz SCS OCC for NPUSCH format 1: - OCC length 2, Slot-level, CDM DMRS with legacy pattern - FFS: CDM details, e.g. with or without spreading For NPRACH transmission, inter-symbol group OCC is not further studied. For support of single-tone OCC for NPUSCH format 1, RAN1 studies: - The parameters that need to be signalled, considering the following: - OCC codeword - Enabling of OCC feature - FFS: other parameters - For dynamic grant in RRC CONNECTED, study which and whether any parameters are signalled via DCI and which and whether any parameters are signalled by RRC - FFS: whether/how to support cases other than dynamic grant in RRC CONNECTED, e.g. Msg3, PUR, CB Msg3 EDT