3GPP TSG-RAN WG2 Meeting #128	R2-2409592

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.4.3

Source:	Huawei, HiSilicon

Title:	Further discussion on the criteria for RRM measurement relaxation and offloading

Document for:	Discussion and Decision

- Introduction

In RAN2#127 meeting [1] and RAN2#127bis meeting [2], the following agreements are reached on RRM measurement relaxation and offloading.

RAN2#127 - RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results. RAN2#127bis Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

The following agreements are reached in RAN4 for RRM measurement relaxation and offloading.

RAN4#111 Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4#112 Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility.

In this contribution, we further discuss the criteria for RRM measurement relaxation and offloading for IDLE/INACTIVE modes.

- Discussion

  - General aspects for criteria

RAN4 has only agreed on case #1 and case #3 so far when serving cell measurement by LP-WUR is being performed:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

If a UE is monitoring LP-WUS, the number of transitions from LR to MR should be minimized to obtain the power saving gain. As stated in TR 38.869 [3], using the MR for RRM measurements has a significant impact on UE energy consumption and reduces the power saving gain. Thus, the RRM measurements by MR should be relaxed (case#3 relaxation) or stopped (case#1 fully offloading) once UE is monitoring LP-WUS, which is aligned with RAN4 agreements.

Proposal 1: RAN2 confirms that if UE is monitoring LP-WUS, RRM measurements by MR are either off (RAN4 case#1: Fully offloading case) or relaxed (RAN4 case#3: Relaxed case).

For now, RAN2 should focus on discussing the criteria for RRM measurement cases that have been agreed by RAN4, i.e., case#1 and case#3.

Proposal 2: RAN2 should focus on discussing the criteria for RRM measurement fully offloading (RAN4 case#1) and RRM measurement relaxation (RAN4 case#3).

For case#1, since neighbor cell of equal or lower priority (including intra-frequency, NR inter-frequency and inter-RAT frequency) is off, it means UE is not likely to perform cell re-selection. For example, UE is near the center of the cell. On the other hand, for case#3, UE may be near the cell edge so that neighboring cell measurement is on. So obviously the RSRP threshold for case#1 should be higher than for case#3.

Observation 1: Case#1 is the RRM measurement behaviors for UEs in center of the cell and Case#3 is the RRM measurement behaviors for UEs near the edge of the LP-WUS coverage.

Proposal 3: The threshold(s) for RRM measurement fully offloading (RAN4 case#1) should be higher than threshold(s) for RRM measurement relaxation (RAN4 case#3).

Based on above, how LP-WUR and MR perform RRM measurement is illustrated in the figure below.

Fig. 1. UE behaviors for RRM measurement fully offloading and relaxation cases

In the following two sections, we will discuss the detailed criteria for RRM measurement relaxation (RAN4 case#3) and RRM measurement fully offloading (RAN4 case#1) separately.

  - Criteria for RRM measurement relaxation (RAN4 case#3)

Technically, we understand separate threshold(s) compared to threshold(s) of Rel-16 neighbor cell RRM measurement relaxation should be introduced. The Rel-19 function should not be coupled with a Rel-16 feature, it should work independently without configuring any Rel-16 parameters. Besides, we understand the Rel-19 threshold is a higher value.

Proposal 4: Separate threshold(s) should be introduced for RRM measurement relaxation (RAN4 case#3), compared to the threshold(s) of Rel-16 neighbor cell RRM measurement relaxation, i.e., these two features should be decupled.

For the criteria for RRM measurement relaxation (RAN4 case#3), we understand ‘UE not at cell edge’ can be reused, which means the criterion like “the serving cell quality is above a threshold” will be defined. Two options (or methods) were discussed previously for RRM measurement relaxation:

- Reuse entry/exit condition of LP-WUS monitoring as entry/exit condition for RRM measurement relaxation.

If the entry condition of LP-WUS monitoring is fulfilled, the UE starts LP-WUS monitoring, and performs RRM measurement relaxation (case#3). If the exit condition of LP-WUS monitoring is fulfilled, the UE starts legacy procedure by MR and performs legacy RRM measurement by MR.

- Define a separate entry/exit condition for RRM measurement relaxation, e.g. by introducing a new Rel-19 condition or reusing the Rel-16 RRM measurement relaxation condition.

It requires that threshold for condition of RRM measurement relaxation be equal to or higher than threshold for condition of LP-WUS monitoring. Otherwise, it is not reasonable that UE begins RRM measurement before entering LP-WUS coverage. On the one hand, introducing a new Rel-19 condition for RRM measurement fully relaxation may lead to great specification impact. On the other hand, if the Rel-16 RRM measurement relaxation condition is reused, it will unnecessarily add network complexity of configuring LP-WUS to fulfill the restriction and impacts LP-WUS coverage, e.g. the entry/exit criteria of LP-WUS monitoring should be tighter than the Rel-16 RRM measurement relaxation condition. Anyway, in the most reasonable case, NW should just configure as option 1) illustrates. If not, there is no power saving gain if UE doesn’t relax RRM measurement by MR after entering LP-WUS coverage.

Based on the analyses above, we prefer 1) as it has less specification impact and less complexity on NW configuration.

Proposal 5: The criteria for entry/exit condition of RRM measurement relaxation is the same as the criteria for entry/exit condition of LP-WUS monitoring.

  - Criteria for RRM measurement fully offloading (RAN4 case#1)

In RAN2#127 meeting, RAN2 discussed the entry/exit condition for RRM measurement fully offloading (case#1) without taking case#3 (which was agreed in parallel by RAN4 during the last meeting) into account. Currently, with both case#1 and case#3 supported, RAN2 needs to further think about the entry/exit condition considering: switch between case#1 and case#3.

If UE is monitoring LP-WUS, the UE needs to perform serving cell measurement by LR, meanwhile the serving cell measurement by MR is relaxed or stopped. This means even if serving cell measurement by MR is not stopped, it will be relaxed many times more than normal serving measurement. In this case, it is very likely that serving cell measurement by MR is not timely enough to be used for the evaluation of the entry condition of fully offloading case. Thus, the serving cell quality measured by LR is more appropriate to be used for determination between RRM measurement fully offloading (case#1) or measurement relaxation (case#3).

Observation 2: In RRM measurement relaxation case, serving cell measurement by MR is not timely enough to be used for the evaluation of the entry condition of fully offloading.

Proposal 6: If UE is currently in RRM measurement relaxation case, serving cell measurement by LR is used for the entry condition of fully offloading.

If the NW configures MR threshold in addition to LR threshold, which means MR measurement result will also be considered for evaluating the entry condition of fully offloading in addition to LR measurement, UE can also consider using the serving cell measurement by MR for the entry condition of fully offloading.

Proposal 7: If both MR threshold is configured besides the LR threshold, UE may consider the serving cell measurement by MR besides the serving cell measurement by LR.

- Conclusion

In this paper, we discuss the criteria for RRM measurement relaxation and offloading, and have the following proposals.

General aspects for criteria

Proposal 1: RAN2 confirms that if UE is monitoring LP-WUS, RRM measurements by MR are either off (RAN4 case#1: Fully offloading case) or relaxed (RAN4 case#3: Relaxed case).

Proposal 2: RAN2 should focus on discussing the criteria for RRM measurement fully offloading (RAN4 case#1) and RRM measurement relaxation (RAN4 case#3).

Observation 1: Case#1 is the RRM measurement behaviors for UEs in center of the cell and Case#3 is the RRM measurement behaviors for UEs near the edge of the LP-WUS coverage.

Proposal 3: The threshold(s) for RRM measurement fully offloading (RAN4 case#1) should be higher than threshold(s) for RRM measurement relaxation (RAN4 case#3).

Criteria for RRM measurement relaxation (RAN4 case#3)

Proposal 4: Separate threshold(s) should be introduced for RRM measurement relaxation (RAN4 case#3), compared to the threshold(s) of Rel-16 neighbor cell RRM measurement relaxation, i.e., these two features should be decupled.

Proposal 5: The criteria for entry/exit condition of RRM measurement relaxation is the same as the criteria for entry/exit condition of LP-WUS monitoring.

Criteria for RRM measurement fully offloading (RAN4 case#1)

Observation 2: In RRM measurement relaxation case, serving cell measurement by MR is not timely enough to be used for the evaluation of the entry condition of fully offloading.

Proposal 6: If UE is currently in RRM measurement relaxation case, serving cell measurement by LR is used for the entry condition of fully offloading.

Proposal 7: If both MR threshold is configured besides the LR threshold, UE may consider the serving cell measurement by MR besides the serving cell measurement by LR.

- References

- R2-2407572, Session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

- R2-2409212, Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD.

- TR 38.869, Study on low-power wake up signal and receiver for NR.