3GPP TSG-RAN WG2 Meeting #128	R2-2409615

Orlando, Florida, USA, 18th Oct – 22th November, 2024

Agenda Item		:	8.8.4

Source		:	LG Electronics Inc.

Title		:	Discussion on support of broadcast service in NTN

Document for		:	Discussion and Decision

1. Introduction

In the last meeting, RAN2 agreed following statement:

RAN2#125bis Agreements: - The intended service area can cover the area of more than one NTN cells (or portions thereof) RAN2#127 Agreements: - The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3) RAN2#127bis Agreement: - For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

In addition, RAN2 have discussed about following topics:

- MII procedure initiation based on the intended service area Proposal 4: For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area. - QC thinks this is up to UE and there is nothing to reflect in the spec. Xiaomi agrees - HW thinks that when entering the area this makes sense, but wonder about the case when the UE leaves the area. - Apple thinks the UE could initiate the MII procedure in any case and then it’s up to NW implementation how to behave. LG agrees with Apple. Oppo also agrees. CMCC thinks in this case the NW has no idea of the UE location in this case. - Come back in the next meeting - Service continuity Proposal 3: Intended service area for broadcast session (which is not targeting for service continuity) is only restricted to serving cell. Proposal 4: Service continuity can be enhanced in two places by considering intended service area. • In MBSBroadcastConfiguration, the intended service area within each neighbor cell is provided for each MBS broadcast service session. • In SIB21, the intended service area is provided for each MBS broadcast service session, or for each FSAI (depending on common understanding selected). - Ericsson thinks we should focus on the scenario where the quasi-Earth-fixed cell is replaced due to satellite movement - ZTE thinks we currently support both and then supports p4 - QC also thinks both scenarios should be supported. - HW thinks this covers the case of UE movement. For the satellite movement there would be no difference wrt previous releases. HW is ok to reflect the second part of p4 - Ericsson disagrees with the proposal as there is no need for service continuity for this. Apple thinks this is needed for cell reselection. - CATT wonders whether we really need to provide the service area for service continuity, in a per neighbour cell way. - Ericsson thinks we need to clarify the UE behavior when receiving this information - Come back in the next meeting

In this contribution, we provide our views on remaining issues to support broadcast service in NTN.

2. Discussion

2-1. Service continuity of RRC_IDLE/INACTIVE UE

In the last meeting, RAN2 have discussed whether the intended service area of associated broadcast service is consistent among the serving and the neighbour cells. One company proposed that the intended service area for broadcast service is only applied within the serving cell. However, RAN2 already agreed that the intended service area can cover the area of more than one NTN cells or portions thereof. In this case, the NW cannot provide the intersection between the intended service area and the serving cell coverage. The UE may choose to receive the broadcast service from the serving cell if the serving cell quality is suitable to camp on. In other words, the UE may not perform cell reselection despite the neighbour cell has better cell quality than the serving cell. The NW does not know exact boarderline where the UE performs cell reselection to the neighbour cell. Therefore, the NW should provide the intedned service area applicable to both serving and neighbour cells.

Observation 1	The intended service area of associated broadcast service is applicable to both serving and neighbour cells when the intended service area covers the area of more than one NTN cells or portions thereof.

The broadcast service session information in MCCH contains the information of one or more neighbour cells providing this broadcast serivce. The neighbour cell information contains the phsical cell ID and its frequency. By this information, the UE in RRC_IDLE/INACITVE may consider the frequency of one of the neighbour cells to be the highest priority when it is needed to perform cell reselection, e.g., the cell quality of serving cell is no longer suitable to camp on. On the other hand, the UE should know which intended service area is applied to receive the broadcast service from the neighbour cell. As agreed in the past meeting, the UE should not establish MRB when it is outside the intended service area. MBS frequency prioritization only can be performed if the UE can receive the broadcast service on this frequency.

According to the agreement made in the last meeting, the broadcast service session information in MCCH also contains the associated intended service area. By the relationship among the session information, the UE can figure out which intended service area is associated with the neighbour cell providing broadcast service.

Observation 2	UE can figure out which intended service area is associated with the frequency of the neighbour cell providing broadcast service.

Proposal 1	Without spec change, UE considers the frequency of neighbour cell providing MBS broadcast service of interest to be the highest priority when it is within the intended service area of the associated MBS broadcast service.

2-2. Possible MII procedure initiation enhancement

The UE may be required to indicate the MII upon leaving the intended service area. The MII message contains the frequency and TMGI of the broadcast service that the UE is receiving or interested to. Also, the MII message contains the information of non-serving cell to receive the broadcast service. As agreed in the past meeting, the UE may release the broadcast MRB upon leaving the intended service area, i.e., the UE stops to receive the broadcast service. Furthermore, the non-serving cell also does not support the broadcast service outside of the intended service area anymore. The UE indicates the updated MII excluding frequency, TMGI of this broadcast service and the information of non-serving cell to receive this broadcast service.

Similarly, the UE may be required to indicate the MII upon entering the intended service area. The entry of non-serving cell information list may be changed upon entering the intended service area. According to the Observation 2, the UE can figure out which neighbour cell provides the broadcast service within the intended service area. Nevertheless, the broadcast service which the UE starts to receive may already have been reported by previous MIIs if the UE is interested in receiving this service, i.e., the frequency and TMGI of this broadcast service may not be changed.

Proposal 2	UE in RRC_CONNECTED may initiate the MII procedure upon entering or leaving the intended service area.

2-3. Providing intended service area

In the last meeting, RAN2 have discussed which message would be used to provide the intended service area. There are two options as follows:

Option 1 – MBSBraodcastConfiguration.

Option 2 – SIB (existing SIB or new SIB).

Main argument point is that whether each Option can support skipping MCCH information acquisition when the UE is not within any of the intended service areas of interest. In fact, both options can support such behavior. According to the current spec, the UE may avoid acquiring MCCH information when it is outside the MBS service area. The only thing the UE to do is obtaining the intended service area or interest.

For Option 1, the UE must acquire MCCH at the very first time. After the UE acquires the intended service area of interest, the UE may choose not to re-acquire MCCH when it is outside any of the intended service area of interest. For Option 2, the UE does not need to acquire MCCH at all when it is outside the intended service area of interest. To do this, the SIB should provide the TMGI of the MBS broadcast service together with the intended service area. Using TMGI, the UE identifies which intended service area is of interest.

Although Option 1 requires at least one MCCH acquisition, there is no major difference between both options. According to the LS from RAN3 [1], the intended service area is determined by the core NW and signalled to the gNB. In our understanding, it is rare to change the intended service area associated with the braodcast service.

Observation 3	It is rare to change the intended service area associated with the broadcast service.

In terms of signalling overhead, Option 2 requires duplicated signalling compared to Option 1. According to the RAN2 agreement, the mapping between the broadcast service session and the intended service area is provided into the session information in MCCH. However, Option 2 requires providing TMGI into the information of the intended service area to support skipping MCCH acquisition aspect.

Considering required UE behavior and signaling overhead, Option 1 is more suitable for providing the intended service area.

Proposal 3	The intended service area is provided via MBSBroadcastConfiguration message.

2-4. Skipping MCCH acquisition

In the past meeting, RAN2 agreed that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. The UE performs MCCH information acquisition upon receiving the MCCH change notification of two cases. The first case is that the notification is due to a change in an ongoing MBS service(s). The second case is that the notification is due to the start of new MBS service(s). We discuss skipping MCCH acquisition aspect considersing these two cases.

For the first case, the UE may not acquire MCCH information upon receiving the change notification when the UE does not have any established broadcast MRB. In the past meeting, RAN2 agreed that the UE may not have any established broadcast MRB outside the intended service area. Therefore, the UE may skip the MCCH acquisition outside of the intended servcie area in this case without any spec change.

Observation 4	When UE is outside the intended service area of interest, the UE can skip acquiring MCCH information   upon receiving MCCH change notification due to the change of on-going MBS broadcast service. No spec change is needed.

For the second case, the UE shall acquire MCCH information upon receiving the change notification regardless of the established broadcast MRB. The rationale for this UE behaviour is that the UE anyway must acquire the MCCH information to receive the newly started broadcast service of interest. Even though the UE is outside any of the intended service area signalled before, a new intended service area may be added to MCCH information. The NW cannot guarantee that all the possible intended service areas will be signalled into the MCCH information.

One may argue that this is the reason of providing the intended service area via SIB. With SIB signalling, the UE knows whether a new broadcast service is associated with a new intended service area covering the UE location or not. When a new intended service area does not cover the UE location, the UE may skip the MCCH acquisition. However, anyway the UE must wake up to acquire the SIB providing the intended service area. Once the UE wake up, acquiring MCCH or SIB has not much difference with respect to the power consumption of the UE. When a new intended service area covers the UE location, the UE should additionally acquire MCCH. In this case, the providing the intended service area via SIB has unnecessary SIB acquisition compared to providing the inteded servcie area via MBSBroadcastConfiguration.

Observation 5	Skipping MCCH acquisition upon receiving the change notification due to the start of new MBS broadcast service has marginal benefit compared to expected signalling impact.

Proposal 4	Even if UE is outside the intended service area of interest, the UE interested in receiving MBS broadcast service shall acquire MCCH information for the change due to the start of a new MBS broadcast service. No spec change is needed.

3. Conclusion

In this contribution, following statements are proposed:

2-1. Service continuity in RRC_IDLE/INACTIVE

Observation 1	The intended service area of associated broadcast service is applicable to both serving and neighbour cells when the intended service area covers the area of more than one NTN cells or portions thereof.

Observation 2	UE can figure out which intended service area is associated with the frequency of the neighbour cell providing broadcast service.

Proposal 1	Without spec change, UE considers the frequency of neighbour cell providing MBS broadcast service of interest to be the highest priority when it is within the intended service area of the associated MBS broadcast service.

2-2. Possible MII procedure initiation enhancement

Proposal 2	UE in RRC_CONNECTED may initiate the MII procedure upon entering or leaving the intended service area.

2-3. Providing intended service area

Observation 3	It is rare to change the intended service area associated with the broadcast service.

Proposal 3	The intended service area is provided via MBSBroadcastConfiguration message.

2-4. Skipping MCCH acquisition

Observation 4	When UE is outside the intended service area of interest, the UE can skip acquiring MCCH information   upon receiving MCCH change notification due to the change of on-going MBS broadcast service. No spec change is needed.

Observation 5	Skipping MCCH acquisition upon receiving the change notification due to the start of new MBS broadcast service has marginal benefit compared to expected signalling impact.

Proposal 4	Even if UE is outside the intended service area of interest, the UE interested in receiving MBS broadcast service shall acquire MCCH information for the change due to the start of a new MBS broadcast service. No spec change is needed.

4. References

[1] R2-2409519, LS on Supporting MBS broadcast service for NR NTN, RAN3.