3GPP TSG-RAN WG2 Meeting #128	R2-2409616

Orlando, USA, Nov. 18th – 22nd, 2024

| Agenda Item: | 8.6.2 (NR_Mob_Ph4-Core) |
| --- | --- |
| Source: | LG Electronics Inc. |
| Title: | Discussion on remaining issues of inter-CU LTM |
| Document for: | Discussion and Decision |

1.	Introduction

In this paper, we discuss the following issues of inter-CU LTM:

- SN security update for inter-CU LTM cell switch, where CU is acting as MN and SN is unchanged.

- Radio bearer handling upon inter-CU LTM cell switch

2.	Discussion

2.1.	SN security update for inter-CU LTM cell switch where CU is acting as MN and SN is unchanged

In RAN2#127 meeting, RAN2 agreed the following:

For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

This means that the legacy SN security update procedure is re-used for the case where CU is acting as MN and SN is unchanged. Note that the legacy SN security update procedure is always performed if sk-Counter is included in the RRCReconfiguration message.

Specifically in legacy, the network sends to the UE the MN RRCReconfiguration message including the new MN security configuration and the single counter value (i.e., sk-Counter) for updating the SN security if the MN security update is needed due to, e.g, MN handover while the SN connection is maintained before/after MN handover. To this end, the SN RRCReconfiguration message including sk-Counter is embedded in the MN RRCReconfiguration message. Upon receiving the RRCReconfiguration message, the UE updates the MN security based on the new MN security configuration (i.e., new KgNB). Subsequently, the UE updates the SN security based on the new KgNB and sk-Counter (i.e. new S- KgNB). To sum up, the SN security key update is based only on the provision of sk-Counter in the case of MN handover with keeping the existing SN connection.

For LTM cell switch, one or more candidate cell configurations are provided to the UE beforehand. Upon receiving an LTM cell switch command (CSC) MAC CE from the network, the UE performs the LTM cell switch toward the candidate cell indicated by the LTM CSC MAC CE, i.e., the UE first applies the stored candidate cell configuration and subsequently the UE accesses the candidate cell based on the stored candidate cell configuration and the LTM CSC MAC CE. In order to update the SN security with legacy Rel-15 RRC reconfiguration with sync procedure in the case of inter-CU MCG LTM while the SN connection is maintained, the pre-configured candidate cell configuration of MCG LTM target cell should include the single counter value, i.e., sk-Counter, for updating SN security.

In RAN2#125bis meeting, RAN2 agreed the following:

Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

Based on the above agreement, the following two scenarios (i.e., Figure 1 and Figure 2) may occur.

Figure 1. Subsequent intra-CU MCG LTM cell switches after inter-CU MCG LTM cell switch in case where the SN connection via a specific cell is maintained.

Figure 1 describes a scenario that subsequent intra-CU MCG LTM cell switches are performed after inter-CU MCG LTM cell switch in case where the SN connection via a specific cell is maintained.

Scene#0: The UE is configured with DC, where Cell#0 belonging to CU#0 is PCell and Cell#5 belonging to CU#5 is PSCell. The MN security key and the SN security key are KgNB#0 and S-KgNB#0, respectively. Cell#1 and Cell#2 belonging to CU#1 are configured with candidate cells for MCG LTM cell switch with keeping the SN connection via the current PSCell (i.e., Cell#5).

Scene#1: Inter-CU MCG LTM cell switch with keeping the existing SN connection is performed, i.e., PCell change from Cell#0 to Cell#1. Due to PCell change, the MN security is updated, i.e., the MN key is updated from KgNB#0 to KgNB#1. Subsequently, the SN security is updated based on KgNB#1 and sk-Counter included in the candidate cell configuration for Cell#1, where sk-Counter value is 1. Consequently, the SN security is updated to S-KgNB#1.

Scene#2: Intra-CU MCG LTM cell switch with keeping the existing SN connection is performed, i.e., PCell change from Cell#1 to Cell#2. Since CU for MCG (i.e., MN) is not changed due to PCell change, the MN security update is not performed, i.e., the MN key is maintained as KgNB#1. On the other hand, for the SN key, the SN security update is performed because the candidate cell configuration for Cell#2 includes sk-Counter. Consequently, the SN key is updated to S-KgNB#2 based on KgNB#1 and sk-Counter value 2. Since the MN security is not updated, the SN security update is redundant and unnecessary, which can result in data interruption at SCG due to the random access procedure for SN security update.

Based on the legacy Rel-15 RRC reconfiguration with sync procedure, whether to perform the SN security update during MN RRC reconfiguration (e.g., MN handover) does not depend on whether the MN security update is performed but rather on whether sk-Counter is provided. This means that the legacy Rel-15 RRC reconfiguration with sync procedure cannot prevent redundant SN security update during subsequent intra-CU MCG LTM cell switches. Hence, as described in scene#2, a redundant SN security update may be performed during subsequent intra-CU MCG cell switches.

Observation 1. During subsequent intra-CU MCG LTM cell switches, a redundant SN security update may be performed due to the provision of sk-Counter into candidate cell configurations.

Scene#3: Intra-CU MCG LTM cell switch with keeping the existing SN connection is performed, i.e., PCell change from Cell#2 to Cell#1. Since CU for MCG (i.e., MN) is not changed due to PCell change, the MN security update is not performed, i.e., the MN key is maintained as KgNB#1. On the other hand, for the SN key, the SN security update is performed because the candidate cell configuration for Cell#1 includes sk-Counter. Consequently, the SN key is updated to S-KgNB#1 based on KgNB#1 and sk-Counter value 1. Here, the SN key that were used in the past (i.e. S-KgNB#1) is re-used, which violates the security requirement that the security key that were used in the past should not be re-used. This violation can incur a severe security related problem, e.g., leakage of the SN key may occur due to repetitive intra-CU MCG LTM cell switches with keeping the existing SN connection.

Observation 2. During subsequent intra-CU MCG LTM cell switches, the SN key that were used in the past may be re-used.

It seems that the problem incurred by the observation 2 is a consequence of the problem incurred by the observation 1. This implies that a violation of security requirement due to duplicated SN security key use during subsequent intra-CU MCG LTM cell switches can be prevented by blocking redundant SN security update during subsequent intra-CU MCG LTM cell switches. To this end, an SN security update during subsequent MCG LTM cell switches with DC (i.e., SN is not changed) needs to be performed only if MN security update is performed during an MCG LTM cell switch. For example, the new Rel-19 RRC ID introduced in the last meeting (i.e., RAN2#127) can be used for UE to perform SN security update if MN security update is performed based on the new Rel-19 RRC ID. Other options given in [1] based approaches are also possible.

Proposal 1. SN security update during subsequent MCG LTM cell switches with DC (i.e., SN is not changed) is performed only if MN security update is performed during an MCG LTM cell switch.

Figure 2. Subsequent inter-CU LTM cell switches in case where the SN connection via a specific cell is maintained.

Figure 2 describes a scenario that subsequent inter-CU MCG LTM cell switches are performed in case where the SN connection via a specific cell is maintained.

Scene#0: the UE is configured with DC, where Cell#0 belonging to CU#0 is PCell and Cell#5 belonging to CU#5 is PSCell. The MN security key and the SN security key are KgNB#0 and S-KgNB#0, respectively. Cell#1 belonging to CU#1 and Cell#2 belonging to CU#2 are configured with candidate cells for MCG LTM cell switch with keeping the SN connection via the current PSCell (i.e., Cell#5).

Scene#1: Inter-CU MCG LTM cell switch with keeping the existing SN connection is performed, i.e., PCell change from Cell#0 to Cell#1. Due to PCell change, the MN security is updated, i.e., the MN key is updated from KgNB#0 to KgNB#1. Subsequently, the SN security is updated based on KgNB#1 and sk-Counter included in the candidate cell configuration for Cell#1, where sk-Counter value is 1. Consequently, the SN security is updated to S-KgNB#1.

Scene#2: Inter-CU MCG LTM cell switch with keeping the existing SN connection is performed, i.e., PCell change from Cell#1 to Cell#2. Due to PCell change, the MN security is updated, i.e., the MN key is updated from KgNB#1 to KgNB#2. Subsequently, the SN security is updated based on KgNB#2 and sk-Counter included in the candidate cell configuration for Cell#2, where sk-Counter value is 2. Consequently, the SN security is updated to S-KgNB#2.

Scene#3: Inter-CU MCG LTM cell switch with keeping the existing SN connection is performed, i.e., PCell change from Cell#2 to Cell#1. Due to PCell change, the MN security is updated, i.e., the MN key is updated from KgNB#2 to KgNB#3. Subsequently, the SN security is updated based on KgNB#3 and sk-Counter included in the candidate cell configuration for Cell#1, where sk-Counter value is 1. Consequently, the SN security is updated to S-KgNB#3. Here, sk-Counter value 1 is re-used for updating the SN key.

Observation 3. During subsequent inter-CU MCG LTM cell switches, the same sk-Counter value may be repetitively used.

In [2], there is the note saying that the SN Counter value (i.e., sk-Counter) input is never reused within or across DC and SCPAC (see the yellow-highlighted part below).

A.16 Derivation of KSN for dual connectivity This input string is used when the MN and UE derive KSN during dual connectivity. The following input parameters shall be used: - FC =0x79, - P0 = Value of the SN Counter as a non-negative integer, - L0 = length of the SN Counter value (i.e. 0x00 0x02). The input key KEY shall be KeNB when the MN is an ng-eNB and KgNB when the MN is a gNB. NOTE: The same input string is used for both DC and SCPAC. Therefore, it is crucial that the SN Counter value input is never reused within or across DC and SCPAC.

Based on the note, repeated use of the same sk-Counter during subsequent inter-CU MCG LTM cell switches may be problematic. For example, the repeated use of the same sk-Counter value may incur a security concern, e.g., limited randomization for generating a new security key by using the same sk-Counter value. In RAN2 perspective, it is difficult to judge whether or not it is problematic to repeatedly use the same sk-Counter during subsequent inter-CU MCG LTM cell switches, as it is outside the scope of RAN2. Therefore, RAN2 needs to consult with SA3 whether the repeated use of the same sk-Counter during subsequent inter-CU MCG LTM cell switches is problematic or not.

Proposal 2. Send LS to SA3 to ask whether or not it is problematic to repeatedly use the same sk-Counter during subsequent inter-CU MCG LTM cell switches.

2.2.	Radio bearer handling upon inter-CU LTM cell switch

2. R19 Set ID and R18 NoResetID (P1 in R2-2408117, Vivo) Proposal 1 (modified): For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18. Agreed. [IDC]: Is the ID associated per cell or per RB? [Vivo]: It is similar to Rel-18, ID is linked to serving cell and candidate configuration (basically per cell). [Samsung]: Agree with Vivo. [IDC]: For Rel-18 LTM, there is no case of PDCP re-establishment. Now for Rel-19, inter-CU LTM is supported and some bearer will be PDCP re-established and some are not (dependent on RB type). [MediaTek]: Not sure how it works. In LTM configuration, we don’t have RB configuration. It sounds very complicated to support RB level operation. [Huawei]: Think proposal is ok. Not sure how IDC’s comment is related to the proposal. [OPPO]: Understand IDC’s scenario is when RB type is changed upon inter-CU LTM. Not sure how it impacts on proposal, we can discuss it further later. [CATT]: Agree with OPPO, and we can agree with proposal 1 for now. [Apple]: Ok with proposal 1, we may comeback if real problem. [Qualcomm]: We can agree with proposal 1 for non-DC case.

Based on the chair note in RAN2#127bis [3] given above, some companies seem to be concerned about radio bearer (RB) handling upon inter-CU LTM cell switch. For example, how to handle DRB(s) associated with SCG if SCG is released at MCG LTM cell switch execution, i.e., FFS the need of RB remapping upon MCG LTM cell switch with SCG release.

With [4], RAN2 discussed the RB remapping issue during Rel-18 (in RAN2#124 meeting) and concluded that further enhancement for RB remapping upon LTM cell switch is not needed. As a rationale of the conclusion, the network can handle the issue, e.g., the network maps DRB(s) associated with SCG into DRB(s) associated with MCG before the LTM cell switch. Further, RB remapping upon the LTM cell switch is considered as an optimization.

Observation 4. In Rel-18, RAN2 discussed the need of RB remapping upon MCG LTM cell switch with SCG release and concluded that further enhancement for RB remapping upon LTM with DC is not needed.

In our view, the concern about RB handling upon inter-CU LTM cell switch raised in RAN2#127bis is similar to the one discussed in Rel-18. This implies that the concern about RB handling upon inter-CU LTM cell switch can be handled by the network. For instance, the network provides the UE with the configuration remapping DRB(s) associated with SCG into DRB(s) associated with MCG before/after the LTM cell switch. Hence, RAN2 can simply follow the Rel-18 conclusion for the need of RB remapping upon LTM cell switch, i.e., further enhancement for RB remapping upon LTM cell switch is not needed.

Proposal 3. Further enhancement for RB remapping upon LTM cell switch is not needed.

3.	Conclusion

In this paper, we have made the following observations and proposals:

SN security update for inter-CU LTM cell switch where CU is acting as MN and SN is unchanged:

Observation 1. During subsequent intra-CU MCG LTM cell switches, a redundant SN security update may be performed due to the provision of sk-Counter into candidate cell configurations.

Observation 2. During subsequent intra-CU MCG LTM cell switches, the SN key that were used in the past may be re-used.

Proposal 1. SN security update during subsequent MCG LTM cell switches with DC (i.e., SN is not changed) is performed only if MN security update is performed during an MCG LTM cell switch.

Observation 3. During subsequent inter-CU MCG LTM cell switches, the same sk-Counter value may be repetitively used.

Proposal 2. Send LS to SA3 to ask whether or not it is problematic to repeatedly use the same sk-Counter during subsequent inter-CU MCG LTM cell switches.

Radio bearer handling upon inter-CU LTM cell switch:

Observation 4. In Rel-18, RAN2 discussed the need of RB remapping upon MCG LTM cell switch with SCG release and concluded that further enhancement for RB remapping upon LTM with DC is not needed.

Proposal 3. Further enhancement for RB remapping upon LTM cell switch is not needed.

4.	References

[1] R2-2403929, LS on security handling for inter-CU LTM in non-DC cases, Apple, 3GPP TSG RAN WG2#125bis, Changsha, China, 15th - 19th April 2024

[2] TS 33.501 Security architecture and procedures for 5G system (Release 18)

[3] R2-2409211, Report from session on V2X/SL, R19 NES and MOB	Vice Chairman (Samsung), 3GPP TSG RAN WG2#127bis, Hefei, China, 14th - 18th October 2024

[4] R2-2311899, Discussion on RRC open issues for LTM, vivo, 3GPP TSG-RAN WG2#124, Chicago, USA, 13th - 17th November 2023