3GPP TSG-RAN WG2 Meeting #128	                                                                    R2-2409621

Orlando, USA, Nov. 18th – 22nd , 2024

Source:	CATT

Title:	Discussion on the Functionality Aspects for Ambient IoT

Agenda Item:	8.2.2

Document for:	Discussion

1. Introduction

This paper further discusses the functionality-related issues [1] with the following aspects,

- Functionalities required for A-IoT

- Remaining aspects of AS ID

- Segmentation

- Information visible to reader

- Protocol stack

The Section 2 gives the detailed discussion, and followed by Section 3 that summarizes this paper.

2. Discussion

Functionalities required for A-IoT

Among the functionality discussion, at least for the energy status reporting, we had agreed to support 1-bit indication (along with A-IoT Msg1/Msg3 or D2R response message) from the device to the reader so that the reader knows that the device has no enough energy to perform the following requested operation,

- Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1,MSG3 and Command Response message), if the device can. 1 bit indication can be captured in the TR. We will capture the use case in the TR. FFS whether it is reader controlled.

From our view, such an AS control-like signaling can be defined similar as legacy MAC CE-like format, i.e., a LCID-like header in order to let the reader know the functionality of this format. The detailed design can be left to WI discussion. However, if the device can send the energy status reporting along with the A-IoT Msg1/Msg3 or D2R response message, it is straightforward to allow the multiplexing between the control-like signaling and the data transmission for A-IoT.

Proposal 1: Support the multiplexing of control-like signaling and D2R transmission data.

Remaining aspects of AS ID

Regarding the AS ID used for D2R scheduling and R2D reception, in last meeting we reached the following agreements,

- RAN2 assumes that if “AS ID” is defined it is used at least for purpose of D2R scheduling and R2D reception. Up to RAN1 to decide whether a “AS ID” is defined. - RAN2 assumes this “AS ID” should be a short AS layer ID, rather than the full upper layer device ID. FFS on the length. FFS if AS ID can be based on partial upper layer device ID. - From RAN2 perspective, following options are possible for “AS ID”: Option 1: a random ID if used in Msg1 can be reused; Option 2: reader assigns this “AS ID”. FFS via which R2D message. RAN2 will aim to define one common design for all RA procedures (if technically possible)

It remains another solution whether the AS ID can be based on partial device ID. According to the SA2 progress,

The permanent Ambient IoT Device Identifier includes the following information: 1) Part1information: - The ID type, which is used to indicate the identifier is assigned by an operator or by a third party. - The operational entity, which is the entity managing and allocating the Identifier: - For operator allocated ID, it is a network identifier (i.e. MCC+MNC and/or NID). - For third party allocated ID, it is information used to identify a 3rd party. 2) Part2 information - the information (e.g. EPC or others) used to distinguish different Ambient IoT Devices within the scope identified by the Part1 information. Figure 8.2.1-1 The Operator allocated ID Figure 8.2.1-2 The third party allocated ID

The part 2 information can be used to distinguish different A-IoT devices within the scope identified by the part 1 information. For the operator allocated ID, the part 2 information (partial device ID) is enough as the reader only schedules the devices registered in the corresponding operator. For the third party allocated ID, there may be the case where the devices from different third parties are registered in the same operator, so only the part 2 information is not enough to ensure the uniqueness among all devices.

Observation 1: Whether the partial device ID can be used for scheduling purpose depends on the device ID format defined by SA2.

Besides, now the reader is not aware of any upper layer content such as device ID information. To support the device ID-based solution for AS scheduling, the reader needs to acquire the device ID information from CN.

Since at least it cannot achieve common design according to Observation 1, we prefer to not use the partial device ID based solution for AS scheduling.

Proposal 2: Do not support the partial device ID to serve as AS ID for scheduling purpose.

Segmentation

For the AS layer segmentation functionality, RAN2 aims to design a simple solution as much as possible with the following baseline,

- Do not support sequence number for A-IoT segmentation functionalities. - Do not support segment number and the number of segments for A-IoT segmentation functionalities. - Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment. FFS the details on how this is designed and number of bits (one or two). - RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s).

It is still unclear how does the segmentation work, i.e., whether 1-bit to indicate the last segmentation is enough or not.

Recalling NR RLC-based segmentation, let’s take the UM mode RLC entity for example, and the following information is needed,

Table I: NR RLC-based segmentation for UM mode RLC entity

| Control information | Intention of the control information |
| --- | --- |
| SN number for segmented RLC SDU | Only the segmented RLC SDU has SN number; If the RLC PDU contains a segment that maps to the last byte of an RLC SDU, then increment the SN state variable by one, i.e., all segments share same SN number to let the receiver know whether the RLC segments belong to the same original RLC SDU; |
| SI indication | Indicate whether an RLC PDU contains a complete RLC SDU or the first, middle, last segment of an RLC SDU; |
| SO indication | Indicate the position of the RLC SDU segment in bytes within the original RLC SDU; |

For the segment number and the number of segments mentioned in A-IoT, it is indeed to indicate the receiver how to reassemble the segments, similar as the functionality of SI/SO indication. Thus, without these indications for A-IoT, if the device performs segmentation for a certain A-IoT MAC PDU, it needs to send the first segment, and followed by the middle segment(s) and the last segment in order. For example, after sending the first segment, the device waits for the scheduling for next transmission (implicitly confirm the successful reception of the previous transmission by the reader). Upon receiving the next scheduling, the device sends the following segments. Upon each segment transmission, the device uses 1-bit to indicate whether it is the end/last segment of the A-IoT MAC PDU. If the A-IoT MAC PDU is not segmented, based on the device implementation, it will indicate this is the last segment, which means no segmentation for this A-IoT MAC PDU.

Proposal 3a: One bit is enough for A-IoT device to indicate the reader whether the data is segmented and whether it is last segment.

Proposal 3b: Without the segment number and the number of segments, the device should send the segments in order, with each segment to use 1-bit to indicate whether it is the last segment.

Proposal 3c: If the A-IoT MAC PDU is not segmented by the device, it will indicate this PDU is the last segment.

Information visible to the reader

On the visibility of A-IoT information to reader, it was generally agreed that,

- At least the following information are considered useful to be visible to the reader from CN - The service type of A-IoT (e.g. inventory, command). FFS if more information on command type (e.g. read/write/disable) is useful - targeted for one or more than one devices; - approximate number of target devices (if available). FFS on mandatory/optional

As agreed above, the reader can know the service type, e.g., inventory or command. When the service type is command, the reader can further determine which procedure to use (command only or inventory + command) for this service request.

Regarding whether to further need the command type such as read, write and disable, this may further depend on the detailed procedure for these commands. For the read command, the device will send the data as per the reader command, thus the reader needs to include the UL grant over the command or other potential ways. For the write or disable commands, whether to have feedback to reader (for example, ACK) needs further clarified. If no any feedback for write or disable commands, the reader needs to know the command type so that it can determine whether to send the UL grant over the command or other potential ways.

Proposal 4: The reader doesn’t need to know the specific command type unless there is clear requirement from SA2 that some specific commands (e.g. write or disable) does not need feedback from the device.

On the targeted devices or approximate number of devices, it is mainly used for slot-ALOHA to allocate the access round/occasion. This seems more or less the optimization for slot-ALOHA.

Proposal 5: It’s optional for CN to provide the approximate number of targeted devices to the reader.

Protocol stack

It still remains open on whether to need a new layer on the top of A-IoT MAC for the radio interface between the device and the reader.

As per the potential functionalities required for A-IoT,

Table II: Supported functionalities required for A-IoT

| Supported functionalities | Whether can be supported in A-IoT MAC |
| --- | --- |
| Providing radio bear for higher layer data | Straightforward |
| RACH procedure by slotted-ALOHA |  |
| Segmentation | Can be supported by the adding the control information in the header of radio bear in the A-IoT MAC layer. |
| Multiplexing/de-multiplexing | Can be easily supported in A-IoT MAC as it only needs to handle the multiplexing between the data channel and the control-like signaling (for data transmission, only one LCH is supported). |
| Message size reporting (if needed from device to reader) | Can be supported by defining MAC CE-like format in A-IoT MAC layer. |
| Energy status reporting |  |

We see no reason to introduce a new layer besides the A-IoT MAC layer and A-IoT PHY layer.

Proposal 6: No need additional layer on the top of A-IoT MAC layer for A-IoT.

3. Conclusion

Based on the analysis in section 2, the observations and proposals are summarized as follows:

Observation 1: Whether the partial device ID can be used for scheduling purpose depends on the device ID format defined by SA2.

Functionalities required for A-IoT

Proposal 1: Support the multiplexing of control-like signaling and D2R transmission data.

Remaining aspects of AS ID

Proposal 2: Do not support the partial device ID to serve as AS ID for scheduling purpose.

Segmentation

Proposal 3a: One bit is enough for A-IoT device to indicate the reader whether the data is segmented and whether it is last segment.

Proposal 3b: Without the segment number and the number of segments, the device should send the segments in order, with each segment to use 1-bit to indicate whether it is the last segment.

Proposal 3c: If the A-IoT MAC PDU is not segmented by the device, it will indicate this PDU is the last segment.

Information visible to the reader

Proposal 4: The reader doesn’t need to know the specific command type unless there is clear requirement from SA2 that some specific commands (e.g. write or disable) does not need feedback from the device.

Proposal 5: It’s optional for CN to provide the approximate number of targeted devices to the reader.

Protocol stack

Proposal 6: No need additional layer on the top of A-IoT MAC layer for A-IoT.

4. References

[1] Draft_RAN2_127b_Meeting_Report_v1