3GPP TSG-RAN WG2 Meeting #128	R2-2409622

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.2.3

Source: 	CATT

Title: 	Discussion on Paging for Ambient IoT

Document for: 	Discussion and Decision

1	Introduction

In this contribution, the A-IoT paging aspects and the required contents for A-IoT in the identified procedures will be further discussed.

2	Discussion

2.1	 Multiple/subsequent paging

In RAN2#127bis, the following progress was made for the mechanism of multiple/subsequent paging. This section will further discuss these related issues.

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

- Based on this information the device determines whether to skip sending the response to paging.

1) Assistance info provided by reader

During the online meeting, candidate solutions on what information is indicated to device to avoid duplicate response were discussed. The most straightforward way is to introduce an ID (e.g., named as service ID) in the paging message for device to distinguish the paging messages belong to the same service. The paging messages with the same service ID belong to the same service request from CN.

Proposal 1: Introduce a service ID in the paging message to assist device to distinguish the subsequent paging messages. The paging messages associated with the same service ID belong to the same service request from CN.

Company point out that SA2’s candidate solutions also involve an ID within the signalling between CN and reader, which is named as session ID or transaction ID [2]. Companies may wonder about the relationship between the ID within SA2’s solutions and the service ID used in the A-IoT interface (as mentioned in proposal 1). They concern that if the ID from SA2 is reused in the A-IoT interface, i.e., included in the paging message, the ID length may be too long and will increase the cost of device.

It is worth noting that, the service ID is used for device distinguishes different service requests in A-IoT interface, while the session ID/ transaction ID proposed by SA2 is used for reader distinguishes different service request from CN. Hence, after the reader distinguishes different service requests from CN (rely on the session ID/transaction ID from CN or by reader implementation), it could independently generate the service ID with a short format, e.g., 2 bits, as long as the service ID can avoid collision over the A-IoT interface. That is, the service ID defined for A-IoT interface does not need to rely on SA2’s progress.

Proposal 2: The service ID in the paging messages over the A-IoT interface does not rely on the session ID/transaction ID between CN and reader (i.e., based on the candidate solutions of SA2). It can be independently generated by reader with a short format, e.g., 2 bits.

2) Device behaviours

Once the device has responded the service request, it should not respond the subsequent paging messages of this service request. Considering the reader may reuse the same service ID for the following service requests without overlapping time, for one certain service request, the “no response” operation for this service ID should be limited before the end of this service procedure to avoid missing the following service requests from CN. Hence, it is proposed:

Proposal 3: Once the device has responded the paging message with a service ID, it does not respond the subsequent A-IoT paging messages with the same service ID before the end of this service request.

To realize that, the device needs to store some status information for the ongoing service request, e.g., the service ID and whether it has responded or not. RAN1 has agreed to support the following two types of memory [3].

- Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage.

Considering the status information is used in short-term and changes frequently, stored in temporary memory is more suitable.

Proposal 4: The device needs to maintain some status information (e.g., service ID, has responded or not) for the service request in its temporary memory during the A-IoT procedure.

The maintained status information should be released when the service procedure in A-IoT interface is ended. As for how the device determines end of the service request, i.e., before which point the paging messages with the same service ID belong to the same service request from CN, some candidate solutions are given as following. RAN2 could have a further investigation on these solutions in WI phase.

- Option 1: A timer is used by device to determine end of the service request, the length of timer could be predefined in spec (e.g., 30s) or configured by reader in the R2D message. FFS the detailed device behaviours related to the timer.

- Option 2: The device is explicitly indicated by the reader with an “end” indication (e.g., to be a one bit indication) in the last paging message of a service request. FFS the detailed devices behaviours.

Proposal 5: As far how the device determines the end of a service request within multiple subsequent paging messages, further study the following solutions in WI phase.

- A timer for devices. The length of timer can be predefined in spec or configured by reader in R2D message.

- Explicitly indicated by the reader with an “end” indication in the last paging message of the service.

3) Serial service or parallel service

Another issue is whether to limit the service request over A-IoT interface to be serial or allow being parallel.

Figure 1(a): Serial service mode over A-IoT interface               Figure 1(b): Parallel service mode over A-IoT interface

Examples are given by the above two figures, in which the reader receives two service requests from CN successively. Serial service flow over A-IoT interface refers to figure 1(a). The reader firstly initiates paging message 2a~2m for service request#1. And only after the procedures over A-IoT interface for service request#1 are finished, the reader initiates paging message 4a~4n for service request#2. Parallel service flow over A-IoT interface refers to figure 1(b). The duration for reader initiate paging messages and performing random access/ data transmission procedure for service request#1 and service request#2 can be overlapped.

Comparing these two options, it is obvious that the end to end latency of serial service mode is several times that of parallel service mode. If the service over A-IoT interface is limited to be serial, the user will has poor quality of experience. From our perspective, support parallel service mode is a better choice.

Observation 1: If the service over A-IoT interface is limited to be serial, the end to end latency of A-IoT service will be quiet long, which will lead to poor user experience.

Proposal 6: Support parallel services over A-IoT interface, i.e., the duration for reader perform procedures over the A-IoT interface for different service requests from CN can be overlapped.

2.2	 Contents of the paging message

1) Whether to support multiple device IDs in single paging message

In RAN2#127, the following agreements were achieved. Whether to support paging multiple device IDs need to be further confirmed.

- From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs.    RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

- If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

Paging multiple device IDs may applicable to two use cases: a) the CN raises multiple service requests to reader for single device, the reader trigger one paging message for these service requests via multiplexing function; b) the CN raises service request to reader for multiple devices at the same time, i.e., within a single service request.

For these two cases, we do not seen significant gain to support multiple device IDs in single paging message, even in the signalling overhead aspect. From service perspective, trigger a paging message with multiple device IDs equals to trigger multiple paging messages with single device ID. That is, for case b), if the CN wants to page multiple devices, it could send service request for these target devices one by one, or the CN send a service request for multiple target devices to the reader with each device ID contained within separate upper layer packet and the reader paging the target devices over the A-IoT interface one by one.

Observation 2: The case of multiple device IDs in single paging message can be replace by multiple paging messages with single device ID. And there is no significant gain on signalling overhead aspect to support multiple device IDs in single paging message.

From another perspective, if this function is agreed, we may need to support segmentation (due to limitation on the TB size) and multiplexing for R2D transmission, which will increase the complexity of the device.

Observation 3: To support multiple device IDs in single paging message, it may also need to support functionalities of segmentation and multiplexing for R2D transmission, which will increase the complexity of the device.

Based on the above observations, it is not sensible to raise the complexity of device to support an unessential function. Considering this may impact RAN3 and SA2 on their signalling design, it is proposed to send LS to RAN3 and SA2 to notify that RAN2 does not have essential requirement on supporting paging multiple device IDs in single paging message.

Proposal 7: Do not support paging multiple device IDs in single paging message over the A-IoT interface, including the following cases:

a) The reader initiates single paging message for multiple service requests from CN by multiplexing;

b) The reader initiates single paging message for single service request for multiple devices from CN.

Proposal 7a: Send LS to RAN3 and SA2 to notify that RAN2 does not have essential requirement on supporting paging multiple device IDs in single paging message.

2) Other contents in paging message

In RAN2#127, we have discussion on indicating access type and access resource for devices. The basic principle is the device can recognize access type from the paging message. However, the detailed solution subjects to whether we will down select from these candidate random access types. After the random access type is decided finally, what information needs to be contained in the paging message is quite straightforward that can be discussed in WI phase.

Proposal 8: The detailed information for indicating access type and access resource in paging message can be discussed in WI phase after the random access type in Rel-19 is determined.

There is also discussion on including other information in paging message, e.g., service type (including inventory, command-only and inventory+command) and reader ID. At present, these discussions seem not essential for the basis function of paging message. Hence, we propose to discuss these issues in WI phase if needed.

Proposal 9: Whether additional information needs to be contained in paging message can be discussed in WI phase (if needed), e.g., service type, reader ID.

4	Conclusion

In this contribution, paging functionality for A-IoT is discussed with giving corresponding observations and proposals:

Multiple/subsequent paging

Proposal 1: Introduce a service ID in the paging message to assist device to distinguish the subsequent paging messages. The paging messages associated with the same service ID belong to the same service request from CN.

Proposal 2: The service ID in the paging messages over the A-IoT interface does not rely on the session ID/transaction ID between CN and reader (i.e., based on the candidate solutions of SA2). It can be independently generated by reader with a short format, e.g., 2 bits.

Proposal 3: Once the device has responded the paging message with a service ID, it does not respond the subsequent A-IoT paging messages with the same service ID before the end of this service request.

Proposal 4: The device needs to maintain some status information (e.g., service ID, has responded or not) for the service request in its temporary memory during the A-IoT procedure.

Proposal 5: As far how the device determines the end of a service request within multiple subsequent paging messages, further study the following solutions in WI phase.

- A timer for devices. The length of timer can be predefined in spec or configured by reader in R2D message.

- Explicitly indicated by the reader with an “end” indication in the last paging message of the service.

Observation 1: If the service over A-IoT interface is limited to be serial, the end to end latency of A-IoT service will be quiet long, which will lead to poor user experience.

Proposal 6: Support parallel services over A-IoT interface, i.e., the duration for reader perform procedures over the A-IoT interface for different service requests from CN can be overlapped.

Contents of the paging message

Observation 2: The case of multiple device IDs in single paging message can be replace by multiple paging messages with single device ID. And there is no significant gain on signalling overhead aspect to support multiple device IDs in single paging message.

Observation 3: To support multiple device IDs in single paging message, it may also need to support functionalities of segmentation and multiplexing for R2D transmission, which will increase the complexity of the device.

Proposal 7: Do not support paging multiple device IDs in single paging message over the A-IoT interface, including the following cases:

a) The reader initiates single paging message for multiple service requests from CN by multiplexing;

b) The reader initiates single paging message for single service request for multiple devices from CN.

Proposal 7a: Send LS to RAN3 and SA2 to notify that RAN2 does not have essential requirement on supporting paging multiple device IDs in single paging message.

Proposal 8: The detailed information for indicating access type and access resource in paging message can be discussed in WI phase after the random access type in Rel-19 is determined.

Proposal 9: Whether additional information needs to be contained in paging message can be discussed in WI phase (if needed), e.g., service type, reader ID.

5	Reference

- Report of 3GPP TSG RAN WG2 meeting #127bis.

- 3GPP TR 23.700-13: Study on Architecture support of Ambient power-enabled Internet of Things.

- Chair's notes RAN1#116 9.4 R19 Ambient IoT