3GPP TSG-RAN WG2 Meeting #128	R2-2409624

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.2.5

Source: 	CATT

Title: 	Discussion on Topology 2 for Ambient IoT

Document for: 	Discussion and Decision

1	Introduction

In RAN2#127bis, the following agreements on topology 2 were achieved.

Agreements

- Dedicated resource configuration is only given to the UE reader via dedicated signalling.  Mechanisms for shared resource pool amongst readers are not considered in this release.

- The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.    FFS how valid resource is determined – we will consider temporary out of connection scenarios.    FFS on resource validity across multiple cells.

- The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

This contribution will continue discussing the identified issues.

2	Discussion

2.1	RRC state of UE reader

In RAN2#127bis, we have an initial discussion on the applicable RRC states of UE reader when there is ongoing A-IoT service, but no conscience was reached on supportive of RRC_INACTIVE and RRC_IDLE. Here in below, the discussion on these two states will be carried with considering two scenarios, i.e., when A-IoT service request arrives from CN/AF and when UE needs forwarding the received A-IoT data towards CN/AF. According to existing mechanisms, analyzation for these scenarios is listed in Table 1.

Table 1: Analyzation on the feasibility of RRC states of UE reader.

| RRC state of UE reader | Scenario A: when A-IoT service request arrives from CN/AF | Scenario B: when UE needs forwarding the received A-IoT data towards CN/AF |
| --- | --- | --- |
| RRC_INACTIVE | The RAN pages the UE into RRC_CONNECTED state and sends the service request in RRC_CONNECTED; or RAN sends the A-IoT service request to UE via SDT procedure. | The UE resumes to RRC_CONNECTED firstly and finish data forwarding in RRC_CONNECTED; or UE sends the received device data via SDT procedure. |
| RRC_IDLE | The CN pages the UE into RRC_CONNECTED, and establish NAS connection and PDU session (for UP solution). | The UE setups RRC connection, NAS connection and establish PDU session (for UP solution) for A-IoT service, and forward the received device data towards CN/AF. |

- Analyzation on RRC_IDLE

For UE reader in RRC_IDLE, under scenario A, the NW could page the UE enters RRC_CONNECTED firstly as legacy, and then triggers A-IoT service. But problem may exist in scenario B. When the UE enters RRC_IDLE, the CN will release the A-IoT service context for the UE. After the UE collects data from A-IoT devices, it re-establishes NAS connection and/or PDU session to forward the received data towards CN/AF. The CN/AF cannot recognize the response is for which A-IoT service before since there is no related context.

Observation 1: If UE reader collects A-IoT data in RRC_IDLE and forwards the data to CN/AF when enters RRC_CONNECTED, CN/AF cannot recognize the A-IoT response from UE reader whose connection is re-established and the A-IoT context was already released previously.

Enhancement is required in SA2 according to the above observation. Hence, it is proposed that:

Proposal 1: Do not support the case that UE reader performs A-IoT service in RRC_IDLE state.

Even if it is expected that the UE reader does not enters RRC_IDLE state during ongoing A-IoT service. However, the RRC state of UE reader cannot be fully controlled by the network as expected, e.g., when UE detects RLF failure and fails to select suitable cell for re-establishment, the UE enters RRC_IDLE.

Observation 2: UE reader may enter RRC_IDLE under abnormal conditions, e.g., the UE reader detects RLF on Uu and fails to select suitable cell for re-establishment.

If UE reader enters RRC_IDLE when there is ongoing A-IoT service, considering the reason mentioned by observation 1, it is meaningless for UE reader to continue finishing the A-IoT service. It is proposed that if the UE reader enters RRC_IDLE, the UE reader terminates the ongoing A-IoT procedures over both Uu and A-IoT interface, if any.

Proposal 1a: When UE reader enters RRC_IDLE, the UE reader terminates the ongoing A-IoT procedures over both Uu and A-IoT interface, if any.

- Analyzation on RRC_INACTIVE

For UE reader in RRC_INACTIVE, the A-IoT service can be performed normally under scenario A and scenario B based on existing mechanisms in theory, e.g., page UE enters RRC_CONNECTED or via SDT. In RAN2#127bis, it was agreed that the UE reader performs the A-IoT procedure on A-IoT interface only if the resource configuration is valid. That is, whether to support UE reader performs A-IoT service in RRC_INACTIVE depends on the mechanism of resource allocation and resource validity aspects. Related discussion will be carried on in section 2.2.

Observation 3: Whether to support the case that UE reader performs A-IoT service in RRC_INACTIVE state depends on the mechanism of resource allocation and validity conditions.

2.2	Resource allocation and validity conditions

According to the agreement achieved in RAN#127bis, configure dedicated resource to UE reader via dedicated signalling is agreed and mechanisms for shared resource pool are not considered in Rel-19. This section will discuss how to configure dedicate resource for UE reader and the validity conditions of the configured resource.

- Trigger and assistance info for RAN node allocates A-IoT interface resource toward UE reader

According to the use case given by SA1 [1], different A-IoT service procedures may have different requirements on the A-IoT radio resource. For example, one inventory procedure is for single device and another inventory procedure is for a group of devices, the latter needs more radio resource than the former. Hence, to avoid congestion or resource waste on the A-IoT interface, some assistance information can be provided to the RAN node for allocating appropriate resources. For this purpose, two way forwards are given as following:

WF 1: UE reader requests resource to RAN node for the received A-IoT service request

After the UE reader receives inventory/command service request from CN/AF, it request the needed radio resource to the RAN node based on the characteristic of this service, e.g., number of the target devices and service type. Then the RAN node could allocate appropriate resource based on UE reader’s request.

WF 2: CN provides assistance info to RAN node when triggering A-IoT service for UE reader

When CN/AF sends A-IoT service request to UE reader, it also indicates assistance info to RAN node (There will be impacts on SA2/RAN3). Take UP solution as an example, the CN/AF could send the assistance info (e.g., number of target devices, service type) to RAN node in the header of IP packet which contains the inventory/command for UE reader. Then the RAN node could allocate appropriate resource to UE reader for this A-IoT service procedure.

These two options both have pros and cons, option 1 can assign resource more accurately, but brings additional Uu signalling overhead; option 2 has less Uu impact than option 1 but needs coordinate with other work groups, and option 2 can only roughly estimate the required resources. Considering which option(s) to be adopted is impacted by the network architecture decision by SA2 and the validity conditions of the configured resource, RAN2 could further discuss and down select from these two options until WI phase.

Proposal 2: Discuss and down select from the following options on how to enable A-IoT resource allocation from RAN to UE reader in WI phase.

- UE reader explicitly request resource towards RAN node after receiving the A-IoT service request;

- CN provide assistance info to RAN node when triggering A-IoT service for UE reader, and RAN node allocates A-IoT resource for the UE reader.

- Validity conditions of the configured resource

For the FFS on resource validity across multiple cells, we think this mechanism seems like an over optimization

On the one hand, the mechanism of resource validity across multiple cells is mainly benefit for the scenario UE in RRC_INACTIVE state and the UE has a certain degree of mobility. An example is the LPHAP feature for Rel-18 positioning WI, for which the target scenario is industrial park and the UE keeps moving across cells. With the resource validity across multiple cells, when UE reselects to a cell within the validity area, it does not need to establish RRC connection to acquire new resource, so that the power consumption can be reduced. However, in the first release of A-IoT, UE reader with frequent movement is not a typical scenario.

Observation 4: The resource validity across multiple cells is mainly benefit for the case UE in RRC_INACTIVE state and keeps moving across cells (the UE could save energy for entering RRC_CONNECTED to acquire new resource), which is not a typical scenario for A-IoT.

On the other hand, to support resource validity across multiple cells, there will be impacts on other groups, including ask RAN1 for evaluating feasibility of this mechanism and discussing means to avoid possible interference in A-IoT interface or Uu interface, and ask RAN3 to study resource coordination methods among cells. This will be a complex mechanism to be specified.

Observation 5: Resource valid across multiple cells is a complex mechanism which needs involve RAN1 for evaluation and involve RAN3 for studying resource coordination methods among cells.

Based on the above analysis, it is proposed to not support resource validity across multiple cells, i.e., the resource is valid within the serving cell which configures resource for the UE reader.

Proposal 3: Do not support resource validity across multiple cells in the first release and only support the configured A-IoT interface resource is valid within the serving cell which configures resource for the UE reader.

- Whether to support RRC_INACTIVE state in view of resource validity aspects

Take proposal 3 as premise, i.e., the resource is valid within the serving cell which configures the resource for UE reader, let us come back to the issue of whether to support UE reader performs A-IoT service in RRC_INACTIVE state as mentioned by observation 3.

Since performing A-IoT service in RRC_INACTIVE is benefit for power saving of UE reader, we prefer to support this case with minimum spec impact. Comparing with RRC_CONNECTED state, there is not significant different on supporting performing A-IoT service in RRC_INACTIVE. The A-IoT resource for UE reader used in RRC_INACTIVE can be configured by suspendConfig of RRCRelease message. And the resource can be released by a timer or be released by RAN node with explicit signalling. And upon cell reselection, the configured resource is invalid. Detailed mechanisms can be discussed in WI phase.

Proposal 4: Support the case that UE reader performs A-IoT service in RRC_INACTIVE state with minimum spec impact, including A-IoT resource configured to UE reader by suspendConfig in RRCRelease message.

2.3	Temporarily “out of coverage” impact

Also take proposal 3 as premise, i.e., the resource is valid within the serving cell which configures the resource for UE reader, this section discusses the behaviour of UE reader when temporarily out of coverage and there is ongoing A-IoT procedure, including HO and RLF detection.

Refer to the KPIs given by SA1 [1], the latency requirement of A-IoT service is low, the latency could be several seconds, that it is not necessary to ensure service continuity when temporarily “out of coverage”. Hence, the simplest treatment during temporarily “out of coverage” is to suspend the A-IoT service at the start point of the “out of coverage” duration and continue the A-IoT service until the connection is recovered. However, the “out of coverage” duration of HO and re-establishment procedure may up to several seconds, reflected by length of T304 or T311. When the UE reader establishes the RRC connection, it is possible that the device becomes out of power, resulting in loss of temporarily stored information in memory, that the previous A-IoT procedure cannot be continued. That is, the previous A-IoT service is failure in A-IoT interface and need to be restarted.

Observation 6: If the A-IoT service over the A-IoT interface is suspended when temporarily “out of coverage”, the A-IoT device may become out of power, resulting in loss of temporarily stored information in memory that the previous A-IoT procedure will fail.

But if the UE reader continues the A-IoT service over the A-IoT interface with the resource configured by RAN previously, there may be potential interference issue. For example, if the UE reader moves in coverage of a neighbour cell after RLF, the UE reader may interfere the Uu or A-IoT interface of the neighbour cell.

Observation 7: When temporarily “out of coverage”, if the UE reader continues the A-IoT service over A-IoT interface with the previous resource, there may be potential interference issue if the UE reader moves in coverage of neighbour cell.

Both of these two options have pros and cons, RAN2 could have a further discussion and decision on this issue.

Proposal 5: Decide the UE reader behaviours over the A-IoT interface when temporarily “out of coverage”.

- Alt 1: Suspend the A-IoT service. The cons is the A-IoT service may fail because the device could be out of power during suspending.

- Alt 2: Continue the A-IoT service. The cons is there is a potential interference issue.

2.4	Others

1) Impacts on UE (re-)selection mechanism

The RAN node in topology 2 should be able to allocate A-IoT radio resource for the UE reader, which is an optional capability in Rel-19. We could not assume all RAN nodes supports this capability. If UE camps on the cells which do not support A-IoT topology 2, the UE reader could not perform A-IoT related progresses. Hence, when the UE operating as normal mode, there is no limitation on its mobility management progress, but when the UE operating as a reader, it should select a cell which supports A-IoT topology 2 to ensure service continuity. This issue could be further discussed in WI phase.

With respect to how to indicate the cell supports A-IoT topology 2 to UE reader. One solution is to introduce an indication in system information, e.g., SIB1. The occurrence of the indication shows the cell supports A-IoT topology 2 related capabilities. The UE could determine whether the cell supports A-IoT topology 2 by reading system information.

Proposal 6: The RAN node should indicate whether it supports A-IoT topology 2 to UE readers, e.g. an indication in SIB1. FFS details in WI.

2) How the RAN node recognize an UE reader

Apart from CN, the RAN node also needs to know the UE supports the capability of operating as a reader. For example, if the RAN node knows the UE’s radio capability on the new A-IoT interface, it could configure suitable radio resource for the UE. Hence, it is proposed that UE supports A-IoT reports its capability on A-IoT topology 2 towards RAN node. The detailed solution can be discussed in WI phase.

Proposal 7: UE reports its capability on A-IoT topology 2 towards RAN node. The detailed solution can be discussed in WI.

3) UE reader selection

In practical needs, the inventory/command request raised by AF may be in format of geographical area. How to select suitable UE reader(s) and which network element make the decision need to be discussed. Reference to the solutions proposed in TR 23700-13 [2], the related mechanisms on reader selection are summarized as the following three kinds.

- AF raised inventory/command request towards AMF/AIoTF, the request include area information optionally. Then AMF/A-IoTF selects reader(s) for this service accordingly. Reference to solution #4, #6, #11, #21, #33.

- AF raised inventory/command request towards AMF/AIoTF, the request include area information optionally. AMF/A-IoTF selects some candidate reader(s) accordingly and sends the candidate reader(s) list to RAN node. The reader who is used to perform this A-IoT service is finally determined by RAN node. Reference to solution #30.

- AF determines reader directly based on the location of UE reader. Reference to solution #22.

After analysing the issue with reference to these candidate solutions, it can be seen that the decision can be totally depends on CN/AF and optional RAN node. The only requirement on UE reader is the NW needs to know UE reader’s location, which can be supported by legacy mechanisms, e.g., GNSS location report or 3GPP positioning feature. No additional RAN2 impact is identified so far.

Proposal 8: How to select suitable UE reader(s) to perform A-IoT service depends on other groups, i.e. RAN3/SA2. RAN2 may further discuss the potential impact if there is within the progress of RAN3/SA2 in WI.

3	Conclusion

In this contribution, A-IoT topology 2 is discussed with giving corresponding observations and proposals:

RRC state of UE reader

Observation 1: If UE reader collects A-IoT data in RRC_IDLE and forwards the data to CN/AF when enters RRC_CONNECTED, CN/AF cannot recognize the A-IoT response from UE reader whose connection is re-established and the A-IoT context was already released previously.

Proposal 1: Do not support the case that UE reader performs A-IoT service in RRC_IDLE state.

Observation 2: UE reader may enter RRC_IDLE under abnormal conditions, e.g., the UE reader detects RLF on Uu and fails to select suitable cell for re-establishment.

Proposal 1a: When UE reader enters RRC_IDLE, the UE reader terminates the ongoing A-IoT procedures over both Uu and A-IoT interface, if any.

Observation 3: Whether to support the case that UE reader performs A-IoT service in RRC_INACTIVE state depends on the mechanism of resource allocation and validity conditions.

Resource allocation and validity conditions

Proposal 2: Discuss and down select from the following options on how to enable A-IoT resource allocation from RAN to UE reader in WI phase.

- UE reader explicitly request resource towards RAN node after receiving the A-IoT service request;

- CN provide assistance info to RAN node when triggering A-IoT service for UE reader, and RAN node allocates A-IoT resource for the UE reader.

Observation 4: The resource validity across multiple cells is mainly benefit for the case UE in RRC_INACTIVE state and keeps moving across cells (the UE could save energy for entering RRC_CONNECTED to acquire new resource), which is not a typical scenario for A-IoT.

Observation 5: Resource valid across multiple cells is a complex mechanism which needs involve RAN1 for evaluation and involve RAN3 for studying resource coordination methods among cells.

Proposal 3: Do not support resource validity across multiple cells in the first release and only support the configured A-IoT interface resource is valid within the serving cell which configures resource for the UE reader.

Proposal 4: Support the case that UE reader performs A-IoT service in RRC_INACTIVE state with minimum spec impact, including A-IoT resource configured to UE reader by suspendConfig in RRCRelease message.

Temporarily “out of coverage” impact

Observation 6: If the A-IoT service over the A-IoT interface is suspended when temporarily “out of coverage”, the A-IoT device may become out of power, resulting in loss of temporarily stored information in memory that the previous A-IoT procedure will fail.

Observation 7: When temporarily “out of coverage”, if the UE reader continues the A-IoT service over A-IoT interface with the previous resource, there may be potential interference issue if the UE reader moves in coverage of neighbour cell.

Proposal 5: Decide the UE reader behaviours over the A-IoT interface when temporarily “out of coverage”.

- Alt 1: Suspend the A-IoT service. The cons is the A-IoT service may fail because the device could be out of power during suspending.

- Alt 2: Continue the A-IoT service. The cons is there is a potential interference issue.

Others

Proposal 6: The RAN node should indicate whether it supports A-IoT topology 2 to UE readers, e.g. an indication in SIB1. FFS details in WI.

Proposal 7: UE reports its capability on A-IoT topology 2 towards RAN node. The detailed solution can be discussed in WI.

Proposal 8: How to select suitable UE reader(s) to perform A-IoT service depends on other groups, i.e. RAN3/SA2. RAN2 may further discuss the potential impact if there is within the progress of RAN3/SA2 in WI.

4	Reference

- 3GPP TR 22.840: "Study on Ambient power-enabled Internet of Things ".

- 3GPP TR 23.700-13: "Study on Architecture support of Ambient power-enabled Internet of Things".

- 3GPP TR 38.331: "Radio Resource Control (RRC) protocol specification".