3GPP TSG-RAN WG2 Meeting #128                                                                    R2-2409626																				xxx

Orlando, USA, Nov. 18th – 22nd, 2024

Source:	CATT

Title:	Discussion on other aspects for SBFD

Agenda Item:	8.11.3

Document for:	Discussion

1. Introduction

The objectives of Sub-band full duplex (SBFD) which are related with RAN2 except random access can be found as below:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier:

  - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

  - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

  - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2]

    - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon

    - Enhancement on resource allocation in frequency domain in SBFD symbols, including

    - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols,

    - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation

    - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

  - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

    - UL resource muting for PUSCH[RAN1, RAN2, RAN4]

    - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework,[RAN1, RAN2, RAN4]

We summarized the progress in RAN1 on SBFD configuration for UE transmission, reception and the new measurement in this contribution.

2. Discussion

RAN1 has discussed the signal and channel-specific procedures. The configurations of PDSCH, CSI-RS, PUCCH, PUSCH will be impacted according to the conclusion in RAN1.

Agreement The number of configured FH offsets of FH offset list for SBFD symbols is the same as the number of FH offsets of FH offset list for non-SBFD symbols. - The number of configured FH offsets of FH offset list is determined based on the UL BWP size as legacy. The number of bits used to indicate FH offset in DCI is determined as legacy. Agreement Support separate SRS configurations for SBFD and non-SBFD symbols. Select one from the following options. - Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage. - SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission. - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols. - SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission. - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols. - FFS applicable usages - Option 2: Support separate configurations for SBFD and non-SBFD symbols in the same SRS-Resource. - At least freqDomainPosition, freqDomainShift and freqHopping are separately configured. - FFS separate configurations of other parameters - FFS applicable usages Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. Agreement: For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with Configuration 1 or Configurat 2 via RRC configuration. Select from the following options - Option 1: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per serving cell basis - Option 2: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per BWP per channel/signal basis - Separate configurations for configured and dynamic transmission/receptions for a given channel/signal are not precluded. - Option 3: Configuration 1 or Configuration 2 is on a per cell basis. - No separate UE capabilities for Configuration 1 and 2.

Here is the summary of progress in RAN1 on these configurations.

| Channel | Required configurations in High layer | RAN1 progress |
| --- | --- | --- |
| Physical channel/signal across SBFD and non-SBFD symbols in same slot | New configurations for SBFD | On going |
| Physical channel/signal across SBFD and non-SBFD symbols in multiple slots | New configurations for SBFD | 1. Support configuration of whether across SBFD symbols and non-SBFD symbols in different slots is supported. 2. In case the transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type for semi-statically configured transmissions/receptions without activation DCI is explicitly configured by RRC. 3 For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. |
| Separate configurations for SBFD symbols and non-SBFD symbols（including PUCCH，PUSCH and SRS） | Required | 1. Separate configurations of startingPRB and secondHopPRB 2. Support separate SRS configurations for SBFD and non-SBFD symbols. 3. Support separate configurations of FH offsets for SBFD symbols and non-SBFD symbols. |
| PDCCH | Not required | Not applicable to PDCCH |

We can find that RAN1 have been discussing the detailed configurations step by step.

Proposal 1: Below configurations of signal and channel on SBFD shall be sent to UE via RRC procedure, relying on RAN1 progress:

- Physical channel/signal across SBFD and non-SBFD symbols in same slot

- Physical channel/signal across SBFD and non-SBFD symbols in multiple slots

- Separate configurations for SBFD symbols and non-SBFD symbols.

RAN1 reached some agreement on L1 UE-to-UE CLI measurement and reporting at RAN1#118bis meeting.

Agreement Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement. - The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic - Note: No new usage needs to be defined for SRS resource sets Agreement For aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured is configured by higher layer parameter. - FFS: Whether to reuse the legacy SRS resource set - FFS: Whether available slot offset list is also reused For aperiodic CLI-RSSP reporting using aperiodic CLI-RSSI measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured is configured by higher layer parameter.

Proposal 2: Wait for the LS from RAN1 on the design of UE-to-UE CLI measurement and reporting.

3. Conclusion

Based on the analysis in section 2, the proposals are summarized as following:

Proposal 1: Below configurations of signal and channel on SBFD shall be sent to UE via RRC procedure, relying on RAN1 progress:

- Physical channel/signal across SBFD and non-SBFD symbols in same slot

- Physical channel/signal across SBFD and non-SBFD symbols in multiple slots

- Separate configurations for SBFD symbols and non-SBFD symbols.

Proposal 2: Wait for the LS from RAN1 on the design of UE-to-UE CLI measurement and reporting.

4. References

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)  Huawei

- 3GPP TR 38.858 Study on Evolution of NR Duplex Operation (Release 18) V2.0.0 (2023-12)

- RAN1 Chair Notes WG1 #118bis