3GPP TSG-RAN WG2 Meeting #128		R2-2409634

Orlando, USA, 18th – 22nd Nov. 2024

Source:	vivo

Title:		Discussion on Service continuity for multi-hop relay

Agenda Item:		8.13.4

Document for:	Discussion and Decision

- Introduction

In the newest WID, the objective of service continuity is as follows[1]:

- Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

This contribution will provide our initial thoughts about how to support these intra-gNB service continuity scenarios for multi-hop relay and which Rel-17/18 procedures may be reused.

- Discussion

  - Intra-gNB multi-hop indirect to direct path switching

In legacy L2 U2N Remote UE switching from (single-hop) indirect to direct path under the same gNB, the procedure is as follows[2]:

Figure 1: Procedure for L2 U2N Remote UE intra-gNB switching from indirect to direct path

In legacy i2d path switch, the Uu measurement configuration and measurement report signalling procedures are performed to evaluate both relay link measurement and Uu link measurement in step 1 above.

For multi-hop i2d path switch, the network still needs to know the Uu link quality of the remote UE before switch it to direct link, so for the Uu link measurement it can reuse the legacy configuration and measurement report signalling procedures as well.

However, for the relay link measurement, now there are more than one hops, i.e.:

- Link 1: the link between remote UE and first relay UE

- Link 2: the link between first relay UE and intermediate relay UE

- Link 3: the link between intermediate relay UE and last relay UE

For link 1, the legacy configuration and measurement report signalling procedures can also be reused. But for link 2, in order to make the gNB known of the quality of link 2, the first relay UE should evaluate the relay link measurement and report it to gNB. What is included in the measurement report can also reuse the legacy SL relay mechanism, i.e. to include the measured intermediate Relay UE’s source L2 ID, serving cell ID and measurement quantity result. For link 3, the situation is similar. Meanwhile the intermediate relay UE may also measure link 2, which may not be restricted, anyway the gNB would finally know the reported measurement result is for which link because the measured relay information would be included.

The different aspects compared to legacy i2d case is that:

- The first relay UE and the intermediate relay UE does not have to perform Uu link measurement, as the gNB only needs the Uu measurement result for remote UE to switch it to direct Uu link.

Therefore, we propose:

Proposal 1: For multi-hop i2d, Remote UE evaluates both relay link (for the link between remote UE and serving first relay UE) and Uu link, where the sidelink relay measurement report shall include at least serving first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

Proposal 2: For multi-hop i2d, first relay UE/ Intermediate relay UE evaluates adjacent relay link, where the sidelink relay measurement report shall include at least measured relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

After the measurement reporting, the following step 2 to step 5 agree generally same for single hop i2d and multi-hop i2d procedures.

For step 6, the gNB needs to reconfigure the connection between the L2 U2N Relay UE and the gNB in legacy i2d scenario, but for multi-hop i2d case, the gNB may also reconfigure the link 2 and link 3, e.g. to release PC5 Relay RLC channel configuration for relaying. However, this may not always be needed, e.g. in approach 2. But we propose to have some agreements even just for one approach, to make a progress.

Proposal 3: For multi-hop i2d, after remote UE is switched to direct link, the gNB may reconfigure the connection between the first relay/ intermediate Relay UE and the gNB (e.g. to release PC5 Relay RLC channel configuration for relaying).

And then the connection between first/intermediate relay UE and the intermediate/last relay UE may also need to be released, e.g. if there is no sidelink traffic anymore between them. But we understand this can be left to first/intermediate relay UE’s implementation when to release it.

Proposal 4: For multi-hop i2d, after remote UE is switched to direct link, when to release the PC5 unicast link between remote UE and first relay UE, is up to remote UE implementation.

Proposal 5: For multi-hop i2d, after remote UE is switched to direct link, when to release the PC5 unicast link between first/ intermediate relay UE and intermediate relay UE/last relay UE, is up to first/ intermediate Relay UE implementation.

After all the discussions, we think most steps of the legacy i2d procedure can be reused for multi-hop i2d scenario, with some different aspects we discussed in Proposal 1 to Proposal 5.

  - Intra-gNB multi-hop indirect to single-hop indirect path switching

The legacy procedure for L2 U2N Remote UE intra-gNB switching from indirect to indirect path is as follows[2].

Figure 2: Procedure for L2 U2N Remote UE intra-gNB switching from indirect to indirect path via a target L2 U2N Relay UE in RRC_CONNECTED

Generally, we understand that most steps of the procedure may also be used in multi-hop indirect to single-hop indirect path switching.

In step 1, in legacy procedure, the L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s), we understand it should remain the same for multi-hop case, where the remote UE should also report one or multiple candidate last Relay UE(s) (or called candidate single-hop relay, to be more accurate), because after all the UE would be connected to gNB via a single hop relay at last. The reported information can be the same as legacy as well.

Proposal 6: For multi-hop i2i, the remote UE reports serving first relay UE and candidate single-hop relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information.

On the other hand, the relay link should also be reported to gNB. In legacy, only sidelink measurement between the L2 U2N Remote UE and the source L2 U2N Relay UE needs to be reported, but for multi-hop i2i, similar as we discussed for multi-hop i2d, for link 2 and link 3, in order to make the gNB known of the quality, the first/ intermediate relay UE should evaluate the relay link measurement and report it to gNB.

Therefore, we propose:

Proposal 7: For multi-hop i2i, first/ intermediate relay UE evaluates adjacent relay links, where the sidelink relay measurement report shall include at least serving intermediate/ last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

Similarly, as we discussed in multi-hop i2d, after remote UE is switched to single-hop relay, the gNB may also reconfigure the SOURCE link 2 and link 3, e.g. to release PC5 Relay RLC channel configuration for relaying. Therefore, similar proposals are given here:

Proposal 8: For multi-hop i2i, after remote UE is switched to single-hop relay link, the gNB may reconfigure the connection between the source first relay/ intermediate Relay UE and the gNB (e.g. to release PC5 Relay RLC channel configuration for relaying).

And then the connection between source first/ intermediate relay UE and the source intermediate/ last relay UE may also need to be released, e.g. if there is no sidelink traffic anymore between them. But we understand this can be left to first relay/ intermediate relay UE’s implementation.

Proposal 9: For multi-hop i2i, after remote UE is switched to single-hop relay link, when to release the PC5 unicast link between remote UE and first relay UE, is up to remote UE implementation.

Proposal 10: For multi-hop i2i, after remote UE is switched to single-hop relay link, when to release the PC5 unicast link between first/ intermediate relay UE and intermediate relay UE/last relay UE, is up to first/ intermediate Relay UE implementation.

  - Path switch scenarios of second priority

For the scenarios C and D, some restrictions are stated as:

The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

However, in the post-email discussion about the approach 1 and 2[3], some concerns were raised as follows about the validity of scenario C and D when applying approach 2, because it may be not guaranteed that ‘all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell’.

| A2.6 Alternative 2 cannot support the agreed WI Objective for supporting Scenario C and D (see the highlighted part) | - The WI objective states that - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. A serious limitation of approach 2 is that Scenario C and D cannot be supported with approach 2 as relay UEs can be in different cell/ different gNB and bringing them to connected state will be extremely complex. |
| --- | --- |

Observation 1: If we apply approach 2 for multi-hop U2N relay UE connection structure, it may be not guaranteed that ‘all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell’ which is the limitation of path switch scenarios C and D.

As anyway the scenario C and D are second priority, we think we can further discuss them after the structure of approach 1 or 2 is determined.

Proposal 11: Path switch scenarios of second priority (C and D) are discussed after we decide approach 1 or 2 (as in Post127-402) of multi-hop U2N relay UE connection structure.

- Conclusion

In this contribution, we discussed the path switch scenarios for first priority, and for second priority ones, we prefer to discuss them after we agree the approach 1 or 2 under connection establishment discussion. We have the following Observations and Proposals:

For multi-hop i2d:

Proposal 1: For multi-hop i2d, Remote UE evaluates both relay link (for the link between remote UE and serving first relay UE) and Uu link, where the sidelink relay measurement report shall include at least serving first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

Proposal 2: For multi-hop i2d, first relay UE/ Intermediate relay UE evaluates adjacent relay link, where the sidelink relay measurement report shall include at least measured relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

Proposal 3: For multi-hop i2d, after remote UE is switched to direct link, the gNB may reconfigure the connection between the first relay/ intermediate Relay UE and the gNB (e.g. to release PC5 Relay RLC channel configuration for relaying).

Proposal 4: For multi-hop i2d, after remote UE is switched to direct link, when to release the PC5 unicast link between remote UE and first relay UE, is up to remote UE implementation.

Proposal 5: For multi-hop i2d, after remote UE is switched to direct link, when to release the PC5 unicast link between first/ intermediate relay UE and intermediate relay UE/last relay UE, is up to first/ intermediate Relay UE implementation.

For multi-hop i2i:

Proposal 6: For multi-hop i2i, the remote UE reports serving first relay UE and candidate single-hop relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information.

Proposal 7: For multi-hop i2i, first/ intermediate relay UE evaluates adjacent relay links, where the sidelink relay measurement report shall include at least serving intermediate/ last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

Proposal 8: For multi-hop i2i, after remote UE is switched to single-hop relay link, the gNB may reconfigure the connection between the source first relay/ intermediate Relay UE and the gNB (e.g. to release PC5 Relay RLC channel configuration for relaying).

Proposal 9: For multi-hop i2i, after remote UE is switched to single-hop relay link, when to release the PC5 unicast link between remote UE and first relay UE, is up to remote UE implementation.

Proposal 10: For multi-hop i2i, after remote UE is switched to single-hop relay link, when to release the PC5 unicast link between first/ intermediate relay UE and intermediate relay UE/last relay UE, is up to first/ intermediate Relay UE implementation.

For scenario C/D:

Observation 1: If we apply approach 2 for multi-hop U2N relay UE connection structure, it may be not guaranteed that ‘all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell’ which is the limitation of path switch scenarios C and D.

Proposal 11: Path switch scenarios of second priority (C and D) are discussed after we decide approach 1 or 2 (as in Post127-402) of multi-hop U2N relay UE connection structure.

- References

- RP-242349, Revised WID on NR sidelink multi-hop relay, LG Electronics, InterDigital, FirstNet, 3GPP TSG RAN Meeting #105;

- 3GPP TS 38.300 V18.3.0 (2024-09);

- R2-xxxxxxx Report of [POST127][402][Relay] MultliHop Relay CP_P2_v20_Rapp;