3GPP TSG-RAN2#128 	R2-2409640

Orlando, USA, 18th – 22th Nov, 2024

Agenda item:		8.12.2 (NR_MIMO_Ph5-Core)

Source:	LG Electronics Inc.

Title: 	Consideration on Asymmetric DL sTRP/UL mTRP

Document for:	Discussion and Decision

1.	Introduction

Based on the Rel-19 MIMO WID [1], RAN2 had initial discussion for asymmetric DL sTRP/UL mTRP in the last meeting.

In this contribution, we analyse RAN2 impacts from asymmetric DL sTRP/UL mTRP.

2.	Discussion

In legacy mTRP scenario, a serving cell has multiple TRPs to improve cell coverage and data rate. Each TRP can schedule DL/UL for a UE, and the UE can receive and transmit DL/UL from/to both TRPs.

In Rel-19, asymmetric DL sTRP/UL mTRP scenario is introduced. The motivation is to improve UL coverage and UL data rate while saving energy consumption from DL transmission. In this scenario, a serving cell has two TRPs, one is DL/UL TRP and the other one is UL-only TRP. The UE can only transmit UL data to UL-only TRP while scheduling/DL and UL data transmission are possible via DL/UL TRP.

Figure 1. mTRP and asymmetric sTRP DL/mTRP UL scenario

In legacy, each TRP transmits DL RS and the UE can estimate PL (pathloss) based on each DL RS. However, in Rel-19 scenario, UL-only TRP does not transmit DL RS, so the UE cannot estimate PL for UL-only TRP.

Thus, for UL-only TRP, RAN1 agreed to configure PL offset in a joint/UL TCI state by RRC and to update the PL offset by MAC CE, and sent LS [2] as follows. The network can estimate PL offset for UL-only TRP by measuring difference of UL RSRP between DL/UL TRP and UL-only TRP. Upon receiving PL offset, the UE calculates PL of UL-only TRP based on DL RS of DL/UL TRP and the PL offset.

Pertaining to the Rel-19 NR MIMO Ph5 WI, RAN1 would like to inform RAN2 about the following: - An initial list of RRC parameters (excel spreadsheet attached) - An agreement requiring MAC CE support: Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). “MAC-CE impact: MAC-CE updates the PL offset value(s) configured in one or more joint TCI states or UL TCI states”

Based on [2], RAN2 need to discuss the configuration of PL offset and MAC CE design.

Firstly, how to configure PL offset should be discussed. RAN1 agreed that PL offset is configured in a joint/UL TCI state by RRC and the value range of PL offset is [-12, 60] and the step size is 4dB.

[RAN1#118] Agreement For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB. - FFS: Extending the range to lower than -10dB - Step size is 4dB [RAN1#118bis] Agreement - The lower limit of the value range of PL offset is extended to -12 dB.

Currently, joint TCI State and UL TCI State are configured by using separate IE. Therefore, RAN2 should introduce PL offset parameter in TCI-State IE and TCI-UL-State-r17 IE respectively, where the value range is [-12, 60] and the step size is 4dB. One example is below.

TCI-State ::=                       SEQUENCE {

tci-StateId                         TCI-StateId,

qcl-Type1                           QCL-Info,

qcl-Type2                           QCL-Info                                                    OPTIONAL,   -- Need R

...,

[[

additionalPCI-r17                   AdditionalPCIIndex-r17                                      OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r17          PathlossReferenceRS-Id-r17                                  OPTIONAL,   -- Cond JointTCI1

ul-powerControl-r17                 Uplink-powerControlId-r17                                   OPTIONAL    -- Cond JointTCI

]],

[[

tag-Id-ptr-r18                      ENUMERATED {n0,n1}                                          OPTIONAL    -- Cond 2TA

]],

[[

plOffset-r19                      ENUMERATED {dB-12, dB-8, dB-4, dB0, dB4, dB8, dB12, dB16, dB20, dB24, dB28, dB32, dB36, dB40, dB44, dB48, dB52, dB56, dB60}                                          OPTIONAL

]]

}

Proposal 1: RAN2 introduce PL offset parameter in TCI-State IE and TCI-UL-State IE respectively, where the value range id [-12, 60] dB and the step size is 4dB.

Next, RAN1 also agreed to update PL offset for a joint/UL TCI state using a MAC CE, so RAN2 need to discuss details for MAC CE design. Based on contributions in the last meeting, discussion points can be listed as follows.

- Point 1. Whether to update PL offset only or to update PL offset with activating TCI state

- Point 2. Whether to indicate absolute PL offset value or delta value of PL offset value configured by RRC

- Point 3. How many PL offset are indicated in MAC CE

- Point 4. Other contents (e.g. CORESET POOL ID, D/U field and BWP ID field)

First above all, currently, there is no MAC CE to update PL offset. Thus, RAN2 should introduce a new MAC CE with new (e)LCID.

Proposal 2. To update PL offset associated with a joint/UL TCI state, RAN2 introduce a new MAC CE with new (e)LCID.

Point 1. Whether to update PL offset only or to update PL offset with activating TCI state

In the current MAC specification, joint/separate TCI states are activated by (Enhanced) TCI state activation MAC CE. Thus, two options were proposed in the last meeting.

- Option 1. Upon receiving MAC CE, update PL offset only

- Option 2. Upon receiving MAC CE, update PL offset and activate TCI state

As mentioned above, the network estimates PL offset for UL-only TRP by measuring difference of UL RSRP between DL/UL TRP and UL-only TRP. Thus, before receiving uplink transmission using a certain TCI state, the network may not be able to measure difference of UL RSRP for the certain TCI state.

Thus, it makes sense that for initial uplink transmission, the UE uses PL offset configured by RRC, then the network may be able to measure difference of UL RSRP and to estimate the actual PL offset. Therefore, the updated PL offset for a certain TCI state can be provided after receiving initial UL transmission using the certain TCI state.

In addition, considering that PL offset can be updated after activating TCI state, TCI state information can be provided redundantly in (Enhanced) TCI state activation MAC CE and new MAC CE.

Therefore, the new MAC CE should be designed to only consider PL offset update and to not consider TCI state activation.

Proposal 3. New MAC CE is designed to consider only PL offset update and to not consider TCI state activation.

Point 2. Whether to indicate absolute PL offset value or delta value of PL offset value configured by RRC

In the last meeting, RAN2 briefly discussed whether PL offset is absolute PL offset value or delta value of PL offset value configured by RRC.

If the new MAC CE indicates absolute PL offset value, PL offset value configured by RRC is replaced by PL offset indicated by the new MAC CE. Some companies raised a concern on this because, generally, a value configured by RRC is kept until a new value is configured, so they argued that PL offset value indicated in the new MAC CE should be delta value of PL offset value configured by RRC.

However, as other companies explained in the last meeting, currently, there are some cases where the state configured by RRC is replaced/changed by MAC CE, e.g. initial state of BWP or initial state of Serving Cell. Thus, even if PL offset value configured by RRC is replaced by PL offset indicated by the new MAC CE, it is not problematic.

In addition, from our perspective, it is a bit strange to indicate the delta value because PL offset itself means delta value. This means, PL offset is derived based difference of UL RSRP between DL/UL TRP and UL-only TRP and is applied to compensate PL between DL/UL TRP and UL-only TRP. Thus, delta value of PL offset value means "delta of delta value". Therefore, it would be desirable to indicate absolute value of PL offset in the new MAC CE.

Proposal 4. Absolute value of PL offset is indicated in the new MAC CE.

Point 3. How many PL offset are indicated in MAC CE

For Point 3, even though there is no clear RAN1 agreement on this, it should be discussed first whether PL offset corresponding to non-active TCI state can be updated by the new MAC CE

Currently, up to 8 joint/UL TCI state can be activated and up to 128 TCI state can be configured. Thus, if PL offset can be updated only for active joint/UL TCI state, up to 8 PL offsets need to be indicated in the new MAC CE, otherwise, up to 128 PL offsets need to be indicated.

As mentioned above, the network can estimate the actual PL offset for a certain TCI state upon receiving initial uplink transmission using the certain TCI state. Thus, PL offset corresponding to non-active TCI state may not be correctly estimated.

In addition, for active TCI state, PL offset needs to be updated quickly because the active TCI state is already used for UL transmission, so updating by MAC CE is desirable. However, PL offset corresponding to non-active TCI state does not need to be updated quickly, so updating by RRC signaling is sufficient.

Therefore, we propose that up to 8 PL offset are indicated in the new MAC CE and Only PL offset corresponding to active TCI state is updated by the new MAC CE.

Proposal 5. Up to 8 PL offset are indicated in the new MAC CE.

Proposal 6. Only PL offset corresponding to active TCI state is updated by the new MAC CE.

Point 4. Other contents (e.g. CORESET POOL ID, D/U field and BWP ID field)

To discuss which contents should be included in the new MAC CE, it would be helpful to refer contents of (Enhanced) Unified TCI state activation MAC CE.

Contents of (Enhanced) Unified TCI state activation MAC CE and our analysis are as follows:

| Contents | Analysis |
| --- | --- |
| Serving Cell ID | Serving Cell ID is needed in the new MAC CE to indicate the identity of Serving Cell for which the new MAC CE applies. Thus, One Serving Cell ID field is needed. |
| CORESET POOL ID | For asymmetric DL sTRP/UL mTRP scenario, sDCI method is used, while CORESET POOL ID is used for mDCI method. Thus, CORESET POOL ID is not needed. |
| UL/DL BWP ID | For joint mode, TCI-State IE is configured in DL BWP, and for separate mode, TCI-UL-State IE is configured in UL BWP. Considering unified TCI state mode (i.e. unifiedTCI-StateType) is configured per Serving Cell, TCI state to which PL offset is applied is configured either DL BWP or UL BWP. Thus, one BWP field is needed, i.e. two BWP field for DL BWP ID and UL BWP ID are not needed. |
| Codepoint/TCI state ID | If P5 is agreeable, one PL offset is mapped to one active TCI state. Considering one codepoint is mapped to one active TCI state and one PL offset is mapped to one active TCI state, one PL offset can be mapped to one codepoint. Therefore, codepoint bit map is needed but TCI state ID is not needed. Some companies proposed to include TCI state ID to indicate the mapping PL offset and TCI state ID, but codepoint is enough to indicate the mapping. Considering sDCI method is used for asymmetric DL sTRP/UL mTRP, 16 bit codepoint bit map is needed, i.e., 8-bit for first joint/UL TCI state and 8-bit for second joint/UL TCI state. |
| D/U | For joint mode, PL offset is mapped to joint TCI state, so D/U field is not used for joint mode. For separate mode, PL offset is mapped to UL TCI State and there is no mapping with DL TCI state, so D/U field is not needed. Therefore, D/U field is not needed. |

Based on above analysis, we propose to design the new MAC CE considering follows.

- One Serving Cell field and one BWP field are included

- 16-bit codepoint bit map is included

- CORESET Pool ID and D/U field are not included.

Proposal 7. RAN2 design the new MAC CE considering follows.

- One Serving Cell field and one BWP field are included

- 16-bit codepoint bit map is included

- CORESET Pool ID field and D/U field are not included

In addition, RAN1 also agreed to support applying PL offset on PDCCH-order based PRACH as follows.

Agreement Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1. - Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification. - Above is subject to a separate UE capability signaling. Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, support Alt3: - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission. - FFS the detailed design of DCI format: e.g., how to indicate one of the indicated joint/UL TCI states or whether to apply the PL offset in the indicated TCI state or not.

When one of joint/UL TCI state is indicated by PDCCH-order, a PL offset associated with the indicated joint/UL TCI state is applied on PDCCH order PRACH transmission and Tx power is calculated based on the PL offset.

In the last meeting, some companies think it is not clear whether PDCCH-order PRACH refers CFRA or CBRA. However, RAN1 clearly agreed that PL offset is applied on PDCCH-order PRACH toward UL-only TRP, and this means PL offset is indicated for CFRA toward UL-only TRP.

For PL offset on PDCCH-order PRACH, from RAN2 point of view, we think no further RAN2 work is needed except RRC configuration to associate PL offset with joint/UL TCI state.

Proposal 8. RAN2 assume that no RAN2 work is needed for PL offset on PDCCH-order based PRACH, except RRC configuration.

According to [1], one of objectives is to apply 2-TAs for asymmetric DL sTRP/UL mTRP scenario reusing Rel-18 specification. In Rel-18, 2-TAs are introduced only for mDCI case, but for asymmetric DL sTRP/UL mTRP, only sDCI is possible, so 2-TAs method needs to be extended considering sDCI.

In order to apply 2-TAs for asymmetric DL sTRP/UL mTRP, RAN1 made agreements as follows.

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

According to RAN1 agreement, almost things are inherited from Rel-18 2-TAs principle. However, for sDCI, coresetPoolIndex is not configured and SRS resource set is used to distinguish TAG, so RAN1 agreed to remove the restriction of of coresetPoolIndex.

In the current RRC specification, 2-TAs can be configured only when two coresetPoolIndex is configured, so the field description needs to be fixed for asymmetric DL sTRP/UL mTRP.

tag2 This field to indicate the second TAG information for the serving cell, it is optionally configured in a serving cell if and only if the serving cell is configured with more than one value for the coresetPoolIndex.

Considering the restriction is still needed for mDCI case, RAN2 need to discuss how to remove the restriction of coresetPoolIndex in the field description of tag2.

Proposal 9. RAN2 discuss how to remove the restriction of coresetPoolIndex in the field description of tag2.

3.	Conclusion

In this document, we analyse RAN2 impacts from asymmetric DL sTRP/UL mTRP, and made proposals as follows.

Proposal 1: RAN2 introduce PL offset parameter in TCI-State IE and TCI-UL-State IE respectively, where the value range id [-12, 60] dB and the step size is 4dB.

Proposal 2. To update PL offset associated with a joint/UL TCI state, RAN2 introduce a new MAC CE with new (e)LCID.

Proposal 3. New MAC CE is designed to consider only PL offset update and to not consider TCI state activation.

Proposal 4. Absolute value of PL offset is indicated in the new MAC CE.

Proposal 5. Up to 8 PL offset are indicated in the new MAC CE.

Proposal 6. Only PL offset corresponding to active TCI state is updated by the new MAC CE.

Proposal 7. RAN2 design the new MAC CE considering follows.

- One Serving Cell field and one BWP field are included

- 16-bit codepoint bit map is included

- CORESET Pool ID field and D/U field are not included

Proposal 8. RAN2 assume that no RAN2 work is needed for PL offset on PDCCH-order based PRACH.

Proposal 9. RAN2 discuss how to remove the restriction of coresetPoolIndex in the field description of tag2.

4.	Reference

- RP-242394	WID revision: NR MIMO Phase 5	Samsung

- R2-2407906	LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5	RAN1