3GPP TSG RAN WG2 #128                                                                                   R2-2409649

Orlando, USA, 18th – 22nd  Nov, 2024

Source:	CATT

Title:	Considerations on R18 leftovers for SDT

Agenda Item:	8.10.5

Document for:	Discussion and Decision

Introduction

In RAN2#127bis meeting [1], for R18 leftover issues for SDT, RAN2 has achieved the agreements as below:

Agreements: 3: For Rel-18 leftovers, regarding SON for SDT, focus on RSRP and data volume in SON reports. Other enhancements (including the additional failure cause) shall be paused unless time permits. For SDT: - For failed SDT case, UE includes the DL RSRP and UL data volume at the time of SDT evaluation in SON report. For successful SDT procedure, the UE does not log. - RAN2 understands for SON/MDT R19 the SDT enhancements only relate to the RA-SDT procedure. This does not rule out the case when the UE falls back from RA-SDT.

In this contribution, we will continue to discuss the R18 leftover issues for SDT, and our views are also given in this paper.

Discussion

Background

SDT consists of R17 MO-SDT and R18 MT-SDT. In the SDT initiation procedure, CG-SDT has a higher priority than RA-based SDT. For MO-SDT, when MO-SDT data arrives at the UE side, the UE will initiate the MO-SDT procedure with RA partitioning resource if CG-SDT initiation is not selected or fails. For MT-SDT, if the paging with MT-SDT indication is received by the UE, the UE will initiate the MT-SDT procedure with the normal RA resource if CG-SDT initiation is not selected or failed. Besides, the MT-SDT based RA procedure could have an MT-SDT specific RSRP threshold. If the MT-SDT specific RSRP threshold is not configured for the MT-SDT based RA procedure, the MO-SDT specific RSRP threshold will be applied.

RSRP and data volume

In RAN2#127bis meeting, RAN2 agreed to log the DL RSRP and UL data volume at the time of SDT evaluation in SON report for failed SDT case and not for successful SDT procedure. Additionally, RAN2 also agreed to not rule out the case when the UE falls back from RA-SDT, which means that RAN2 could discuss the case that UE will trigger the common RA procedure if the condition for RA-SDT initiation is failed and RA-SDT could not be triggered. In this case, rather than introducing an explicit indication to indicate the RA-SDT is triggered prior to a common RA procedure, it is more beneficial to log the DL RSRP and UL data volume for not triggered RA-SDT as an implicit indication. This could assist the network in setting the thresholds for SDT initiation.

Proposal 1: In the case of UE falling back from RA-SDT, UE logs the DL RSRP and UL data volume for the failed triggering of RA-SDT in RA report.

Failure cause

Similarly to SI request failure and beam recovery failure, SDT failure indication has been recorded in R18 RA report for RA-partitioning. In RAN2#127 meeting, RAN2 agreed that UE logs and reports the failure cause for SDT to the network. Based on the companies’ views and the specification on SDT procedure, the SDT failure could happen in below cases:

- Case 1: upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached is received while SDT procedure is ongoing.

- Case 2: upon random access problem indication is received from MCG MAC while SDT procedure is ongoing.

- Case 3: upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing.

- Case 4: upon integrity check failure indication is received from lower layers while SDT procedure is ongoing.

- Case 5: upon T319a expires.

- Case 6: upon network sends UE to RRC_IDLE in response to the resume procedure initiated for SDT.

- Case 7: upon network sends UE to RRC_INACTIVE in response to the resume procedure initiated for SDT.

- Case 8: unsuccessfully completed upon cell re-selection

For above cases, case 5 has been agreed in RAN2#126 meeting, the case1 and 2 are aligned with the legacy failure report cause and can be supported as well. Case 3 is associated to CG procedure, not to RA procedure, but it will be also beneficial to help network adjust the Timer, in our view it can also be supported. Furthermore, it can be seen that most of above SDT failure cases are not be differentiated as the CG-SDT or RA-SDT in legacy specification, it is suggested to introduce an indication to identify it is for CG-SDT or RA-SDT, which could help network to know the failure is caused by CG-SDT or RA-SDT and determine which parameters needs to be adjusted. For case 4/6/7, it is not sure what the benefits could be if supported. For case 8, the UE will move to RRC_IDLE mode, when SDT is failed because of poor cell channel quality in camping cell, so the cause reporting for case 8 could be supported and aligned to the rule to indicate the channel status in legacy RLF report.

Proposal 2: The following failure causes can be logged for failed SDT in RA report:

- upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is ongoing;

- upon random access problem indication is received from MCG MAC while SDT procedure is ongoing;

- upon T319a expires;

- upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing;

- unsuccessfully completed upon cell re-selection.

Proposal 3: To introduce an indication to differentiate whether it’s CG-SDT failure or RA-SDT failure in RA report, which could help network to know the failure is happened in CG-SDT or RA-SDT procedure and determine which parameters need to be adjusted.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: In the case of UE falling back from RA-SDT, UE logs the DL RSRP and UL data volume for the failed triggering of RA-SDT in RA report.

Proposal 2: The following failure causes can be logged for failed SDT in RA report:

- upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is ongoing;

- upon random access problem indication is received from MCG MAC while SDT procedure is ongoing;

- upon T319a expires;

- upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing;

- unsuccessfully completed upon cell re-selection.

Proposal 3: To introduce an indication to differentiate whether it’s CG-SDT failure or RA-SDT failure in RA report, which could help network to know the failure is happened in CG-SDT or RA-SDT procedure and determine which parameters need to be adjusted.

Reference

- Chairman’s Notes RAN2#127bis, Hefei, China, Oct 14th –18th, 2024.