3GPP TSG RAN WG2 #128                                                                         	  R2-2409653

Orlando, USA, Nov 18th – 22nd, 2024

Source:	CATT

Title:	Consideration on NW side data collection

Agenda Item:	8.1.3

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting [1], some agreements have been achieved for NW side data collection in the aspect of BM use case:

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

In this contribution, we further discuss the data collection for NW-side model training for BM and Pos use cases.

- Beam management

  - Mechanism of NW side data collection

It has been agreed that immediate MDT is the baseline framework for OAM-centric training data collection. In legacy, immediate MDT reuses the measConfig field in RRCReconfiguration to indicate the MDT configuration. To enhance this field for AI/ML purpose, additional parameters are necessary:

- Enhancement of configuration for AI/ML training data collection;

For immediate MDT, the UE measurement and report mechanism is entirely under NW control. The NW could configure a set of MeasObjectNR(s) and a set of ReportConfigNR(s), only the MeasObjectNR and ReportConfigNR associated by a measID could be actually performed for measurement. Multiple measId(s) could be configured to the UE for different measurements. Therefore based on the immediate MDT, it is easy for the NW to configure multiple AI/ML purposed measurements, and the activation/deactivation of measurements can be controlled by the presence/absence of measId in RRCReconfiguration. Since RAN1 has agreed that “ For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability” and “The Activated functionalities may be enabled based on CSI framework”, we think for training data collection, the similar mechanism could be utilized to configure and activate more than one set of parameters.

Proposal 1: Support multiple configurations at UE side for AI/ML training data collection.

Proposal 2: Activation/deactivation of measurements for AI/ML training data collection can be implicitly controlled by the presence/absence of measId in RRCReconfiguration.

The gNB should configure UE to perform data logging and reporting based on the potential AI/ML training requirements, such as:

- trigger conditions of measurement result logging;

- trigger conditions of availability indication reporting.

Based on the legacy immediate MDT, the reference signal related information in the serving/neighboring cell(s) to be measured could be included in the MeasObjectNR IE, and the logging trigger conditions and reporting (availability indication) trigger conditions could be introduced in the ReportConfigNR IE which including the logging type, reporting type, logging/reporting quantity, report Interval/Amount .etc.

Proposal 3: For AI/ML NW side training data collection, network configures data logging trigger conditions and availability indication reporting trigger conditions related information in the ReportConfigNR IE, and configures the reference signal related information in the MeasObjectNR IE.

- Enhanced data logging in UE;

It is agreed in last RAN2 meeting that both periodic logging and event-triggered logging will be supported.

For periodic recording, it is more akin to a combination of immediate MDT and logged MDT. The UE can record one or multiple entries related to AL/ML of UE measurement results within one reporting period according to the network-configured interval parameters. Moreover, the recording will commence immediately after the UE receives the training data collection configuration.

For event-based recording, the UE can record only one entry if the pre-configured event is triggered. For example, when a handover failure (HOF) occurs or the UE measurement fulfills the event A3 threshold. Or, it can periodically record the measurement results in the UE variable according to the network-configured recording trigger conditions and interval parameters after the recording trigger conditions are met.

To compare among these recording mechanism, different triggering conditions may apply to different data collection content requirements, and the event-triggered periodic data logging could make a trade-off between the one-shot event based logging and the pure periodic logging.

Proposal 4: Support event-triggered periodic data logging to make a trade-off between the one-shot event based logging and the pure periodic logging.

For event-triggered logging, the trigger condition is FFS. In the logged MDT, an area scope will be configured to the UE. The area scope indicates the area for which UE is requested to perform measurement logging. If not configured, measurement logging is not restricted to specific cells or tracking areas but applies as long as the RPLMN is contained in the MDT PLMN list. For the AI/ML NW side training data collection, an area scope could also be used to restrict the data collection geographical area, to avoid unnecessary logging and power waste. The area scope could be one cell, a cell list or TA/PLMN wide (as in logged MDT).

Proposal 5: Introduce an area scope to restrict the UE to log data only within this geographical area. The area scope can be specified as a single cell, a list of cells, or a list of TA/PLMN as in logged MDT.

- Enhanced data reporting by UE;

It is agreed in last RAN2 meeting that on-demand reporting of the logged measurements will be specified instead of periodic reporting, and the UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection similar as logged MDT does. The details could follow the logged MDT report content. The logged MDT report content is as follows:

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...

}

- For training collection time, the absoluteTimeStamp could be the absolute time when the training data configuration is received by the UE;

- For training data marker, the associated ID may be used to instead of the trace related ID of traceReference and traceRecordingSessionRef;

- For training collection node, TCE or other node index could be used;

- The main content of training data collection could be a list similar as logMeasInfoList;

- If the size of the training data memorized in UE exceeds the size a report message can carry, the available indicator will be added in the report message until all data is sent to the NW.

Proposal 6: The details of the training data reporting message could include:

- For training data collection time, the absoluteTimeStamp could be the absolute time when the training data configuration is received by the UE;

- For training data marker, the associated ID may be used to instead of the trace related ID of traceReference and traceRecordingSessionRef;

- For training data collection node, TCE or other node index could be used;

- The main content of training data collection could be a list like logMeasInfoList;

- If the size of the training data memorized in UE exceeds the size a report message can support, the availability indicator will be added in the report until all data is sent to the NW.

The availability indicator is agreed to be used but the trigger/definition is FFS. The availability indicator of legacy logged MDT will be sent to the network to indicate whether the UE stores the available measurement result. But for training data collection, since the data is non-time sensitive and all the data will be used for model training together, the UE could aware of the NW the power state condition is not fulfilled, and then stop the data collection and send available indicator to the NW. In a similar way, if the logged AI/ML measurement results reached the AS buffer limitation, the available indicator could also be sent to the NW. After receiving the available indicator, NW could decide whether to request for the measurement result reporting based on NW implementation.

Proposal 7: The trigger condition of the event-based availability indicator can be the UE power state is low, or the UE AS buffer is full.

  - Memory for NW side training data

It is agreed that UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. But it is FFS on the memory size. For logged MDT, the minimum memory size of MDT logged measurements is 64KB. Therefore 64KB could also be assumed as the minimum memory size of AI/ML training data.

Proposal 8: Set the minimum memory size of AI/ML training data logging to be 64KB.

  - Transmission priority of NW side training data

It is agreed in last RAN2 meeting that low priority SRB will be used. But whether to use new SRB or use SRB4 is FFS. The legacy logged MDT report is transmitted on SRB2. The priority between SRBs is described in TS38.331 as below:

- SRB1 is for RRC messages (which may include a piggybacked NAS message) as well as for NAS messages prior to the establishment of SRB2, all using DCCH logical channel (except SRB1 of L2 U2N Remote UE); - SRB2 is for NAS messages and for RRC messages which include logged measurement information, all using DCCH logical channel (except SRB2 of L2 U2N Remote UE). SRB2 has a lower priority than SRB1 and may be configured by the network after AS security activation; - SRB3 is for specific RRC messages when UE is in (NG)EN-DC or NR-DC, all using DCCH logical channel; - SRB4 is for RRC messages which include application layer measurement report information, all using DCCH logical channel. SRB4 has a lower priority than SRB1 and can only be configured by the network after AS security activation. - SRB5 is for RRC messages which include application layer measurement report information, all using DCCH logical channel. SRB5 has a lower priority than SRB1 and SRB3 and can only be configured by the SN serving the SCG when the UE is in NR-DC, after AS security activation.

From the spec, the SRB2/SRB4/SRB5 has a lower priority than SRB1. But there is no fixed priority comparison between SRB2 and SRB4/SRB5. SRB2 could be configured with higher/lower/equal priority with SRB4/SRB5.

Observation 1: There is no fixed priorities among SRB2 and SRB4/SRB5. SRB2 can be configured to have a higher, lower, or equal priority as that of SRB4/SRB5

On one hand, the NW is capable on when to request the UE to report the large amount of AI/ML training data or logged MDT data to avoid the conflict with NAS message transmission, on the other hand, the UE could decide how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. Since the AI/ML training data may have the same minimum memory size as for logged MDT measurements, SRB2 could also be used to transmit the AI/ML training data. There is no need to define a new SRB or use SRB4/5 since the NW could require the data when the traffic load is low, e.g. in the midnight.

Observation 2: NW is capable to schedule the UE to report the large amount of AI/ML training data of logged MDT data and avoid the conflict with NAS message transmission.

Proposal 9: Utilize SRB2 as in the logged MDT report for AI/ML training data transmission.

- Positioning accuracy enhancements

Among all 5 Pos cases, UE side model cases include:

- Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

Among all 5 Pos cases, NW side model cases include:

- Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

- Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

It is approved that RAN1 will focus on the first priority issues (case1/3a/3b) until end of 2024 in RAN#105 meeting [2]. And inside these 3 first priority cases, RAN3 has already discussed on case 3a and 3b and some agreements have been achieved in last RAN3 meeting. Therefore, RAN2 should first focus on the specification impact on PoS case 1 even it is a UE side model.

Observation 3: RAN3 has already discussed on case 3a and 3b and some agreements have been achieved in last RAN3 meeting.

Proposal 10: RAN2 first focus on the specification impact of training data collection on PoS case 1.

Training data collection for BM case is enhanced on MDT mechanism which involves UE, gNB, and TCE node. Some CN node may also be utilized for MDT configuration transmission without information change, i.e. for signaling based MDT. Since training data collection is non-time sensitive, the UE can log the training data over a period of time, and then send them to the training node to reduce the air-interface overhead and to avoid frequent connection establishment. PoS case1 is UE-based positioning with UE-side model, and UE side data collection is still under discussion in SI phase. For PoS case1, two options may be used for training data collection with 3GPP protocol implications:

- Option 1: To collect training data by OAM first, similar as BM case, and OAM sends all data to training node.

- Option 2: To collect training data by LMF first, using LPP signaling;

By using option1, the similar training data collection mechanism agreed for BM can be applied for Pos use case between UE and OAM, but the configuration for PoS training data collection, the report content should all be defined in Uu as new parameters which seem more complex.

Observation 4: It is complex and many PoS related parameters should be defined in Uu, if using a similar training data collection mechanism in BM case between UE and OAM for Pos use case.

By using option2, training data for Pos use case should be transmitted from UE to LMF. The location in Deferred 5GC-MT-LR procedure as specified in TS 23.273 can be reported by UE by periodic, triggered and UE available location events [3]. For a request for the UE available location event, a one-shot report will be performed. If periodic or triggered location was requested, the LMF sends a supplementary services LCS Periodic-Triggered Invoke Request to the UE, and the UE should monitors for occurrence of the trigger or periodic event requested. When a trigger or periodic event is detected, the UE should report the event and perform the positioning procedure if fulfill all the conditions (e.g. access an allowed access type, and inside the event report allowed area). So if we choose this option, the data logging, the available indication and on-demand report mechanism agreed for training data collection in BM case cannot be applied to PoS case without SA2 impact based on current LCS procedure.

But since the LCS procedure is not as frequent as RSRP/beam-id measurement in BM case, some legacy NW request LCS procedures could be utilized for training data collection, e.g. NI-LR procedure, 5GC-MT-LR procedure, Deferred 5GC-MT-LR procedure.

Proposal 11: Some legacy LCS procedures initiated by NW could be utilized for training data collection, e.g. NI-LR, 5GC-MT-LR, Deferred 5GC-MT-LR.

- Conclusion

In this contribution, we provide our views on the training data collection for NW side model, and the observation and proposals are summarized as follows:

For beam management

- Mechanism:

Proposal 1: Support multiple configurations at UE side for AI/ML training data collection.

Proposal 2: Activation/deactivation of measurements for AI/ML training data collection can be implicitly controlled by the presence/absence of measId in RRCReconfiguration.

Proposal 3: For AI/ML NW side training data collection, network configures data logging trigger conditions and availability indication reporting trigger conditions related information in the ReportConfigNR IE, and configures the reference signal related information in the MeasObjectNR IE.

Proposal 4: Support event-triggered periodic data logging to make a trade-off between the one-shot event based logging and the pure periodic logging.

Proposal 5: Introduce an area scope to restrict the UE to log data only within this geographical area. The area scope can be specified as a single cell, a list of cells, or a list of TA/PLMN as in logged MDT.

Proposal 6: The details of the training data reporting message could include:

- For training data collection time, the absoluteTimeStamp could be the absolute time when the training data configuration is received by the UE;

- For training data marker, the associated ID may be used to instead of the trace related ID of traceReference and traceRecordingSessionRef;

- For training data collection node, TCE or other node index could be used;

- The main content of training data collection could be a list like logMeasInfoList;

- If the size of the training data memorized in UE exceeds the size a report message can support, the availability indicator will be added in the report until all data is sent to the NW.

Proposal 7: The trigger condition of the event-based availability indicator can be the UE power state is low, or the UE AS buffer is full.

- Memory:

Proposal 8: Set the minimum memory size of AI/ML training data logging to be 64KB.

- Priority:

Observation 1: There is no fixed priorities among SRB2 and SRB4/SRB5. SRB2 can be configured to have a higher, lower, or equal priority as that of SRB4/SRB5

Observation 2: NW is capable to schedule the UE to report the large amount of AI/ML training data of logged MDT data and avoid the conflict with NAS message transmission.

Proposal 9: Utilize SRB2 as in the logged MDT report for AI/ML training data transmission.

For positioning accuracy enhancements

Observation 3: RAN3 has already discussed on case 3a and 3b and some agreements have been achieved in last RAN3 meeting.

Proposal 10: RAN2 first focus on the specification impact of training data collection on PoS case 1.

Observation 4: It is complex and many PoS related parameters should be defined in Uu, if using a similar training data collection mechanism in BM case between UE and OAM for Pos use case.

Proposal 11: Some legacy LCS procedures initiated by NW could be utilized for training data collection, e.g. NI-LR, 5GC-MT-LR, Deferred 5GC-MT-LR.

- References

- Chairman’s Notes RAN2#127, Maastricht, Netherlands, Aug 19th – 23rd, 2024

- RP-242387	Moderator's summary on offline discussion about non-RAN2 aspects on REL-19 WI NR_AIML_Air	RAN1 vice-chair (CMCC)

- TS23.273, 5G System (5GS) Location Services (LCS); Stage 2