3GPP TSG-RAN WG2 Meeting #128	R2- 2409666

Orlando, USA, Nov 18th – 22th, 2024

Agenda Item		:	8.8.2

Source		:	LG Electronics Inc.

Title		:	Discussion on downlink coverage enhancement

Document for		:	Discussion and Decision

1. Introduction

During RAN2 #127bis meeting, RAN2 made some progress with the following statements:

Agreements:

1.	If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE)

2.	If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit)

3.	(also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

In this contribution, we provide our views on DL coverage enhancements.

2. Discussion

Access Control

In the RAN2#127bis meeting, RAN2 agreed to introduce a new barring mechanism to control access of UEs supporting DL-CE, if it turns out that there is a need to bar UEs not supporting DL-CE. It is needed to discuss how to introduce a barring mechanism for the UE supporting DL-CE. Similar to the existing barring mechanism that is used to control UE access for a new feature, a new bar bit should be introduced in SIB1 to control access of UEs supporting DL-CE.

Proposal 1	If we need to bar UEs supporting DL-CE, a new bar bit should be introduced in SIB1 to control access of UEs supporting DL-CE.

There are three possible options for the new bar bit with code points {barred}, {notBarred}, and {barred, notBarred}.

Option 1. {barred}

As the new bar bit can be set to “barred” only, then the absence of new bar bit indicates “notBarred”. If the new bar bit is absent in a cell, the UEs supporting DL-CE cannot determine whether the cell is operating with DL-CE or not. Then, the UE cannot determine whether the UE should consider the cell as notBarrred, assuming the cell is operating with DL-CE, or whether it should consider the legacy cellBarredNTN, assuming that the cell is not operating with DL-CE. To enable UEs to determine whether the cell is operating DL-CE, RAN2 should introduce a new indication in SIB1 that the cell is operating with DL-CE.

Option 2. {notBarred}

The new bar bit can be set to “notBarred” only. Absence of the new bar bit indicates that DL-CE is not supported in this cell. Then, if the UE supporting DL-CE sees the new bar bit absent in a cell, the UE considers the cell as legacy NTN cell and hence follows the legacy cellBarredNTN for access control. But, when UEs supporting DL-CE are barred from the cell, UEs cannot know whether the cell is DL-CE cell or not.

Option 3. {barred, notBarred}

In this option, the new bar bit is set to either “barred” or “notBarred”. The new bar bit shall be present in a cell operating with DL-CE. Then, there is no need to introduce a new indication that the cell is operating with DL-CE, because the presence of new bar bit already indicates that the cell is operating with DL-CE. If the new bar bit is absent, UEs supporting DL-CE consider the cell as non-DL-CE cell and hence consider the existing cellBarredNTN bit for access control as legacy.

Based on the analysis, option1 is less desirable, as it requires the new indication that the cell is operating with DL-CE. Option2 and option3 can provide necessary access control, without requiring the new indication. The only difference between option2 and option3 is whether UEs supporting DL-CE can know if a cell is operating as a DL-CE cell when they are barred from the cell actually operating with DL-CE (in option3, the UE can know that the cell is DL-CE cells from the presence of the new bar bit, whereas in option2, the UE cannot distinguish the cell from the legacy non-DL-CE cell). We think this difference is not important because we do not see any necessity for UEs to know whether the barred cell is DL-CE cell. So, we suggest to adopt the option3{barred, notBarred} just to follow the same code-points of cellBarredNTN.

Proposal 2	The new bar bit shall be present in a cell operating with DL-CE, with code points {barred, notBarred}.

Proposal 3	If the new bar bit is absent in SIB1, UE supporting DL-CE considers that the cell does not support DL-CE and follows cellBarredNTN.

Cell Reselection Enhancement

As discussed in the previous meeting, UEs not supporting DL-CE should be prevented from reselecting DL-CE cells, when the serving cell is not operating DL-CE. This is because if UEs that do not support DL-CE reselect DL-CE cells, they will be barred and measure additionally to reselect among other candidate cells. This would increase the delay of the reselection process. In the worst case, if the cell is an earth-moving cell, UEs might be unable to camp normally on a suitable cell and go to any cell selection state. As some UEs that do not support DL-CE are pre-Rel19 UEs, it is not possible to introduce a new IE to prevent UEs not supporting DL-CE from reselecting DL-CE cells. Therefore, the DL-CE cell reselection enhancement should consider the backward compatibility issue.

The simplest way to enhance DL-CE cell reselection without causing backward compatibility issue is using the existing neighbor cell list, such as ExcludedCellList in SIB3,4 or ntn-NeighCellConfigList in SIB19. In detail, if the ExcludedCellList is configured to include DL-CE cells, UEs not supporting DL-CE will not reselect DL-CE cells. Also, we can use ntn-NeighCellConfigList in SIB19. As NTN UEs only can measure and evaluate the neighboring cells configured in the ntn-NeighCellConfigList and ntn-NeighCellConfigListExt in SIB19, the network can configure the DL-CE cells to be excluded from the ntn-NeighCellConfigList and ntn-NeighCellConfigListExt. And UEs not supporting DL-CE will not measure and reselect DL-CE cells.

Proposal 4	Network can use the existing neighbor cell list, such as ExcludedCellList in SIB3,4 and ntn-NeighCellConfigList in SIB19, to prevent UEs not supporting DL-CE from reselecting DL-CE cells, without backward compatibility issue.

However, if the existing neighbor cell list is used to prevent UEs that do not support DL-CE from reselecting DL-CE cells, UEs that support DL-CE will also be unable to reselect DL-CE cells. To allow UEs supporting DL-CE to reselect DL-CE cells, we need to introduce a new excludedCellList or a new ntn-NeighCellConfigList. For example, the network can configure a new ExcludedCellList in SIB3,4. As UEs supporting DL-CE are Rel-19 UEs, UEs supporting DL-CE can be configured to consider only a new ExcludedCellList and ignore existing ExcludedCellList. Also, the network can configure a new ntn-NeighCellConfigList in SIB19. UEs supporting DL-CE can be configured to consider both new ntn-NeighCellConfigList and existing ntn-NeighCellConfigList to measure and evaluate the neighbor cells for re-selection.

Proposal 5	RAN2 should introduce a new neighbor cell list, either/both a new ExcludedCellList in SIB3,4 or/and a new ntn-NeighCellConfigList in SIB19, to allow UEs supporting DL-CE to reselect DL-CE cells.

Proposal 6	If new ExcludedCellList for DL-CE is present, UEs supporting DL-CE only consider new ExcludedCellList for DL-CE and ignore existing ExcludedCellList.

Proposal 7	If new ntn-NeighCellConfigList for DL-CE is present, UEs supporting DL-CE consider both new ntn-NeighCellConfigList for DL-CE and existing ntn-NeighCellConfigList.

SMTC impacts due to beam-hopping / larger SSB periodicity

In the RAN2#127bis, there was a discussion on whether extending the number of SMTC is needed or not due to SSB periodicity extension and beam hopping. However, before discussing the required number of SMTCs, we need to analyze how beam hopping affects the SMTC configuration, which requires a precise understanding of how the SSBs are transmitted via beam hopping. So far, RAN1 has not discussed the details of beam hopping. Without a sufficient understanding of how beam hopping is performed possibly with SSB periodicity extension, it is premature for RAN2 to discuss its impact on SMTC configuration.

Observation 1	The decision regarding the extension of the SSB periodicity is still ongoing and there is no progress on beam hopping in RAN1.

The argument for increasing the number of SMTCs per frequency was based on the idea that, if the SSB periodicity is extended, different SSB transmission timings might be applied among cells through SSB beam-hopping transmission. To address the scenario where the six neighbor cells adjacent to a serving cell have different SSB transmission timings, it was proposed to increase the number of SMTCs per frequency to six.

In principle, even if the SSB periodicity is extended and the network transmits SSBs via beam hopping, multiple cells will transmit SSBs with the same SSB transmission timing. The network can group the cells adjacent to each other, ensuring that adjacent cells are placed in the same SSB beam hopping area. The maximum number of cells in one group is the total number of simultaneously active beams, approximately 10% of the cells deployed in the satellite for the Set1-1 scenario [1]. The cells included in the same group may have the same SBB transmission timing. The UE may experience three kinds of the SSB transmission timing in worst case. Therefore, the network implementation will ensure that the number of SMTCs in the current specification is sufficient to measure all the neighbor cells without requiring any enhancement.

Figure 1. Illustrating the SSB beam hopping area and the worst-case scenario.

Observation 2	If the network properly configures beam hopping for SSB transmission, the current number of SMTCs will be sufficient for neighbor cells measurement.

Given the observation1 and 2, unless RAN1 provides additional input on beam hopping, the current SMTC configuration is sufficient with network implementation. We think that RAN2 does not need to further discuss the impact of beam-hopping /the extension of SSB periodicity on SMTC.

Proposal 8	From RAN2's perspective, there is no SMTC issue related to beam hopping or SSB periodicity extension unless further input is provided by RAN1.

DTX

From the previous meetings, many companies proposed enhancing the Rel-18 NES DTX operation to support beam hopping operation in NTN. However, since many aspects of the DTX operation in NTN are dependent on RAN1, they should first be discussed in RAN1 before being discussed in RAN2. For example, extending the periodicity of SSB affects how often SSBs should be transmitted in a cell, which affects the DTX pattern of the cell. Similar to the SSB transmission, before discussing the DTX operation in RAN2, it is necessary to discuss how the beam hopping operation impacts physical channels or which beam hopping conditions are required to support initial cell access, paging, and RACH procedure.

Proposal 9	RAN2 postpones the discussion on DTX operation until it receives sufficient input from RAN1 regarding the DTX operation.

3. Conclusion

In this contribution, following statements are proposed:

Proposal 1	If we need to bar UEs supporting DL-CE, a new bar bit should be introduced in SIB1 to control access of UEs supporting DL-CE.

Proposal 2	The new bar bit shall be present in a cell operating with DL-CE, with code points {barred, notBarred}.

Proposal 3	If the new bar bit is absent in SIB1, UE supporting DL-CE considers that the cell does not support DL-CE and follows cellBarredNTN.

Proposal 4	Network can use the existing neighbor cell list, such as ExcludedCellList in SIB3,4 and ntn-NeighCellConfigList in SIB19, to prevent UEs not supporting DL-CE from reselecting DL-CE cells, without backward compatibility issue.

Proposal 5	RAN2 should introduce a new neighbor cell list, either/both a new ExcludedCellList in SIB3,4 or/and a new ntn-NeighCellConfigList in SIB19, to allow UEs supporting DL-CE to reselect DL-CE cells.

Proposal 6	If new ExcludedCellList for DL-CE is present, UEs supporting DL-CE only consider new ExcludedCellList for DL-CE and ignore existing ExcludedCellList.

Proposal 7	If new ntn-NeighCellConfigList for DL-CE is present, UEs supporting DL-CE consider both new ntn-NeighCellConfigList for DL-CE and existing ntn-NeighCellConfigList.

Observation 1	The decision regarding the extension of the SSB periodicity is still ongoing and there is no progress on beam hopping in RAN1.

Observation 2	If the network properly configures beam hopping for SSB transmission, the current number of SMTCs will be sufficient for neighbor cells measurement.

Proposal 8	From RAN2's perspective, there is no SMTC issue related to beam hopping or SSB periodicity extension unless further input is provided by RAN1.

Proposal 9	RAN2 postpones the discussion on DTX operation until it receives sufficient input from RAN1 regarding the DTX operation.

4. References

[1] R1-2401844	FL Summary #2: NR-NTN downlink coverage enhancements	Moderator (THALES).