3GPP TSG-RAN WG2 Meeting #128	 R2-2409672

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.8.6

Source: 	CATT

Title: 	Discussion on the remaing issue for LTE TN to NR NTN IDLE mode mobility

Document for: 	Discussion and Decision

Introduction

In RAN2 #127bis, the remaining issues on LTE TN to NR NTN IDLE mode mobility were discussed, and agreements were made for most of these issues, except whether dedicated NR NTN frequencies can be included in RRCConnectionRelease: [1]

Proposal 3: RAN2 confirms the understanding that dedicated NR NTN frequency is not configured in the RRCConnectionRelease message, and confirms that this can be left to NW implementation (w/o Spec impact). - QC is ok with this but thinks it’s fine to clarify this somewhere - Samsung thinks we should separate the case of providing dedicated frequencies from the redirection case. Xiaomi agrees, redirection could be performed providing assistance information in the release message. LG shares similar view as Samsung but thinks the NW does not need to provide assistance info - China Telecom thinks satellite information would be needed to support redirection to NR NTN - QC thinks that either we go for proposal 3 or we need to introduce a UE capability for this. Come back in the next meeting

This contribution addresses this remaining issue for the LTE_TN_NR_NTN_mob work item.

Discussion

Based on last meeting discussion, companies did not reach consensus on whether redirection from LTE TN to NR NTN can be supported even for Rel-17/18.

According to current TS36.331 and TS36.306 [2][3], we can find that since Rel-15, UE has already been able to report its capability on the support of NR SA and the supported bands for NR SA. The network can determine whether to configure the NR frequency for redirection or cell reselection purpose, based on these reported UE capabilities.

- TS 36.331 IRAT-ParametersNR-v1540 ::= SEQUENCE { eutra-5GC-HO-ToNR-FDD-FR1-r15 ENUMERATED {supported} OPTIONAL, eutra-5GC-HO-ToNR-TDD-FR1-r15 ENUMERATED {supported} OPTIONAL, eutra-5GC-HO-ToNR-FDD-FR2-r15 ENUMERATED {supported} OPTIONAL, eutra-5GC-HO-ToNR-TDD-FR2-r15 ENUMERATED {supported} OPTIONAL, eutra-EPC-HO-ToNR-FDD-FR1-r15 ENUMERATED {supported} OPTIONAL, eutra-EPC-HO-ToNR-TDD-FR1-r15 ENUMERATED {supported} OPTIONAL, eutra-EPC-HO-ToNR-FDD-FR2-r15 ENUMERATED {supported} OPTIONAL, eutra-EPC-HO-ToNR-TDD-FR2-r15 ENUMERATED {supported} OPTIONAL, ims-VoiceOverNR-FR1-r15 ENUMERATED {supported} OPTIONAL, ims-VoiceOverNR-FR2-r15 ENUMERATED {supported} OPTIONAL, sa-NR-r15 ENUMERATED {supported} OPTIONAL, supportedBandListNR-SA-r15 SupportedBandListNR-r15 OPTIONAL } - TS 36.306 - 4.3.34.3 supportedBandListNR-SA-r15 This field indicates whether UE supports standalone NR, as specified in TS 38.331 [35], and includes the supported NR bands as defined in TS 38.101-1 [33] and TS 38.101-2 [34]. The presence of this field also indicates that the UE can perform both NR SS-RSRP and SS-RSRQ measurement in the included NR band(s) as specified in TS 38.215 [36]. - 4.3.34.12 sa-NR-r15 This field indicates whether the UE supports standalone NR as specified in TS 38.331 [35].

Observation 1: Since Rel-15, eNB can determine whether to configure the NR frequency for redirection or cell reselection in RRCConnectionRelease for a UE, based on the UE capabilities reported in sa-NR-r15 and supportedBandListNR-SA-r15.

The value range of the supportedBandListNR-SA-r15 is 1~1024 as follows:

SupportedBandListNR-r15 ::=		SEQUENCE (SIZE (1..maxBandsNR-r15)) OF SupportedBandNR-r15

SupportedBandNR-r15 ::=			SEQUENCE {

bandNR-r15							FreqBandIndicatorNR-r15

}

FreqBandIndicatorNR-r15 ::=			INTEGER (1.. maxFBI-NR-r15)

maxFBI-NR-r15				INTEGER ::= 1024	-- Highest value FBI range for NR.

Also note that currently the NR NTN operating bands include n256/n255/n256/n510/n511/n512 [4]. That is, if the UE supports NR NTN, it is allowed to report the corresponding NR NTN operating bands via current Rel-15 signaling.

- 5.2.2 Operating bands with conducted requirements NTN satellite is designed to operate in the operating bands defined in Table 5.2.2-1. Table 5.2.2-1: NTN satellite bands in FR1-NTN NTN satellite operating band; Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high; Duplex mode. n256; 1980 MHz – 2010 MHz; 2170 MHz – 2200 MHz; FDD. n255; 1626.5 MHz – 1660.5 MHz; 1525 MHz – 1559 MHz; FDD. n254; 1610 – 1626.5 MHz; 2483.5 – 2500 MHz; FDD. NOTE: NTN satellite bands are numbered in descending order from n256.. - 5.2.3 Operating bands with radiated requirements NTN satellite is designed to operate in the operating bands defined in Table 5.2.3-1. Table 5.2.3-1: Satellite operating bands in FR2-NTN Satellite operating band; Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high; Duplex mode. n512(Note 1); 27500 MHz - 30000 MHz; 17300 MHz - 20200 MHz; FDD. n511(Note 2); 28350 MHz - 30000 MHz; 17300 MHz - 20200 MHz; FDD. n510(Note 3); 27500 MHz - 28350 MHz; 17300 MHz - 20200 MHz; FDD. NOTE 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the USA subject to FCC 47 CFR part 25. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202)..

Observation 2: A UE can report its supported NR NTN bands via Rel-15 capability signaling (sa-NR-r15 and supportedBandListNR-SA-r15).

Based on the above observations, we think even in Rel-17/18, there is no restriction that E-UTRAN cannot configure a frequency on NR NTN operating bands to redirect the UE to NR NTN, and the network may determine to redirect the UE to one NR NTN frequency based on the supported NR NTN bands indicated in UE capability reporting.

Observation 3: In Rel-17/18, there is no restriction in the specification that E-UTRAN shall not configure a frequency on NR NTN operating band to a UE for the redirection purpose. In this case, the Rel-17/18 UE may just perform the cell (re)selection without NR NTN satellite assistance information available.

If companies do not consider observation 3 as a problem for Rel-17/18, then the situation in Rel-19 can be understood as follows. When UE receives a dedicated NR NTN frequency for redirection purpose in RRCConnectionRelease:

- If the UE supports the Rel-19 LTE TN to NR NTN IDLE mode mobility enhancement and the eNB provides the NR NTN satellite assistance information in SIB24 and SIB33, the UE performs cell reselection/measurement according to TS36.304 by using the satellite assistance information (with how to use the assistance information up to UE implementation without need of specified UE behavior).

- If the UE does not support the Rel-19 LTE TN to NR NTN IDLE mode mobility enhancement or the eNB does not provide the NR NTN satellite assistance information in SIB24 and SIB33, the UE may perform cell reselection/measurement according to TS36.304 without NR NTN satellite assistance information, which is the similar case as in Rel-17/Rel-18.

Observation 4: If the UE supports NR NTN band(s) and it receives the dedicated NR NTN frequency in RRCConnectionRelease message, it can perform cell reselection/measurement according to TS36.304, with (based on its Rel-19 "LTE TN to NR TN IDLE" IDLE mode mobility capability) or without (similar to Rel-17/18 case) the NR NTN assistance information.

Hence, we think E-UTRAN can determine whether to configure the dedicated NR NTN frequency in RRCConnectionRelease message based on current Rel-15 UE capability signaling, and based on the intended UE behavior illustrated in Observation 4. No further spec impact is needed.

Proposal 1: RAN2 confirms the understanding that whether dedicated NR NTN frequency is configured in the RRCConnectionRelease message for redirection purpose can be based on existing UE capability (sa-NR-r15/supportedBandListNR-SA-r15) and NW implementation. No Spec impact is needed.

Conclusion

In this contribution, in the remaining issue for LTE_TN_NR_NTN_mob WI on whether dedicated NR NTN frequency can be provided in RRCConnectionRelease issue is discussed, with the following observations and proposals:

Observation 1: Since Rel-15, eNB can determine whether to configure the NR frequency for redirection or cell reselection in RRCConnectionRelease for a UE, based on the UE capabilities reported in sa-NR-r15 and supportedBandListNR-SA-r15.

Observation 2: A UE can report its supported NR NTN bands via Rel-15 capability signaling (sa-NR-r15 and supportedBandListNR-SA-r15).

Observation 3: In Rel-17/18, there is no restriction in the specification that E-UTRAN shall not configure a frequency on NR NTN operating band to a UE for the redirection purpose. In this case, the Rel-17/18 UE may just perform the cell (re)selection without NR NTN satellite assistance information available.

Observation 4: If the UE supports NR NTN band(s) and it receives the dedicated NR NTN frequency in RRCConnectionRelease message, it can perform cell reselection/measurement according to TS36.304, with (based on its Rel-19 "LTE TN to NR TN IDLE" IDLE mode mobility capability) or without (similar to Rel-17/18 case) the NR NTN assistance information.

Proposal 1: RAN2 confirms the understanding that whether dedicated NR NTN frequency is configured in the RRCConnectionRelease message for redirection purpose can be based on existing UE capability (sa-NR-r15/supportedBandListNR-SA-r15) and NW implementation. No Spec impact is needed.

Reference

- R2-2409213	Report from Break-out session on NR-NTN and IoT-NTN	Session Chair (ZTE Corporation)

- 3GPP TS 36.331: "Universal Terrestrial Radio Access (UTRAN); Radio Resource Control (RRC)"

- 3GPP TS 36.306:"Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities"

- 3GPP TS 38.101-5:"NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements"