3GPP TSG-RAN WG2 Meeting #128	R2-2409677

Orlando, United States, 18th – 22nd November, 2024

Agenda Item:	8.7.4.1

Source: 	OPPO

Title:  	Discussion on LCP enhancements for XR

Document for:	Discussion and Decision

# Introduction

In Rel-19 XR WID [1] revised in RANP#105 meeting, the objectives of scheduling enhancements are updated as follows.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This contribution would like to provide our considerations on LCP enhancements for scheduling.

# Discussion on LCP enhancements

In current LCP procedure, for an uplink grant allocating uplink resource for a new transmission, UE needs to select the LCHs that satisfy all LCH restrictions, and then the uplink resource is allocated for selected LCHs in a decreasing LCH priority order. During the phase of resources allocation, it is divided into two rounds, i.e.,

- 1st-round: Initial resources allocation, i.e., Bj-based resources allocation to achieve PBR-based fairness for all selected LCHs,

- 2nd-round: Remaining resources allocation, i.e., if any resource remains, allocating resource for all selected LCHs regardless of the number of tokens (i.e., Bj).

In RAN2#127bis meeting, RAN2 discussed on which round during LCP procedure the additional LCH priority should be applied, and the following agreements were made.

Agreements on LCP enhancements - As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. - We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

UE determines whether to apply the additional LCH priority or not when starting the 1st rould of resource allocation procedure. If additional LCH priority is determined to use for an LCH, the additional LCH priority is applied for an LCH in both 1st and 2nd rounds of resource allocation procedure in LCP as a baseline. However, it is still FFS on whether UE is allowed to change the priority for the 2nd rould to ensure fairness. Besides, a compermised solution to leave it to UE implementation is also suggested to consider.

In our understanding, the UE complexity and tight timeline of LCH procedure would be more essential issues. From UE perspective, MAC behaviours to update/change LCH priority for 2nd round may require UE to perform additional cross-layer interactions between MAC layer and PDCP layer. Too many cross-layer interactions would have significantly negative impact on the timeline of UE’s LCP procedure. Even though MAC behaviours to further update/check of LCH priority are finished within MAC layer (i.e., no need of cross-check before 2nd round), it still has impact on the timeline since UE has to identify whether there is still LCH priority-adjusted data within the LCH, which is of difficult in case that there is also non-LCH priority-adjusted data ahead within the LCH. On the contrary, thanks to BSR and enhanced DSR reporting, the fairness issue could be dealt with by the proper scheduling of networks. Therefore, we propose MAC entity should determine the priority of a LCH only once before executing the LCP procedures (i.e., before 1st round of resource allocation), and the determined priority for a LCH should be applied consistently across both rounds of resource allocation. It implies that there should be no further checks, updates, or fallback mechanisms during the 2nd round of remaining resource allocation in the LCP procedure. In this way, LCP implementation complexity from UE perspective would be simplest, and UE can avoid negative impacts on LCP timeline as much as possible.

- From UE perspective, LCP implementation would be simplest if MAC entity determines the priority of a LCH only once before executing the LCP procedures.

- Further update/check of LCH priority would have negative impact on the timeline of LCP procedure for UE.

- The determined priority for a LCH is applied consistently for both 1st and 2nd rounds of resource allocation in LCP procedure, i.e., no need to change the priority for the 2nd round.

Another issue raised by companies is the enhancement of PBR (i.e., Bj) in case of an LCH with LCH priority-adjusted data. The potential solutions include 1) set Bj to infinity, 2) temporarily raise Bj.

We understand that Bj adaptation would break the logic of token bucket algorithm in the LCP procedure. If set Bj to infinity in case of an LCH with LCH priority-adjusted data, the MAC entity shall allocate resources for all the data that is available for transmission on the LCH before meeting the PBR of the lower priority LCHs. Therefore, it would have negative impacts on fairness for both 1st and 2nd round, which is more significant than the fairness issue caused by using additional LCH priority for LCHs with LCH priority-adjusted data.

Besides, for temporarily raising Bj, it is also hard for UE to determine a proper value to raise Bj to allow all LCH priority-adjusted data in buffer for transmission on the LCH. For example, when UE receives DCI in PDCCH indicating an uplink grant, UE firstly needs to perform cross-layer interaction (e.g., at time T1) to know the presence/data volume of LCH priority-adjusted data, so that UE can determine the LCH priority and set value of Bj. And then UE’s MAC entity needs to perform multiplexing and assembly including LCP procedure (e.g., at time T2) before the actual time of PUSCH transmission scheduled by this uplink grant. The time T1 and T2 are fully up to UE implementation within the time offset K2 between PDCCH and the scheduled PUSCH transmission. So, it might be more additional LCH priority-adjusted data generated after the cross-layer interaction at time T1, but it is failed to be counted into Bj.

Therefore, we propose not to consider the enhancement of Bj in case of LCH with LCH priority-adjusted data.

- Not to consider the enhancement of Bj in case of LCH with LCH priority-adjusted data.

In RAN2#125bis meeting, the following agreement was made on SRB impact due to LCP enhancement, but no further conclusion on this.

RAN2 should consider potential impact on traffic from SRBs.

In our understanding, a proper network implementation would solve this issue easily. Networks can ensure the LCH of SRB always has higher LCH priority than the additional LCH priority used for the LCH with LCH priority-adjusted data.

- Up to network implementation to avoid SRB impact due to LCH prioritization-based LCP enhancement, i.e., configure higher priority for LCH of SRBs than LCH with LCH priority-adjusted data.

In RAN2#127 meeting, the following agreement was made on intra-UE prioritization enhancement.

FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

In legacy intra-UE prioritization, for the MAC entity configured with lch-basedPrioritization, the priority of an uplink grant is determined based on the priorities of the LCHs that are multiplexed or can be multiplexed. In detail, it is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions. Similarly, for the MAC entity configured with lch-basedPrioritization, the priority of an SR is also determined based on the priority of the LCH triggering the SR.

For LCH with LCH priority-adjusted data, whether/how additional LCH priority to be applied for intra-UE prioritization procedure or not is still FFS. There are two different understandings for the impact on intra-UE prioritization:

- The priority of logical channel in intra-UE prioritization procedure refers to legacy LCH priorities, regardless of LCH priority adjustment due to LCH priority-adjusted data,

- The priority of logical channel in intra-UE prioritization procedure refers to the current LCH priorities including the additional LCH priorities if configured, i.e., if an LCH is adjusted to additional LCH priority due to LCH priority-adjusted data, use the additional LCH priority in intra-UE prioritization procedure.

From our perspective, we tend to understanding-2. If an LCH is adjusted to additional LCH priority due to LCH priority-adjusted data, it is straightforward to use the additional LCH priority as the current LCH priority in intra-UE prioritization procedure, so that the uplink grant/SR associated to LCH priority-adjusted data could be prioritized in case of overlapping uplink grants/SRs, otherwise LCH priority-adjusted data would not be transmitted in time. Furthermore, RAN2 may further discuss whether the additional LCH priority applies for the cases: 1) the priority of uplink grant for new transmission, 2) the priority of uplink grant for retransmission, and 3) the priority of SR.

- The additional LCH priority also applies to intra-UE prioritization procedure. FFS whether it applies for the cases including 1) the priority of uplink grant for new transmission, 2) the priority of uplink grant for retransmission, and 3) the priority of SR.

Another issue needed to be started to discuss is the capability for LCP enhancement. The LCP enhancement introduced in Rel-19 XR should be an optional feature for a Rel-19 UE to support, thus needing an optional capability reported to network to keep a common understanding between UE and network. R2 can confirm the introduction of the optional capability with signalling.

- Introduce a new capability on whether the UE supports LCP enhancement for Rel-19 XR, which is an optional capability with signalling.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- From UE perspective, LCP implementation would be simplest if MAC entity determines the priority of a LCH only once before executing the LCP procedures.

- Further update/check of LCH priority would have negative impact on the timeline of LCP procedure for UE.

- The determined priority for a LCH is applied consistently for both 1st and 2nd rounds of resource allocation in LCP procedure, i.e., no need to change the priority for the 2nd round.

- Not to consider the enhancement of Bj in case of LCH with LCH priority-adjusted data.

- Up to network implementation to avoid SRB impact due to LCH prioritization-based LCP enhancement, i.e., configure higher priority for LCH of SRBs than LCH with LCH priority-adjusted data.

- The additional LCH priority also applies to intra-UE prioritization procedure. FFS whether it applies for the cases including 1) the priority of uplink grant for new transmission, 2) the priority of uplink grant for retransmission, and 3) the priority of SR.

- Introduce a new capability on whether the UE supports LCP enhancement for Rel-19 XR, which is an optional capability with signalling.

# 4. Reference

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia