3GPP TSG-RAN WG2 Meeting #128	R2-2409689

Orlando, USA, 18th – 22nd Nov, 2024

Agenda Item:	8.9.2

Source:	ZTE Corporation, Sanechips

Title:	Further consideration on S&F operation in IoT NTN

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed and in RAN#103, the WID is further refined in [1]. The main objectives related to support of Store&Forward (S&F) satellite operation are as following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In RAN2#125bis meeting, RAN2 has had some high level discussion on this topic and made the following assumptions:

RAN2 assumptions: - S&F implies that at least the full eNB will be onboard - An IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied, either via NAS or AS (wait for SA2 progress on this) - The S&F satellite operation is common for NB-IoT and eMTC. - The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture)

In RAN2#126 meeting, RAN2 has made more agreements for this objective as following:

1. For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite): - The UE sends uplink data signalling to eNB when service link is available and the eNB stores it. - When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN. - The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available). - The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again 2. S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed 3. RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

In RAN2#127 meeting, based on the latest conclusions in SA2, RAN2 focus the discussion on RAN2 impacts of full CN and spit-MME payload architecture options and achieved the following agreements:

Agreements: - RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options) - RAN2 will consider both single satellite pass and multiple satellite pass scenarios - RAN2 will consider both MO and MT data within scope - UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature) - RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected. - RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In RAN2#127B meeting, RAN2 focus the discussion about S&F operation indication and the cell bar mechanism, and achieved the following agreements:

Agreements: - The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed) - RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided. - An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed - If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure - When present, the “S&F operation” indication has two possible settings: ‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell) ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

In this paper, we will further discuss the remaining issues from last RAN2 meeting and give our proposals.

Discussion

According to 8.2 in 23.700-29[2], the conclusions in SA2 to support of Store and Forward Satellite operation is as below:

The following option is agreed for supporting Store and Forward operation with a Split MME architecture with the following (informative) principles: 1) In the Split MME architecture, HSS is on the ground. 2) MME functionality is split into two parts: MME-onboard - the MME part which is onboard the satellite and MME-ground - the MME part which is on the ground network with an interface out of scope of 3GPP. 3) The MO data is stored in the MME-onboard when the service link is available and the feeder link is unavailable, and transferred to the ground when the feeder link becomes available. The MT data is stored in the MME-ground or in S-GW when the feeder link is unavailable and transferred to the MME-onboard when the feeder link becomes available. The MT data is stored in the MME-onboard when the feeder link is available and service link is unavailable, and transferred to the UE when service link becomes available. All types of data traffic (e.g. IP, etc.) can be supported and transferred using the existing user plane and control plane procedures defined in EPS. 4) For MO SMS, upon reception of the MO SMS the MME-onboard stores the MO-SMS and may immediately send the delivery report(i.e. RP-ACK) to the UE i.e. as if the MO-SMS has already been successfully delivered to the Service Centre (SC).

It can been seen that if the satellite is operating in S&F mode, the network is still able to provide MO or MT service at least for R19 UE supporting S&F, as long as the network has store the UE context or data.

2.1 Switching timing information

There is an open issue on whether to introduce some switching timing information about S&F. we will give our discussion for the following two scenarios:

1) the satellite operating in S&F mode is going to switch to normal mode

If the satellite is operating in S&F mode, there is a risk that the access triggered by UE may be refused by the network due to the absence of UE context. For example, MME rejects Attach or TAU procedure triggered by UE as below.

2) When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes: a) A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network. b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

In order to avoid the potential failure, UE could delay to access until the satellite has switched into normal mode. For example, UE could initiates Attach or TAU procedure after the S&F mode stops. In this case, the switching timing information is beneficial for UE. Hence, we suggest to provide the timing information about S&F to UE in SIB. Moreover, the remaining time of S&F is a relative time based on the transmission occasion of the SIB carry this remaining time, it is too complex and may cause frequent SIB update. So the start time of normal mode is more straightforward and simple.

From specification perspective, for R19 UE supporting S&F, it is up to UE’ implementation to select when to access the network based on the start time of normal mode.

Proposal 1a: The start time of normal mode could be provided in SIB for the serving satellite.

Proposal 1b: For R19 UE supporting S&F, it is up to UE’s implementation to select when to access the network based on the start time of normal mode.

Moreover, if the start time of normal mode about neighbour satellite can be provided, it could assist R19 UE supporting S&F and legacy UE to select a suitable cell. For example, UE could research the neighbour satellite only after the start time of normal mode of this satellite. However, if to introduce a new start time indication (it may be later than the t-ServiceStartNeigh which corresponds to the start of coverage of neighbor satellite), considering legacy UE cannot identify this new indication, it may happen that the legacy UEs measure and reselect to a neighbour satellite but later barred due to that this satellite is still in S&F mode. In order to avoid such case, another way may be to set the t-ServiceStartNeigh in SystemInformationBlockType33 as the start time of normal mode for neighbour satellite in the continuous coverage case and this can be left to eNB’ implementation.

Proposal 2a: The t-ServiceStartNeigh in SystemInformationBlockType33 could be set as the start time of normal mode for neighbour satellite in the continuous coverage case. This can be left to NW implement.

Similarly, t-ServiceStart in SystemInformationBlockType32 can also be set to the start time of normal mode for neighbour satellite in the discontinuous coverage case. Furthermore, it can be left to UE implementation on when to start measurement for neighbour satellite according to the start time of normal mode.

Proposal 2b: The t-ServiceStart in SystemInformationBlockType32 could also be set as the start time of normal mode for neighbour satellite in the discontinuous coverage case. This can be left to NW implement.

2) the satellite operating in normal mode is going to switch to S&F mode

If the satellite is operating in normal mode, the satellite could fetch UE contexts and data from the ground. In this way, if the satellite has switches to S&F mode, according to the above conclusion from SA2, at least for R19 UE supporting S&F, the data transmission still can be performed. Hence, for R19 UE supporting S&F, UE could try to access to the network as long as the serving cell is not barred.

For legacy UE, if the Attach or TAU procedure is triggered during the satellite is operating in S&F mode, the procedure may fail. However, if the Attach or TAU procedure is triggered during the satellite is operating in normal mode, the procedure could be completed successfully. Even after the satellite has switched to S&F mode, the data could be transferred using the existing user plane and control plane procedures as long as the satellite could maintain the UE context and store data. Hence, if the satellite is operating in S&F mode, it isn’t reasonable to prohibit all the legacy UE to access the satellite via setting t-service of the serving cell as the stopping time of normal mode.

Moreover, if UE has established the connection with the satellite in normal mode, the data transmission could be resumed after the satellite has switched to S&F mode. If all the data stored in the satellite has been sent, the network could release the connection. That is, it is under the control of the network whether to resume the data transmissions. UE is not allowed to release the connection automatically based on the the stopping time of normal mode.

As above, there is no need for UE to know the stopping time of normal mode, hence, the duration or stop time of normal mode isn’t introduced.

Proposal 3: The duration or stop time of normal mode needn’t be introduced.

2.2 UP solution

If the satellite operating in normal mode is going to switch to S&F mode, the eNB could initiate a RRC connection suspend procedure to UE via RRC release message. Upon the reception of this message, UE could enter into RRC idle mode and retain the UE AS context. And then, UE could translate from RRC_IDLE into RRC_CONNECTED and resume the connection with the next eNB via a RRC connection resume procedure. During RRC connection resume procedure, a resume ID and short MAC-I provided by UE are transferred to the eNB on purpose of authentication. Only the eNB that has stored the UE context can resume the connection successfully. Hence, UE is expected to resume the connection with the satellite that has fetched the UE context.

In order to support that, some candidate satellite ID could be supplied to R19 UE supporting S&F via RRC release message. For example, if the serving satellite is going to switch to S&F mode, eNB could indicate the next satellite in normal mode. For UE, the satellite information in SIB 33 could be related with satellite ID, these satellites could be prioritized for UE to resume the connections.

Proposal 4: Some candidate satellite ID that is prioritized for R19 UE supporting S&F to resume the connections could be supplied via RRC release message.

Conclusion

Based on the above discussion, the following proposals are given:

Proposal 1a: The start time of normal mode could be provided in SIB for the serving satellite.

Proposal 1b: For R19 UE supporting S&F, it is up to UE’s implementation to select when to access the network based on the start time of normal mode.

Proposal 2a: The t-ServiceStartNeigh in SystemInformationBlockType33 could be set as the start time of normal mode for neighbour satellite in the continuous coverage case. This can be left to NW implement.

Proposal 2b: The t-ServiceStart in SystemInformationBlockType32 could also be set as the start time of normal mode for neighbour satellite in the discontinuous coverage case. This can be left to NW implement.

Proposal 3: The duration or stop time of normal mode needn’t be introduced.

Proposal 4: Some candidate satellite ID that is prioritized for R19 UE supporting S&F to resume the connections could be supplied via RRC release message.

Reference

[1] RP-240776 Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things MediaTek Inc., RAN#103, March, 2024

[2] 3GPP TR 23.700-29 V19.0.0, Study on integration of satellite components in the 5G architecture; Phase 3 (Release 19), 2024-06