3GPP TSG-RAN WG2 Meeting #128                                        R2-2409690

Orlando, USA, 18th – 22nd November, 2024

Source:	ZTE Corporation, Sanechips

Title:	Further consideration on UL capacity enhancements in IoT NTN

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed and in RAN#105, the WID is further refined in [1]. The main objectives related to support of capacity enhancements for uplink are as following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In previous RAN2 meetings, RAN2 has discussed some general aspects for this objective and the feasibility of contention-based Msg3 transmission, and also gave comparison among different anti-collision schemes. The related RAN2 agreements are as below:

RAN2#125bis agreements: - Both NB-IoT and eMTC are within scope of uplink capacity enhancements - Both C-plane and U-plane solutions are within scope of uplink capacity enhancements. - Only CIoT EPS is within scope of uplink capacity enhancements RAN2#126 agreements: - RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). - RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) - If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3. RAN2#127 agreements: - RAN2 will continue working on a CB-msg3 EDT-like mechanism. - RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts - In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1. - For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition RAN2#127b agreements: - RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19. - RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed. - At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details. - CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA) - RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA) - At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission - The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA) - CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

In this document, we will firstly give analysis on the main issues of Msg3 and Msg4 transmission, and further give analysis on some specific issues for supporting DSA schemes. Based on the analysis, some proposals are given.

Discussion

The resources configuration for contention-based Msg3 transmission

Considering the resource configured for EDT transmission is a shared resource, it can be taken as reference when we discuss the shared resources configuration for contention-based Msg3 transmission (Hereafter referred to as CB-msg3 EDT). For example, the NPRACH configuration for EDT is as below:

NPRACH-ConfigSIB-NB information elements

-- ASN1START

NPRACH-ConfigSIB-NB-r13 ::=			SEQUENCE {

nprach-CP-Length-r13				ENUMERATED {us66dot7, us266dot7},

rsrp-ThresholdsPrachInfoList-r13	RSRP-ThresholdsNPRACH-InfoList-NB-r13	OPTIONAL,	-- Need OR

nprach-ParametersList-r13		NPRACH-ParametersList-NB-r13

}

NPRACH-ConfigSIB-NB-v1330 ::=		SEQUENCE {

nprach-ParametersList-v1330			NPRACH-ParametersList-NB-v1330

}

NPRACH-ConfigSIB-NB-v1450 ::=		SEQUENCE {

maxNumPreambleAttemptCE-r14			ENUMERATED {n3, n4, n5, n6, n7, n8, n10, spare1}

}

NPRACH-ConfigSIB-NB-v1530 ::=		SEQUENCE {

tdd-Parameters-r15					SEQUENCE {

nprach-PreambleFormat-r15			ENUMERATED {

fmt0, fmt1, fmt2, fmt0-a, fmt1-a},

dummy								ENUMERATED {

n1, n2, n4, n8, n16, n32, n64, n128,

n256, n512, n1024},

nprach-ParametersListTDD-r15		NPRACH-ParametersListTDD-NB-r15

}	OPTIONAL,		-- Cond TDD

fmt2-Parameters-r15					SEQUENCE {

nprach-ParametersListFmt2-r15		NPRACH-ParametersListFmt2-NB-r15 OPTIONAL,	-- Need OR

nprach-ParametersListFmt2EDT-r15	NPRACH-ParametersListFmt2-NB-r15 OPTIONAL	-- Cond EDT2

}	OPTIONAL,		-- Need OR

edt-Parameters-r15					SEQUENCE {

edt-SmallTBS-Subset-r15				ENUMERATED {true}				OPTIONAL,	-- Need OR

edt-TBS-InfoList-r15				EDT-TBS-InfoList-NB-r15,

nprach-ParametersListEDT-r15		NPRACH-ParametersList-NB-r14	OPTIONAL	-- Need OR

}	OPTIONAL		-- Cond EDT1

}

//skip

NPRACH-ParametersList-NB-r14 ::=		SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF

NPRACH-Parameters-NB-r14

NPRACH-Parameters-NB-r14 ::=			SEQUENCE {

nprach-Parameters-r14					SEQUENCE {

nprach-Periodicity-r14					ENUMERATED {ms40, ms80, ms160, ms240,

ms320, ms640, ms1280, ms2560}

OPTIONAL,	-- NEED OP

nprach-StartTime-r14					ENUMERATED {ms8, ms16, ms32, ms64,

ms128, ms256, ms512, ms1024}

OPTIONAL,	-- NEED OP

nprach-SubcarrierOffset-r14				ENUMERATED {n0, n12, n24, n36, n2, n18, n34, spare1}

OPTIONAL,	-- NEED OP

nprach-NumSubcarriers-r14				ENUMERATED {n12, n24, n36, n48}

OPTIONAL,	-- NEED OP

nprach-SubcarrierMSG3-RangeStart-r14	ENUMERATED {zero, oneThird, twoThird, one}

OPTIONAL,	-- NEED OP

npdcch-NumRepetitions-RA-r14			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1}

OPTIONAL,	-- NEED OP

npdcch-StartSF-CSS-RA-r14				ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64}

OPTIONAL,	-- NEED OP

npdcch-Offset-RA-r14					ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

OPTIONAL,	-- NEED OP

nprach-NumCBRA-StartSubcarriers-r14		ENUMERATED {n8, n10, n11, n12, n20, n22, n23, n24,

n32, n34, n35, n36, n40, n44, n46, n48}

OPTIONAL,	-- NEED OP

npdcch-CarrierIndex-r14					INTEGER (1..maxNonAnchorCarriers-NB-r14)

OPTIONAL,	-- Need OP

...

}	OPTIONAL	-- Need OR

}

//skip

}

For time domain resource, the start time and periodicity are used to represent the resource occasion:

- As above, the periodicity of NPRACH for EDT is {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560}. Considering that the conditions for using CB-msg3 EDT feature will be stricter than those for using EDT, we can reasonably assume that the number of UEs using CB-msg3 EDT will generally be less than the number of UEs using the EDT, thus, it can be assumed that the periodicity in the time domain resources for CB-msg3 EDT is sparer than NPRACH for EDT. Hence, the periodicity of the resource for CB-msg3 EDT could be in the unit of tens of, or hundreds of, even thousands of milliseconds. So the radio frame, e.g., SFN (e.g., 10~10240 ms) as unit may be enough for the definition of periodicity. Meanwhile, subframe as unit of periodicity means more dense time resource, and seems not so necessity. And H-SFN (i.e., larger than 10.24s) as unit of periodicity may leading to increased latency, also isn’t suggested. In final, only SFN is suggested to be used as unit for the periodicity of CB-msg3 EDT. Furthermore, based on the considerations of feasibility and simplifying the complexity, we think such definition way for the periodicity of the shared resources for CB-msg3 EDT can be applicable to both eMTC NTN and NB-IoT NTN.

- For start time, the start subframe and start SFN are generally needed to represent a specific time occasion within a periodicity.

For frequency domain resource, multiple occasions could create more transmission opportunities and reduce conflicts, thus, multiple PRB/subcarrier resource could be considered:

- For eMTC, the start PRB index and PRB number could represent a frequency domain resources pool for CB-msg3 EDT.

- For NB-IoT, a number of resource units are used to allocate a UL resource in a carrier and each of the resource unit consists of a set of contiguously allocated subcarriers (see below definition in RAN1 spec for reference). As a result, carrier index, a list of subcarrier set index, and RU number for NB-IoT could be used to represent a frequency domain resources pool for CB-msg3 EDT:

TS 36.213 16.5.1.1 Resource allocation The resource allocation information in uplink DCI format N0 for NPUSCH transmission or configured by higher layers for NPUSCH transmission using preconfigured uplink resource indicates to a scheduled UE - a set of contiguously allocated subcarriers () of a resource unit determined by the Subcarrier indication field, or by the higher layer parameter npusch-SubCarrierSetIndex in PUR-Config-NB - a number of resource units () determined by the resource assignment field according to Table 16.5.1.1-2, or by the higher layer parameter npusch-NumRUsIndex in PUR-Config-NB - a repetition number () determined by the repetition number field according to Table 16.5.1.1-3, and for a NPUSCH transmission using preconfigured uplink resource, the UE shall use the repetition number configured by higher layers; except for NPUSCH with 16QAM where NRep=1.

For code domain resource, according to companies’ simulations in last meeting (e.g., in [2]), it can be seen that OCC-based SA scheme can achieve the best performance for some cases. Therefore, RAN2 have made a general assumption that OCC (orthogonal cover codes) sequence can be further applied to (N)PUSCH (Msg3) transmission to reduce the collision caused by using shared time/frequency domain resources and facilitate more UEs multiplexing on a same set time/frequency domain resources. So from RAN2 perspective, the OCC sequences will be a new resource type which needs to be introduced in the shared (N)PUSCH resources configuration for CB-msg3 EDT transmission.

For CE level aspect, RAN2 has agreed that the (N)PUSCH resources for CB-msg3 EDT transmission are provided per CE level in last meeting. Thus, a repetition number needs to be configured and taken as the baseline dimension for configuring other time domain/frequency domain/code domain resources. Furthermore, it needs to be noted that, in legacy system, for a certain CE level, the repetition number for (N)PRACH and (N)PUSCH can be different. So the repetition number configuration of PUR is more suitable as a reference for the relevant configuration for CB-msg3 EDT.

Besides, (N)PDCCH resource configuration is also needed for UE to monitor the response for Msg3.

Based on the discussion above, the proposal about the necessary contents for shared resources configuration for CB-msg3 EDT transmission is given as below:

Proposal 1a: RAN2 can discuss the following resources types for shared resources configuration for CB-msg3 EDT:

- Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN)

- Frequency domain resources for (N)PUSCH occasions: PRB index and PRB number for eMTC; Carrier index, a list of subcarrier set index and RU number for NB-IoT

- OCC resources

- repetition number

- (N)PDCCH resource

Proposal 1b: The periodicity of the time domain resources for (N)PUSCH occasions could be with the unit of SFN, the detailed value range can be FFS.

Efficient delivery of Msg4

In this section, we will discuss RAN2 aspects for supporting efficient delivery of Msg4 which corresponds to the CB-msg3 EDT transmission.

Contention resolution window

Taking legacy contention-based RA procedure or contention-based EDT procedure as reference, in order to guarantee the reception of Msg4, a contention resolution window is necessary.

Per our understanding, CB-msg3 EDT will use similar Msg3 and Msg4 transmission (without Msg1 and Msg2) as that in PUR and the main difference is that CB-msg3 EDT needs an additional contention procedure, so we assume the definition of Msg3 response window in PUR can be referred when defining the contention resolution window for CB-msg3 EDT.

The start time of the Msg3 response window in PUR is determined based on some factors, e.g., the RTT, the processing time of Msg3. According to the following references from RAN1 and RAN2, after Msg3 transmission, UE needs to monitor PDCCH after an interval of 3 subframes. Hence, the contention resolution window of CB-msg3 EDT can be also started at the subframe of Msg3 transmission plus 4 subframes plus UE-eNB RTT.

TS 36.213: - If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe start from subframe n'+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n'+1 to subframe n+Kmac+3. TS 36.321: After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer: - if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes plus UE-eNB RTT. - else: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes.

Proposal 2a: For CB-msg3 EDT procedure, a contention resolution window needs to be introduced, which starts at the subframe of Msg3 transmission plus 4 subframes plus UE-eNB RTT.

For the length of contention resolution window, we assume the length of Msg3 response window in PUR can be referred. The value of pur-ResponseWindowTimer are as below and seems long enough:

| for eMTC | pur-ResponseWindowTimer-r16 ENUMERATED {sf240, sf480, sf960, sf1920, sf3840, sf5760, sf7680, sf10240} |
| --- | --- |
| For NB-IoT | pur-ResponseWindowTimer-r16 ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64} |

Hence, we suggest the length of contention resolution window CB-msg3 EDT could reuse that of the pur-ResponseWindowTimer.

Proposal 2b: The same value range as pur-ResponseWindowTimer can be used to configure the length of contention resolution window for CB-msg3 EDT procedure.

RNTI calculation

In last meeting, RAN2 has agreed that RNTI for scheduling Msg4 transmission is derived based on the resource associated to the (N)PUSCH occasion used for CB-msg3 EDT transmission.

According to our analysis in 2.1, the resource associated to the PUSCH occasion includes the time domain resource, frequency domain resource and OCC resource. In below, we will discuss one by one whether and how these resource information should be used in the RNTI calculation:

- Regarding the time domain resource, the calculation of RNTI has a close relationship with the length of contention resolution window. If UEs select different transmission occasions in time domain but use the same RNTI, and monitor the same RNTI within the overlapped contention resolution window, they cannot correctly distinguish the Msg4 via RNTI. Therefore, it should ensure that, within the length of a contention resolution window, for two UEs who select two transmission occasions in different periodicity, RNTI calculated by the UEs should be different. According to our Proposal 1b, due to the periodicity will be with the unit of tens of or hundreds of milliseconds, the subframe will keep unchanged in different periodicities and just SFN is used to distinguish the transmission occasion. Hence, SFN index should be used as a factor in the formula of RNTI, meanwhile, subframe index is not needed.

  - Besides, according to some previous discussion for RA-RNTI, to avoid a too large RNTI, it can be considered to compress the part where SFN index is involved. On one hand, since resources are configured according to periodicity, not all SFN indices will be used; that is, only SFN indices at certain intervals will be considered valid resource positions during resource selection. Hence, SFN index can be divided by a minimum periodicity in the formula. On the other hand, since each UE only monitors the DL for a limited time period, e.g., during the contention resolution window, to receive Msg4, the RNTIs of two UEs need only be different during this period. Hence, it also can be considered to modulo the SFN index by the length of contention resolution window.

- Regarding the frequency resource, according to our Proposal 1a, for eMTC, a UL resource allocation includes the start PRB index, PRB number. As long as UEs select the different start PRB index, Msg3 from UEs will not conflict. Hence, the start PRB index could be used as a parameter in the formula of RNTI. Meanwhile, for NB-IoT, a UL resource allocation includes subcarrier set index, RU number and carrier index. As long as UEs select the different subcarrier set and carrier, Msg3 from UEs will not conflict. Hence, the subcarrier set index and carrier index could also be considered in the formula of RNTI.

- Regarding the code domain resource, the OCC index could be used to differentiate the resources in code domain. In order to further differentiate UEs to avoid conflicts, the OCC index could also be considered in the formula of RNTI.

Based on the above discussion, the following proposals are given for RNTI calculation:

Proposal 3a: The following parameters can be taken into account in the RNTI calculation for CB-msg3 EDT:

- Time domain resource: SFN index, no need of subframe index

- Frequency domain resource: PRB index for eMTC; subcarrier set index and carrier index for NB-IoT

- Code domain resource: OCC index

Proposal 3b: For the RNTI calculation for CB-msg3 EDT, to avoid a too large RNTI, it can be considered to compress the part where SFN index is involved, for example, the SFN index can be divided by the minimum periodicity, or modulo by the length of contention resolution window.

Contention resolution identity

Similar as that in legacy contention-based procedures, the collision caused by using shared resources for Msg3 transmission cannot be completely resolved by RNTI design. The additional contention resolution scheme is still needed.

The straightforward way is to reuse the contention resolution scheme in legacy random access procedure. That is, after recpetion of Msg3, eNB can send a DL MAC PDU in which a Contention Resolution Identity is included. For UE side, it needs to check whether the UE Contention Resolution Identity included in the DL MAC PDU matches the 48 first bits of the CCCH SDU transmitted in Msg3.

Proposal 4: The contention resolution scheme in legacy random access procedure can be reused for CB-msg3 EDT procedure, e.g., a UE Contention Resoution Identity can be included in the DL MAC PDU and UE checks whether the UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in Msg3.

Backoff/Fallback schemes

Due to skipping Msg1/Msg2 transmission, the CB-msg3 EDT is a kind of challenging technology. If the channel quality is not stable enough, the Msg3/Msg4 transmission may fails. In UE side, such failure in RRC layer would be informed to NAS and NAS may trigger another access after a while. In order to avoid triggering higher layers to perform retransmissions which may increase the delay, it can be considered to prioritize remediation at the AS layer.

Generally, two schemes for improving the performance robustness in AS layer, e.g., Backoff and Fallback, can be considered.

Regarding Backoff, it mainly refers to, upon failure occurrence, retriggering the same procedure after a certain time interval. Therefore, for CB-msg3 EDT, when the collision or congestion is detected by eNB, the eNB can provide kind of backoff indication via the Msg4 (or DL MAC CE before Msg4) to require backoff. Via backoff scheme:

- On one hand, the UE can have additional opportunity to retransmit another CB-msg3 EDT, e.g, direct Msg3/Msg4 transmission without Msg1/Msg2, rather than immediately falling back to the 4-step access procedure. This is friendly and reasonable for the UE, as the previous failure was likely due to the selected resources by this UE is conflicted with another UE's choice. If given another chance to select resources, it would increase the likelihood of successful transmission.

- On the other hand, even UE is provided with another transmitting opportunity, UE still needs to delay a backoff time, then can choose another time domain resources within the shared resources for Msg3 transmission. This is also be helpful to avoid collisions possibly caused by another direct Msg3/Msg4 transmission. Here we think the range of backoff time given/configured by the network would be important as network can suitably provide this based on the real collision situation.

- Furthermore, for the OCC-based SA scheme, with combined with Backoff scheme, it may have similar benefit as DSA that is brought by more transmission opportunities. But OCC-based SA + Backoff may be better, as in DSA, multiple replicas are always needed. Meanwhile, for OCC-based SA + Backoff, the additional transmission only occurs (may be seldom) when the previous transmission fails. So OCC-based SA + Backoff scheme can achieve a better balance between more transmission opportunities and reduced resource occupation.

Observation 1: OCC-based SA can be combined with Backoff scheme and it may have similar benefit as DSA or may be better than it as the additional transmission is also allowed but only occurs (may be seldom) when the previous transmission fails.

Proposal 5a: RAN2 is suggested to support backoff scheme for CB-msg3 EDT (e.g., when CB-msg3 EDT fails, after a backoff time, UE can choose an occasion to perform another Msg3 transmission), FFS details.

Regarding Fallback, the straightforward way is to fall back to legacy procedures with Msg1 and Msg2 transmission when direct Msg3/Msg4 transmission failure occurs.

Proposal 5b: The fallback mechanism can also be considered for CB-msg3 EDT (e.g., fallback to EDT or legacy RACH when CB-msg3 EDT fails), FFS details.

DSA

General procedure

In this section, we will give our analysis on the additional RAN2 impacts for supporting DSA for CB-msg3 EDT transmission.

In last meeting, DSA has been agreed to be introduced. In DSA, UE can select randomly multiple transmission occasions to transmit multiple replicas for one packet. For eMTC and NB-IoT UE, it couldn’t support to transmit multiple (N)PUSCH in different frequency domain locations simultaneously. Hence, for DSA, we will not consider the case that multiple replicas are transmitted on same time domain resources but different frequency domain resources. We only consider the case that UE selects randomly different transmission occasions in time domain for multiple replicas, here in each transmission occasion, it’s still possible for UE to select randomly a frequency domain resource.

After the UE selects multiple time domain transmission occasions and starts to transmit the replicas of Msg3, it’s possible that the response of the previous replica is received before the time that is selected for transmitting the next replica. In this case, the next replica needs to be canceled. As showed in the Figure 1, for DSA with 2 replicas, at beginning, replica 1 is transmitted in SFN 4 and replica 2 is to be transmitted in SFN 16. Assuming that the RTT= 40ms, if the response for replica 1 is received in SFN 12, sending replicas 2 is meaningless. Therefore, a reasonable assumption should be that UE can only select time domain resources for multiple replicas within a certain range. Most importantly, for NB-IoT UE, due to the limited capability, it may take more time to switch to DL reception from UL transmission. Hence, NB-IoT UE needs to transmit all replicas before monitoring the downlink. Furthermore, for the purpose of reducing latency, it also should not allow the transmission intervals between multiple replicas to be too far apart. Hence, a replica selection window for selecting transmission occasions needs to be introduced.

Figure 1

Proposal 6: For DSA, a replica selection window for selecting time domain transmission occasions needs to be introduced. UE could select randomly multiple transmission occasions within this replica select window for multiple replicas transmission. And in each time domain transmission occasions, UE could select randomly a frequency domain resource.

In DSA, UE transmits multiple replicas to eNB. eNB may receives one or more replicas from the UE. Generally, the eNB has no idea about whether a received replica is the first replica, or the subsequent one or the last one. Therefore, with intention to reduce access latency, eNB can respond the received replica immediately without waiting for other remaining replicas. Correspondingly, UE also needs to start monitoring the possible Msg4 after sending the first replica, not after sending all the replicas.

Proposal 7a: For DSA, once the eNB receives any one of the multiple replicas, it can respond it immediately without waiting for the remaining replica(s).

Proposal 7b: For DSA, UE needs to start monitoring the possible Msg4 after sending the first replica.

From UE perspective, if multiple replicas are received successfully by network, it’s expected for network not to respond Msg4 for multiple times, e.g., for each replica. But this mainly relys on eNB’ implementation, e.g., eNB can store the received replica and determine the duplicated replica received later by some internal comparison. And it’s expected that after eNB determines that all the replicas have been transmitted, the stored replicas can be discarded. Per our understanding, the knowledge of replica selection window could give assistance to the eNB to determine whether a replica is duplicated replica and when all the replicas are transmitted. Therefore, it’s suggested to let eNB configure explicitly the replica selection window for the UE.

Proposal 8a: For DSA, if eNB receives remaining replica, whether or not to respond it can be left to eNB’s implementation. It’s expected for network not to respond Msg4 for duplicated replicas.

Proposal 8b: The knowledge of replica selection window could give assistance to the eNB to determine whether a replica is duplicated replica, so it’s suggested that the replica selection window can be configured by eNB.

Specific configurations for DSA

According to the simulation results presented by various companies in previous meetings, it has been observed that DSA could reduce the collision probability among UEs, and thus show better performance than SA in term of lower PLR. However, under the case that repetition is configured, if more consumed occasions brings more collision in DSA, the performance of DSA may deteriorate. On one hand, if the small number of repetition transmission for CB-msg3 EDT is configured, e.g., n2 for CE level 1, the more transmission occasion in DSA may not cause a significant increase in conflicts. On the other hand, if the larger number of repetitions transmission is configured, e.g., n128 for CE level 3, DSA will inevitably increase conflicts and lead to performance degradation. Hence, it is reasonable to enable or disable the DSA per CE level, e.g., according to the required repetition number, and this can be configured by the network.

Proposal 9a: The DSA could be enabled/disabled by eNB per CE level.

According to the simulation and discussion in previous meetings, a reasonable number of replicas can not only make full use of resources, but also reduce conflicts. So a configurable replica numbers could help to achieve tradeoff between the resource usage and probability of collision. Besides, according to the simulation from companies, DSA with 2 replicas shows lower PLF under most cases. Hence, DSA with 2 replicas is suggested as a default value.

Proposal 9b: For DSA, the number of replicas (e.g., 2 by default) can be configured by eNB.

According to the above Proposal 8b, it’s suggested that the replica selection window can be configured by eNB. Moreover, from UE perspective, the following alternatives can be considered to apply the replica selection window:

Figure 2

- A sliding replica selection window (Alt1): The start time of a sliding replica selection window is determined by the first selected transmission occasion, so it’s flexible. As showed in above figure 2, if SFN 4 is selected for first transmission occasion, replica selection window 1 starts from SFN 4. The disadvantage of this solution is that no matter which replica eNB receives, eNB needs to wait for a certain length of replica selection window before discarding it.

- A fix replica selection window (Alt2): the window is defined as an integer multiple of the configured periodicity of the resources for CB-msg3 EDT, its start time is determined by the configured start time of transmission occasion, so it’s fixed. As showed in above figure 2, selection window 2 starts from SFN 1. The disadvantage of this solution is that the collision probability of the transmission occasion at the end of the window may be high.

Since both alternatives have their advantages and disadvantages, RAN2 is suggested to discuss all of them.

Proposal 9c: In DSA, RAN2 is suggested to discuss the following alternatives for applying the replica selection window:

- Alt1: a sliding replica selection window, its start time is determined by the first selected transmission occasion.

- Alt2: a fix replica selection window, its start time is determined by the configured start time of transmission occasion.

RNTI calculation

In DSA, UE could transmit multiple replicas on multiple time transmission occasions. It is very likely that eNB can receive only one of them and respond for it. eNB can’t know the other transmission occasions. Hence, the RNTI is derived only based on the transmission occasion for the corresponding replica transmission. For example, in below figure 3, for DSA with 2 replicas, if eNB receives replica 1 and response Msg4, the RNTI 1 calculated based on SFN 4 is used for scrambling Msg4. if eNB receives replica 2 and response Msg4, the RNTI 2 calculated based on SFN 10 is used for scrambling Msg4.

Proposal 10: For DSA, the RNTI used to schedule Msg4 transmission is derived only based on the resource associated to the transmission occasion used for the corresponding replica.

Contention resolution window

For DSA, as mentioned in Proposal 7a, when UE transmits multiple replicas, eNB may respond Msg4 for any one of them, hence, UE has to monitor any possible Msg4.

The following alternatives can be considered for UE to apply the contention resolution window:

- Multiple contention resolution windows (Alt1): Each contention resolution window corresponds to a transmitted replica. It is started at the transmission occasion of one replica plus 4 subframes plus UE-eNB RTT. Within each window, UE monitors a Msg4 addressed by the RNTI derived based on transmission occasion of the corresponding transmitted replica. For example, in figure 3, for DSA with 2 replica, during window 1, UE monitor RNTI 1 derived based on SFN 4, and during window 2, UE monitor RNTI 2 derived based on SFN 10.

- One contention resolution window (Alt2): It is started at the transmission occasion of first transmitted replica plus 4 subframes plus UE-eNB RTT. Within this window, UE monitors all the Msg4 addressed by each RNTI derived based on transmission occasion of each transmitted replica. For example, in figure 3, for DSA with 2 replica, during window 3, UE monitor both RNTI 1 derived based on SFN 4 and RNTI 2 derived based on SFN 10.

Figure 3

From specification perspective, Alt 1 could clearly specify UE behavior, but introduce complexity. Hence, we suggest RAN2 to discuss both of them.

Proposal 11: For DSA, RAN2 is suggested to discuss the following alternatives for applying contention resolution window:

- Alt1: multiple contention resolution window: within each window, UE monitors a RNTI which is derived based on transmission occasion of a replica.

- Alt2: one contention resolution window: within this window, UE monitors all RNTIs, one of which is derived based on transmission occasion of a replica.

Conclusion

Based on the above discussion, the following proposals are given:

Proposal 1a: RAN2 can discuss the following resources types for shared resources configuration for CB-msg3 EDT:

- Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN)

- Frequency domain resources for (N)PUSCH occasions: PRB index and PRB number for eMTC; Carrier index, a list of subcarrier set index and RU number for NB-IoT

- OCC resources

- repetition number

- (N)PDCCH resource

Proposal 1b: The periodicity of the time domain resources for (N)PUSCH occasions could be with the unit of SFN, the detailed value range can be FFS.

Proposal 2a: For CB-msg3 EDT procedure, a contention resolution window needs to be introduced, which starts at the subframe of Msg3 transmission plus 4 subframes plus UE-eNB RTT.

Proposal 2b: The same value range as pur-ResponseWindowTimer can be used to configure the length of contention resolution window for CB-msg3 EDT procedure.

Proposal 3a: The following parameters can be taken into account in the RNTI calculation for CB-msg3 EDT:

- Time domain resource: SFN index, no need of subframe index

- Frequency domain resource: PRB index for eMTC; subcarrier set index and carrier index for NB-IoT

- Code domain resource: OCC index

Proposal 3b: For the RNTI calculation for CB-msg3 EDT, to avoid a too large RNTI, it can be considered to compress the part where SFN index is involved, for example, the SFN index can be divided by the minimum periodicity, or modulo by the length of contention resolution window.

Proposal 4: The contention resolution scheme in legacy random access procedure can be reused for CB-msg3 EDT procedure, e.g., a UE Contention Resoution Identity can be included in the DL MAC PDU and UE checks whether the UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in Msg3.

Proposal 5a: RAN2 is suggested to support backoff scheme for CB-msg3 EDT (e.g., when CB-msg3 EDT fails, after a backoff time, UE can choose an occasion to perform another Msg3 transmission), FFS details.

Proposal 5b: The fallback mechanism can also be considered for CB-msg3 EDT (e.g., fallback to EDT or legacy RACH when CB-msg3 EDT fails), FFS details.

Proposal 6: For DSA, a replica selection window for selecting time domain transmission occasions needs to be introduced. UE could select randomly multiple transmission occasions within this replica select window for multiple replicas transmission. And in each time domain transmission occasions, UE could select randomly a frequency domain resource.

Proposal 7a: For DSA, once the eNB receives any one of the multiple replicas, it can respond it immediately without waiting for the remaining replica(s).

Proposal 7b: For DSA, UE needs to start monitoring the possible Msg4 after sending the first replica.

Proposal 8a: For DSA, if eNB receives remaining replica, whether or not to respond it can be left to eNB’s implementation. It’s expected for network not to respond Msg4 for duplicated replicas.

Proposal 8b: The knowledge of replica selection window could give assistance to the eNB to determine whether a replica is duplicated replica, so it’s suggested that the replica selection window can be configured by eNB.

Proposal 9a: The DSA could be enabled/disabled by eNB per CE level.

Proposal 9b: For DSA, the number of replicas (e.g., 2 by default) can be configured by eNB.

Proposal 9c: In DSA, RAN2 is suggested to discuss the following alternatives for applying the replica selection window:

- Alt1: a sliding replica selection window, its start time is determined by the first selected transmission occasion.

- Alt2: a fix replica selection window, its start time is determined by the configured start time of transmission occasion.

Proposal 10: For DSA, the RNTI used to schedule Msg4 transmission is derived only based on the resource associated to the transmission occasion used for the corresponding replica.

Proposal 11: For DSA, RAN2 is suggested to discuss the following alternatives for applying contention resolution window:

- Alt1: multiple contention resolution window: within each window, UE monitors a RNTI which is derived based on transmission occasion of a replica.

- Alt2: one contention resolution window: within this window, UE monitors all RNTIs, one of which is derived based on transmission occasion of a replica.

Reference

- RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc., RAN#105, September, 2024

- R2-2408334, Comparison of solutions for UL capacity enhancements in IoT NTN, ZTE, RAN2#127b, October, 2024

[3] R2-2408547, Repetitions and Delay Considerations about SA and DSA, ESA, Eutelsat Group, Viasat, Inmarsat, Novamint, Echostar, Sateliot, Toyota ITC, RAN2#127b, October, 2024

[4] R2-2409181, UL capacity enhancements for IoT NTN, Ericsson, RAN2#127b, October, 2024