3GPP TSG-RAN WG2 Meeting #128                                                                    R2-2409721

Orlando, USA, Nov.  18th – 22nd, 2024

Source:	CATT

Title:	Discussion on RRC and MAC Impacts for Asymmetric DL sTRP/UL mTRP

Agenda Item:	8.12.2

Document for:	Discussion and Decision

- 1. Introduction

For Rel-19 MIMO, according to the WID as shown below [1], objectives relevant to asymmetric DL sTRP/UL mTRP deployment are identified to have significant RAN2 impacts in terms of RRC configuration and MAC procedure as following.

RAN1: - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above.

In this contribution, we will discuss the detail impacts on RRC and MAC for asymmetric DL sTRP/UL mTRP deployment.

- 2. Discussion on RRC impacts

During RAN1#118bis meeting, RAN1 agreed the lower limit of the value range of PL offset is extended to -12 dB as following.

Agreement The lower limit of the value range of PL offset is extended to -12 dB.

The corresponding RRC parameter plOffset can be updated as following.

| NR_MIMO_Ph5 | TCI-State and TCI-UL-State | plOffset | New | This parameter is used to inform pathloss offset value applied to the joint or UL TCI state. | [-12, 60] dB, with 4dB step size |
| --- | --- | --- | --- | --- | --- |

Thus, a new RRC parameter is needed to configure the pathloss offset for the joint or UL TCI state. The value of pathloss offset is from -12dB to 60dB with 4dB step size.

Proposal 1: A new RRC parameter plOffset is introduced to configure the pathloss offset for the joint or UL TCI state. The values of plOffset are defined in an ENUMERATED structure from -12dB to 60dB with 4dB step size.

In RAN1#118bis, RAN1 agreed to introduce a new RRC parameter to configure the presence of 1-bit DCI field, which is used to indicate PL offset for PDCCH-order PRACH. The detail agreement is as following.

Agreement For indicating PL offset for PDCCH-order PRACH, introduce a new 1-bit DCI field in DCI format 1_0: - This DCI field exists when the corresponding RRC parameter (which is a new RRC used to configure the presence of this 1-bit DCI field) is enabled and at least one TCI state is configured with PL offset. - When one joint/UL TCI state is indicated in Rel-17 unified TCI, - the bit field index 0 of this field indicates that PL offset is not included in the PRACH transmission power calculation - the bit field index 1 of this field indicates that the PL offset associated with the indicated TCI state is included in the PRACH transmission power. - FFS: Whether the bit field can be used to indicate other information - FFS: When two joint/UL TCI states are indicated in Rel-18 unified TCI

Proposal 2: A new RRC parameter is introduced to configure the presence of 1-bit DCI field, which is used to indicate PL offset for PDCCH-order PRACH.

During RAN1#118bis meeting, RAN1 discussed the extended range of starting bit position in DCI format 2_3 and update the value X in the following agreement.

Agreement About the extended value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19, support Alt1: - Alt1: X = 45 (to be captured in RAN2 spec) - This feature is a separate UE capability and is applicable to any Rel-19 UE who supports this UE capability, regardless this UE supports two separate SRS CLPC adjustment states or not. - Note: X=45 can be used for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise

Thus, the corresponding RRC parameter is updated as following.

| NR_MIMO_Ph5 | SRS-TPC-CommandConfig | startingBitOfFormat2-3 | Existing | The value range of this parameter is extended from 1~31 to 1~X45（FR1 in shared spectrum or FR2-2）and 1~43 (otherwise). FFS: the value of X>31. | INTEGER (1..X) X=45（FR1 in shared spectrum or FR2-2）and X=43 (otherwise). FFS: X>31 |
| --- | --- | --- | --- | --- | --- |

Therefore, the existing RRC parameter startingBitOfFormat2-3 can be extended to a CHOICE structure. The value of startingBitOfFormat2-3 can be choice of INTEGER (1..45) for FR1 in shared spectrum or FR2-2, and INTEGER (1..43) for otherwise.

Proposal 3: The RRC parameter startingBitOfFormat2-3-r19 can be defined with the values of INTEGER (1..45) with the field description that INTEGER (1..45) is configured for FR1 in shared spectrum or FR2-2, and INTEGER (1..43) is configured for otherwise.

- 3. Discussion on MAC impacts

In RAN1#118 meeting, RAN1 discussed the issue of PL offset configuration and update, and sent the LS [2] to RAN2 as follows:

Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). “MAC-CE impact: MAC-CE updates the PL offset value(s) configured in one or more joint TCI states or UL TCI states”

In the following, we will analyze the MAC CE impact for PL offset updates.

When to update the PL offset and what channels are applicable to

Based on RAN1 agreement, PL offset associated with joint or UL TCI state(s) can be configured by RRC and updated by MAC CE. In current MAC Spec, the MAC CE can activate and deactivate the joint or separate TCI state(s). Thus, there are two options regarding when to update the PL offset.

- Option1: PL offset is updated together with the corresponding joint or separate TCI state(s) activation MAC CE;

- Option2: PL offset update MAC CE is separate with the joint or separate TCI state(s) activation MAC CE.

According to RAN1 discussion, PL offset is updated due to, e.g., UE movement, in the scenario of asymmetric DL sTRP/UL mTRP deployment. Different UE locations may cause different PL offset between DL PL and UL PL. Thus, the PL offset may be updated after activating the corresponding joint or UL TCI state(s). And it’s meaningless that PL offset is updated when there is no any activated joint or UL TCI state(s) for the UE. Thus, Option 2 is more flexible than Option1.

Moreover, based on RAN1 agreements, the PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH. Thus, it’s not suitable to update the PL offset together with the corresponding joint or separate TCI state(s) activation MAC CE. For example, for SRS, the DCI-indicated TCI state is applied if followUnifiedTCI-StateSRS is configured, otherwise for periodic SRS resource TCI state is configured by RRC, and for SP/AP SRS resource TCI state is indicated by MAC CE.

Based on above analysis, we have the following proposals.

Proposal 4: For PL offset updates, a new MAC CE is designed separately with the joint or separate TCI state(s) activation MAC CE(s).

Proposal 5: The PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

PL offset updated indication

In RAN1#118 meeting, RAN1 discussed the value range of PL offset and made the following agreement.

Agreement For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB - FFS: Extending the range to lower than -10dB - Step size is 4dB

Furthermore, RAN1 discussed the FFS on extending the range to lower than -10dB and have the following agreement.

Agreement The lower limit of the value range of PL offset is extended to -12 dB.

Thus, based on the above agreements, the value range of PL offset includes [-12, 60]dB and Step size is 4dB. There are two alternatives to indicate the value of PL offset in the new MAC CE.

- Alt1: PL offset is indicated with an absolutely value, which is [-12, 60]dB with a step size of 4dB;

- Alt2: PL offset is indicated with a delta offset with respect to the RRC configured value. The value range of delta offset is [-72, 72] with a step size of 4dB.

Based on the current RAN1 agreement, i.e., one PL offset value is configured in a joint or UL TCI state by RRC and a MAC CE can update the PL offset value(s) for joint or UL TCI state(s), the PL offset updated MAC CE can indicate the absolutely value of PL offset which is overrode the value configured by RRC, i.e., Alt1. Since the value range of PL offset is [-12, 60]dB with a step size of 4dB, 5bits in the new MAC CE is enough to represent the value range of PL offset.

However, it’s may be a problem if we configure a PL offset value in RRC and then modify via MAC CE. Thus, Alt2 can avoid this issue that the new MAC CE just indicates a delta offset with respect to the RRC configured value. Thus, the value range of delta offset is [-72, 72] with a step size of 4dB. 6bits in the new MAC CE is needed to represent the value range of delta offset.

Proposal 6: RAN2 to discuss the following alternatives for PL offset updated indication:

- Alt1: PL offset is indicated with an absolutely value, which is [-12, 60]dB with a step size of 4dB. In the new MAC CE, 5bits are enough to represent the value range of PL offset;

- Alt2: PL offset is indicated with a delta offset with respect to the RRC configured value. The value range of delta offset is [-72, 72] with a step size of 4dB. In the new MAC CE, 6bits are needed to represent the value range of delta offset with respect to the RRC configured value.

How to indicate the associate TCI state(s) in the new MAC CE

Based on the above discussion, the PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH. Different channel may have different TCI state indication methods. Thus, it’s better to indicate the TCI state ID in the new PL offset updated MAC CE to explicit indicate the PL offset updated associate with which TCI state.

Proposal 7: TCI state ID is included in the new MAC CE.

Other fields in the new MAC CE

Similarly with the joint or separate TCI state(s) activation/deactivation MAC CE, the Serving Cell ID is needed in the new MAC CE to indicate the identity of the Serving Cell for which the MAC CE applies. The length of the Serving Cell ID field is 5 bits. Moreover, DL/UL BWP ID is also needed in the new MAC CE to indicate a DL/UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212. DL BWP ID is referred to the joint TCI state(s) which PL offset associated with, while UL BWP ID is referred to the UL TCI state(s). However, the PL offset is updated after the corresponding joint or UL TCI state(s) activation. And a UE cannot be configured joint and separate TCI states simultaneously. Thus, the UE knows what kind of TCI state(s), i.e., joint or UL, is associated with the PL offset in the new MAC CE. Only including one field of DL/UL BWP ID is enough in the new MAC CE. The length of the DL/UL BWP ID field is 2 bits.

Proposal 8: Serving Cell ID is included in the new MAC CE to indicate the identity of the Serving Cell for which the MAC CE applies. The length of the Serving Cell ID field is 5 bits.

Proposal 9: One field of DL/UL BWP ID is included in the new MAC CE to indicate a DL/UL BWP for which the MAC CE applies. The length of the DL/UL BWP ID field is 2 bits.

- 3. Conclusion

This paper discusses the RRC and MAC Impacts for Asymmetric DL sTRP/UL mTRP. The contribution of this paper is summarized as follows.

Proposal 1: A new RRC parameter plOffset is introduced to configure the pathloss offset for the joint or UL TCI state. The values of plOffset are defined in an ENUMERATED structure from -12dB to 60dB with 4dB step size.

Proposal 2: A new RRC parameter is introduced to configure the presence of 1-bit DCI field, which is used to indicate PL offset for PDCCH-order PRACH.

Proposal 3: The RRC parameter startingBitOfFormat2-3-r19 can be defined with the values of INTEGER (1..45) with the field description that INTEGER (1..45) is configured for FR1 in shared spectrum or FR2-2, and INTEGER (1..43) is configured for otherwise.

Proposal 4: For PL offset updates, a new MAC CE is designed separately with the joint or separate TCI state(s) activation MAC CE(s).

Proposal 5: The PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

Proposal 6: RAN2 to discuss the following alternatives for PL offset updated indication:

- Alt1: PL offset is indicated with an absolutely value, which is [-12, 60]dB with a step size of 4dB. In the new MAC CE, 5bits are enough to represent the value range of PL offset;

- Alt2: PL offset is indicated with a delta offset with respect to the RRC configured value. The value range of delta offset is [-72, 72] with a step size of 4dB. In the new MAC CE, 6bits are needed to represent the value range of delta offset with respect to the RRC configured value.

Proposal 7: TCI state ID is included in the new MAC CE.

Proposal 8: Serving Cell ID is included in the new MAC CE to indicate the identity of the Serving Cell for which the MAC CE applies. The length of the Serving Cell ID field is 5 bits.

Proposal 9: One field of DL/UL BWP ID is included in the new MAC CE to indicate a DL/UL BWP for which the MAC CE applies. The length of the DL/UL BWP ID field is 2 bits.

- 4. Reference

- RP-242394 WID revision: NR MIMO Phase 5

- R2-2407906, LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5 (R1-2407285; contact: Samsung)