3GPP TSG-RAN WG2 #128	R2-2409728

Orlando, US, November 18-22, 2024

Agenda Item:	8.13.2

Source:	NEC

Title:	Discussion on UE discovery and relay reselection

Document for:	Discussion, Decision

Introduction

Considering the WID of NR sidelink multi-hop relay has been set up, and this is the first meeting for discussion. Therefore, in this contribution, we will discuss some initial consideration of relay discovery and relay selection/reselection

Discussion

Relay discovery

In Rel-17, U2N relay is introduced of which the main purpose is to ensure a better network coverage. When it comes to Rel-18, in order to ensure the service quality, multi-path U2N relay is introduced, which allows U2N remote UE to connect to the network via both a relay link and a direct Uu link, so that both service reliability and throughput can be ensured.

Currently, in Rel-19, multi-hop U2N relay is for discussion, one may consider in some low-density network area, UE, i.e., U2N remote UE, may be hardly under network coverage or reach the edge of network coverage. Therefore, the U2N remote UE may need to set up a multi-hop connection so that to perform a Uu service. That is the main motivation to work for multi-hop U2N relay.

Furthermore, in discovery procedure, it is necessary to differentiate the UE type so that to allow U2N remote UE perform corresponding relay service initiation, i.e., the UE needs to know whether it is setting up a single hop U2N relay service or a multi-hop U2N relay service. How to differentiate such UE type shall be up to SA2 discussion. So RAN2 shall send the corresponding LS, which is captured in the annex, towards SA2 to trigger their discussion.

It is up to SA2 decision on how to differentiate the UE type, i.e., Intermediate relay UE and U2N relay UE within discovery messages.

RAN2 shall send the LS towards SA2 to trigger their discussion on how to differentiate the UE type, i.e. Intermediate Relay UE and Lsat Relay UE, within the discovery messages.

In Rel-17/18, a U2N relay UE can only be selected when its Uu radio measurement result is above a minimum configured threshold, and below a maximum configured threshold. The reason to have the maximum configured threshold is assuming when the U2N relay UE has a very good radio condition, which means it should be in the centre of network coverage, therefore, its surrounding U2N remote UEs should also have a qualified radio condition, so the U2N remote UE does not need to initiate relay service. While it is not a must that the U2N relay UE is in RRC_CONNECTED state.

When it comes to Rel-19, in order to avoid radio interference from U2N relay UE and network, similar principle shall be applied, that is both upper Uu RSRP threshold and lower Uu RSRP threshold shall be applied for a UE which will act as a U2N relay UE.

In Rel-19, a UE can act as a U2N relay UE only if its Uu radio signal is between an upper Uu RSRP threshold and a lower Uu RSRP threshold.

On the other hand, in Rel-17/18, a U2N remote UE can be initiated for discovery procedure only if its Uu RSRP is lower than a configured threshold. It can be simply reused for U2N remote UE in Rel-19. Furthermore, for multi-hop U2N, the U2N remote UE is reasonably out of coverage to have Uu service. So it is suggested that either the U2N remote UE’s Uu RSRP is lower than a configured threshold, or the UE is out of coverage, can initiate discovery procedure.

In Rel-19, a U2N remote UE can be initiated for discovery procedure when its Uu RSRP is lower than a configured threshold, or the UE is out of coverage.

Furthermore, for Intermediate relay UE, where the concept is not existing in Rel-17/Rel-18, it will directly connect to the U2N remote UE, so their distance will not be too far. So similar configuration principle of Rel-17/18 U2N relay UE can be applied to the Rel-19 Intermediate relay UE, that is the Intermediate relay UE is allowed for discovery procedure only if its Uu RSRP is above a minimum Uu threshold and below a maximum Uu threshold.

In Rel-19, a Intermediate relay UE can be initiated for discovery procedure when its Uu RSRP is above than a minimum threshold and below than a maximum threshold.

In Rel-17/18, a dedicated discovery resource pool may be configured for both U2N remote UE and U2N relay UE to transmit and monitor discovery messages. In details, both U2N remote UE and U2N relay UE shall only use the dedicated discovery resource pool for discovery message transmission if it is configured, otherwise, data transmission resource pool shall be used for discovery message transmission. When it comes to Rel-19, similar principle shall be used, that is all of the U2N remote UE, U2N relay UE and Intermediate relay UE shall use dedicated discovery resource pool for discovery message transmission if it is configured, otherwise, data transmission resource pool shall be used for discovery message transmission.

In Rel-19, all of the U2N remote UE, U2N relay UE and Intermediate relay UE shall use dedicated discovery resource pool for discovery message transmission if it is configured; otherwise, communication resource pool shall be used for discovery message transmission.

Relay selection/reselection

As mentioned before, in Rel-19 multi-hop U2N relay service, there are three types of UE, which are U2N relay UE, Intermediate relay UE and U2N remote UE. Suppose a U2N remote UE would like to initiate a relay service, both Intermediate relay UE selection and U2N remote UE selection shall be performed so that can set up a complete multi-hop U2N relay link.

Intermediate relay UE selection/reselection

In Rel-17/18 U2N relay, the relay selection/reselection procedure is captured in the following table

The U2N Remote UE performs radio measurements at PC5 interface and uses them for U2N Relay selection and reselection along with higher layer criteria, as specified in TS 23.304 [48]. When there is no unicast PC5 connection between the U2N Relay UE and the U2N Remote UE, the U2N Remote UE uses SD-RSRP measurements to evaluate whether PC5 link quality towards a U2N Relay UE satisfies relay selection criterion. For relay reselection, U2N Remote UE uses SL-RSRP measurements towards the serving U2N Relay UE for relay reselection trigger evaluation when there is data transmission from U2N Relay UE to U2N Remote UE, and it is left to UE implementation whether to use SL-RSRP or SD-RSRP for relay reselection trigger evaluation in case of no data transmission from U2N Relay UE to U2N Remote UE. A U2N Relay UE is considered suitable by a U2N Remote UE in terms of radio criteria if the PC5 link quality measured by U2N Remote UE towards the U2N Relay UE exceeds configured threshold (pre-configured or provided by gNB). The U2N Remote UE searches for suitable U2N Relay UE candidates that meet all AS layer and higher layer criteria (see TS 23.304 [48]). If there are multiple such suitable U2N Relay UEs, it is up to U2N Remote UE implementation to choose one U2N Relay UE among them. For L2 U2N Relay (re)selection, the PLMN ID and cell ID can be used as additional AS criteria.

When it comes to Rel-19 Intermediate relay UE selection, the similar procedure can be reused. That is the U2N remote UE performs radio measurements at PC5 interface and uses them for Intermediate relay UE selection and reselection along with higher layer criteria. When there is no unicast PC5 connection between the Intermediate relay UE and U2N remote UE, the U2N remote UE uses SD-RSRP measurements to evaluate whether PC5 link quality towards a Intermediate relay UE satisfies configured criterion. For Intermediate relay UE reselection, U2N remote UE uses SL-RSRP measurements towards the serving Intermediate relay UE for Intermediate relay UE reselection trigger evaluation when there is data transmission from Intermediate relay UE to U2N remote UE, and it is left to UE implementation whether to use SL-RSRP or SD-RSRP for Intermediate relay UE reselection trigger evaluation in case of no data transmission from Intermediate relay UE to U2N remote UE. A Intermediate relay UE is considered suitable by a U2N remote UE in terms of radio criteria if the PC5 link quality measured by U2N remote UE towards the Intermediate relay UE exceeds configured threshold. The U2N remote UE searches for suitable Intermediate relay UE candidates that meet all AS layer and higher layer criteria. If there are multiple such suitable Intermediate relay UEs, it is up to U2N remote UE implementation to choose one Intermediate relay UE among them.

In Rel-19, when there is no unicast PC5 connection between the Intermediate relay UE and U2N remote UE, the U2N remote UE uses SD-RSRP measurements to evaluate whether PC5 link quality towards a Intermediate relay UE satisfies configured criterion.

In Rel-19, U2N remote UE uses SL-RSRP measurements towards the serving Intermediate relay UE for Intermediate relay UE reselection trigger evaluation when there is data transmission between Intermediate relay UE and U2N remote UE.

It is left to UE implementation whether to use SL-RSRP or SD-RSRP for Intermediate relay UE reselection trigger evaluation in case of no data transmission between Intermediate relay UE and U2N remote UE.

A Intermediate relay UE is considered suitable by a U2N remote UE in terms of radio criteria if the PC5 link quality measured by U2N remote UE towards the Intermediate relay UE exceeds configured threshold.

Since the Intermediate relay UE is not the endpoint which connect to the network, so whether it is the inter cell coverage or intra cell coverage with U2N relay UE/U2N remote UE is not important. Therefore, the PLMN ID and cell ID does not necessarily to be used as additional AS criteria.

In Rel-17/18, the U2N relay selection/reselection criteria is shown in the following table:

The U2N Remote UE triggers U2N Relay selection in following cases: - Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold; - Indicated by upper layer of the U2N Remote UE. The U2N Remote UE may trigger U2N Relay reselection in following cases: - PC5 signal strength of current U2N Relay UE is below a (pre)configured signal strength threshold; - Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by U2N Relay UE via PC5-RRC signalling; - When U2N Remote UE receives a PC5-S link release message from U2N Relay UE; - When U2N Remote UE detects PC5 RLF; - Indicated by upper layer.

When it comes to Rel-19, for Intermediate relay UE selection trigger, it might be slightly different, normally a U2N remote UE would like to initiate multi-hop U2N relay service when it is under the following two conditions:

- The U2N remote UE previously worked as normal Uu UE

- The U2N remote UE previously work as single hop U2N remote UE

For the first condition, the U2N remote UE would trigger Intermediate relay UE selection when its direct Uu signal strength of current serving cell of the U2N remote UE is below a configured signal strength threshold.

For the second condition, the U2N remote UE would trigger Intermediate relay UE selection when its PC5 signal strength between it and the serving single hop U2N relay UE is below a configured signal strength threshold.

Similar as Rel-17/18, upper layer can always indicate the U2N remote UE to perform Intermediate relay UE selection.

U2N remote UE would like to initiate multi-hop U2N relay service when it is under the following two conditions:

- The U2N remote UE previously worked as normal Uu UE

- The U2N remote UE previously work as single hop U2N remote UE

In Rel-19, U2N remote UE would trigger Intermediate relay UE selection when its direct Uu signal strength of current serving cell of the U2N remote UE is below a configured signal strength threshold

In Rel-19, U2N remote UE would trigger Intermediate relay UE selection when its PC5 signal strength between it and the serving single hop U2N relay UE is below a configured signal strength threshold.

In Rel-19, U2N remote UE would trigger Intermediate relay UE selection when it is indicated by upper layer.

Furthermore, for triggering of intermediate UE reselection, either there is radio issue happened on the link between U2N relay UE and Intermediate relay UE, or on the link between Intermediate relay UE and U2N remote UE, shall trigger Intermediate relay UE reselection. In details, if Intermediate relay UE indicate the following information towards the U2N remote UE:

- the PC5 signal strength between the serving U2N relay UE and the serving Intermediate relay UE is below a configured threshold,

- PC5 RLF is happened on the link between the serving U2N relay UE and the serving Intermediate relay UE

- PC5 link release message is sent either from Intermediate relay UE towards the U2N relay UE or from U2N relay UE towards the Intermediate relay UE

U2N remote UE shall trigger U2N intermediate reselection.

In Rel-19, the U2N remote UE shall trigger Intermediate relay UE reselection if Intermediate relay UE indicate the following information towards the U2N remote UE:

- the PC5 signal strength between the serving U2N relay UE and the serving Intermediate relay UE is below a configured threshold,

- PC5 RLF is happened on the link between the serving U2N relay UE and the serving Intermediate relay UE

- PC5 link release message is sent either from Intermediate relay UE towards the U2N relay UE or from U2N relay UE towards the Intermediate relay UE

In addition, at U2N remote UE side, if:

- It receives a PC5-S link release message from Intermediate relay UE

- It detects PC5 RLF on the link between U2N remote UE and Intermediate relay UE

- Receive indication from upper layer

It shall also trigger Intermediate relay UE reselection.

In Rel-19, the U2N remote UE shall trigger Intermediate relay UE reselection for the following cases:

- It receives a PC5-S link release message from Intermediate relay UE

- It detects PC5 RLF on the link between U2N remote UE and Intermediate relay UE

- Receive indication from upper layer

In Rel-17/18, for L2 U2N remote UEs in RRC_IDLE or RRC_INACTIVE and L3 U2N remote UEs, the cell (re)selection procedure and relay (re)selection procedure run independently. If both suitable cells and suitable U2N relay UEs are available, it is up to the U2N remote UE implementation to select either a cell or a U2N relay UE. A L3 U2N remote UE may select a cell and a L3 U2N relay UE simultaneously and this is up to implementation of L3 U2N remote UE.

When it comes to Rel-19, for L2 U2N remote UEs in RRC_IDLE or RRC_INACTIVE and L3 U2N remote UEs, the U2N remote UE shall be based on the QoS requirement to decide whether to perform a single hop U2N relay UE (re)selection or a multi-hop Intermediate relay UE (re)selection.

In Rel-19, for L2 U2N remote UEs in RRC_IDLE or RRC_INACTIVE and L3 U2N remote UEs, the U2N remote UE shall be based on the QoS requirement to decide whether to perform a single hop U2N relay UE (re)selection or a multi-hop Intermediate relay UE (re)selection.

U2N Relay UE selection/reselection

In Rel-17/18, it is the U2N remote UE perform relay selection/reselection because it is the single hop relay which only contains two nodes in a relay link, which are U2N remote UE and U2N relay UE. However, when it comes to Rel-19 multi-hop U2N relay, there are three types of UE, which are U2N relay UE, Intermediate relay UE and U2N remote UE. So whether it is the U2N remote UE or the Intermediate relay UE to perform relay (re)selection shall be discussed. In details, there are respective pros and cons for these two options.

Firstly, if the traditional principle is followed, that is to allow the U2N remote UE to perform relay (re)selection. Since the Uu traffic is generated by the U2N remote UE, it has a better knowledge of the QoS requirement of the service. Correspondingly, the U2N remote UE can clearly know which U2N relay UE shall be selected that can satisfy its service QoS. However, since the U2N remote UE does not directly connect to the U2N relay UE, it cannot perform a direct measurement on which U2N relay UE is better for selection.

On the other hand, if Intermediate relay UE performing U2N relay UE selection/reselection, it can directly perform radio measurement so that to select a U2N relay UE with a best radio quality. However, since the service traffic is coming from U2N remote UE, the Intermediate relay UE cannot make the final decision so that it has to request the U2N remote UE to confirm the final selection.

So before RAN2 is going to work on any details of U2N relay UE (re)selection, RAN2 should firstly decide which UE is going to perform U2N relay UE (re)selection.

RAN2 should discuss which UE to perform U2N relay UE (re)selection, U2N remote UE or Intermediate relay UE, or both

Another principal issue which shall be discussed in RAN2 is whether U2N relay UE (re)selection will also trigger Intermediate relay UE reselection.

RAN2 should discuss whether U2N relay UE (re)selection will also trigger Intermediate relay UE reselection.

Conclusion

In this contribution, it is our first discussion for multi-hop U2N relay, a brunch of proposals and observations are given in below:

Observation 1	It is up to SA2 decision on how to differentiate the UE type, i.e., Intermediate relay UE and U2N relay UE within discovery messages.

Observation 2	U2N remote UE would like to initiate multi-hop U2N relay service when it is under the following two conditions:

-	The U2N remote UE previously worked as normal Uu UE

-	The U2N remote UE previously work as single hop U2N remote UE

Proposal 1	RAN2 shall send the LS towards SA2 to trigger their discussion on how to differentiate the UE type, i.e. Intermediate Relay UE and Lsat Relay UE, within the discovery messages.

Proposal 2	In Rel-19, a UE can act as a U2N relay UE only if its Uu radio signal is between an upper Uu RSRP threshold and a lower Uu RSRP threshold.

Proposal 3	In Rel-19, a U2N remote UE can be initiated for discovery procedure when its Uu RSRP is lower than a configured threshold, or the UE is out of coverage.

Proposal 4	In Rel-19, a Intermediate relay UE can be initiated for discovery procedure when its Uu RSRP is above than a minimum threshold and below than a maximum threshold.

Proposal 5	In Rel-19, all of the U2N remote UE, U2N relay UE and Intermediate relay UE shall use dedicated discovery resource pool for discovery message transmission if it is configured; otherwise, communication resource pool shall be used for discovery message transmission.

Proposal 6	In Rel-19, when there is no unicast PC5 connection between the Intermediate relay UE and U2N remote UE, the U2N remote UE uses SD-RSRP measurements to evaluate whether PC5 link quality towards a Intermediate relay UE satisfies configured criterion.

Proposal 7	In Rel-19, U2N remote UE uses SL-RSRP measurements towards the serving Intermediate relay UE for Intermediate relay UE reselection trigger evaluation when there is data transmission between Intermediate relay UE and U2N remote UE.

Proposal 8	It is left to UE implementation whether to use SL-RSRP or SD-RSRP for Intermediate relay UE reselection trigger evaluation in case of no data transmission between Intermediate relay UE and U2N remote UE.

Proposal 9	A Intermediate relay UE is considered suitable by a U2N remote UE in terms of radio criteria if the PC5 link quality measured by U2N remote UE towards the Intermediate relay UE exceeds configured threshold.

Proposal 10	In Rel-19, U2N remote UE would trigger Intermediate relay UE selection when its direct Uu signal strength of current serving cell of the U2N remote UE is below a configured signal strength threshold

Proposal 11	In Rel-19, U2N remote UE would trigger Intermediate relay UE selection when its PC5 signal strength between it and the serving single hop U2N relay UE is below a configured signal strength threshold.

Proposal 12	In Rel-19, U2N remote UE would trigger Intermediate relay UE selection when it is indicated by upper layer.

Proposal 13	In Rel-19, the U2N remote UE shall trigger Intermediate relay UE reselection if Intermediate relay UE indicate the following information towards the U2N remote UE:

Proposal 14	In Rel-19, the U2N remote UE shall trigger Intermediate relay UE reselection for the following cases:

Proposal 15	In Rel-19, for L2 U2N remote UEs in RRC_IDLE or RRC_INACTIVE and L3 U2N remote UEs, the U2N remote UE shall be based on the QoS requirement to decide whether to perform a single hop U2N relay UE (re)selection or a multi-hop Intermediate relay UE (re)selection.

Proposal 16	RAN2 should discuss which UE to perform U2N relay UE (re)selection, U2N remote UE or Intermediate relay UE, or both

Proposal 17	RAN2 should discuss whether U2N relay UE (re)selection will also trigger Intermediate relay UE reselection.

Reference

3GPP TSG-RAN WG2 Meeting #127 R2-24NNNNN Maastricht, Netherlands, Aug 19th -23rd, 2024 Title: [DRAFT] LS on UE type differentiation Response to: - Release: Release 19 Work Item: NR_SL_relay_enh2 Source: NEC [RAN WG2] To: SA WG2 Cc: Contact Person: #### Name: Boyuan Zhang E-mail Address: zhang_boyuan@nec.cn Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org Attachments: - 1. Overall Description: In RAN2 #127 meeting, it has been agreed that to introduce the UE type, Intermediate relay UE, for multi-hop sidelink relay, where it connects to U2N remote UE at one end, and connects to U2N relay UE at the other end. Therefore, RAN2 would like to check how to differentiate the UE type, i.e., U2N relay UE, U2N remote UE and Intermediate relay UE via discovery procedure. 2. Actions: To SA WG2 group. ACTION: RAN2 respectfully asks SA2 to reply to the above questions and update their specifications if necessary. 3. Date of Next TSG-RAN WG2 Meeting: RAN2#128 from 2024-10-15 to 2024-10-19 China, CN RAN2#129 from 2024-11-20 to 2024-11-24 USA