3GPP TSG-RAN WG2 #128	R2-2409734

Orlando, USA, November 2024

Agenda Item:	8.7.3

Source:	OPPO

Title:	Discussion on Measurement Gap Enhancements

Document for:	Discussion, Agreement

Introduction

In RAN2 #127bis meeting, RAN2 discussed potential L2 impacts due to skipping of measurement and the need for semi-static/UE-initiated solution besides the DCI-based solution to enable measurement gap skipping. In this paper, the RAN2 impacts will be further discussed.

Discussion

Issue-1: L2 impact

For the impact on L2 features, the impact to LCP, DSR, and DRX are discussed, and whether there are any impacts to DRX and DRS are to be further evaluated.

RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact what would require some specification changes/enhancements. RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear.

Firstly, for DSR, it is triggered for an LCG if the smallest remaining PDB among all associated SDUs is below a configured threshold. The larger/smaller value of threshold configuration from the network controls the earlier/later trigger of DSR.

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel.

The earlier/later trigger of DSR is controlled by the network via remainingTimeThreshold configuration.

Therefore, RAN2 made the agreement that at least some impact on DSR from MG skipping can be avoided by network implementation. The further question is whether there is any DSR impact from MG skipping that cannot be avoided by network implementation.

To further analyze the potential impact, this issue is divided into two sub-cases:

- Case-1: the measurement gap is not indicated as skipped.

- Case-2: the measurement gap is indicated as skipped.

In Case-1, if the measurement gap is not skipped, it is the same as the no measurement gap skipping scenario, which means there are no measurement gap skipping-specific impacts on DSR in case the measurement gap is not indicated as skipped.

In Case-2, if the measurement gap is indicated as skipped, as RAN1 agreed, UE will receive/transmit as it would without any gaps, which means there is no measurement gap skipping specific impacts to DSR in case the measurement gap is indicated as skipped as well.

- From RAN1 perspective, when an occasion(s) of gaps/restrictions that are caused by RRM measurements are cancelled/skipped fully, UE is assumed to receive/transmit in the gaps/restrictions that are caused by RRM measurements as it would without any (measurement etc. related) gaps/restrictions that are caused by RRM measurements.

There is no specific impact on DSR due to measurement gap skipping, regardless of whether the measurement gap is indicated as skipped or not. Since the UE's transmission/reception behavior remains the same.

No need to have DSR enhancements specific for MG skipping.

Then for DRX, the concern for the DRX impact from measurement gap skipping is on whether the DRX active time, MG to be skipped, and the scheduling can be fully aligned to avoid the miss decoding of the PDCCH from the network. The understanding is this issue will not happen since DRX configuration, measurement gap configuration, and PDCCH scheduling are all controlled by the network, i.e., network has the full knowledge of all the information and thus will only schedule the UE if it is/will be in active time.

The Network implementation is sufficient to handle any interaction between measurement gap skipping and DRX since the network controls DRX configuration, measurement gap configuration, and PDCCH scheduling.

No need to have DRX enhancements specific for MG skipping.

Issue-2: UAI

Firstly, on how to enable the transmission and reception in MG/scheduling restriction occasions, RAN1 has agreed on the following solution based on dynamic indication from the network.

For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it.

As shown in the above agreements, it is network who decides whether to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements, and there is a discussion in RAN1 that whether the assistance information from the UE is needed for network’s decision. There is no consensus in RAN1, one LS[1] is sent to RAN4 and CC RAN2. In the LS, the candidate assistance information is listed as follows.

RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information.

The main motivation of assistance information from the UE is whether/how much the RRM performance is impacted by skipping measurement occasions. This relates to how the RRM requirement is defined for measurement gap skipping scenario. I.e.,

- If reusing the existing RRM requirement, the assistance information may be helpful for the network to know whether/which measurement gap to skip without impacting the RRM requirement;

- If defining new RRM requirement for measurement gap skipping scenario that is fully compatible with the skipping behavior, there seems less need for the assistance information.

This should be evaluated by RAN4, and RAN4 has begun their discussion.

On the RRM requirements

Issue 1-1: Workscope < Agreement>: - As starting point, RAN4 to discuss the Tx/Rx in occasions of L3 measurement. - The progress in RAN1/2 should be taken into account. - FFS whether/when to start the discussion of Tx/Rx in occasions of scheduling restriction due to L1 operation. - RAN4 to discuss - The potential impact on measurement requirements due to measurement cancellation - The corresponding solution to address the impact if needed.

And the need for UAI

## Issue 2-1: General on UAI <Way forward>: - Discuss if UAI is needed and which aspects to consider.

Whether the assistance information from the UE is necessary depends on RAN4 evaluation on the RRM requirement for measurement gap skipping scenario, e.g., whether new RRM requirement is defined.

For measurement gap skipping, RAN2 wait for RAN4 evaluation before working on the UE assistance information message design.

If RAN4 confirms that assistance information is needed, for the detailed information to be carried, the following candidates are discussed in RAN1

- FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not

Among the above information, the first FFS on assistance information related to measurement occasions is more of RAN4 scope, and the last three FFS related to channel quality and traffic are more of RAN2 scope which can be discussed in RAN2.

Within the candidates of UAI that R1 identified, the first one is more related to measurement occasions and is in RAN4 scope, while the others are more in RAN2 scope.

But anyway, the discussion at RAN2 can happen after seeing RAN4 LS reply.

Issue-3: Semi-static/UE initiated approach

For the need of semi-static or UE-initiated approach, it was discussed in the last RAN2 meeting, and the following agreements were made:

RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

It is agreed in RAN2 that the need for other solutions will be decided after RAN1/4 evaluation, and RAN4 has also discussed this issue in their last meeting but no conclusion has been made.

RAN4 is discussing/evaluating the solutions for measurement gap skipping.

To avoid duplex discussion and conflict conclusion, RAN2 should wait for further RAN4/RAN1 conclusions of the need for other solutions for measurement gap skipping.

RAN2 wait for further RAN4/RAN1 conclusion of the need for other solutions for measurement gap skipping.

- Conclusion

We have the following observations:

Observation 1	The earlier/later trigger of DSR is controlled by the network via remainingTimeThreshold configuration.

Observation 2	There is no specific impact on DSR due to measurement gap skipping, regardless of whether the measurement gap is indicated as skipped or not. Since the UE's transmission/reception behavior remains the same.

Observation 3	The Network implementation is sufficient to handle any interaction between measurement gap skipping and DRX since the network controls DRX configuration, measurement gap configuration, and PDCCH scheduling.

Observation 4	Whether the assistance information from the UE is necessary depends on RAN4 evaluation on the RRM requirement for measurement gap skipping scenario, e.g., whether new RRM requirement is defined.

Observation 5	Within the candidates of UAI that R1 identified, the first one is more related to measurement occasions and is in RAN4 scope, while the others are more in RAN2 scope.

Observation 6	RAN4 is discussing/evaluating the solutions for measurement gap skipping.

We have the following proposals:

Proposal 1	No need to have DSR enhancements specific for MG skipping.

Proposal 2	No need to have DRX enhancements specific for MG skipping.

Proposal 3	For measurement gap skipping, RAN2 wait for RAN4 evaluation before working on the UE assistance information message design.

Proposal 4	RAN2 wait for further RAN4/RAN1 conclusion of the need for other solutions for measurement gap skipping.

Reference

- R1-2405736, LS on UE assistance information