3GPP TSG-RAN WG2 Meeting #128			     R2-2409741

Orlando, US, November 18 – 22, 2024

Agenda Item:	8.7.6 (NR_XR_Ph3-Core)

Source: 	LG Electronics Inc.

Title:         	Rate control signaling for XR

Document for: 	Discussion and Decision

Introduction

This contribution discusses rate control signalling for XR based on the below RAN2 agreement.

- FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc. - RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications: - Extend the Bit Rate field - Define a new bit rate table to provide sufficient granularity for XR traffic - Introduce new values for the bitRateMultiplier - Send LS to SA4 asking about range/granularity which is required

Discussion

For QoS requirement enforcement, one IP flow is mapped to one QoS flow and one DRB can be mapped with multiple QoS flows which have same/similar QoS requirement. With this understanding, the proponent of per DRB thinks that per DRB is sufficient for XR rate control since legacy rate control, i.e., RBR MAC CE, is already for LCH level which is associated with DRB level and an application can decide whether to adapt to bitrate based on this per DRB indication. However, we think that situation is different and per QoS flow indication should be considered for XR rate control signalling due to following reasons.

The RBR MAC CE is targeted for VoIP and one QoS flow for VoIP may be associated with one DRB, and hence per DRB indication would be sufficient because anyway one DRB may be mapped with one VoIP. Of course, one DRB may be mapped with multiple VoIP services, but typically multiple QoS flows for multiple VoIP services would not be activated simultaneously and only one QoS flow for one VoIP is activated in one DRB. In this situation, per DRB indication may be sufficient since anyway one QoS flow for the activated VoIP service in one DRB would be actually adapted based on the per DRB indication in the RBR MAC CE. However, one XR service is comprised of multiple QoS flows and each QoS flow in one XR service may have different QoS requirement. This means that one QoS flow in a XR service can be mapped with a DRB together with other QoS flows having same QoS requirement for non-XR services. Even though QoS flows for a specific XR service should need to be adapted based on the XR rate signalling, if per DRB indication is used, the UE cannot exactly know for which QoS flow is targeted and other unintended QoS flows in the DRB would be unnecessarily impacted. The proponent of per DRB indication may argue that an application can decide whether to adapt to bitrate based on this per DRB indication and there is no impact to other unintended QoS flows. In our view, it is doubtful how the UE denies the bitrate adaptation after receiving rate control indication from the lower layer because each application is independent program and each application definitely knows that if they denies the bitrate adaptation, their ongoing application service quality would be deteriorated.

Observation 1. If per DRB indication is introduced, the unintended QoS flow in the same DRB can be unnecessarily impacted.

Observation 2. Per DRB indication needs to rely on application’s decision for bitrate adaptation, but it may be infeasible to decide the bitrate adaptation by application itself.

According to the contribution (RP-242224), the issue observed is that in most network deployments, the L4S markings at the IP header can be removed at intermediate nodes outside of the RAN network and this issue prevents the L4S solution from functioning properly in the UL. To overcome this observed issue on L4S, the XR rate control signalling is considered as complementary solution together with existing L4S and this should be per QoS flow indication since one IP flow is mapped to one QoS flow and the L4S is used to control one IP flow using ECN (explicit congestion notification) field in IP header. If per DRB indication is introduced, we don’t think that the XR rate control signalling can resolve the observed issue on L4S. As a result, per DRB indication in the XR rate control would not be a complementary solution to L4S since this cannot provide per QoS flow control like L4S.

Observation 3. Per DRB indication would not be a complementary solution to L4S since this cannot provide per QoS flow control like L4S.

Based on the observations above, we think that per QoS flow indication should be considered for the XR rate control signalling.

Proposal 1. Per QoS flow indication is defined for the XR rate control signalling.

For other details, it is better to wait for the SA4 reply for now.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Observation 1. If per DRB indication is introduced, the unintended QoS flow in the same DRB can be unnecessarily impacted.

Observation 2. Per DRB indication needs to rely on application’s decision for bitrate adaptation, but it may be infeasible to decide the bitrate adaptation by application itself.

Observation 3. Per DRB indication would not be a complementary solution to L4S since this cannot provide per QoS flow control like L4S.

Proposal 1. Per QoS flow indication is defined for the XR rate control signalling.