3GPP TSG-RAN WG2 Meeting #128		R2-2409780

Orlando, U.S., Nov 18 – 22, 2024

Agenda Item:	8.10.2

Source: 	LG Electronics

Title:  	MRO for MOB LTM

Document for:	Discussion

Introduction

In this contribution, we focus on the Rel-18 LTM MRO based on the last agreements.

Discussion

Failure logging

      - Explicit indication

At the RAN2#127, RAN2 made the following agreement but has not yet finalized more specific details [1]:

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

We think that the RLF report should explicitly include the access type information for the attempted LTM cell switch. This is because, based on implicit access information, the network will not be able to accurately determine which access type the UE used for the entire LTM cell switch attempt in some cases.

A typical example is when the UE attempts a RACH-less LTM cell switch based on a dynamic grant (i.e., DG-based RACH-less LTM). In this case, the UE receives a RACH-less LTM CSC (cell switch command) from the source cell and monitors the PDCCH in the target cell for the UL grant. However, for certain reasons from the network side, the target cell may command the UE to use RACH for LTM. Then, if RLF is detected in this case, the UE may include both RACH-less and RACH information as the access information. Without explicit access type information, the network cannot determine which access type the UE attempted until RLF unless the network receives this information explicitly.

By above reason, we think the implicit access type information will leave a fundamental lack of trust in the access type information reported by the UE seems not be enough. Thus, we think that the UE should log explicit access type information to provide clear information preventing any potential confusion from the network side.

Proposal 1: In the RLF report for MCG LTM, the UE explicitly logs the access type.

In the above example of DG-based RACH-less LTM, we confirmed that the UE may face a situation where RACH-less LTM CSC is commanded but RACH-based LTM should be performed. We think RAN2 need to consider more about this case where the UE actually tried different access type from the access type commanded by the network in LTM CSC.

In the above example, it is unclear whether the UE should log the RACH actually performed as the access type instead of the RACH-less commanded by the network. If the UE logs the access type as RACH, the network may not know that the LTM CSC has been indicated to RACH-less, the network may misunderstand that the cause of HOF/RLF is due to improper early synchronization before LTM. Otherwise, if the UE logs the access type as RACH-less, the network doesn’t know that the UE had actually tried RACH based LTM until HOF occurs and the network may misunderstand that the UE failed RACH-less procedure during LTM.

Since the above inaccurate logging information cannot help the network’s MRO, we think, for this case, the UE should be able to report both the access type actually performed by the UE and the access type commanded by the network. The access type commanded by the network can only need to be optionally logged if the information is different from the access type actually performed by the UE.

Proposal 2: In the RLF report for MCG LTM, the UE logs not only the access type but also the commanded access type which was indicated LTM CSC if the access type is different from the commanded access type.

      - Beam information

In addition, we think beam information should be newly considered in RLF-report

The network triggers the RACH-less LTM, the network needs to indicate which beam to be applied for the RACH-less LTM. For the beam to be applied for the mobility, the network can indicate SSB index or TCI state ID associated with the target cell. For the association, the network needs to be pre-configured the UE with the SSB configuration/TCI state configuration of the target cell. Then, the UE applies the received beam information, during the RACH-less mobility, for monitoring of PDCCH at the target cell for receiving dynamic grant for transmitting mobility completion message or the received beam information is applied for transmission of mobility completion message via configured grant. The network may indicate TA value to be applied for the mobility or may indicate the UE to apply the UE-calculated TA value for the mobility.

The problem is that the received beam information may not be suitable for the UE due to, e.g., the beam information is outdated, or the UE’s local movement makes the beam information unsuitable. If such unsuitable beam is used for the RACH-less LTM, the mobility fails, i.e. the received TA or the UE calculated TA value may not be suitable for the UE, and in that case, the mobility fails. In case LTM failure happens, the current RLF-report cannot provide any information of the beam information used by the UE for the mobility. Hence, it is difficult for network to analyse the failure reason.

Thus, we propose to include the beam information for LTM trial in RLF-report.

For the beam information, the UE may log below information for logging the failed RACH-less LTM:

- SSB and TCI state information

This is for indicating which SSB index and associated TCI state was used for RACH-less LTM

- Other qualified beam information

This is for indicating whether there is any other qualified beam information for RACH-less LTM

Proposal 3: In RLF-report for MCG LTM, the UE includes SSB and TCI state with other qualified beam to indicate which beam was used and can be used for LTM in RLF-report.

Near failure logging

      - Candidate cell list optimisation

At the RAN2#127, RAN2 made the following agreement to deduce the unnecessary LTM candidate cells [1]:

We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

choCandidate provides information on whether the neighbor cell was a candidate cell when including measurement results for the neighbor cell in SHR [2]:

choCandidate-r17                    ENUMERATED {true}                                     OPTIONAL,

For the purposes of this agreement, there seems to be no reason to provide ltmCandidate information implicitly. To introduce ltmCandidate similarly to choCandidate, we believe it would be simplest to define ltmCandidate as an explicit field of the same enum type.

Proposal 4: In the SHR for MCG LTM, ltmCandidate is explicitly defined as the same enum type as choCandidate.

In addition, to deduce the LTM candidate cell more properly, we think there is another case to be considered that the candidate cell had selected for LTM cell switch but the candidate cell may be unnecessary candidate cell.

For example, let's assume that the UE has configured candidate cell 1, candidate cell 2, and candidate cell 3 for subsequent conditional mobility. If the UE performs subsequent mobility consecutively in the order of candidate cell 1 to candidate cell 3, and the time spent in candidate cell 2 is very short, there might be no problem even if mobility is performed directly from candidate cell 1 to candidate cell 3. Since the network doesn’t know this information, candidate cell 2 would keep being configured unnecessarily for the subsequent mobility.

To address this issue, we believe it is necessary for the UE to make determinations based on pre-defined/configured information between the network and the UE, such as the actual time the UE had stayed on the candidate cell. If detailed discussion seems necessary, it would be beneficial to discuss this together when discussing candidate cell list optimization above.

Proposal 5: In the SHR for MCG LTM, RAN2 discuss that the UE log candidate cells not only for those candidate cells unused but also for those candidate cells selected yet not requiring actual cell switching.

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1: In the RLF report for MCG LTM, the UE explicitly logs the access type.

Proposal 2: In the RLF report for MCG LTM, the UE logs not only the access type but also the commanded access type which was indicated LTM CSC if the access type is different from the commanded access type.

Proposal 3: In RLF-report for MCG LTM, the UE includes SSB and TCI state with other qualified beam to indicate which beam was used and can be used for LTM in RLF-report.

Proposal 4: In the SHR for MCG LTM, ltmCandidate is explicitly defined as the same enum type as choCandidate.

Proposal 5: In the SHR for MCG LTM, RAN2 discuss that the UE log candidate cells not only for those candidate cells unused but also for those candidate cells selected yet not requiring actual cell switching.

References

- R2-2407576 Report from session on maintenance, CovEnh, SON/MDT and eRedCap Session chair (Ericsson)

- 3GPP TS 38.331 V18.2.0 (2024-06)