3GPP TSG-RAN WG2 Meeting #128							   R2-2409783

Orlando, USA, Nov, 2024

Source: 	ZTE Corporation, Sanechips

Title: 	A-IoT functionality

Agenda item:	8.2.2

Document for: 	Discussion and Decision

# Introduction

At RAN2#127-bis meeting, it was agreed to support segmentation at least for the D2R direction. In this contribution, we discuss the remaining open aspects including the following issues:

- Whether segmentation is needed also in R2D direction

- Whether ACK/NACK is needed for D2R segments

In this contribution we discuss these aspects and propose a way forward.

# Protocol for D2R data transmission

# ACK/NACK for D2R data

In case of D2R it was agreed that segmentation is supported. Further, every D2R message is scheduled. So, the device has to wait for a “grant” before transmitting a given D2R message. Since grant is required, the reader can provide an indication whether the previous message has been received or not within this grant. Furthermore, at least for MSG3, it seems some form of success/failure indication will be specified. If this is the case, then it would be better to unify the design and have a success/failure indication every D2R segment. This will then enable a simple device implementation and the device doesn’t need to care whether it is MSG3 or a subsequent message and would simply follow the reader instruction for data transfer.

In general, for acknowledged data transfer, a window size is maintained at the transmitter and receiver and the transmitter waits for ack/nack indication for the data transmitted within this window size. The simplest protocol that would implement acknowledged data transfer would use a window size of (i.e. stop and wait protocol). The advantage of this is that the number of bits needed for ack/nack are minimised and the device complexity is also low. Based on this, we propose a stop and wait protocol of window size 1 for D2R transmissions. This is depicted in Figure 1 below.

Figure 1: UP protocol for D2R transmissions

Proposal 1: Feedback for D2R data (ACK/NACK//NDI – details up to stage-3) is supported and the feedback can be included in R2D message (from MSG2 onwards) along with the grant (TB size)

Proposal 2: If ACK is received, the device transmits the next segment of data from upper layers (size of the segment is the TB size in the latest grant)

Proposal 3: If NACK is received, the device retransmits the upper layer bits starting from the beginning of the previous segment (again size of the segment is the TB size in the latest grant)

# Protocol for R2D data transmission

# Need for segmentation for R2D

As far as segmentation is concerned, D2R and R2D are different because the R2D message may be broadcast (e.g. paging message) or unicast (MSG2 onwards) unlike a D2R message which is always unicast (i.e. from a device to the reader). The question is then whether it is feasible to segment all these messages and if so, what mechanisms are used to do so.

Segmenting broadcast/multicast messages comes with additional complexity from the protocol perspective since the segments need to be addressed to a group of devices and all the devices need to store all these segments even if the data is not necessarily for that device. This is really needed only if for instance a large number of individual device IDs are included in the paging message or if the paging message contains one-to-one mapping of paging IDs to MSG1 resources (e.g. CFRA) for a large number of devices. However, both these cases are optional and can be up to the implementation of the reader. A reader can potentially split the paging procedure into multiple rounds instead and include a subset of paging IDs/CFRA resources in individual paging rounds. So, segmentation of the paging message (i.e. MSG0) is not needed.

Observation 1: Paging message is broadcast and supporting segmentation for broadcast messages is more complex and is not needed as the reader can split the device IDs into multiple paging rounds and/or minimise the CFRA resources if the paging message exceeds the maximum R2D TB size

Proposal 4: Segmentation is not supported for paging message (i.e. MSG0)

Then, the question is whether segmentation is needed for other R2D messages (e.g. command). This depends on the upper layer payload size. If the upper layer payload size in R2D direction can exceed the maximum R2D payload size, then segmentation may be needed. So, this can be left until SA2 responds to our earlier LS.

Proposal 5: Wait for SA2 input before deciding whether unicast R2D messages (i.e. MSG2 onwards) can be segmented.

# ACK/NACK for D2R data

Unlike the D2R direction, supporting similar functionality for ACK/NACK in R2D direction is a lot more complex. The issue is that ACK/NACK for R2D messages need to be generated by the device and generating this and transmitting ACK/NACK for every R2D message/segment may be complex and would need additional functionality at the device side, which may not be practical in simple tags/devices.

Proposal 6: ACK/NACK is not supported for R2D data

# AS ID for scheduling

MSG0 will be broadcast and is addressed to all devices. However, MSG2 and subsequent R2D messages are unicast and are addressed to a specific device. So, an AS ID is necessary to address the device. Given that the random number included in MSG1 is used for contention resolution (as agreed at RAN2#126), it can be assumed that the random number is unique for the A-IoT session and hence can be used as the AS ID for addressing all messages from MSG2 onwards. The alternative is for the reader to allocate a dedicated AS ID and this is complex since the device then needs to successfully receive and store this AS ID and then use it for subsequent transmissions. The advantage of using a reader allocated ID over reusing RN16 as AS ID is hence not clear.

Proposal 7: The random number included in MSG1 (RN16) is used as the AS ID for scheduling subsequent A-IoT messages.

# Conclusion and proposals

The following observations/proposals are made in this contribution:

For D2R data:

Proposal 1: Feedback for D2R data (ACK/NACK//NDI – details up to stage-3) is supported and the feedback can be included in R2D message (from MSG2 onwards) along with the grant (TB size)

Proposal 2: If ACK is received, the device transmits the next segment of data from upper layers (size of the segment is the TB size in the latest grant)

Proposal 3: If NACK is received, the device retransmits the upper layer bits starting from the beginning of the previous segment (again size of the segment is the TB size in the latest grant)

For R2D data:

Observation 1: Paging message is broadcast and supporting segmentation for broadcast messages is more complex and is not needed as the reader can split the device IDs into multiple paging rounds and/or minimise the CFRA resources if the paging message exceeds the maximum R2D TB size

Proposal 4: Segmentation is not supported for paging message (i.e. MSG0)

Proposal 5: Wait for SA2 input before deciding whether unicast R2D messages (i.e. MSG2 onwards) can be segmented.

Proposal 6: ACK/NACK is not supported for R2D data

AS ID:

Proposal 7: The random number included in MSG1 (RN16) is used as the AS ID for scheduling subsequent A-IoT messages.

# References