3GPP TSG-RAN WG2 Meeting #128	R2-2409791

Orlando, FL, US, Nov 18-22 2024

Agenda Item:	8.1.2.3

Source:	NEC

Title:	LCM for Positioning use case

Document for:	Discussion, Decision

# Introduction

In the previous RAN2 meetings, the following agreement was reached for LCM for Positioning use case:

Agreements for positioning

1. The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline:

Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message.

Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side.

Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).

Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message.

Step 5: The LMF requests the inferred location information using the LPP request location information message.

Step 6: UE reports the inferred location using LPP provide location information message.

2.Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress).

3.Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress).

4. FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively).  FFS the signalling details.

5. RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

This paper discusses the remaining issues for life cycle management (LCM) for Positioning use case with AIML assistance.

# Discussion

## Applicable capability for AI positioning case 1

According to the discussion in the last RAN2 meeting, there is FFS on whether LMF controls the UE sending unsolicited LPP provide capabilities. In step-4 of the agreed basic procedure for LCM for UE sided model for AI positioning case 1, the UE reports the applicable functionality to the LMF by the LPP provide capabilities message. However, it is not clear if this message can be sent by the UE in a proactive manner or reactive manner.

Dependent on the positioning method and UE model/implementation, a change of some of the assistance data for UE positioning calculation may lead to availability or unavailability of AIML model, which may be subject to the change of network conditions also. Hence, in our opinion, for positioning Case 1, UE should be able to report a change in applicable functionality by sending applicable functionalities upon change via LPP signaling (i.e., LPP provide capabilities in step-4), and this can be seen as unsolicited information transfer.

Meanwhile, UE should be also able to provide applicable functionality reporting as a response to LMF request (solicited information transfer), which may be similar to the legacy positioning method.

Proposal-1: For AI/ML based positioning case 1, both unsolicited and solicited LPP provide capabilities message transmission should be supported.

## Model inference

For positioning use case, considering the specification impact will be mainly caused by the UE sided model, RAN2 mainly focus on cases 1, 2a and 2b for specification impact analyses.

The following agreements were reached during RAN1#116 with regards to model input for UE sided model:

Agreement

For AI/ML based positioning case 2b, at least the following types of time domain channel measurements are supported for UE reporting to LMF:

- timing information;

- paired timing information and power information.

Agreement

For AI/ML assisted positioning Case 2a, at least LOS/NLOS indicator and/or timing information are supported for reporting.

- If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214.

- If timing information is reported, the timing information at least can be reported via DL RSTD or UE Rx-Tx time difference as defined in 38.215.

- Note: details of the report are pending further discussion.

Agreement

For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input.

The following agreements were reached during RAN1#118bis with regards to model input for UE sided model:

Agreement

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference,

- for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other;

For case 1, it is UE-based positioning with UE-sided model, direct AI/ML positioning, it should be the UE that generates the measurement for AI/ML model inference based on the reception of DL PRS transmission. Meanwhile, according to RAN1 agreement, there may be a need for the LMF to indicate the purpose of the assistance data for UE-based positioning. The LPP signalling design at RAN2 should follow RAN1 progress.

Proposal 2: For AI/ML based positioning case 1, RAN2 may need to design the LPP signalling to indicate the purpose of the assistance data for UE-based positioning. (waiting for further RAN1 progress)

For case 2a, according to RAN1’s agreement, at least LOS/NLOS indicator and/or timing information are supported for reporting, i.e., the inference output of UE-sided model needs to be reported to LMF.

For case 2b, according to RAN1’s agreement, the timing information or the	paired timing information and power information needs to be reported to LMF.

At RAN1#116 meeting, there was a discussion on the possibility to support an indication in the UE measurement report to indicate that the reported timing information is generated by an AI/ML model. However that indication was not agreed, due to the fact that the companies believed that LMF should know if there is any model/functionality activated by the UE and then LMF can implicitly classify that UE measurement report as AIML model based report.

In legacy LPP signaling, the reporting of LOS/NLOS indicator and timing information have been supported for both LTE and NR.

RAN1 will be still discussing the detailed information for UE reporting, especially the timing information. Based on the detailed information for UE reporting, we can discuss whether using an existing LPP reporting procedure or a new reporting procedure. If an existing LPP signalling procedure is used, the semantics description of such legacy measurement report should be extended to cover the purpose of AIML based positioning.

Proposal 3: For AI/ML based positioning case 2a/2b, whether using an existing LPP reporting procedure or a new reporting procedure for UE measurement reporting should depend on further RAN1 discussion on the detailed information for UE reporting (e.g. timing information).

Proposal 3a: If an existing LPP signalling procedure is used, the semantics description of such legacy measurement report should be extended to cover the purpose of AIML based positioning.

## Performance Monitoring

In RAN1, there were many discussions during RAN1#116bis and RAN1#118 for performance monitoring for AI positioning use case. In summary, the following agreements reached are listed for model performance monitoring for Position use case for UE sided model.

RAN1#116bis Agreement

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label:

- Option A. The target UE side performs monitoring metric calculation.

- Option B. The LMF performs monitoring metric calculation.

RAN1#118 Agreement

Model monitoring related

For case 3a, both gNB and LMF performs monitoring metric calculation are feasible from RAN1 perspective. Further selection left to RAN3’s decision. Method to perform the metric is up to gNB/LMF implementation.

For case 1, RAN1 will not discuss Option A-4(monitoring data transferred from PRU to target UE with specification transparent way).

Meanwhile, there are different options identified for both Option A and Option B for UE sided model. Different options require a bit different signalling exchange between the UE and the LMF. As RAN1 will continue the discussion and RAN3 may discuss the metric calculation in either gNB or LMF. However, to support the the metric calculation at gNB or LMF, air interface based performance data may be required and signalling based approach may be subject to RAN2 discussion .of the options, RAN2 can wait for the outcome and design the signalling accordingly when it is ready.

Proposal-4: For model performance monitoring for Position use case for UE sided model, wait for RAN1 and RAN3 progress.

Proposal-5: RAN2 discuss the signalling support to transfer to the performance data from UE to gNB or LMF.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: For AI/ML based positioning case 1, both unsolicited and solicited LPP provide capabilities message transmission should be supported.

Proposal 2: For AI/ML based positioning case 1, RAN2 may need to design the LPP signalling to indicate the purpose of the assistance data for UE-based positioning. (waiting for further RAN1 progress)

Proposal 3: For AI/ML based positioning case 2a/2b, whether using an existing LPP reporting procedure or a new reporting procedure for UE measurement reporting should depend on further RAN1 discussion on the detailed information for UE reporting (e.g. timing information).

Proposal 3a: If an existing LPP signalling procedure is used, the semantics description of such legacy measurement report should be extended to cover the purpose of AIML based positioning.

Proposal-4: For model performance monitoring for Position use case for UE sided model, wait for RAN1 and RAN3 progress.

Proposal-5: RAN2 discuss the signalling support to transfer to the performance data from UE to gNB or LMF.

# Annex: Agreement made by RAN1#118bis for AI/ML based positioning

Agreement

For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is:

- When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455;

  - FFS: details on how to associate quality indicator to timing information

Conclusion

For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

Note: the UE can be a PRU and/or a Non-PRU UE.

Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Agreement

From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) Channel measurement;

- (Optional) Quality of the channel measurement;

  - FFS: details of the quality

- (Mandatory) Time stamp of the channel measurement.

Agreement

From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) timing information;

- (Optional) Quality of the timing information;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp.

FFS: LOS/NLOS indicator.

Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

Agreement

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference,

- for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other;

- Companies can provide inputs on further enhancements of existing assistance data, including new information

- Note: this does not mean that training and inference phases are mentioned in assistance data.

Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable.

|  | Information | UE-based DL-TdoA | UE-based DL-AoD |
| --- | --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |  |  |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 3 | DL-PRS configuration of candidate NR TRPs |  |  |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |  |  |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |  |  |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |  |  |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |  |  |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 9 | PRS-only TP indication |  |  |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID |  |  |
| 11 | LOS/NLOS indicators |  |  |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |  |  |
| 13 | Validity Area of the Assistance Data |  |  |
| 14 | PRU measurements together with the location information of the PRU |  |  |
| 15 | Data facilitating the integrity results determination of the calculated location |  |  |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |  |  |
| 17 | Expected Angle Assistance information |  |  |
| 18 | PRS priority list |  |  |