3GPP TSG-RAN WG2 Meeting #128	R2-2409792

Orlando, FL, US, 18 – 22 Nov 2024

Agenda Item:	8.2.4

Source:	NEC

Title:	Random Access for Ambient IoT device

Document for:	Discussion, Decision

# Introduction

In the last RAN2 meeting, the following agreements were reached for the random access procedure for Ambient IoT device:

Agreements for Random Access

- In case of D2R data transmission failure, device follows the reader instruction:

	it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and

	reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R).

2	Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.  FFS for following D2R data.

3	For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access.  The re-access is always controlled by reader.

4	For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure.

5	R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message.   FFS additional indications to differentiate if needed in paging message.    FFS if other message can be used.

6	From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader, as the baseline for CBRA.  FFS if this is applicable to FDMA.  Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA.

This paper intends to discuss the controversial issues following the email discussion above.

# Discussion

## Selection of 2-step vs 3-step CBRA

Currently, both 2-step and 3-steps RA are supported in A-IoT, on the association between available ROs and 2-step and/or 3-steps RA, we have following options:

- Option 1: available ROs are associated to 2-step RA only;

- Option 2: available ROs are associated to 3-step RA only;

- Option 3: available ROs are associated to both 2-setp and 3-steps

On how to indicate the association between available ROs and 2-step and/or 3-steps RAs, we see following two alternatives:

- Alternative 1: AIoT paging message explicitly indicates association between available ROs and 2-setp and/or 3-steps ROs;

- Alternative 2: Pre-define association between available ROs and 2-step and/or 3-steps RA in the specification.

Since Alternative 1 is much more flexible, we propose to support Alternative 1 in A-IoT system.

Proposal-1: AIoT paging message explicitly indicates following associations between available ROs and 2-setp and/or 3-steps ROs:

- available ROs are associated to 2-step RA only;

- available ROs are associated to 3-step RA only;

- available ROs are associated to both 2-setp and 3-steps.

When AIoT paging message explicitly indicates that available ROs are associated to 2-step RA only or 3-step only, RA type supporting can be considered as an AS layer filter criteria for intended devices, i.e., each device matching the AS layer criteria (i.e., supporting 2-step RA or supporting 3-step RA) is expected to respond to the certain A-IoT paging message. On the other hand, when AIoT paging message explicitly indicates that available ROs are associated to both 2-step and 3-step RAs (or implicitly indicates it by not containing the RA type supporting information in the A-IoT paging message), then devices supporting either 2-step or 3-step RA are expected to respond the certain A-IoT paging message. In such a case, to simplify reader reception processing, it would be beneficial to early indicate RA type of the responded device. Like early indication supporting in the NR, two options can be studied further:

- Option 1: early indicates the RA supporting type via separated random access resource for 2-step and 3-step RA provided by the A-IoT paging message (like NR Msg.1 early indication)

- Option 2: early indicates RA supporting type via indication in MAC subheader for differentiating Msg.1 of 2-step and 3-step (like NR Msg.3 early indication)

Proposal-2: For early indicating the RA type, RAN2 agree to capture the following two options to TR.

- Option 1: early indicates RA supporting type via separated random access resource for 2-step and 3-step RA provided by the A-IoT paging message (like NR Msg.1 early indication)

- Option 2: early indicates RA supporting type via indication in MAC subheader for differentiating Msg.1 of 2-step and 3-step (like NR Msg.3 early indication)

One more issue is whether down selection for RA types (2step, 3step, CFRA) in SI/WI phase is needed or not. Compared to 3-step RA, 2-step RA can achieve a shorter “AIoT service latency” (for example, a shorter inventory latency). Also with Proposals 1 and 2, we believe both 2-step and 3-step can be supported without significant complexity increasing.

Proposal-3: It is no need to down select RA type (2step, 3step, CFRA) in SI/WI phase. Both 2-step and 3-step can be supported without significant complexity increasing.

## RACH resource selection for Ambient IoT device

In order to select a time slot to respond to the reader via random access procedure, the AIoT device needs to know the exact time slots that can be used to initiate its random access. In addition, according to the discussion at RAN1, the FDM approach was also discussed on top of the TDM based access. FDM based approach may be used to afford large number of AIoT device access, which allows different devices to access the reader via a different frequency bandwidth at the same time slot. Based on this background, we discuss below what type of parameters the reader should announce before the AIoT device can initiate its random access.

The initial triggering message sent by the reader should include a set of access occasions allocated for RA (e.g. M time domain resources and N frequency domain resource) within one frame or one access round.

Firstly, we focus on the discussion of time domain aspects. Assuming intended devices information (i.e., filter criteria from CN) is also provided in the initial trigger message, each device matching the filter criteria may generate a random number according to the number of the available time resources(e.g. M) in the initial triggering message for random access. The random number should be in the range [0, (M-1)].

Proposal-4: Each AIoT device matching the filter criteria generates a random number for time resource determination according to the number of the available time resources in the initial triggering message for random access if slotted ALOHA based access is chosen for AIoT device.

Based on the random number generated, the AIoT device may start “RACH-time resource” counting down since receiving “initial triggering message” command.

For RFID, the slotted ALOHA access mechanism highly relies on the running of “QUERYREP” command, which allows the tag to count down when receiving such command at every time resource. If AIoT device uses the similar approach, the AIoT device may need to count down whenever receiving a “Repeat” (or start indication of a new time resource) command from the reader following the initial triggering message. The “Repeat” (or start indication of a new time resource) command may be used by the device to maintain the synchronization or clock tracking. It should be noted that the running of “Repeat” (or start indication of a new time resource) command is a big signalling overhead for the AIoT air interface and also it requires the device to monitor the message from the reader to decide the start of each time resource, which may cause energy consumption for AIoT device.

Observation-1: the run of “QUERYREP” command (for RFID) is a big signalling overhead for the AIoT air interface and also it requires the device to monitor the message from the reader for determining the start of each time resource time slot, which may cause energy consumption for AIoT device.

RAN1 is studying TDMA for Msg.1 transmissions from multiple devices corresponding to a PRDCH providing a triggering message. Agreements of the last RAN1# meeting was:

Agreement@RAN1#118bis

RAN1 studies following:

- A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s).

- The study includes

  - Study X=1 and X>1 and X>=1, the maximum value of X>1 should be set considering the device implementation complexity, device power consumption, the resource usage efficiency affected at least by SFO, and inventory latency.

  - Size(s) for resource allocation in the time domain

  - Determination of the X time domain resource(s) by the device

  - Addressing timing errors for adjacent time domain resources due to residual SFO of the device

In our understanding, start indication of a new time resource will be needed only for the case X=1. Therefore, Proposal 4 can be considered as the baseline and RAN2 can further discuss necessary mechanism for X>1 and X>=1 cases after RAN1 making the final decision.

In addition to the RA time resource selection, the device also needs to randomly selects one frequency resource among the available frequency resources (e.g. N) in the initial triggering message for sending MSG-1 to the reader.

Proposal-5: in addition to the RA time resource selection, the device also needs to randomly selects one frequency resource among the available frequency resources (e.g. N) in the initial triggering message for sending MSG-1 to the reader.

Meanwhile, it is very important to note that when there are multiple frequency domain resources for performing  RA, RAN2 may need to study additional mechanisms or enhancements for AIoT device access, since anyway the slotted ALOHA mechanism cannot be simply reused any more.

As discussed above, an RO in A-IoT can be defined as a combination of a time resource  and a frequency resource . RAN2#126 agreed that “AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message).”, therefore it is nature to assume that the total available ROs (for example, M time resources and N frequency resources) is indicated by AIoT paging message.

Proposal-6: the total available ROs (including both time and frequency resources) is indicated by AIoT paging message.

## AIoT Device Re-Access

In the last meeting, it was agreed that R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message. There was an FFS on additional indications to differentiate if needed in paging message. In our understanding, in a particular access round, when the repeated AIoT paging message is received by the device, the AIoT device will now this paging message is the same within the current access round. This mechanism is like the “QUERYREP” command used by RFID. We assume that a particular access round be governed by a specific time window. Within that time window, repeated response is not expected from the AIoT device. For example, for an access round window, there may be 3 AIoT paging messages sent over AIoT air interface, some AIoT device may respond to the AIoT reader when receiving the first AIoT paging messages, the others may respond the second or the third AIoT paging message. If one AIoT device experiences access failure during its response to the first AIoT paging message, it can re-access following its reception of the second or the third AIoT paging message.

In this manner, the A-IoT paging message do not need to indicate the device about whether it is for a new service request or a re-transmission for the old service request. Indicating the service request for a new service request or a re-transmission limits the devices to access, which is not resource efficient.

Proposal-7: AIoT device can re-access AIoT reader following the repeated AIoT paging messages within a time window defined for an Access Round.

Proposal-8: The A-IoT paging message do not need to indicate the device about whether it is for a new service request or a re-transmission for the old service request.

# Conclusion and Proposal

We have the following observations and proposals:

Observation-1: the run of “QUERYREP” command (for RFID) is a big signalling overhead for the AIoT air interface and also it requires the device to monitor the message from the reader for determining the start of each time resource time slot, which may cause energy consumption for AIoT device.

Proposal-1: AIoT paging message explicitly indicates following associations between available ROs and 2-setp and/or 3-steps ROs:

- available ROs are associated to 2-step RA only;

- available ROs are associated to 3-step RA only;

- available ROs are associated to both 2-setp and 3-steps.

Proposal-2: For early indicating the RA type, RAN2 agree to capture the following two options to TR.

- Option 1: early indicates RA supporting type via separated random access resource for 2-step and 3-step RA provided by the A-IoT paging message (like NR Msg.1 early indication)

- Option 2: early indicates RA supporting type via indication in MAC subheader for differentiating Msg.1 of 2-step and 3-step (like NR Msg.3 early indication)

Proposal-3: It is no need to down select RA type (2step, 3step, CFRA) in SI/WI phase. Both 2-step and 3-step can be supported without significant complexity increasing.

Proposal-4: Each AIoT device matching the filter criteria generates a random number for time resource determination according to the number of the available time resources in the initial triggering message for random access if slotted ALOHA based access is chosen for AIoT device.

Proposal-5: in addition to the RA time resource selection, the device also needs to randomly selects one frequency resource among the available frequency resources (e.g. N) in the initial triggering message for sending MSG-1 to the reader.

Proposal-6: the total available ROs (including both time and frequency resources) is indicated by AIoT paging message.

Proposal-7: AIoT device can re-access AIoT reader following the repeated AIoT paging messages within a time window defined for an Access Round.

Proposal-8: The A-IoT paging message do not need to indicate the device about whether it is for a new service request or a re-transmission for the old service request.

# Reference

- Report of [POST127][033][AIoT] Random Access (Huawei)

- TS38.300