3GPP TSG-RAN WG2 Meeting #128	R2-2409798

Orlando, US, October 18th – 22th, 2024

Agenda Item:	8.7.5

Source: 	Canon Research Centre France

Title:	Discussion on RLC AM Enhancements

Document for:	Discussion

# Introduction

During the RAN2#127bis meeting at Hefei ([1]), RAN2 discussed RLC AM and reached the following agreements:

Agreements on RLC timely retransmissions - RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. - Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay Agreements on avoiding unnecessary retransmissions - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

In this contribution we discuss both how to avoid unnecessary retransmission (2.1), and how to ensure timely RLC retransmission (2.2).

# Discussion

## Avoid unnecessary retransmission

It has been agreed that the RLC receiving window must be updated prior to the transmitting window. It has also been agreed that TX and RX side will discard PDUs, based on different timers.

As explained in [2], RX side timers are inaccurate, and will most likely elapse after the TX side timers. The time difference between TX and Rx side timer elapsed constitutes the delay for updating both windows.

Contrary to TX side, RX side has no knowledge of PDU Sets at any layer. This is because PDU Sets information, including PDU Set boundary and sequence numbers, are not included inline. How RX side can determine that an SDU is obsolete is therefore a difficult issue.

It is likely that the RX side timer would be started upon a sequence number gap detection and would be set to a value related to some delay budget. But the time to detect a sequence number gap is unpredictable, with a worst case when the last PDU of a PDU Set is lost, the sequence number gap will only be detected at the successful transmission of a PDU of a further PDU Set.

During the extra time induced by the sequence number gap detection, the transmitting and receiving windows are out of synchronization.

Observation 1: Window updating is stalled for the duration of TX to RX timer setting difference.

The goal of the combined TX/RX approach is to avoid unnecessary retransmission, but it seems to have an undesirable side effect that affects the system latency by stalling the TX and RX window update.

It is therefore proposed to add a signalling from Tx to Rx allowing faster TX to RX window synchronization.

Proposal 1: To speed-up the TX to RX window synchronization, an indication is sent from Tx to Rx, when the TX side discards SDUs.

The RX window will be updated based on the discard notification received from the transmitting entity. Thus, it is possible to adopt the Move Receiving Window (MRW) command as defined in UMTS ([6]):

An SDU can be discarded with explicit signaling when MaxDAT number of retransmissions is reached or the transmission time exceeds a predefined value (Timer_Discard) for an SDU in acknowledged mode RLC. Move Receiving Window (MRW) command is sent to the receiver so that AMD PDUs carrying that SDU are discarded in the receiver and the receiver window is updated accordingly…

In order to limit the signalling overhead, we propose to merge the MRW command with the discard notification signalling.

Proposal 2: Adopt as baseline MRW signalling for RX side and Tx side signalling

## Timely RLC retransmission

### Autonomous retransmission

One way to reduce the retransmission delay is to anticipate the retransmission of PDUs with unknown status. The unknown status is indicative of the PDU having been transmitted to the receiver but not acknowledged as being received or detected lost by the receiver.

This enables earlier and more efficient retransmission of some PDUs, which thereby minimizes the overhead when compared to systematic retransmission.

The overhead may be reduced if the autonomous retransmission of PDUs with unknown status is not systematically triggered.

The autonomous retransmission of PDUs having an unknown status may occur following the first transmission of all SDUs of a PDU Set.

The autonomous retransmission of PDUs having an unknown status may occur when the transmit and retransmit buffers are empty.

To decrease even more the needed resources, the autonomous retransmission trigger can be further extended to a remaining time threshold applied to the transmit window. If the transmit window is stalled up to the remaining time threshold of a PDU Set, then autonomous retransmission may be triggered.

Observation 2: Careful design of autonomous retransmission trigger allows drastic reduction of unnecessary SDU retransmission

### Enhanced polling

The maximum polling frequency can be configured as one polling bit every 4 packets. But polling frequency cannot match the PDU Sets boundaries because PDU Set boundaries are variable.

In [5], the authors suggest to reduce the t-PollRetransmit timer below 5 ms.

Observation 3: Increasing the polling frequency can result in a large number of non-useful status reports being sent.

In [3] and [4], the authors suggest initiating polling by the transmitter based on remaining time information.

However, if the polling bit triggers a status report based on PDB, but the t-Reassembly timer is still running, there will be no nack information sent back to the transmitter.

Also, the prohibit timer might prevent the receiver to send back a status report.

Observation 4: There is a need for certain polling bits to be interpreted differently at the receiving side.

For example, a polling bit sent based on remaining time information is enhanced with dedicated signalling that allows the receiving entity to handle differently the reassembly and prohibit timers.

Proposal 3: Enhance the polling bit signalling to indicate a low remaining time condition.

### Comparison

In term of packet delay, autonomous retransmission achieves better performances since the retransmission decision does not rely on a prior exchange of signalling with the receiving side. The gain in delay reduction can be substantially higher in favour of autonomous retransmission if the conditions of polling bit triggering are not enhanced.

Observation 5: Autonomous retransmission achieves better retransmission delay

In term of capacity cost, the autonomous transmission may trigger some unnecessary SDU retransmission while the enhanced polling generates unnecessary signalling exchange. The difference is not obvious to quantify, there are numerous factors to take into account like the design of autonomous retransmission trigger, the size of the SDUs, or the frequency of the polling.

Observation 6: Capacity impact of autonomous retransmission can be managed through careful design of triggering condition.

In term of complexity, autonomous retransmission has impact only on the TX side while the polling needs to be enhanced at TX side for triggering, and at RX side for timer management.

Observation 7: Autonomous retransmission is less complex than polling enhancement.

Proposal 4: RAN2 to adopt autonomous retransmission of PDUs with unknown status.

# Conclusion

In this contribution, we conclude the following observations and proposals.

Observation 1: Window updating is stalled for the duration of TX to RX timer setting difference.

Proposal 1: To speed-up the TX to RX window synchronization, an indication is sent from Tx to Rx, when the

TX side discards SDUs.

Proposal 2: Adopt as baseline MRW signalling for RX side and Tx side signalling

Observation 2: Careful design of autonomous retransmission trigger allows drastic reduction of unnecessary SDU retransmission

Observation 3: Increasing the polling frequency can result in a large number of non-useful status reports being sent.

Observation 4: There is a need for certain polling bits to be interpreted differently at the receiving side.

Proposal 3: Enhance the polling bit signalling to indicate a low remaining time condition.

Observation 5: Autonomous retransmission achieves better retransmission delay

Observation 6: Capacity impact of autonomous retransmission can be managed through careful design of triggering condition.

Observation 7: Autonomous retransmission is less complex than polling enhancement.

Proposal 4: RAN2 to adopt autonomous retransmission of PDUs with unknown status.

# References

- R2-2409215, Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei)

- R2-2409208, Discussion on RLC re-transmission related enhancements	OPPO, China Telecom, Sharp, Samsung, HONOR, CMCC, KDDI, Apple, Intel, vivo, MediaTek, ZTE, Canon

- R2-2404850, RLC re-transmission enhancements for XR, ITRI

- R2-2405445, Solutions for RLC AM retransmission enhancement, TCL

- R2-2405032, Discussion on RLC Retransmission Enhancements in XR, CMCC

- UMTS TS 125 322 V4.0.0 (2001-03)