3GPP TSG-RAN WG2 Meeting #128	R2-2409819

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.7.5

Source:	Nokia, Nokia Shanghai Bell

Title:	RLC Enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#127bis:

RLC timely retransmissions RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. Exclude enhanced status reporting. Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay Avoiding unnecessary retransmissions RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

Based on those, this contribution investigates further details of avoiding unnecessary retransmissions and guaranteeing timely RLC retransmissions.

# 2	Avoiding unnecessary retransmissions

## 2.1	How to indicate abandoned SDUs to transmitter

The agreements in RAN2#127bis assumed as a baseline that also RLC SDUs not received and abandoned by the receiver are indicated to the transmitter using RLC ACK. As a result, if a PDCP PDU is RLC ACKed, but the transmitting RLC and PDCP entities do not know whether the PDU is successfully delivered, or whether it has been abandoned, the RLC ACKs will no longer serve their current purpose. While this is straightforward from RLC point of view, received RLC ACKs are also used locally by PDCP, for instance to determine how high-numbered PDCP SDUs can be transmitted by PDCP:

NOTE 1: Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

Observation 1: If a received RLC ACK passed to PDCP no longer reliably indicates successful delivery to the peer PDCP entity, regular PDCP status reporting is needed for the transmitting PDCP to know how high-numbered PDCP SDUs it can proceed to transmit.

As a replacement, regular Status reporting would seem to be needed at PDCP.

Proposal 1: For indicating abandoned RLC SDUs from RLC receiver to transmitter, in order that the transmitting PDCP reliably knows how high-numbered PDCP SDUs it can proceed to transmit, RAN2 select between:

A)	RLC ACK, combined with regular PDCP status reporting to keep the PDCP transmitter reliably informed of successful delivery; or

B)	A new explicit RLC indication separate from ACK, of SDUs abandoned by the receiver.

## 2.2	Handling of PDCP Control PDUs

One drawback mentioned against the RX-initiated approach is the handling of control PDUs [R2-2409208]. The following lists the PDCP Control PDUs currently defined and discusses the severity if abandoned at RLC by the receiver:

-	PDCP Status Report:

-	The purpose of this is to minimize unnecessary PDCP retransmissions at RLC-layer discontinuities (for instance after handover). If abandoned by RLC, it is anyway likely to be too delayed at RLC to have benefited PDCP even if successfully delivered at that time.

-	PDCP SN Gap Report:

-	The purpose is to minimize unnecessary PDCP reordering delay. If abandoned by RLC, it is anyway likely to be too delayed at RLC to have benefited PDCP even if successfully delivered at that time.

-	Feedback for ROHC, EHC, or UDC compression:

-	The first two also apply to bearers mapped on RLC UM, meaning that their occasional loss at RLC can be expected.

-	UDC feedback only applies to bearers mapped on RLC AM, because so does UDC itself, whose operation was deemed to rely on reliable and in-sequence delivery of data [R2-1710721]. The sole purpose of this feedback is to request the UE to reset its compression buffer because the network detected a checksum error at reception. The UE then confirms this reset by setting the FR bit (as defined in TS 38.323 section B.2.2.2) in a following uplink UDC header. If the feedback is abandoned at RLC, the network will not observe the expected FR bit and can re-send the feedback Control PDU.

Observation 2: RLC abandoning PDCP Control PDUs for Status report, SN Gap report, or ROHC or EHC feedback will not cause problems. Abandoning of UDC feedback can be handled by network implementation.

Proposal 2: No mechanism is introduced at RLC to prevent abandoning of PDCP Control PDUs.

## 2.3	Discard indication from transmitter to receiver

As already apparent from the Rel-18 discussions part of introducing PDCP SN gap report, whether or not the protocol performs reordering is pivotal in terms of whether or not a discard indication from transmitter to receiver is useful in addition to receiver-autonomous advancing of the receiving window. RAN2 was unanimous in their view that when the protocol delivers received SDUs to upper layer out of order, the discard indication is not a useful addition, and thus not configured [38.331]:

-	sn-GapReport: Indicates whether the PDCP entity is configured to send a PDCP SN gap report in the uplink, as specified in TS 38.323 [5]. This field is only configured for DRBs. This field can be included only if outOfOrderDelivery is not configured.

Observation 3: an indication of abandoned SDUs from transmitter to receiver is only a useful addition when reordering is performed.

Given that also RLC AM does not reorder received SDUs, the same conclusion should be valid.

Proposal 3: No discard indication from RLC transmitter to receiver is introduced.

# 3	Timely retransmissions

## 3.1	Autonomous retransmissions

Since autonomous retransmissions do not rely on explicit knowledge of failed RLC SDUs (and are not triggered in case of timely RLC ACKs), new conditions should be defined to trigger such retransmissions. In the following we list some of the options that have been proposed during previous meetings:

1.	Timer-based triggers: the TX RLC will start retransmitting the RLC SDUs when a timer associated with the corresponding SDU falls below a preconfigured threshold. Three possible alternatives:

a.	Relying on the existing PDCP indication of the RLC SDU becoming delay-critical. This option is coupled with DSR being configured which creates unnecessary dependencies between the features.

b.	Relying on the remaining time of a SDU based on PDCP discard timer. This option might need to factor in the shorter PDCP discard timer applied to low importance PDU in case of congestion (so that they do not get prioritized wrongly).

c.  A new RLC retransmission timer. This new timer may start upon submission of RLC PDU to MAC.

2.	HARQ-aware triggers: this option relies on the knowledge from the HARQ status of first attempt of the RLC transmission and tries to initiate RLC retransmission if HARQ does not seem to be able to deliver the TB(s) originated from the RLC AM entities’ SDU(s) in a timely manner. The main benefit of this option is that the RLC will have a more informed decision for retransmissions based on the status of the first transmission which in turn will reduce the negative capacity impact of autonomous methods. One main concern that needs to be addressed for the HARQ-aware schemes is to have knowledge about the mapping between TBs and RLC SDUs. Two possible solutions for triggering timely retransmissions based on HARQ-awareness:

a.	Indication from HARQ regarding several failed retransmissions: one challenge with this solution in UL is that the UE only knows about a failed HARQ transmission once it gets a scheduling grant for a new retransmission, at which point an ARQ retransmission can be initiated together with the upcoming HARQ retransmission. Therefore, the lack of explicit HARQ feedback in DL results in added delays for triggering RLC retransmissions which hinders the timeliness on them.

b.	Explicit indication from HARQ for last HARQ retransmission attempted failure: Based on an explicit indication from the gNB, informing the UE that the last attempt of retransmission for a certain HARQ process has failed. This requires new PHY procedures that need to be discussed in RAN1.

3.	Available PUSCH resources: relying on the bursty properties of XR traffic, a simple indication that RLC retransmissions are needed would be the scheduling of a TB after transmitting all RLC PDUs of a burst once.

The simpler trigger seems to be option 1.c (new RLC retransmission timer), while option 2 should be ruled out due its high complexity with regards to unknown mapping of RLC SDUs and TBs and also involving layer 1 coordination. Option 3 needs to be studied further

Proposal 4: for autonomous retransmissions, a new RLC retransmission timer is introduced. The availability of PUSCH resources could also be investigated further. HARQ-aware triggers are ruled out.

The main concerns expressed with autonomous retransmissions had to do with capacity implications. However, the autonomous retransmissions can be kept to a minimum provided that RLC ACKs are provided rapidly. What makes the current RLC Status PDU difficult for rapid acknowledgment is that, among the SDUs that are acknowledged, all the other SDUs are negatively acknowledged. The t-Reassembly timer avoids such premature NACKs but also holds back the ACKs.

Observation 4: among the SDUs that are acknowledged in a RLC Status PDU, all the other SDUs are always negatively acknowledged.

Since RAN2 agreed to focus specifically on timely uplink retransmissions, a complete solution to this problem would be to make it configurable whether the UE considers SDUs for retransmission when NACKed. A UE that would not, would allow the network to send RLC ACKs rapidly without any concerns about premature NACKs.

Proposal 5: to minimize autonomous retransmissions, allow the network to send frequent ACKs, by introducing a possibility to configure the UE not to consider SDUs for retransmission based on received NACKs (to eradicate the problem of premature NACKs).

Another mechanism that may hold back RLC ACKs is LCP. Currently in LCP, there is no consideration regarding prioritizing control PDUs such as RLC Status PDU. The RLC Status PDU is prioritized over data PDU only within the same bearer/LCH when the LCH is treated in LCP. As a result, during LCP, the LCHs with RLC Status PDU awaiting transmission, may end up not getting enough allocation to put such PDUs in the MAC PDU being generated for transmission, delaying transmission of these PDUs.

Proposal 6: RAN2 to discuss possible enhancements in LCP regarding high priority RLC Status PDUs.

## 3.2	Polling enhancements

As discussed above, the current t-Reassembly guards against sending premature NACKs. On the other hand, as soon as t-Reassembly expires and qualifies one or more new missing SDUs for confident NACKing, status reporting is triggered immediately. No matter how much polling is increased, this process cannot be expedited. Therefore, enhancing polling can only expedite ACKs and not NACKs. and the gains from expedited ACKs for helping to trigger timely retransmissions is questionable.

Observation 5: the current RLC already sends a NACK as soon as t-Reassembly ensures that it is not premature. Therefore increased polling can only expedite ACKs, not NACKs, and cannot help with timely retransmissions.

Proposal 7: RAN2 exclude polling enhancements and focus on autonomous retransmissions.

# 4	Conclusion

This document has made the following proposals:

Proposal 1: For indicating abandoned RLC SDUs from RLC receiver to transmitter, in order that the transmitting PDCP reliably knows how high-numbered PDCP SDUs it can proceed to transmit, RAN2 select between:

A)	RLC ACK, combined with regular PDCP status reporting to keep the PDCP transmitter reliably informed of successful delivery; or

B)	A new explicit RLC indication separate from ACK, of SDUs abandoned by the receiver.

Proposal 2: No mechanism is introduced at RLC to prevent abandoning of PDCP Control PDUs.

Proposal 3: No discard indication from RLC transmitter to receiver is introduced.

Proposal 4: for autonomous retransmissions, a new RLC retransmission timer is introduced. The availability of PUSCH resources could also be investigated further. HARQ-aware triggers are ruled out.

Proposal 5: to minimize autonomous retransmissions, allow the network to send frequent ACKs, by introducing a possibility to configure the UE not to consider SDUs for retransmission based on received NACKs (to eradicate the problem of premature NACKs).

Proposal 6: RAN2 to discuss possible enhancements in LCP regarding high priority RLC Status PDUs.

Proposal 7: RAN2 exclude polling enhancements and focus on autonomous retransmissions.

In the Annex, we also provide a baseline text proposal for the Combined approach for avoiding unnecessary retransmissions.

# Annex – Text proposal for Combined approach

### 5.2.3	AM data transfer

#### 5.2.3.2	Receive operations

##### 5.2.3.2.1	General

The receiving side of an AM RLC entity shall maintain a receiving window according to the state variable RX_Next as follows:

-	a SN falls within the receiving window if RX_Next <= SN < RX_Next + AM_Window_Size;

-	a SN falls outside of the receiving window otherwise.

When receiving an AMD PDU from lower layer, the receiving side of an AM RLC entity shall:

-	either discard the received AMD PDU or place it in the reception buffer (see clause 5.2.3.2.2);

-	if the received AMD PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly and t-Abandon as needed (see clause 5.2.3.2.3).

When t-Reassembly or t-Abandon expires, the receiving side of an AM RLC entity shall:

-	update state variables and start t-Reassembly or t-Abandon, respectively, as needed (see clause 5.2.3.2.4 or 5.2.3.2.X, respectively).

##### 5.2.3.2.3	Actions when an AMD PDU is placed in the reception buffer

When an AMD PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall:

-	if x >= RX_Next_Highest:

-	update RX_Next_Highest to x+ 1.

-	if all bytes of the RLC SDU with SN = x are received:

-	reassemble the RLC SDU from AMD PDU(s) with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer;

-	if x = RX_Highest_Status:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN > current RX_Highest_Status for which not all bytes have been received.

-	if x = RX_Next:

-	update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received.

-	if t-Reassembly is running:

-	if RX_Next_Status_Trigger = RX_Next; or

-	if RX_Next_Status_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or

-	if RX_Next_Status_Trigger falls outside of the receiving window and RX_Next_Status_Trigger is not equal to RX_Next + AM_Window_Size:

-	stop and reset t-Reassembly.

-	if t-Reassembly is not running (includes the case t-Reassembly is stopped due to actions above):

-	if RX_Next_Highest> RX_Next +1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

-	if t-Abandon is running:

-	if RX_Abandon_Trigger = RX_Next; or

-	if RX_ Abandon _Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or

-	if RX_Abandon_Trigger falls outside of the receiving window and RX_Abandon_Trigger is not equal to RX_Next + AM_Window_Size:

-	stop and reset t-Abandon.

-	if t-Abandon is configured and is not running (includes the case t-Abandon is stopped due to actions above):

-	if RX_Next_Highest> RX_Next +1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	start t-Abandon;

-	set RX_Abandon_Trigger to RX_Next_Highest.

##### 5.2.3.2.X	Actions when t-Abandon expires

When t-Abandon expires, the receiving side of an AM RLC entity shall:

-	update RX_Next to the SN of the first SN >= RX_Abandon_Trigger for which not all bytes have been received;

-	discard all segments with SN < updated RX_Next;

-	if RX_Next_Highest > RX_Next + 1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the RLC SDU associated with SN = RX_Next before the last byte of all received segments of this RLC SDU:

-	start t-Abandon;

-	set RX_Abandon_Trigger to RX_Next_Highest.

## 5.4	SDU discard procedures

When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity configured with t-Abandon shall discard the indicated RLC SDU along with any corresponding RLC Data PDUs.

When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity not configured with t-Abandon or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity not configured with t-Abandon shall not introduce an RLC SN gap when discarding an RLC SDU.

## 7.1	State variables

<…>

The receiving side of each AM RLC entity configured with t-Abandon shall maintain the following state variables:

a) RX_Abandon_Trigger – t-Abandon state variable

This state variable holds the value of the SN following the SN of the RLC SDU which triggered t-Abandon.

<…>

## 7.3	Timers

The following timers are configured by TS 38.331 [5]:

a) t-PollRetransmit

This timer is used by the transmitting side of an AM RLC entity in order to retransmit a poll (see clause 5.3.3).

b) t-Reassembly

This timer is used by the receiving side of an AM RLC entity and receiving UM RLC entity in order to detect loss of RLC PDUs at lower layer (see clauses 5.2.2.2 and 5.2.3.2). If t-Reassembly is running, t-Reassembly shall not be started additionally, i.e. only one t-Reassembly per RLC entity is running at a given time.

c) t-StatusProhibit

This timer is used by the receiving side of an AM RLC entity in order to prohibit transmission of a STATUS PDU (see clause 5.3.4).

X) t-Abandon

When configured, this timer is used by the receiving side of an AM RLC entity in order to abandon unreceived AMD PDUs (see clause 5.2.3.2). If t-Abandon is running, t-Abandon shall not be started additionally, i.e. only one t-Abandon per RLC entity is running at a given time.