3GPP TSG-RAN WG2 Meeting #128	R2-2409830

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:	8.1.3

Source:	Samsung

Title:	Discussion on NW side data collection

Document for:	Discussion & Decision

Introduction

This document discusses on NW side data collection for AI/ML air interface WI.

Discussion

Memory capability

The following agreement was made in RAN2#127, in order for UE to log training data for NW side data collection:

<Agreements (RAN2#127)> - UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases.

In legacy, there are existing memory requirements for UE to log measurement results. Firstly, for logged MDT, the fixed size (i.e., 64KB) of minimum memory requirement is defined. This is commonly mandatory for UEs who support logged MDT, and no separate capability is used for this memory requirement.

<TS 38.306> loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE state. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The minimum memory size of MDT logged measurements is 64KB. For eRedCap UE supporting this feature, the minimum memory size of MDT logged measurements is 16KB.; UE; No; No; No.

Meanwhile, there are two separate memories defined for QoE measurement.

- Memory 1. This is the memory for UEs to log QoE reports in RRC_CONNECTED while QoE reporting is temporarily paused by NW. The mandatory minimum requirement for this memory is 64KB as highlighted below.

- Memory 2. This is the memory for UE to log QoE reports in RRC_IDLE and RRC_INACTIVE for MBS broadcast service. The mandatory minimum requirement for this memory is 64KB as highlighted below. Besides, UE optionally may support more than 64KB for this memory. In this case, UE indicates the size (i.e., larger than 64KB) it supports to NW via an explicit capability indication.

<TS 38.306> # 6 Conditionally mandatory features without UE radio access capability parameters Features; Condition. Acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList; It is mandatory to support acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList for UEs which support the acquisition of the posSIB types in posSchedulingInfoList as specified in TS 38.331 [9].. Acquisition of SI messages with explicit SI window positions; It is mandatory to support acquisition of SI messages with explicit SI window positions for UEs which support the SIB types in schedulingInfoList2 as specified in TS 38.331 [9].. AS layer memory size for QoE paused measurement reports; It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17.. AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE; It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports". <SKIP> ## 4.2 UE Capability Parameters <SKIP> qoe-AdditionalMemoryMeasReport-r18 Indicates the minimum AS layer memory size the UE supports for QoE measurement in RRC_IDLE and RRC_INACTIVE in addition to the "AS layer memory size for QoE paused measurement reports". Value kB128 means the UE supports at least 128 kilobytes for this purpose, and so on. A UE supporting this feature shall also support qoe-IdleInactiveMeasReport-r18.; UE; No; No; No. <TS 38.331> #### – AppLayerMeasParameters The IE AppLayerMeasParameters is used to convey the capabilities supported by the UE for application layer measurements. AppLayerMeasParameters information element -- ASN1START -- TAG-APPLAYERMEASPARAMETERS-START AppLayerMeasParameters-r17 ::= SEQUENCE { qoe-Streaming-MeasReport-r17 ENUMERATED {supported} OPTIONAL, qoe-MTSI-MeasReport-r17 ENUMERATED {supported} OPTIONAL, qoe-VR-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ran-VisibleQoE-Streaming-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ran-VisibleQoE-VR-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ul-MeasurementReportAppLayer-Seg-r17 ENUMERATED {supported} OPTIONAL, ..., [[ qoe-IdleInactiveMeasReport-r18 ENUMERATED {supported} OPTIONAL, qoe-NRDC-MeasReport-r18 ENUMERATED {supported} OPTIONAL, qoe-AdditionalMemoryMeasReport-r18 ENUMERATED {kB128, kB256, kB512, kB1024} OPTIONAL, qoe-PriorityBasedDiscarding-r18 ENUMERATED {supported} OPTIONAL, srb5-r18 ENUMERATED {supported} OPTIONAL ]] } -- TAG-APPLAYERMEASPARAMETERS-STOP -- ASN1STOP

If UE does not include this optional capability in UECapabilityInformation message, it means the UE only supports the mandatory 64 KB memory for QoE measurement in RRC_IDLE and RRC_INACTIVE. The motivation for this optional capability is UEs may have different memory capabilities and NW may want to select the UE who has big memory capability for more and consistent data collection.

We assume the same benefit can be expected, if we have similar capability for NW side data collection. i.e., UE mandatorily supports the fixed size of minimum AS layer memory without UE capability signalling, and optionally reports via UE capability a larger minimum AS layer memory size. i.e., Taking different sizes of buffer capability from UEs into account, NW can select/configure UEs to perform NW-side data collection.

Proposal 1. UE mandatorily supports the fixed size of minimum AS layer memory without UE capability signalling, and optionally reports via UE capability a larger minimum AS layer memory size.

Availability for logging

From the last meeting, RAN2 agreed periodic logging and event-triggered logging for NW-side data collection:

<Agreements (RAN2#127bis)> - Periodic logging is supported for training data collection procedure in R19 - Event-triggered data logging will be supported. At least radio condition based event triggered logging will be supported. FFS the details of radio condition based event. FFS if other events are supported.

Meanwhile, regarding data reporting, RAN2 agreed to support only on-demand request based reporting:

<Agreements (RAN2#127bis)> - Periodic reporting of logged data is not supported. - On-demand reporting of the logged measurements will be specified - UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. FFS of details of the message

In addition, the availability by UE was agreed as trigger for UEInformationRequest from NW.

<Agreements (RAN2#127bis)> - The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. FFS trigger/definition of availability indication. and FFS how data availability indication is sent to the network.

Taking all aforementioned agreements into account, we assume the following procedure for NW side data collection:

- Step 1. NW provides measurement configuration for NW side data collection to UE (e.g., via RRCReconfiguration message).

- Step 2. UE logs the measurement data based on periodic logging and/or event-triggered logging.

- Step 3. UE indicates the availability indication for logged data to NW.

- Step 4. NW sends UEInformationRequest message to UE to request transmission of logged data.

- Step 5. UE reports the logged data via UEInformationResponse message.

Based on this procedure, we think there is no need for UE’s autonomous reporting (e.g., AS buffer event based reporting), which was left as FFS in the RAN2#127:

<Agreements (RAN2#127)> - FFS whether AS buffer event based reporting is supported. FFS if we send availability indication or full report if it is supported

Given multiple UEs may be configured in a cell to collect sufficient data for completion of NW side model training, UE’s autonomous reporting may cause severe network congestion. It is safer for UE to send reports based on NW’s on-demand request.

Proposal 2. UE’s autonomous reporting (including AS buffer event based reporting) is NOT supported.

For the availability indication UE sends in Step 3, we assume the availability indication can be transferred via UAI. We do not see the need to design a new separate message for this indication. Another issue for availability indication is when UE triggers transmission of the availability indication. Assume NW configures CSI-RS resource to measure for NW-side data collection. Then, UE immediately starts to measure and log the measurement results. If UE transmits the availability indication as soon as a single instance of measurement result is obtained, it may not be the useful information to NW. This is because UE is likely to report the availability indication immediately after NW configures CSI-RS configuration. Therefore, we think there should be a threshold for logged data size to trigger the transmission of the availability indication, and it should be configurable by NW depending on its needs.

Proposal 3. UE indicates the availability indication via UAI, when the size of logged data exceeds a threshold which is configured by NW.

Meanwhile, RAN2 agreed not to support RRC segmentation for NW side data collection.

<Agreements (RAN2#127)> - The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

So, when UE stores a big size of logged data, all the logged data may not be accommodated in a single UEInformationResponse message. In this case, it would be beneficial if UE can indicate the availability indication for the residual logged data when sending the first UEInformationResponse message. Based on this information, NW can request further UE’s reports by sending an additional UEInformationRequest message. This is similar to logMeasAvailable indication in UEInformationResponse message in logged MDT.

Proposal 4. UE can indicate the availability indication in UEInformationResponse message to indicate the further residual logged data.

Conclusion

RAN2 is requested to discuss and agree the following proposals:

Proposal 1. UE mandatorily supports the fixed size of minimum AS layer memory without UE capability signalling, and optionally reports via UE capability a larger minimum AS layer memory size.

Proposal 2. UE’s autonomous reporting (including AS buffer event based reporting) is NOT supported.

Proposal 3. UE indicates the availability indication via UAI, when the size of logged data exceeds a threshold which is configured by NW.

Proposal 4. UE can indicate the availability indication in UEInformationResponse message to indicate the further residual logged data.

References