3GPP TSG-RAN WG2 Meeting #128	          R2-2409869

Orlando, USA, Nov.  18th – 22nd , 2024

Agenda Item:	8.3.5

Source:	Mediatek Inc.

Title:	Simulation Assumption of SLS, measurement event prediction, RLF prediction and generalization study

Document for:	Discussion, Decision

# Introduction

In previous #127 and #127bis meeting, RAN2 has made the following agreements on measurement event prediction, RLF prediction and SLS evaluation for AI/ML for mobility.

Agreements on inputs/outputs and KPIs - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window. - A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT). - As baseline, we will use RLF event prediction KPI: - for indirect: F1 score. the following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction. - for direct: F1 score. The following can be reported: missed event detection, false event detection, - Continue discussion over email discussion to see if there is a difference.

Agreements

- The System level performance (e.g. HO performance) evaluation is optional (i.e. companies can bring results if they chose).

  - System level performance for measurement event prediction can be prioritized by companies if they chose to do it.

- RAN2 will prioritize discussions on intermediate KPI discussion before discussing system level performance for the corresponding use case.

2: 	Discussion on what (type of information)/how generalization study can take place in Nov. meeting

3: The SLS simulation assumption discussion is covered in the post#127bis email discussion by assuming:

	The simulation assumptions agreed for measurement event prediction and RLF prediction is taken as baseline for SLS in principle

	The HO model in 36.839 is taken as baseline

	The HO performance will be HOF and number of HO only and definition in 36.839 is taken as baseline

	The baseline of HO performance is R15 legacy measurement and HO procedure

In this paper, we will discuss the simulation assumptions for SLS, measurement event prediction and RLF prediction. Furthermore, we initiate the investigation into generalization for RRM measurement prediction.

# Discussion

## Assumptions of UE/NW behaviour for System-Level Simulation

One of the controversial parts of the system-level simulation (SLS) is the assumption of UE/NW behaviour with AI prediction. In this email discussion [1], we notice that companies start to discuss the detailed timing of each action in the entire HO procedure, e.g., predicting events, reporting predicted results, triggering MR, HO command ...etc as given below.

The HO model for FR2 temporal domain case A is defined as following: If a predicted A3 event at future t1 is reported at t0 and one potential real A3 event is triggered at t2, then HO command is transmitted at t3, where t3=t0+ max(HO prep time, min(t1-t0,t2-t0)). After that one fixed execution time is assumed. For direct prediction, t1 is the middle of the event occurrence window. The HO model for FR1 temporal domain case B is defined as following: If a predicted A3 event at t1 is reported at t0 (t0<=t1) then HO command is transmitted at t3, where t3=t0+max(HO prep time, t1-t0). After that one fixed execution time is assumed.

However, from our point of view, before delving into the details, we must align on the assumption of UE/NW behaviour, especially when the assumption may vary for different applications. For example, once we use AI to predict a measurement event, e.g., an A3 event, what is the UE/NW’s next action? Can UE directly trigger the A3 event report based on the prediction, or will UE send additional information to NW so that NW can perform early HO preparation and wait for the true A3 event report? Another example is that when we use AI to predict an RLF event, UE can send a message to NW for early HO or UE can directly trigger the early RRC reestablishment. From these examples, it is clear that the UE/NW behaviour will depend on the specific application we aim to achieve with the help of AI prediction. Thus, it is unreasonable to define a fixed UE/NW behaviour for all use cases and applications.

Observation 1: The assumption of UE/NW behavior is related to the application of AI prediction. It is not reasonable to define a fixed UE/NW behavior for all use cases and corresponding applications in system-level simulation.

As we know, there are two general goals of AI mobility. Goal 1 is to reduce the measurement overhead, and Goal 2 is to enhance AI mobility performance. For Goal 1, what we would like to examine through SLS is whether the performance degradation (due to the measurement overhead reduction) is acceptable or not. It is natural to assume that UE and NW behaviour will follow the legacy (non-AI, rule-based) mechanism, i.e., R15 L3 HO procedure. The only difference is that the measurement report is now triggered by predicted measurement and optional historical measurement instead of pure historical measurement.

Proposal 1: For Goal 1, i.e., measurement overhead reduction, RAN2 assume the UE and NW behavior follows the legacy mechanism, i.e., R15 L3 HO procedure, in system level simulation. The only difference is that the measurement report is triggered by predicted measurement and historical measurement instead of pure historical measurement.

For Goal 2, UE and NW behaviour are more complicated and related to the applications. Since RAN2 agreed to prioritize measurement event prediction for SLS, we first focus on the possible behaviour for the measurement event prediction case. From our current understanding, there are three different applications to enhance the mobility performance.

- Application (a): Use AI to predict events so that NW can perform early HO preparation (which can save roughly 50ms HO latency). In this application, UE can transmit a message to NW once a measurement event is predicted, allowing NW to start the early HO preparation. However, NW will not send the HO command to the UE until it receives the measurement report (triggered by pure historical measurement without AI prediction). Once NW receives MR, it can send HO command immediately without HO preparation delay. This application can improve mobility performance by saving the HO preparation time from NW.

- Application (b): Use AI prediction to early trigger MR. In this application, the UE can transmit MR based on the prediction results. For example, when the UE predicts an A3 event will occur soon with high probability or an A3 event is valid, UE can directly send MR to early trigger the HO and thus HO to the target cell in time. In this application, NW can act following the traditional mechanism, i.e., receive MR and then send HO command accordingly.

- Application (c): Use AI to avoid unnecessary HO, e.g., ping-pong, short ToS HO. In this application, AI can be used to predict whether the incoming event is valid or invalid. If the event is invalid which may imply the target cell is unstable, the UE can decide not to trigger a measurement report, provided that the quality of the serving cell is sufficient and does not experience RLF.

The UE and network behaviours for different applications of measurement event prediction is summarized in Table 1.

| Applications | UE action | NW action |
| --- | --- | --- |
| (a) Early HO preparation | - Transmit one MR to NW when an event is predicted with the prediction results. - Transmit another MR with actual measurement when the event occurs | - Do early preparation when receiving predicted event message from UE - Send HO command after receiving the MR with actual measurement |
| (b) Early trigger MR | Transmit MR when an event is predicted with prob higher than a threshold or the event is valid. Note: MR triggering and reporting is based on the prediction instead of pure measurements. | Act following the traditional mechanism, i.e., send HO command after receiving MR. |
| (c) Avoid unnecessary HO (e.g., ping-pong) | UE doesn’t trigger MR if the event is invalid | No action |

Table 1: UE and NW behaviour in SLS for measurement event prediction use cases.

Proposal 2: For Goal 2, RAN2 discuss the UE and NW behavior in the SLS for measurement event prediction use cases in different applications:

- Application (a): Use AI to predict events so that NW can perform early HO preparation.

- Application (b): Use AI prediction to early trigger MR.

- Application (c): Use AI to avoid unnecessary HO, e.g., ping-pong, short ToS HO.

We understand that companies have different views about the timing of actions in the entire HO procedure, e.g., predicting events, reporting predicted results, triggering MR, HO command...etc, as discussed in the email discussion [1]. However, before we delve into the details, we must ensure that we are aligned with the same UE and network behaviour assumptions for SLS, otherwise, there is no chance to make convergent assumptions, especially when companies actually consider different applications.

Proposal 3: The detailed timing of actions in the entire HO procedure, e.g., predicting events, reporting predicted results, triggering MR, HO command...etc, can be FFS once RAN2 agree on the applications and their corresponding UE and NW behaviour assumption.

## Measurement event prediction

### Clarification on how to obtain ground-truth

For direct and indirect measurement event prediction, a key issue during the model training phase is how to get the ground-truth. Whether direct or indirect prediction, it is necessary to obtain the event's occurrence time instance or the probability of occurrence within a window to process the data and get the ground-truth.

However, the labeling of the ground-truth is entirely different when performing with HO versus without HO. When HO is executed, the ground-truth are obvious, whereas without HO, the resulting ground-truth are not clearly defined. As shown in figure (a), if we perform HO for each event, the serving cell and the ground-truth event constantly changes with the HO procedures. And in the figure(b), if the HO is not modelled, how do we process the data to get the ground-truth? If we crudely keep the serving cell unchanged, the resulting ground-truth will differ significantly from those with HO execution and are likely to be unreasonable due to the lack of HO, as indicated by the dashed arrows, showing consecutive satisfied events in a short time.

Figure 1: Ground-truth with HO and without HO

Observation 2: The ground-truth without HO may results in unreasonable event and does not align with ground-truth with HO.

To address this issue, certain solutions need to be considered. Therefore, we propose following options:

Proposal 4: For measurement event prediction, RAN2 consider the following option for obtaining ground-truth with HO:

- Consider the switch of the serving cell and neighbour cell during data processing to record all events and ultimately derive the metrics.

Proposal 5: For measurement event prediction, RAN2 consider the following options for obtaining the ground-truth without HO:

- Option1: Switch the serving cell to the strongest one at the event condition is met.

- Option 2: Fixed serving cell and reset TTT at the event condition is met.

- Option 3: Terminate the trajectory when the event condition is met.

### Utilize AI to predict event is about to happen

For direct event prediction, the AI can be used to predict the event about to happen. In this kind of utilization of direct event prediction, the model output is the probability of event occurrence within a time window. During email discussion [1], we saw companies have different understanding on the implementation of direct measurement event prediction and the time window.

Before we discuss the interpretation of time window, we want to clarify the definition of prediction range and prediction tolerance range. The two concepts are for different purpose, the prediction range is the time window of making prediction, while the prediction tolerance range refers to the permissible error tolerance window for deriving performance metrics. To differentiate them, we use "prediction window" and "tolerance window" to refer to the two “time window”. Since the tolerance window is for purpose of deriving the intermediate performance metrics, it may not have specification impact in the future.

Observation 3: The prediction window and tolerance window are two different concepts and with different usage, i.e. one is for inference operation and the other is for deriving performance metrics for intermediate KPI.

In interpretation 1, the two concept of prediction window and tolerance window are blurred combined vaguely into the blue part time window, and the output is within t1 to t2, which will cause issue that if event is occur during t0 to t1, the event will be missed. Even that issue may only happen when every time the AI is initialized during OW+(t1 – t0), but when considering AI LCM, the AI and non-AI method may switch back and forth and may cause frequently missed detection.

Figure 2(a): Interpretation 1 for time window

In interpretation 2, the issue mentioned above is not exist anymore. The starting point of time window is the ending point of OW. The predictions are continuous and will not cause any missed detection.

Figure 2(b): Interpretation 2 for time window

If we would like to consider Interpretation 2, there is no clear concept distinguish between prediction window and tolerance window.

We depict the prediction window and tolerance window separately in the figure 3. The AI will predict the event that occurs in the prediction window (PW), i.e., t0 to tp. The AI model output the probability of prediction window, where the probability is higher than certain threshold and we think there is an event occurrence at t1. If a real event occurs within the [t1 - tolerance window/2, t1 + tolerance window/2] of the predicted event time t1, we consider it a True event prediction. If an event occurs but is predicted as not happening, it is a Missed event prediction. If no event occurs within this tolerance window but is predicted to happen, it is a False event prediction. With this interpretation, we can clearly distinguish between prediction window and tolerance window.

Figure 2(c): Interpretation 3 of prediction window and tolerance window

Proposal 6: The time window is interpretated to include a prediction window and a tolerance window. The prediction window is the time window for model inference output.  The tolerance window is the time window used to derive the intermediate KPI (i.e., F1 score) based on the output of prediction window.

Proposal 7: The prediction window is consecutive to the observation window starts from t0. The AI will predict the event that occurs in the prediction window (PW), i.e., t0 to tp.

As shown in Figure3, if an event occurs at a specific time instance within the window, and if we only output one probability for the prediction window, it's hard to determine the exact predicted occurrence time within the window. Therefore, the AI model's output can be the probability of the prediction window and the event occurrence probabilities for all time points within the PW. We propose to take the largest probability higher than certain threshold within the prediction window as the timing of the predicted event occurrence.

Figure 3: The output/label of the direct prediction

Proposal 8: For direct measurement event prediction, the model output should be the probability of each time instance of within the prediction window and the highest probability time instance higher than certain threshold is the time instance of event occurs in the prediction window.

Proposal 9: For direct measurement event prediction, with the highest probability time instance higher than certain threshold in prediction window, the tolerance window can be used to determine the True event prediction”, “Missed event prediction”, and “False event prediction” to derive the F1 score.

### Utilize AI to predict event is valid or not

For the utilization of AI for direct event prediction, we think that AI can not only predict when an event is going to happen but also possible to predict whether an event that satisfied the entering condition is valid or not.

As showed in Figure 4, we can perform AI inference when the entry condition of A3 event is met (t0), predicting whether this potential A3 event is valid or not. If the event that meets the entering condition leads to an unnecessary HO in the future, such as a ping-pong, short ToS, or HOF, the event can be considered invalid; otherwise, it is valid.

Figure 4: Predict event is valid or not

There may have concern on how to determine whether a measurement event is valid or not. Is it done by the NW or the UE? In fact, it is not necessary to predefine additional thresholds for event valid/invalid prediction. According to our results, the prediction can be done directly using historical measurements. Therefore, it differs from the case of setting a longer TTT, which may result in higher latency or, worse, a too-late HO. Besides, it’s difficult to make accurate prediction for the time instances far from current time. The benefit and the complexity of the AI model for predicting when and whether certain event is about to happen remain unclear.

There have two possible applications with the predicted results:

  - UE can decide whether to send the MR based on the prediction. In this case, it does not imply UE will not send MR for a long time, since the MR is only not reported if the target cell is unstable. For normal target cells, the MR should be predicted as a valid event.

  - UE will still transmit MR following the original rule. However, with such predictions, UE can provide additional information in the MR, allowing the NW to decide whether to send the HO command upon receiving such MR with the additional information from the prediction, which indicates it is possible to have some unintended event like ping-pong if HO is performed to such unstable target cell.

Similar to predicting when an event will occur, the AI output can still provide relevant measurement and prediction information to the NW, assisting the NW in making HO decisions.

Proposal 10: For direct measurement event prediction, the AI can be used to predict if certain event satisfied the entering condition is valid or not in the future. If the event that satisfied the entering condition leads to an unnecessary HO (e.g., Ping-Pong, short ToS) in the future, the event can be considered invalid; otherwise, it is valid.

## RLF prediction

Before discussing the details of the RLF prediction flow, we need to first identify how to utilize the AI prediction result and what is our goal to achieve. From our perspective, there are two possible applications of AI RLF prediction:

- For Early HO to Prevent RLF: The prediction distance should be sufficiently long to allow the UE and the gNB adequate time to react, such as performing an HO procedure, before the channel quality deteriorates significantly, for instance, before the T310 timer starts running. This ensures that the network can maintain connectivity by managing the handover in advance of the RLF condition worsening.

- For Early RRC Reestablishment: The prediction distance can be shorter, allowing the UE to initiate RRC reestablishment proactively based on the RLF prediction, even while the T310 timer is running. This application focuses on quickly reestablishing a stable connection to another cell once an RLF is anticipated, thereby minimizing disruption.

As we can see from Fig 5(a) and 5(b). One of the key differences between these two applications is the “predicted distance”.  To enable a proactive response from the UE/network to prevent or mitigate the impacts of a potential RLF event, the UE must predict the RLF occurrence significantly ahead of the present moment. For application a), the predicted distance must be larger than the T310 timer, whereas for application b), it can be much shorter, shorter than the T310.

Figure 5(a): Illustration of RRM prediction for early HO, Pd>T310.

Figure 5(b): Illustration of RRM prediction for early re-establishment, Pd<T310.

As discussed in the email discussion, we consider different interruptions of observation/prediction timing flow. Since in RLF prediction, the predicted distance is relatively large, interpretation 1 should be more suitable (which is aligned with our Fig 5(a) and 5(b)). There is no need to monitor nearby regions because, even if we predict an RLF between T0 to T1, we may not have enough time to react. Additionally, we think this interruption can be applied to both direct and indirect RLF predictions, both of which are agreed to be considered in RAN2 127 as shown below.

Agreements

-	Both direct and indirect are allowed.  Companies should indicate what they used and what inputs they are using

-	Output for indirect: predicted SINR.  Based on predicted SINR the time instance the RLF occurs can be determined without further AI/ML models.

-	Output for direct: probability of RLF within an window

-	Companies should report the prediction time window they have used in the simulations

For indirection RLF prediction, the AI model output is predicted SINR, however, it will be used to derive the predicted RLF event at some time instance (e.g., inference time instance + predicted distance). If we consider fixed predicted distance, the timing flow is similar to the direct RLF prediction. The only difference part is that the final output is an RLF event in that time instance or a probability of RLF occurring within a given timing window (or tolerance window). However, from our point of view, they are basically the same, especially when we agreed to consider the same intermediate KPI “predict a RLF event will happen in the given time window” for both direct and indirect RLF prediction.

Proposal 11: RAN2 consider two utilizations of RLF prediction, such as (a) for initiating Early HO to Prevent RLF, and (b) for triggering Early RRC Reestablishment.

Proposal 12: RAN2 consider different predicted distances i.e., the interval between the current time slot and the predicted time slot, for different applications. For a) early HO, the predicted distance should be larger than T310, and for b) early RRC reestablishment, the predicted distance can be shorter than T310.

Proposal 13: For both direct and indirect RLF prediction, RAN2 consider interreption 1 with the predicted distance as the baseline. The predicted distance, depending on the applications, can be FFS.

## Generalization

It is currently extremely challenging to use a single model to fit all scenarios, configurations, and areas, in this case, model update/switch is necessary. If the model is pre-downloaded from the UE server to the UE, this requires the UE to store a large number of models. Alternatively, if the model is transferred from the network to the UE based on changes in the environmental location, it means that frequently changing models (parameters) will need to be transmitted during the UE movement. This leads to increased network load and potential delay, which are unacceptable in high-speed contexts or delay-sensitive service scenarios.

Observation 4: It is challenging to use a single model for all situations, which may cause frequent model updates, heavy storage and transmission overhead.

Due to mobility characteristics, considering generalization in mobility scenarios is necessary. If a model is trained with area-based data, the model may not perform well need to switch/update model for AI prediction when the UE enters different area with different scenario and configurations. The scope of the area can be divided based on cell, site, or cluster. Therefore, we propose the following definition for model generalization:

Proposal 14: RAN2 use the definition of model generalization:

  - Model generalization, i.e., using one model that is generalizable to different scenarios/configurations/areas.

Following 3 cases have been considered on R18 AI/ML to evaluate the generalization performance of an AI/ML model over various scenarios/configurations:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

The basic logic of the aforementioned cases is to train on a dataset containing multiple elements and perform inference on the same dataset; train on one dataset and perform inference on other datasets that do not contain these settings; and train on multiple datasets and perform inference on datasets that include these settings.

We propose to consider following cases for mobility scenario generalization study:

Proposal 15: To evaluate the impact of site, RAN2 consider following cases can be considered:

  - Case 1(baseline): The AI/ML model is trained based on training Dataset #A with setting(scenarios/configurations/areas), and then the AI/ML model performs inference/test on a dataset from the same Dataset #A.

  - Case 2: The AI/ML model is trained based on training dataset from one Dataset #A, and then the AI/ML model performs inference/test on a different dataset than Dataset #A, e.g., Dataset #B, Dataset #C that  with different settings.

  - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple sites including Dataset #A and different datasets than Dataset #A, e.g., Dataset #B, Dataset #C, and then the AI/ML model performs inference/test on a dataset with a setting composed of settings in Dataset #A, Dataset #B, Dataset #C.

In RRM frequency domain prediction, we assume that cells in different frequency bands are co-located. However, our preliminary results show that whether the co-located base station beam patterns are identical or not can significantly impact model accuracy. Therefore, it is also necessary to consider the effect of beam pattern on model generalization. Therefore, such elements in simulation may affect AI model performance, like UE speed, beam pattern, and the area of model training data need to be considered.

Proposal 16: RAN2 consider to study the impact of setting(scenarios/configurations/areas) on model generalization, including UE speed, beam pattern, and the collection area of data.

To simplify, we initially use the controlled variable method to study the impact of specific elements on model generalization.

Proposal 17: To evaluate the impact of speed, considering following options:

- Model trained with dataset of 30km/h, perform model inference on dataset with 30km/h.

- Model trained with dataset of 30km/h, perform model inference on different dataset than 30km/h, e.g., 60km/h, 90km/h or 120km/h.

- Model trained with dataset mixture of 30km/h, 60km/h, 90km/h and 120km/h, perform model inference on one of dataset 30km/h, 60km/h, 90km/h or 120km/h.

Proposal 18: To evaluate the impact of data collection area, considering following options:

- Model trained with dataset of one certain cell, apply inference on certain cell (one model for one cell).

- Model trained with dataset of cluster(all cells), apply inference on any one cell of the cluster (one model for cluster);

- Model trained with dataset of one cell, apply inference on any other of cells within the cluster (one model for cluster);

And before we study the impact of beam pattern, firstly we need to consider the possible beam pattern options.

Above options balance model generalization and complexity, ensuring acceptable model complexity and training difficulty while reducing the frequency of model switching during mobility management, thus enabling more flexible model management.

# Conclusion

System-Level Simulation

Observation 1: The assumption of UE/NW behavior is related to the application of AI prediction. It is not reasonable to define a fixed UE/NW behavior for all use cases and corresponding applications in system-level simulation.

Proposal 1: For Goal 1, i.e., measurement overhead reduction, RAN2 assume the UE and NW behavior follows the legacy mechanism, i.e., R15 L3 HO procedure, in system level simulation. The only difference is that the measurement report is triggered by predicted measurement and historical measurement instead of pure historical measurement.

Proposal 2: For Goal 2, RAN2 discuss the UE and NW behavior in the SLS for measurement event prediction use cases in different applications:

- Application (a): Use AI to predict events so that NW can perform early HO preparation.

- Application (b): Use AI prediction to early trigger MR.

- Application (c): Use AI to avoid unnecessary HO, e.g., ping-pong, short ToS HO.

Proposal 3: The detailed timing of actions in the entire HO procedure, e.g., predicting events, reporting predicted results, triggering MR, HO command...etc, can be FFS once RAN2 agree on the applications and their corresponding UE and NW behaviour assumption.

Clarification on how to obtain ground-truth for measurement event prediction

Observation 2: The ground-truth without HO may results in unreasonable event and does not align with ground-truth with HO.

Proposal 4: For measurement event prediction, RAN2 consider the following option for obtaining ground-truth with HO:

- Consider the switch of the serving cell and neighbour cell during data processing to record all events and ultimately derive the metrics.

Proposal 5: For measurement event prediction, RAN2 consider the following options for obtaining the ground-truth without HO:

- Option1: Switch the serving cell to the strongest one at the event condition is met.

- Option 2: Fixed serving cell and reset TTT at the event condition is met.

- Option 3: Terminate the trajectory when the event condition is met.

Utilize AI to predict event is about to happen

Observation 3: The prediction window and tolerance window are two different concepts and with different usage, i.e. one is for inference operation and the other is for deriving performance metrics for intermediate KPI.

Proposal 6: The time window is interpretated to include a prediction window and a tolerance window. The prediction window is the time window for model inference output.  The tolerance window is the time window used to derive the intermediate KPI (i.e., F1 score) based on the output of prediction window.

Proposal 7: The prediction window is consecutive to the observation window starts from t0. The AI will predict the event that occurs in the prediction window (PW), i.e., t0 to tp.

Proposal 8: For direct measurement event prediction, the model output should be the probability of each time instance of within the prediction window and the highest probability time instance higher than certain threshold is the time instance of event occurs in the prediction window.

Proposal 9: For direct measurement event prediction, with the highest probability time instance higher than certain threshold in prediction window, the tolerance window can be used to determine the True event prediction”, “Missed event prediction”, and “False event prediction” to derive the F1 score.

Utilize AI to predict event is valid or not

Proposal 10: For direct measurement event prediction, the AI can be used to predict if certain event satisfied the entering condition is valid or not in the future. If the event that satisfied the entering condition leads to an unnecessary HO (e.g., Ping-Pong, short ToS) in the future, the event can be considered invalid; otherwise, it is valid.

RLF prediction

Proposal 11: RAN2 consider two utilizations of RLF prediction, such as (a) for initiating Early HO to Prevent RLF, and (b) for triggering Early RRC Reestablishment.

Proposal 12: RAN2 consider different predicted distances i.e., the interval between the current time slot and the predicted time slot, for different applications. For a) early HO, the predicted distance should be larger than T310, and for b) early RRC reestablishment, the predicted distance can be shorter than T310.

Proposal 13: For both direct and indirect RLF prediction, RAN2 consider interreption 1 with the predicted distance as the baseline. The predicted distance, depending on the applications, can be FFS.

Generalization

Observation 4: It is challenging to use a single model for all situations, which may cause frequent model updates, heavy storage and transmission overhead.

Proposal 14: RAN2 use the definition of model generalization:

  - Model generalization, i.e., using one model that is generalizable to different scenarios/configurations/areas.

Proposal 15: To evaluate the impact of site, RAN2 consider following cases can be considered:

  - Case 1(baseline): The AI/ML model is trained based on training Dataset #A with setting(scenarios/configurations/areas), and then the AI/ML model performs inference/test on a dataset from the same Dataset #A.

  - Case 2: The AI/ML model is trained based on training dataset from one Dataset #A, and then the AI/ML model performs inference/test on a different dataset than Dataset #A, e.g., Dataset #B, Dataset #C that  with different settings.

  - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple sites including Dataset #A and different datasets than Dataset #A, e.g., Dataset #B, Dataset #C, and then the AI/ML model performs inference/test on a dataset with a setting composed of settings in Dataset #A, Dataset #B, Dataset #C.

Proposal 16: RAN2 consider to study the impact of setting(scenarios/configurations/areas) on model generalization, including UE speed, beam pattern, and the collection area of data.

Proposal 17: To evaluate the impact of speed, considering following options:

- Model trained with dataset of 30km/h, perform model inference on dataset with 30km/h.

- Model trained with dataset of 30km/h, perform model inference on different dataset than 30km/h, e.g., 60km/h, 90km/h or 120km/h.

- Model trained with dataset mixture of 30km/h, 60km/h, 90km/h and 120km/h, perform model inference on one of dataset 30km/h, 60km/h, 90km/h or 120km/h.

Proposal 18: To evaluate the impact of data collection area, considering following options:

- Model trained with dataset of one certain cell, apply inference on certain cell (one model for one cell).

- Model trained with dataset of cluster(all cells), apply inference on any one cell of the cluster (one model for cluster);

- Model trained with dataset of one cell, apply inference on any other of cells within the cluster (one model for cluster);

# Reference

[1] [POST127bis][022][AI mobility] Simulation Assumptions (OPPO)

[2] R2-2407849 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

[3] R2_127_ChairNotes_24-08-23_17-00_eom

[4] 3GPP TR 36.839.