3GPP TSG-RAN WG2 Meeting #128	R2-2409870

Orlando, USA, Nov. 18th~ Nov 22nd, 2024

Agenda Item:	8.1.2.2

Source:	Mediatek Inc.

Title:	Further Discussion on LCM for UE-side Model

Document for:	Discussion, Decision

# Introduction

During the RAN2#127 meeting, significant progress was made concerning the functionality-based LCM for the UE-side model, specifically for the use case of beam management. Additionally, one LS[1] was sent to RAN1, containing questions whose answers are essential for facilitating further progress in RAN2.

During the RAN2#127bis meeting, the following agreement was made regarding the configuration of the UE for data collection and model training:

Data collection initiation and configuration for data collection is under network control. FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

Furthermore, the applicable functionality report was discussed and the following agreement was reached:

Agreements for BM

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

- FFS if inference configuration can be signalled in step3.

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

- Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

- For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

- For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models.  Can come back to this discussion later.

In this contribution, we further discuss the necessary signaling/protocol for configuring and initating data collection for UE-side model training, handling of the non-applicable functionality as well as the applicability functionality report during/after handover.

# Discussion

## Data Collection for UE-side model training

### Data Categorization and Model Development

Assuming the UE-side model is trained by a server, such as an OTT server, datasets from many UEs are used for model training. Based on the discussion of different UE-side data collection options (1a, 1b, 2, and 3), the dataset is first stored on the server for UE-side model training. During data collection, it is impractical for the UE to transfer data sample by sample to the server. Instead, the UE should log the data and then transfer the logged data to the server. Since data logging is already supported for network-side data collection, it should also be supported for UE-side data collection.

Observation 1: Data logging is required for data collection for UE-side model training, as it is impractical to transfer data sample by sample to the server.

Proposal 1: Data logging is supported for data collection for UE-side model training.

These datasets are categorized based on the scenario, RAN configuration, and network conditions, such as the associated ID. Various UE-side models can be trained for different scenarios, RAN configurations, and network conditions, allowing the models to accurately adapt to real-time network environments. The processing of datasets collected under different RAN configurations and network conditions, as well as the development of UE-side models, are implementation details and do not need to be standardized. However, to develop AI/ML models for specific scenarios, RAN configurations, and network conditions, the server for data collection for UE-side model training should be able to request data collection from the network for the relevant scenario, RAN configuration, and network conditions. This can be done either through the UE or directly from the network if there are interfaces that allow interaction between the server and the network.

Observation 2: Data categorization and AI/ML model development are implementation-specific. The data targeted for specific scenarios, RAN configuration and network condition may be required for AI/ML model training to improve the performance.

Proposal 2: The server for data collection for UE-side model training/OTT server should be able to request data collection for specific scenarios, RAN configurations, and network conditions to develop the targeted AI/ML models.

### Data Collection Initiation and Configuration

For different data collection options (1a, 1b, 2, and 3), a common issue is how to initiate the data collection procedure and provide the data collection configuration from the network to the UE. According to observation 2 and proposal 2, the server for UE-side model training or OTT server should be able to request data collection for specific scenarios, RAN configurations, and network conditions. There are two alternatives for requesting data collection configuration and initiation, depending on whether there are interfaces allowing interactions between the server and the network entity responsible for data collection configuration and initiation, such as the NG-RAN. Figure 1 illustrates the example signaling flows for different alternatives.

- Alternative 1: UE request over the air interface: The UE sends a request for data collection configuration and initiation. The network then provides the data collection configuration and initiates the data collection procedure based on the UE's request.

- Alternative 2: Server request over network interfaces: The server for data collection for UE-side model training sends the request and coordinates with the network on the data collection policy. The network then provides the data collection configuration and initiates the data collection procedure towards the UE based on the server's coordination.

For both Alternative 1 and Alternative 2, the network can reject requests from either the UE or the server for data collection for UE-side model training under certain conditions, such as lack of measurement resources or network overload. Alternative 1 is evaluated within the RAN2 domain, while Alternative 2 requires involvement from SA2.

Alternative 1 allows the UE to request measurement resources for data collection and model training, and log the data. This is applicable to all discussed data collection options (1a, 1b, 2, and 3). The server/OTT request to the UE for data collection is optional and applicable to options 1a and 1b if there is a UP tunnel and direct interaction between the UE and the server through the application layer.

Alternative 2 assumes interfaces between the network and the server for data collection for UE-side model training/OTT server, allowing coordination between them. It is applicable to options 1b, 2, and 3. Alternative 1 and Alternative 2 are not exclusive but complementary between each other if the data collection option 1b, 2 and 3 are considered.

Data Collection Initiation via UE request over the air interface

Data Collection Initiation via Server request over network interfaces

Figure 1 Data Collection Configuration/Initiation

Observation 3: Alternative 1 can be used for all the data collection options (1a, 1b, 2, and 3). The server/OTT's request to the UE for data collection is optional and applies to options 1a and 1b if there is a UP tunnel between the UE and the server.

Observation 4: Alternatave 2 assumes that there are interfaces and coordination between the network and the server for data collection for UE-side model training/OTT server, it is applicable to option 1b, option 2 and option 3.

Proposal 3: RAN2 consider the following two alternatives for data collection configuration and initiation for UE-side model training:

- Alternative 1-UE request over the air interface: The UE sends a request for data collection configuration and initiation. The network then provides the data collection configuration and initiates the data collection procedure based on the UE's request.

- Alternative 2- Server request over network interfaces: The server for data collection for UE-side model training server sends the request and coordinates with the network on the data collection policy. The network then provides the data collection configuration and initiates the data collection procedure towards the UE based on the server's coordination.

Proposal 4: The network can reject the request from the UE or the server for data collection for UE-side model training for data collection configuration and initiation.

## Applicability Reporting

### Terminology of ‘Functionality’

In RAN2#118bis meeting, RAN1 discussed the terminologies defined by RAN2 and make following agreements:

Agreement Incorporating below text into the general part of the LS In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs - The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality“ for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing.

Although RAN1 created the term ‘functionality’, it lacks a clear definition, and discussions on its granularity have been suspended for some time. Additionally, the term 'functionality' may have different meanings for various procedures, such as UE capability report, applicability report, and activation/deactivation.

It appears that RAN1 has abandoned the term 'functionality' and is now focusing on the impact on signaling for inference configuration. RAN1 is studying three options for the applicability of inference for UE-side models, where applicability refers to CSI-ReportConfig for inference configuration or one/multiple sets of inference-related parameters.

Agreement RAN 1 further study the following options for applicability for inference for UE-side model: Option 1: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied. - FFS on whether some IEs in the CSI report configuration can be removed or modified - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig - FFS on one or more of the above CSI-ReportConfig to be reported - FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4 - FFS: whether Step 5 is needed, Option 2: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - NW configures one set or multiple sets of inference related parameters - Note: the set of inference related parameters is not configured by CSI-ReportConfig - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - The associated ID(s) may be configured - wherein the associated ID(s) may be - FFS: a) part of one set of the inference related parameters, or - FFS: b) independently from the one set of the inference related parameters. - In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated. - In Step 5, NW configures configuration(s) for CSI report for inference Option 3: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter. - In Step 4, UE reports by UAI - the applicable one or multiple sets of inference related parameters may be included. - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - Note: not applicable may also be replied by UE - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4. - the associated ID(s) may be included - FFS: a) as part of the inference related parameters, or - FFS: b) independently from the set of the inference related parameters. - In Step 5, NW configures configuration(s) for CSI report for inference. Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration.

From the RAN2 perspective, it is unclear whether it is still efficient to continue using this terminology and define procedures based on it, or if RAN2 should focus on inference configuration.

Observation 5: For different procedures, e.g. UE capability report, applicability report and activation/deactivation, the term of ‘functionality’ in RAN1 may have different meaning. RAN1 has abanded the term of ‘functionality’ and focuses more on the impact on the signaling for inference configuration.

Proposal 5: RAN2 assumes that the applicability refers to the applicability of the CSI-ReportConfig for inference configuration or one/multiple sets of inference related parameters.

### Non-applicability Report

Taking both the RAN1 discussion and RAN2 agreement into account, the UE needs to report to the network when the applicable CSI-ReportConfig for inference configuration or the set of inference-related parameters becomes non-applicable. The detailed signaling will depend on RAN1's progress and decision on which option to choose.

Proposal 6: The UE reports to the network when CSI-ReportConfig for inference configuration or one/multiple sets of inference related parameters becomes non-applicable. The detailed signaling depends on RAN1 progress.

As the applicability considers the applicable CSI-ReportConfig for inference configuration or one/multiple applicable sets of inference-related parameters, it is likely that the applicable configuration may be in an activated or deactivated state. According to RAN1's agreement, activated functionalities may be enabled based on the CSI framework. Even for the CSI-ReportConfig or the set of inference-related parameters that are not in use (or are deactivated), these configurations may become non-applicable due to changes in the UE's internal conditions.

Proposal 7: The CSI-ReportConfig or the sets of inference-related parameters that become non-applicable from applicable can be either activated or deactivated configurations based on the CSI framework.

The next question is how to handle the CSI-ReportConfig for inference configuration or one/multiple sets of inference-related parameters when they become non-applicable. Considering the dynamic change in the applicability of these configurations and the possibility that non-applicable configurations may become applicable later, it is not appropriate for the UE to release the configuration autonomously. However, if the configuration is being used and is in an activated state, the UE should deactivate the configuration. The UE should report the non-applicable configuration to the network and retain the configuration.

Proposal 8: When the CSI-ReportConfig for inference configuration or one/multiple sets of inference-related parameters become non-applicable, the UE should keep the CSI-ReportConfig or the inference-related parameters and deactivate the configuration if it is activated.

### Applicability Report During HO

Duirng UE mobility when UE moves from one cell to another, how to update the applicable ‘functionality’ needs to be considered. Now the applicable ‘functionality’ refers to the CSI-ReportConfig for inference configuration or the inference-related parameters.

As the baseline method, the UE can send the applicability report to the target cell after handover via UAI. This method is suitable when different cells may provide different CSI-ReportConfig for inference configuration or sets of inference-related parameters.

Proposal 7: As the baseline method, the UE sends the applicability report to the target cell after handover via UAI.

However, considering the requirement on the generalization performance of the AI/ML models, the AI/ML model can be trained targeting to the multiple cells, taking into account of the RAN configuration and network condition among those cells. Therefore, the CSI-ReportConfig for inference configuration or the set of inference-related parameters can be common among the multiple cells.

Observation 6: Considering the requirement on the generalization performance of the AI/ML models, the CSI-ReportConfig for inference configuration or the set of inference-related parameters can be common among the multiple cells.

In this case, the source cell can coodinates with the target cell on the applicable CSI-ReportConfig for inference configuration or the set of inference-related parameters in handover preparision procedure and the target cell can provide the inference configuration in the handover command. As the response to the handover command, the UE can response the applicable configuration in the RRCReconfigurationRelease message to the taget cell.

It has following merits:

- Leverages the generalization capabilities of AI/ML models, reduces the need for inference reconfiguration during handovers and ensures consistent performance across different cells.

- Mnimizes disruption and ensures a smooth transition, as the target cell is prepared with the appropriate configuration before the handover occurs.

Proposal 8: RAN2 study the method that the UE sends the applicability report as a response to handover command in the RRCReconfigurationComplete message to the target cell.

# Conclusion

Data Categorization and Model Development

Observation 1: Data logging is required for data collection for UE-side model training, as it is impractical to transfer data sample by sample to the server.

Observation 2: Data categorization and AI/ML model development are implementation-specific. The data targeted for specific scenarios, RAN configuration and network condition may be required for AI/ML model training to improve the performance.

Proposal 1: Data logging is supported for data collection for UE-side model training.

Proposal 2: The server for data collection for UE-side model training/OTT server should be able to request data collection for specific scenarios, RAN configurations, and network conditions to develop the targeted AI/ML models.

Data Collection Initiation and Configuration

Observation 3: Alternative 1 can be used for all the data collection options (1a, 1b, 2, and 3). The server/OTT's request to the UE for data collection is optional and applies to options 1a and 1b if there is a UP tunnel between the UE and the server.

Observation 4: Alternatave 2 assumes that there are interfaces and coordination between the network and the server for data collection for UE-side model training/OTT server, it is applicable to option 1b, option 2 and option 3.

Proposal 3: RAN2 consider the following two alternatives for data collection configuration and initiation for UE-side model training:

- Alternative 1-UE request over the air interface: The UE sends a request for data collection configuration and initiation. The network then provides the data collection configuration and initiates the data collection procedure based on the UE's request.

- Alternative 2- Server request over network interfaces: The server for data collection for UE-side model training server sends the request and coordinates with the network on the data collection policy. The network then provides the data collection configuration and initiates the data collection procedure towards the UE based on the server's coordination.

Proposal 4: The network can reject the request from the UE or the server for data collection for UE-side model training for data collection configuration and initiation.

Terminology of ‘Functionality’

Observation 5: For different procedures, e.g. UE capability report, applicability report and activation/deactivation, the term of ‘functionality’ in RAN1 may have different meaning. RAN1 has abanded the term of ‘functionality’ and focuses more on the impact on the signaling for inference configuration.

Proposal 5: RAN2 assumes that the applicability refers to the applicability of the CSI-ReportConfig for inference configuration or one/multiple sets of inference related parameters.

Non-applicability Report

Proposal 6: The UE reports to the network when CSI-ReportConfig for inference configuration or one/multiple sets of inference related parameters becomes non-applicable. The detailed signaling depends on RAN1 progress.

Proposal 7: The CSI-ReportConfig or the sets of inference-related parameters that become non-applicable from applicable can be either activated or deactivated configurations based on the CSI framework.

Proposal 8: When the CSI-ReportConfig for inference configuration or one/multiple sets of inference-related parameters become non-applicable, the UE should keep the CSI-ReportConfig or the inference-related parameters and deactivate the configuration if it is activated.

Applicability Report During HO

Observation 6: Considering the requirement on the generalization performance of the AI/ML models, the CSI-ReportConfig for inference configuration or the set of inference-related parameters can be common among the multiple cells.

Proposal 7: As the baseline method, the UE sends the applicability report to the target cell after handover via UAI.

Proposal 8: RAN2 study the method that the UE sends the applicability report as a response to handover command in the RRCReconfigurationComplete message to the target cell.

# Reference

[1] R2-2407848, LS on applicable functionality reporting for beam management UE-sided model