3GPP TSG-RAN WG2 Meeting #128	R2-2409877

Orlando, US, Nov. 18th – 22nd, 2024

Agenda Item:	8.9.3

Source: 	OPPO

Title:  	Discussion on CB-msg3 EDT and msg4 enhancement

Document for:	Discussion and Decision

Introduction

In last RAN2 meeting, further progress was made for CB-msg3 EDT and msg4 enhancement and the agreements were made as follows:

Agreements:

1.	RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

2.	RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed.

3.	At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

4.	CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

5.	RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

6.	At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

8.	CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

In this contribution, we would like to provide our considerations on open issues for CB-msg3 EDT and msg4 enhancement.

Discussion

Trigger condition

In last RAN2 meeting, it was agreed that a running TAT is not taken as one of the trigger conditions for CB-msg3 EDT according to the feedback from RAN1/RAN4.

6. At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission

While it is still unclear on other conditions to initiate CB-msg3 EDT. For legacy EDT with UE connected to EPC, the trigger conditions are specified as follows in current TS36.331.

#### 5.3.3.1b Conditions for initiating EDT A BL UE, UE in CE or NB-IoT UE can initiate EDT when all of the following conditions are fulfilled: 1> if the UE is connected to EPC: 2> for CP-EDT, the upper layers request establishment of an RRC connection, the UE supports CP-EDT, and SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes cp-EDT; or 2> for UP-EDT, the upper layers request resumption of an RRC connection, the UE supports UP-EDT, SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes up-EDT, and the UE has a stored value of the nextHopChainingCount provided in the RRCConnectionRelease message with suspend indication during the preceding suspend procedure; …… 1> the establishment or resumption request is for mobile originating calls and the establishment cause is mo-Data or mo-ExceptionData or delayTolerantAccess; or 1> the establishment or resumption request is for mobile terminating calls, the UE has a stored mt-EDT indication and the establishment cause is mt-Access; 1> the establishment or resumption request is suitable for EDT as specified in TS 36.300 [9], clause 7.3b.1; 1> SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes edt-Parameters; 1> for mobile originating calls, the size of the resulting MAC PDU including the total UL data is expected to be smaller than or equal to the TBS signalled in edt-TBS as specified in TS 36.321 [6], clause 5.1.1; 1> EDT fallback indication has not been received from lower layers for this establishment or resumption procedure;

According to the WID, the objective aims to reduce the necessary uplink and downlink signalling to complete an EDT transaction. Therefore, the trigger conditions of legacy EDT can be simply followed for CB-msg3 EDT.

At least the following conditions needs to be considered for a UE to initiate CB-msg3 EDT:

- CB-msg3 EDT for CP/UP solution is supported by both UE and NW.

- the establishment/resumption request is for MO calls and the establishment cause is mo-Data or mo-ExceptionData or delayTolerantAccess.

- the establishment or resumption request is for mobile terminating calls, the UE has a stored mt-EDT indication and the establishment cause is mt-Access.

- CB-msg3 EDT resources are configured in the cell.

- the size of the resulting MAC PDU is expected to be no larger the TBS configured for CB-msg3 EDT.

- Fallback indication has not been received from lower layers for this establishment or resumption procedure, e.g., CB-msg3 EDT resources for the selected CE is not available.

Resource configuration/selection

In last RAN2 meeting, the following agreements were made on how to configure CB-msg3 EDT resources.

3. At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details. 4. CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

For EDT, the PRACH resources can be configured on anchor carrier and non-anchor carriers for each CE level as specified in TS36.321. Also for load balance purpose, the same way of configuration can be followed for CB-msg3 EDT.

- for EDT, each enhanced coverage level has zero or one anchor carrier PRACH resource present in nprach-ParametersList-EDT and zero or one PRACH resource for each non-anchor carrier signalled in ul-ConfigList.

For each CE level, the shared uplink resources for CB-msg3 transmission can be configured on anchor and/or non-anchor carriers.

For NB-IoT UEs, the SCS of the UL grant in RAR can be configured as 3.75kHz or 15KHz and the NW selects the numerology to be scheduled based on the radio link quality. For CB-msg3 EDT, the resources with different numerologies need to be considered to adapt to the UEs in different radio link conditions.

For NB-IoT, CB-Msg3 EDT resources can be configured to support different numerologies (e.g. 3.75kHz SCS and 15kHz SCS) for each CE level.

As agreed, the UE determines the CE level on basis of RSRP threshold. If the CB-msg3 EDT resources are further configured on multiple carriers and configured with different numerologies as discussed above, the UE needs to perform carrier selection and numerology selection for the selected CE level. In our understanding, the carrier can be selected based on probability factor as EDT and the numerology can be selected based on RSRP threshold. For the selection order, the following alternatives can be considered:

- Alt1: The UE firstly selects the numerology. If CB-msg3 EDT resources with the selected numerology are configured on multiple carriers, the UE further selects the carrier.

- Alt2: The UE firstly selects carrier. For the selected carrier, if CB-msg3 EDT resources are configured with different numerologies, the UE further selects the numerology.

Both alternatives can work. Alt1 is more beneficial for performance since a proper numerology can be selected based on radio link quality. Alt2 has an advantage in load balancing.

For the selected CE level, the UE further selects carrier and numerology if the CB-msg3 EDT resources are configured on multiple carriers and configured with different numerology.

RAN2 further discuss the order for carrier and numerology selection from the following two alternatives:

  - The UE firstly selects the numerology. If CB-msg3 EDT resources with the selected numerology are configured on multiple carriers, the UE further selects the carrier.

  - The UE firstly selects carrier. For the selected carrier, if CB-msg3 EDT resources are configured with different numerologies, the UE further selects the numerology.

DL response reception

For CB-msg3 EDT, collision occurs when more than one UEs select the same PUSCH resources for UL transmission. To address the collision, existing procedure in RACH can be simply followed, i.e., the UE starts a contention resolution timer after msg3 transmission, during which the UE monitors DL response and consider the contention resolution as successful if a matched contention resolution ID MAC CE is received. Regarding the DL in response to CB-msg3 transmission, at least the contention resolution ID needs to be included.

For SA, legacy mac-ContentionResolutionTimer is reused for CB-msg3 EDT.

If the contention is resolved, at least the contention resolution ID needs to be included in the DL response.

In last RAN2 meeting, it was agreed that DSA is introduced for CB-msg3 EDT and the UE can be configured to transmit more than one replica of CB-msg3. With the support of DSA, further discussion is needed on how to start the mac-ContentionResolutonTimer. We may have the following two options:

- Option1: A single mac-ContentionResolutionTimer is used to monitor DL response for all of multiple msg3 replicas.

- Option2: Separate mac-ContentionResolutionTimer is used for each msg3 replica.

As agreed, the RNTI used for DL monitoring is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission. For DSA, different RNTI can be calculated for each replica. With Option1, the UE needs to keep monitoring the DL responses addressed to all these RNTI during the time window, which is power consumption. Therefore, Option2 is the preferred solution for DSA.

For DSA, separate mac-ContentionResolutionTimer is maintained for each msg3 replica

If contention is resolved, UE stops all the mac-ContentionResolutionTimers.

Efficient delivery of msg4

As per WID, the other aspect on EDT enhancement is to support efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete. In legacy EDT, after UE transmit UL data together with RRCEarlyDataRequest or RRCConnectionResumeRequest message in Msg3, the UE will not consider the EDT as successfully completed until it receives RRCEarlyDataComplete or RRCConnectionRelease message, i.e. only RRC message can be used to indicate the successful reception of the EDT transmission by the eNB.

For PUR, DL response optimization is specified for CP solution, i.e., eNB may terminate the PUR procedure by sending a L1 ACK optionally containing a TAC, a TAC or RRC response message with no data. By introducing L1/L2 ACK, the performance of resource efficiency, UE power consumption as well as MO latency can be further improved compared to EDT.

For CP-solution EDT, if there is no follow-up DL data, L1/L2 signalling can be considered for improvement of msg4 efficiency. The following solutions can be considered for further discussion:

- Solution1: If the NW is able to determine there is no follow-up DL data when responding msg4, the EDT termination indication can be delivered to the UE together with the contention resolution ID MAC CE.

- Solution2: If there is no follow-up DL data after contention resolution, L1 signalling scheduling by UE-dedicated RNTI  can be used to indicate EDT termination.

The following solutions can be considered for msg4 enhancement if there is no follow-up DL data:

  - L1 ACK scheduled by UE-dedicated RNTI.

  - A new MAC CE including contention resolution ID and EDT termination indication.

Conclusion

Based on the discussion in section2, we have following observation proposals:

Proposal 1	At least the following conditions needs to be considered for a UE to initiate CB-msg3 EDT:

-	CB-msg3 EDT for CP/UP solution is supported by both UE and NW.

-	the establishment/resumption request is for MO calls and the establishment cause is mo-Data or mo-ExceptionData or delayTolerantAccess.

-	the establishment or resumption request is for mobile terminating calls, the UE has a stored mt-EDT indication and the establishment cause is mt-Access.

-	CB-msg3 EDT resources are configured in the cell.

-	the size of the resulting MAC PDU is expected to be no larger the TBS configured for CB-msg3 EDT.

-	Fallback indication has not been received from lower layers for this establishment or resumption procedure, e.g., CB-msg3 EDT resources for the selected CE is not available.

Proposal 2	For each CE level, the shared uplink resources for CB-msg3 transmission can be configured on anchor and/or non-anchor carriers.

Proposal 3	For NB-IoT, CB-Msg3 EDT resources can be configured to support different numerologies (e.g. 3.75kHz SCS and 15kHz SCS) for each CE level.

Proposal 4	For the selected CE level, the UE further selects carrier and numerology if the CB-msg3 EDT resources are configured on multiple carriers and configured with different numerology.

Proposal 5	RAN2 further discuss the order for carrier and numerology selection from the following two alternatives:

a.	The UE firstly selects the numerology. If CB-msg3 EDT resources with the selected numerology are configured on multiple carriers, the UE further selects the carrier.

b.	The UE firstly selects carrier. For the selected carrier, if CB-msg3 EDT resources are configured with different numerologies, the UE further selects the numerology.

Proposal 6	For SA, legacy mac-ContentionResolutionTimer is reused for CB-msg3 EDT.

Proposal 7	If the contention is resolved, at least the contention resolution ID needs to be included in the DL response.

Proposal 8	For DSA, separate mac-ContentionResolutionTimer is maintained for each msg3 replica

Proposal 9	If contention is resolved, UE stops all the mac-ContentionResolutionTimers.

Proposal 10	The following solutions can be considered for msg4 enhancement if there is no follow-up DL data:

a.	L1 ACK scheduled by UE-dedicated RNTI.

b.	A new MAC CE including contention resolution ID and EDT termination indication.