3GPP TSG-RAN2 Meeting #128	R2- 2409913

# Orlando, United States, Nov 18th – 22nd, 2024

Agenda item:		8.11.2 (NR_duplex_evo)

Source:	LG Electronics Inc.

Title: 	Discussion on Random Access procedure in SBFD

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 SBFD[1], it is agreed to support the Random Access procedure using the SBFD symbols.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - … - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - …

This document further discusses RAN2 aspects on RA procedure for SBFD.

2.	Discussion

Signaling of RACH configuration with SBFD symbol

In previous RAN1 meetings [2-3], it is agreed to support Option 1 with Alt 1-1 and Option 2 for the configuration of RA resource in SBFD symbol, but enabling both options at the same time is not supported from UE side:

[RAN1#117 agreement] Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options. [RAN1#118bis agreement] Agreement For RACH configuration Option 1, support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols - FFS: Details of the separate power control parameters

Based on the RAN1 agreements on RACH configuration Options of SBFD RO, it is agreed in RAN2#127bis meeting [4] to support the RACH configuration Option 1 and Option 2 and to further discuss on the details of RRC signaling.

The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. The RACH configuration for SBFD is transmitted via SIB1. FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

Given that both RACH configuration options are considered to configure RA resources in SBFD symbol, the first discussion point is whether the network may simultaneously configure RACH configuration Option 1 and RACH configuration Option 2 in the same cell. Note that it was already discussed in RAN1#118bis meeting [3], but there was no consensus on whether the cell may configure both configurations:

Conclusion There is no consensus in RAN1 to support the following in Rel-19: - Enabling both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side.

In detail, following alternatives can be considered on configuration of RA resource in SBFD symbol:

- Alt 1: the network may configure the RACH configuration Option 1 and RACH configuration Option 2 simultaneously in the same cell.

- Alt 2: the network configures only one type of RACH configuration option.

For Alt 1, it could be better to support the SBFD-aware UE, when the SBFD-aware UE supports only one type of RACH configuration option. However, since it is not supported to enable both RACH configuration options at the UE side (as agreed in RAN1), the additional mechanism is needed to determine which RACH configuration option should be used when the UE supports both RACH configuration options. In addition, if both configuration options are provided in the same cell, some of SBFD-aware UEs are using RACH configuration Option 1 while the other SBFD-aware UEs are using RACH configuration Option 2, causing additional discussions on SBFD RA procedure such as:

- whether the SBFD-aware UE supporting both configuration options is allowed to switch from one type of configuration (e.g., RACH configuration option 2) to another type of configuration (e.g., RACH configuration option 1) for load balancing among SBFD ROs, and

- whether some of RA parameters (e.g., rsrp-ThresholdMsg1-RepetitionNum2/4/8) should be separately configured for each RACH configuration option.

Based on the discussion, additional complexity on RA procedure can be occurred, e.g., by determining of RACH configuration option for SBFD-aware UEs supporting both options and/or by additional fallback procedure from one RACH configuration Option to another RACH configuration Option, if it is supported.

For Alt 2, the SBFD RA procedure would be much simpler, since the SBFD-aware UE may use SBFD ROs only if the UE supports RACH configuration Option configured by the cell. If the UE does not support RACH configuration Option provided by the cell, the SBFD-aware UE performs RA procedure using legacy RO since the SBFD-aware UE would not be able to identify SBFD RO, which does not cause any critical issue.

Given that current RA procedure with RACH partitioning framework is already complicated, it would be better to simplify SBFD RA procedure as much as possible without mixing SBFD-aware UEs using RACH configuration Option 1 and SBFD-aware UEs using RACH configuration Option 2 in the same cell. In addition, further discussion on non-critical issues should be avoided on SBFD RA procedure, considering limited TU (0.5 TU) for SBFD WI in RAN2. Therefore, the network should configure only one type of RACH configuration Option in the same cell (i.e., Alt 2).

Proposal 1. Do not support enabling both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side.

On signaling details of RACH configuration Option 1, only some extension of existing RACH configuration is needed, since SBFD-aware UE determines SBFD RO based on the additional rule based on legacy RACH configuration. The details of additional parameters (e.g., separate power control parameters, additional condition to select SBFD RO for initial RA attempt, number of RA attempt for fallback procedure, etc.) can be determined later based on further RAN1/RAN2 discussion.

For RACH configuration Option 2, as agreed in RAN2#127bis meeting, additional RACH configuration for SBFD RO (i.e., ‘SBFD RACH configuration’) should be defined in order to configure SBFD RO and corresponding RA parameters. According to the current RA procedure, following methods were defined in order to configure separated ROs for various purpose:

- Method 1: define a new IE for the separated RACH configuration, e.g., RACH-ConfigCommonIAB defined for IAB

- Method 2: define a feature for RACH partitioning, and use RACH configuration within AdditionalRACH-Config, e.g., RO partitioning configured for SDT, Slicing, Msg1 repetition, etc.

Similarly, one of the above methods can be reused to define SBFD RACH configuration.

Meanwhile, in RAN2#127bis meeting [4], it is agreed that SBFD is not defined as a new feature in RACH partitioning framework for preamble partitioning, since SBFD early indication using preamble partitioning is not needed.

From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

Based on the above agreement, it would be better to not define as a new feature for RO partitioning as well. In this sense, defining SBFD RACH configuration using Method 1 (i.e., new IE for SBFD RACH configuration) is slightly preferred.

Proposal 2. For the details of configuring the SBFD RACH configuration for RACH configuration Option 2, define the new IE of SBFD RACH configuration, e.g., RACH-ConfigCommonSBFD IE.

RA resource selection between SBFD RO and legacy RO

For RA resource selection for initial PRACH transmission, it is agreed to select one RO between SBFD RO and legacy RO based on certain prioritization/conditions, for both RACH configuration Option 1 and Option 2 [4]:

Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

In our view, for SBFD-aware UE, SBFD RO should be prioritized over legacy RO, since the legacy UE cannot use the SBFD RO. However, if the SBFD-aware UE perform RA procedure using SBFD RO in bad channel condition, CLI impact would occur as followings:

- When the UE in bad channel condition performs RA procedure using SBFD RO, the transmission of RA preamble may not be successful due to CLI impact caused by DL subband or DL slot on the network side.

- When the UE performs RA procedure using SBFD RO even though the UE is in cell edge, the UE would perform the power ramping procedure for re-attempt of the RA procedure, causing additional CLI impact to other UEs (e.g., to UEs in neighboring cell).

Therefore, in order to minimize the aforementioned CLI impact of using SBFD RO, it is suggested to use SBFD RO only if the channel is good enough to perform the RA procedure using SBFD RO. In other words, if the SBFD RACH configuration is configured for SBFD-aware UE,

- SBFD RO is used when the channel condition is better than the threshold (i.e., when the RSRP is larger than the RSRP threshold)

- Otherwise, legacy RO is used for the current RA procedure.

Proposal 3. For initial attempt of the RA procedure, SBFD-aware UE prioritizes SBFD RACH occasion if the channel condition is better than the RSRP threshold. Otherwise, SBFD-aware UE performs RA procedure using legacy RACH occasion.

For re-attempt in one random access procedure, it is agreed to support the fallback procedure from SBFD RO to legacy RO, after the certain number of RA attempt using SBFD RO:

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

Meanwhile, when the fallback procedure is defined for Msg1 repetition using the RACH partitioning framework, the fallback procedure is allowed only for increasing the Msg1 repetition number (e.g., from Msg1 repetition number 2 to Msg1 repetition number 4) using the same feature combination, in order to minimize additional complexity of RA procedure with RACH partitioning framework.

Similarly, the fallback procedure for SBFD RA procedure should be defined as simple as possible. Therefore, in order to avoid the re-selection of RACH partition after the fallback from SBFD RO to legacy RO, it is suggested to allow fallback from SBFD RO to legacy RO only if the same RACH partition (i.e., same feature combination and Msg1 repetition number) is configured in the legacy RACH configuration.

Proposal 4. For PRACH transmission re-attempt in one RA procedure, UE is allowed to switch from SBFD RACH occasion to legacy RACH occasions within the same feature combination and Msg1 repetition number.

In last RAN2 meeting, it was remained as FFS on whether the SBFD-aware UE is allowed to switch from legacy RO to SBFD RO. However, the fallback procedure from legacy RO to SBFD RO does not ensure successful transmission of RA preamble, since SBFD RO has additional CLI impact and may cause additional RA attempt failure due to signalling quality of RA preamble. That is, the benefits of fallback procedure from legacy RO to SBFD RO is marginal, compared to the fallback procedure from SBFD RO to legacy RO. Note that current RA procedure allows fallback procedure for only one direction (e.g., from 2-step RA to 4-step RA and from low Msg1 repetition number to high Msg1 repetition number), and it is allowed to ensure the robustness of Msg1 transmission.

Rather, if the fallback procedure from legacy RO to SBFD RO is additionally supported, lots of follow-up discussion may be needed including followings:

- Whether the additional condition to switch from legacy RO to SBFD RO is needed (e.g., RSRP Threshold),

- Whether/how to change the power related variables after performing the fallback procedure, and

- Whether the sequential fallback procedure is allowed (e.g., legacy RO  SBFD RO  legacy RO)

Therefore, if the fallback procedure is supported between SBFD RO and legacy RO for both directions, it would be very complicated to determine RO and performing RA parameter re-initialization, without any critical benefits. In this sense, for simplicity, once the SBFD-aware UE selects the legacy RO, the legacy RO should be used for the rest of the random access procedure, i.e., not supporting fallback from legacy RO to SBFD RO.

Proposal 5. Do not support switching from legacy RACH occasions to SBFD RACH occasions for SBFD-aware UE.

Early indication for SBFD aware UE

During the discussion in RAN2#127bis [4], early indication for SBFD-aware UE using the preamble partitioning is not supported, while early indication using SBFD RO is supported without any spec impact:

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

Then, if the Msg3-based early indication is not supported, the network cannot identify SBFD aware UE in RRC_IDLE/INACTIVE state for following cases:

- If the SBFD-aware UE performs RA procedure using legacy RO, e.g., based on fallback procedure or additional condition to select SBFD ROs,

- If the SBFD-aware UE does not support RACH configuration option provided by network, or

- If the network does not configure SBFD RO even though the network supports SBFD symbols in the cell.

Then, even though the UE is SBFD-aware UE, the network cannot allocate the uplink grant using the UL subband within SBFD symbols, until the UE transfers its UE capability information with SBFD indication.

Therefore, Msg3-based early indication seems beneficial, since the UE and the network need to perform message exchanges (e.g., including NAS messages for initiation setup, security activation, and RRC reconfiguration) before UE transfers UE capability information, and it would be better to transmit these initial messages using SBFD symbols for latency reduction. In our understanding, Msg3-based early indication can be easily implemented, as the only works for Msg3-based early indication is to define additional LCIDs for SBFD-aware UE.

Proposal 6. Support Msg3-based early indication for SBFD-aware UE.

3.	Conclusion

In this contribution, it is discussed about RAN2 aspects on Random Access operation for SBFD. This document includes following proposals.

Proposal 1. Do not support enabling both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side.

Proposal 2. For the details of configuring the SBFD RACH configuration for RACH configuration Option 2, define the new IE of SBFD RACH configuration, e.g., RACH-ConfigCommonSBFD IE.

Proposal 3. For initial attempt of the RA procedure, SBFD-aware UE prioritizes SBFD RACH occasion if the channel condition is better than the RSRP threshold. Otherwise, SBFD-aware UE performs RA procedure using legacy RACH occasion.

Proposal 4. For PRACH transmission re-attempt in one RA procedure, UE is allowed to switch from SBFD RACH occasion to legacy RACH occasions within the same feature combination and Msg1 repetition number.

Proposal 5. Do not support switching from legacy RACH occasions to SBFD RACH occasions for SBFD-aware UE.

Proposal 6. Support Msg3-based early indication for SBFD-aware UE.

4.	Reference

[1] RP‑241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2] Report of 3GPP TSG RAN WG1 #117

[3] Report of 3GPP TSG RAN WG1 #118bis

[4] Draft Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China, 14 - 18 October, 2024