3GPP TSG-RAN WG2 #128	R2-2409935

Orlando, USA, Nov. 18th – 22nd 2024

Source:                    	DENSO CORPORATION

Title:  	Discussion on Store & Forward operation

Document for:        	Discussion and decision

Agenda Item:         	8.9.2	Support of Store & Forward

- Introduction

In Rel-19, the objectives for Store & Forward operation of IoT-NTN are described in the WID of Rel-19 IoT-NTN [1] as follows:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

Based on the chair notes for last meeting RAN2#127bis[2], the following agreements are made for S&F operation:

Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

3.	An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

This paper attempts to investigate about consideration on S&F operation based on the above objectives and agreements.

- Discussion

2.1 Remaining time of current operation mode

In RAN2 #127, it was discussed that the remaining time of current operation mode can be informed via System Information broadcast, and it was not concluded. We support the proposal so that UE can adjust its behavior, e.g.) not to start communication which takes long time just before the operation mode is switched. Such UE behavior would be beneficial to reduce unnecessary resource consumption due to retransmission or communication delay at the operation mode switch. Furthermore, UE may be able to prioritize to communicate with a cell which has longer remaining time of “real-time mode” if the remaining time of neighbor satellite’s current operation mode is also informed somehow, e.g., including in the neighbor satellite information list of SIB33.

Observation 1: Broadcasting the remaining time of the satellite’s current operation mode would be beneficial to reduce unnecessary resource consumption.

Proposal 1: RAN2 to introduce broadcasting the remaining time of current operation mode.

Proposal 2: S&F related information for neighbor satellites (current operation mode, remaining time) could be informed to UE.

2.2 Existing RRC connection at the mode switch

It has been already agreed that legacy UE (not supporting S&F capability) should be barred on “S&F mode”. However, it has not been discussed explicitly whether connected legacy UE should be released or not when the satellite is going to “S&F mode” from “real-time mode” in case that the RRC connection of the legacy UE was established during “real-time mode”. We think the RRC connection could be released when the satellite is going to “S&F mode” from “real-time mode”, since most likely the legacy UE does not expect the significant delay on S&F operation. Additionally, if the RRC connection is released, it should be performed explicitly with RRCConnectionRelease message, since the legacy UE may not have way to know the S&F operation mode switch.

Proposal 3: RRC connection of legacy UE could be explicitly released with RRCConnectionRelease message when the satellite is going to “S&F mode” from “real-time mode”.

On the other hand, in case that connected UE supports both “real-time mode” and “S&F mode”, it is not clear whether the connection should be released. It seems to be simpler to release all connection regardless of UE capability. However, in case of the mode switch going to “real-time mode” from “S&F mode”, UE would not have any disadvantage with keeping RRC connection. Therefore, we would like to propose to discuss the case.

Proposal 4: RAN2 to discuss whether the UE which supports both “real-time mode” and “S&F mode” should be released or not when the satellite switches the S&F operation mode.

2.3 UE requirement indication

It could be expected that UE’s UL data may have period of validity even though S&F UE should be delay-tolerant, since the delay on S&F operation could be significantly longer than the delay expected on the legacy delay-tolerant communication. Based on this assumption, storing & forwarding the UL data would be wasteful after the actual delay exceeds UE’s tolerable delay. Therefore, we would like to propose to introduce a mechanism to indicate UE’s requirement for UL data transmission (e.g., level of delay-tolerance, data expiration time) from UE to satellite, so that satellite can discard the data for storing & forwarding when the actual delay exceeds the requirement.

Proposal 5: RAN2 to consider whether and how UE can indicate its requirement for the UL data transmission (e.g., level of delay-tolerance, data expiration time) to satellite

2.4 EDT support on S&F mode

Introducing Early Data Transmission (EDT) is discussed on Uplink Capacity Enhancement. We think EDT is beneficial to reduce the number of signaling and the total delay even for S&F operation. Therefore, RAN2 may discuss about considerations for using EDT on S&F operation, however, we would like to clarify if EDT is applicable for S&F operation before the discussion.

Proposal 6: RAN2 to clarify if EDT is applicable for S&F operation

- Summary and proposal

This paper discussed about consideration on S&F operation and proposed the following:

Observation 1: Broadcasting the remaining time of the satellite’s current operation mode would be beneficial to reduce unnecessary resource consumption.

Proposal 1: RAN2 to introduce broadcasting the remaining time of current operation mode.

Proposal 2: S&F related information for neighbor satellites (current operation mode, remaining time) could be informed to UE.

Proposal 3: RRC connection of legacy UE could be explicitly released with RRCConnectionRelease message when the satellite is going to “S&F mode” from “real-time mode”.

Proposal 4: RAN2 to discuss whether the UE which supports both “real-time mode” and “S&F mode” should be released or not when the satellite switches the S&F operation mode.

Proposal 5: RAN2 to consider whether and how UE can indicate its requirement for the UL data transmission (e.g., level of delay-tolerance, data expiration time) to satellite

Proposal 6: RAN2 to clarify if EDT is applicable for S&F operation

- References

[1] RP-242397, “New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

[2] R2-2409213, “Report from Break-out session on NR-NTN and IoT-NTN”