# 3GPP TSG RAN WG2 Meeting #128      	                           R2-2409943
Orlando, US, 18th – 22th Nov., 2024

Agenda item:	8.1.2.2

Source:	Apple

Title:	Remaining issues on LCM procedure of UE-sided model for AI/ML based beam management

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The updated Rel-19 WID of AI/ML for air interface (WID RP-242399) was agreed in RAN#105 [1]. The WI objective on general LCM framework, including applicable functionalities, is led by RAN2 which is copied below:

In RAN2#127 [2], baseline LCM procedure of UE-sided model of AI/ML based beam management was agreed, and some questions with potential RAN1 impacts were identified to be included in the LS to RAN1 (R2-2407848) [3]. Then, in RAN2#127b [4], the following further agreements on LCM procedure were made with some FFSs. Among the FFSs, some need to wait RAN1 inputs, but the other highlighted ones need RAN2 further discussion:

Agreements for BM

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

- FFS if inference configuration can be signalled in step3.

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

- Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

- For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

- For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models.  Can come back to this discussion later.

In this contribution, we further discuss RAN2 remaining issues on LCM procedure (except the questions for RAN1 in [3]), including the above highlighted FFSs in RAN2#127b agreements.

# 2 Discussion

According to agreements in RAN2#127 [2] and RAN2#127b [4], the basic applicable functionality reporting framework can be illustrated in Figure. 1 with highlighted FFSs to be discussed in this contribution:

Figure.1: Illustration of basic LCM procedure of UE-sided model for beam management with FFS labelled

## 2.1 FFS1: signalling of data collection configuration and initiation

RAN2#127b [4] had initial discussion on data collection configuration and made below agrement:

- Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

We first discusss data collection configuration, and then discuss the initiation procedure.

### 2.1.1 Data collection configuration

According to RAN1 progress, we understand that the UE needs to collect data for UE-sided model training based on L1 measurements towards CSI-RS (e.g. L1-RSRP and/or beam ID). Thus, CSI-RS configuration needs to provide to the UE. And it is straight forward to reuse CSI framework to provide CSI-RS configuraiton. Meanwhile, RAN2 is also discussing NW-sided data collection via enhanced MDT with L1 measurement logging. Because it can also provide CSI-RS configuration for measurement, we think it also can be considerd as one candidate solution. But at this stage, as the details of the enhanced MDT are still under discusison, we think RAN2 can further discuss it.

Proposal 1: RAN2 assume CSI framework can configure data collection for UE-sided model training in AI/ML based beam management. FFS whether enhanced MDT with L1 measurement logging is also supported.

Agreement

For UE sided model in beam management, support associated ID

- [Working Assumption]

  - The associated ID at least can be configured within CSI framework

    - FFS on details

    - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s)

- UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID

  - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

Meanwhile, RAN1#118 [5] has confirmed the WA of associated ID and further agreed that associated ID at least can be configured within CSI framework, where the wording “at least” is captured due to RAN2 discussion on applicability reporting (i.e. whether it can be configured in OtherConfig of RRCReconfiguration in step 4).

Because associated ID is used to ensure consistency across training and inference, we believe it is necessary to include it in data collection configuration. Thus, we propose:

Proposal 2: Associated ID(s) are included in data collection configuration for UE-sided model training.

### 2.1.2 Data collection initialization

During online discussion of RAN2#127b [4], it was discussed how the NW determines whether data collection should be initiated, and it was agreed to capture it as an FFS. We think it is necessary to introduce UE request on data collection initialization due to below reasons:

- The UE has the local / internal information on whether the data collection is needed, e.g. whether the device local model is ready for interence or needs further training.

- The UE has the local / internal informaion on when the data collection can start, e.g. available remaining battery and memory information, whether any on-going operations which can’t be interrputed by data collection.

  - The UE can’t share such information with the NW because it may be UE’s internal implementation.

- The UE has the local / internal informaion on what assistance information and/or reference signal (RS) from NW are needed for model training, e.g. required CSI-RS resource for measurements.

  - The UE may provide its suggestion to the NW and it is up to NW to decide whether to acommendate the UE requrst as legacy.

Observation 1: It is necessary to introduce UE request on data collection initialization due to below reasons:

- The UE has the local / internal information on whether the data collection is needed, e.g. whether the device local model is ready for interence or needs further training.

- The UE has the local / internal informaion on when the data collection can start, e.g. remaining battery / memory status, whether any on-going operations which can’t be interrputed by data collection.

- The UE has the local / internal informaion on what assistance information and/or reference signal (RS) from NW are needed for model training, e.g. required CSI-RS resource for measurements.

Meanwhile, as legacy, whether the UE is allowed to request / response data collection should be configured by the NW. Thus, we propose:

Proposal 3: Introduce UL signaling to allow UE to request / response data collection configuration and initialization under NW configuration.

On the detailed signaling, we think both UE initiated solution and NW initiated solution should be supported:

- UE initiated: via UAI message if configured in OtherConfig of RRCReconfiguartion.

- NW initiated: via RRCReconfigurationComplete message as response to data collection configuration in RRCReconfiguartion.

Proposal 4: On signaling of UE request / response, support both UE initiated solution and NW initiated solution:

- UE initiated request: via UAI message if configured in OtherConfig of RRCReconfiguartion.

- NW initiated response: via RRCReconfigurationComplete message as response to data collection configuration in RRCReconfiguartion.

Then, we discuss the contents of UE request. According to Observation 1, we think at least below information are necessary:

- Indication on whether initializing data collecton

- It is intended to address requirements 1) and 2) in Observation 1 on whether / when to initiate data collection.

- Suggested / preferred data collection configuration (e.g. Set A and/or Set B configuration)

- It is intended to address requirements 3) in Observation 1 on what RS from NW are needed for measurements.

Note that above 1) and 2) are just high-level information, but the detailed contents need further discussion. Thus, we propose:

Proposal 5: The UE request signaling may include the indication on whether initializing data collection and suggested / preferred data collection configuration. FFS detailed signaling contents.

Finally, we want to clarify one more important point: the reception of data collection configuration doesn’t mean the UE has to immediately start model training. As captured in Section 1 of TR 38.843 [6], the specific AI/ML models are not expected to be specified and left to implementation.

From Section 1 of TR 38.843:

Note 1:	Specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved.

Note 2:	The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In our understanding, the UE may not immediately start model training due to various UE internal reasons, e.g. the UE may need to perform pre-processing on the collected data before transferring the dataset to its server for training. Meanwhile, the UE may also stop performing model training due to various UE internal reasons, e.g. model is ready for inference or insufficient memory / battery.

Observation 2: After reception of data collection configuration, the UE may not immediately start model training due to various UE internal reasons, e.g. the UE may need to perform pre-processing on the collected data before transferring the dataset to its server for training.

Observation 3: The UE may also stop performing model training due to various UE internal reasons, e.g. model is ready for inference or insufficient memory / battery.

Thus, we believe that it is up to UE implementation when to start / stop training of the UE-side model.

Proposal 6: After reception of data collection configuration, it is up to UE implementation when to start / stop training of the UE-side model.

## 2.2 FFS2: if report “available functionalities” in step 4

This FFS is to address the following issue in email discussion summary report [7].

Proposal 6: RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities.

In our understanding, the intention to introduce available functionality is to allow the UE to notify NW the case that one functionality has available model, but it is not applicable (e.g. due to un-satisfaction of NW-side additional condition or un-satisfaction of UE-side additional condition). However, we are not convinced it is necessary because the NW can’t differentiate whether it is due to un-satisfaction of NW-side additional condition or un-satisfaction of UE-side additional condition and thereby doesn’t have clue how to react.

Observation 4: Even if available functionality is introduced, the NW can’t differentiate whether it is due to un-satisfaction of NW-side additional condition or un-satisfaction of UE-side additional condition and thereby doesn’t have clue how to react.

Thus, we propose:

Proposal 7: RAN2 don’t introduce available functionalities separately from applicable functionalities.

## 2.3 FFS3: how to handle handover in step 4

This issue was raised in post-meeting email discussion [8]:

- Whether neighboring cell’s applicability needs to be considered in applicable functionality reporting to serving cell?

We think this issue is important because it determines whether the UE’s AI/ML operation can continue after handover. Please note that AI/ML operation continuity during handover is much more important than legacy beam management operation. In legacy NR, beam management procedure is not expected to take a long time. Thus, there is no requirement on continuity of beam management during handover, i.e. the UE may stop beam management in source cell after reception of HO command and re-initiate beam management procedure in target cell after handover completion. However, we believe it is important to ensure AI/ML based beam management continuity during handover because it may lead to re-train / re-download applicable AI/ML model and thereby a quite long interruption time. As illustrated in Figure.2, if associated ID is cell specific, it means that the same value of associated ID in two cells may represent different NW implementation (e.g. beam pattern). Thus, the UE needs to train cell specific AI/ML model because UE is not aware of the underlaying NW implementation behind the ID. Then, if the UE can’t report applicable functionalities of neighbor cells, its AI/ML operation may be interrupted when the UE performs handover to another cell. Specifically, for each UE handover, the UE may have to re-train / re-download applicable AI/ML model and autonomously trigger the applicable functionality reporting, which will significantly degrade AI/ML performance.

Observation 5: Different from legacy beam management, it is important to ensure continuity of AI/ML based beam management during handover. Otherwise, the UE may have to re-train / re-download applicable AI/ML model and autonomously trigger applicability reporting during handover, which leads to a long interruption.

Figure.2 Illustration of AI/ML operation interruption issue during handover (assume that associated ID is cell specific, i.e. Cell A and B may use same associated ID but NW implementation behind the same ID is different)

Thus, we propose RAN2 to study this issue.

Proposal 8: RAN2 discuss how to guarantee the continuity of the AI/ML operation during handover.

Regarding to solutions, we think it depends on whether associated ID is applicable to multiple cells or only cell specific, which is still under discussion in RAN1:

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Our analysis on the required solutions in different cases:

- Case 1: associated ID is applicable to multiple cells (e.g. PLMN specific or area specific)

  - In this case, source cell and target cell share the same associated ID space. Thus, we can reuse the legacy handover preparation procedure to ensure the continuity. Specifically, the source cell can exchange UEs’ applicable functionalities with neighbor cells via inter-node RRC message and provide target cell’s inference configuration in HO command as legacy handover procedure.

  - The example procedure is illustrated in Figure. 3-Case 1.

  - The required spec impacts are highlighted in orange font.

- Case 2: associated ID is applicable to only single cell

  - In this case, source cell and target cell don’t share the same associated ID space, i.e. the same value of associated ID may represent different NW implementation (e.g. beam pattern) in source and target cell. Thus, we can’t reuse the legacy handover preparation procedure to ensure the continuity because it goes against intention of associated ID (i.e. hide NW implementation). Instead, it is necessary to allow the UE to report neighbor cells’ applicable functionalities based on neighbor cells’ NW-side additional conditions.

  - The example procedure is illustrated in Figure. 3-Case 2.

  - Compared with Case 1, the extra spec impacts are highlighted in green font. Thus, one more inter-node coordination procedure and one more applicability reporting are needed on top of Case 1.

Figure.3 Illustration of required specification changes for AI/ML continuity in different cases

Observation 6: The solution for AI/ML continuity during handover depends on whether associated ID is applicable to multiple cells or cell specific:

  - If associated ID is applicable to multiple cells, the legacy handover preparation procedure can be reused to ensure the continuity via inter-node exchange on UE’s applicable functionalities.

  - If associated ID is cell specific, it is necessary to introduce UE reporting of neighbor cells’ applicable functionalities.

## 2.4 FFS4: whether to introduce “configured functionalities” in step 5

This FFS is to address the following issue in summary report [7].

Proposal 2: RAN2 discuss further whether definition on configured functionalities is needed.

Proposal 4: RAN2 discuss further relationship between configured functionalities and applicable functionalities as a part of discussion on the need of defining configured functionalities.

For above Proposal 2, we don’t think it is necessary to introduce the definition of “configured functionalities” due to below reasons:

- The definition is too obvious because anyway gNB/LMF will configure for functionalities.

- At least stage 3 specification (either RRC or MAC) doesn’t need this definition because what NW configured to UE is always clearly specified in RRC and MAC spec. Thus, whether it is configuration for training or inference, or monitoring should be crystal clear from text before and after in stage 3 specification.

Then, for above Proposal 4, we think RAN2 only need to clarify that the functionalities configured in Step 5 is a subset of applicable functionalities reported by the UE in Step 4. It should be RAN2 common understanding.

Thus, we propose:

Proposal 9: No need to introduce definition of “configured functionalities”. RAN2 clarify that the functionalities configured in Step 5 is a subset of applicable functionalities reported by the UE in Step 4.

## 2.5 FFS5: how to handle functionality becoming non-applicable

This issue was discussed in RAN2#127b [4], and below agreement was made with FFS.

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

First, please note that the detailed content / format of applicable functionality reporting is one explicit question (Q5) in the LS to RAN1 (R2-2407848) [4]. Thus, RAN2 doesn’t need to discuss it now and can wait RAN1 input.

Then, we don’t think it is necessary to introduce a different reporting format for the functionality which becomes non-applicable because the NW can identify which functionality becomes non-applicable by comparing UE’s latest reporting with its last reporting. In this way, RAN2 don’t need extra spec efforts and thereby should be preferred.

Observation 7: It is unnecessary to introduce a different reporting format for the functionality which becomes non-applicable because the NW can identify which functionality becomes non-applicable by comparing UE’s latest reporting with its last reporting.

Thus, we propose:

Proposal 10: No need to introduce explicit non-applicable functionality reporting, i.e. rely on NW to identify the functionality which becomes non-applicable by comparing UE’s latest reporting with its last reporting.

However, we think it will be useful if the UE can provide the NW the cause value on why one functionality becomes non-applicable because the potential NW reaction may depend on the cause value. For example, if the UE reports the cause is due to “model downloading is not complete”, the gNB may just wait the UE to complete the model downloading. As another example, if the UE reports the cause is due to “NW-sided additional condition is not met”, the gNB may reconfigure its inference configuration. The detailed cause value can be further discussed in RAN2.

Observation 8: It will be useful if the UE can provide the NW the cause value on why one functionality becomes non-applicable because the potential NW reaction may depend on the cause value (e.g. wait UE to complete model downloading or reconfigure its inference configuration).

Thus, to facilitate NW taking suitable reaction, we propose that the UE may include cause value in the reporting.

Proposal 11: For the functionality which becomes non-applicable, the UE may include cause value on why it becomes non-applicable. FFS details of candidate cause values.

# 3 Conclusion

In this contribution, we discuss remaining issues on LCM procedure of UE-sided model for beam management. Our observations are:

Observation 1: It is necessary to introduce UE request on data collection initialization due to below reasons:

- The UE has the local / internal information on whether the data collection is needed, e.g. whether the device local model is ready for interence or needs further training.

- The UE has the local / internal informaion on when the data collection can start, e.g. remaining battery / memory status, whether any on-going operations which can’t be interrputed by data collection.

- The UE has the local / internal informaion on what assistance information and/or reference signal (RS) from NW are needed for model training, e.g. required CSI-RS resource for measurements.

Observation 2: After reception of data collection configuration, the UE may not immediately start model training due to various UE internal reasons, e.g. the UE may need to perform pre-processing on the collected data before transferring the dataset to its server for training.

Observation 3: The UE may also stop performing model training due to various UE internal reasons, e.g. model is ready for inference or insufficient memory / battery.

Observation 4: Even if available functionality is introduced, the NW can’t differentiate whether it is due to un-satisfaction of NW-side additional condition or un-satisfaction of UE-side additional condition and thereby doesn’t have clue how to react.

Observation 5: Different from legacy beam management, it is important to ensure continuity of AI/ML based beam management during handover. Otherwise, the UE may have to re-train / re-download applicable AI/ML model and autonomously trigger applicability reporting during handover, which leads to a long interruption.

Observation 6: The solution for AI/ML continuity during handover depends on whether associated ID is applicable to multiple cells or cell specific:

  - If associated ID is applicable to multiple cells, the legacy handover preparation procedure can be reused to ensure the continuity via inter-node exchange on UE’s applicable functionalities.

  - If associated ID is cell specific, it is necessary to introduce UE reporting of neighbor cells’ applicable functionalities.

Observation 7: It is unnecessary to introduce a different reporting format for the functionality which becomes non-applicable because the NW can identify which functionality becomes non-applicable by comparing UE’s latest reporting with its last reporting.

Observation 8: It will be useful if the UE can provide the NW the cause value on why one functionality becomes non-applicable because the potential NW reaction may depend on the cause value (e.g. wait UE to complete model downloading or reconfigure its inference configuration).

Based on observations, our proposals can be found below:

Signaling of data collection configuration and initiation

Proposal 1: RAN2 assume CSI framework can configure data collection for UE-sided model training in AI/ML based beam management. FFS whether enhanced MDT with L1 measurement logging is also supported.

Proposal 2: Associated ID(s) are included in data collection configuration for UE-sided model training.

Proposal 3: Introduce UL signaling to allow UE to request / feedback data collection configuration and initialization under NW configuration.

Proposal 4: On signaling of UE request / response, support both UE initiated solution and NW initiated solution:

- UE initiated request: via UAI message if configured in OtherConfig of RRCReconfiguartion.

- NW initiated response: via RRCReconfigurationComplete message as response to data collection configuration in RRCReconfiguartion.

Proposal 5: The UE request signaling may include the indication on whether initializing data collection and suggested / preferred data collection configuration. FFS detailed signaling contents.

Proposal 6: After reception of data collection configuration, it is up to UE implementation when to start / stop training of the UE-side model.

If report “available functionalities” in step 4

Proposal 7: RAN2 don’t introduce available functionalities separately from applicable functionalities.

How to handle handover in step 4

Proposal 8: RAN2 discuss how to guarantee the continuity of the AI/ML operation during handover.

Whether to introduce “configured functionalities” in step 5

Proposal 9: No need to introduce definition of “configured functionalities”. RAN2 clarify that the functionalities configured in Step 5 is a subset of applicable functionalities reported by the UE in Step 4.

How to handle functionality becoming non-applicable

Proposal 10: No need to introduce explicit non-applicable functionality reporting, i.e. rely on NW to identify the functionality which becomes non-applicable by comparing UE’s latest reporting with its last reporting.

Proposal 11: For the functionality which becomes non-applicable, the UE may include cause value on why it becomes non-applicable. FFS details of candidate cause values.

# 4 References

[1] RP-242399, New WID: New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#127, Chair Notes.

[3] R2-2407848, LS on applicable functionality reporting for beam management UE-sided model.

[4] RAN2#127b, Chair Notes.

[5] RAN1#118, Chair Notes.

[6] TR 38.843-v2.0.1, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 2023-12.

[7] R2-2407165	, Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 1, Samsung.

[8] R2-2406381, Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2, Intel Corporation.