# 3GPP TSG RAN WG2 Meeting #128      	                        R2-2409945
Orlando, US, 18th – 22th Nov., 2024

Agenda item:	8.1.3

Source:	Apple

Title:	Remaining issues on NW-sided data collection

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The updated Rel-19 WID of AI/ML for air interface (WID RP-242399) was agreed in RAN#105 [1]. One of its sub-objectives is to specify data collection for NW-sided model, as highlighted copied below:

In RAN2#126 [2], RAN2#127 [3] and RAN2#127b [4], NW-sided data collection was discussed. It was agreed to design a unified measurement framework, and immediate MDT was agreed to support L1 measurement logging and reporting with some leftover issues. In this contribution, we will address the key remaining issues.

Meanwhile, RAN2 didn’t discuss NW-sided data collection of AI/ML based positioning and required data type for both NW-side model and UE-side model, which are included in the guidance of agenda item:

8.1.3 NW side data collection

Contributions should focus on the mechanisms and principles identified for data collection for network side model training during rel-18.   Contributions should discusss type of data required to be collected for NW sided model and UE sided model (common to NW sided and different).  Question to RAN1 should also be identified.

In this contribution, we further discuss NW-sided data collection from below aspects:

- NW-sided data collection for AI/ML based beam management

- NW-sided data collection for AI/ML based positioning

- Collected data type of NW-sided data collection

- Collected data type of UE-sided data collection

# 2 Discussion

## 2.1 NW-sided data collection for AI/ML based beam management

Based on latest agreements in RAN2#127b [4], our understanding on the whole procedure, including OAM centric and gNB centric data collection, is illustrated in Figure .1.

Figure. 1 Overall procedure of OAM centric and gNB centric NW-sided data collection

In the following, we discuss the below remaining issues based on Figure.1.

- Overall procedure

- AS procedure step 1: RRC configuration of L1 measurement logging

- AS procedure step 2: How UE logs L1 measurement

- AS procedure step 3: Trigger reporting of logged L1 measurement

- AS procedure step 4: Reporting of logged L1 measurement

### Overall procedure

As illustrated in Figure.1, our understanding is:

- CN procedure (i.e. procedure between RAN and CN/OAM/UDM) is labelled in blue font.

  - We think RAN2 can assume this procedure reuses existing procedure of immediate MDT.

  - One possible exception is that maybe a new server instead of TCE can be introduced to store training data, but it is up to RAN3/SA5 to decide.

- AS procedure (i.e. procedure between RAN and UE) is labelled in orange font.

  - This procedure includes 4 steps as illustrated in Figure. 1 and is common to both OAM centric and gNB centric data collection.

  - Difference between OAM centric and gNB centric data collection is which NW entity (OAM vs gNB) to trigger data collection and generate data collection configuration.

Observation 1: Difference between OAM centric and gNB centric data collection is which NW entity (OAM vs gNB) to trigger data collection and generate data collection configuration. Their AS procedure is same.

We have UE privacy concern on gNB centric data collection because the collected data is generated by the UE which may include UE’s privacy sensitive information (e.g., the UE’s location information). Please note that the UE privacy requirement is already satisfied by OAM centric data collection because user consent for MDT is required before NW configures the UE to perform immediate MDT, according to TS 32.422 [5]:

4.9.1 Signalling based MDT

In case of signalling based MDT getting user consent before activating the MDT functionality is required because of privacy and legal obligations. It is the Operator responsibility to collect user consent before initiating an MDT for a specific IMSI, IMEI number or SUPI.

Collecting the user consent shall be done via customer care process. The user consent information availability should be considered as part of the subscription data and as such this shall be provisioned to the UDM database.

Observation 2: UE privacy requirement is preserved in OAM centric data collection due to UE consent mechanism in MDT, but it is not clear how to satisfy UE privacy requirement in gNB centric data collection.

Since RAN2 agreed to aim the same measurement framework for both gNB-centric data collection and OAM-centric data collection, we think similar UE consent mechanism in MDT should also be supported in gNB-centric data collection, i.e. user consent is required before NW configures the UE to gNB-centric data collection. The detailed signaling can be further discussed.

Proposal 1: To strive for a common measurement framework, similar UE consent mechanism in MDT is supported in gNB-centric data collection, i.e. UE consent is required before NW configures the UE gNB-centric data collection. FFS signaling details.

### Step 1: RRC configuration of L1 measurement logging

#### 2.1.2.1 RRC framework

In legacy immediate MDT and L3 RRM, measurement configuration (i.e. MeasConfig) is included in RRCReconfiguration message. And whether it is immediate MDT or L3 RRM is transparent to the UE. The existing RRC structure is illustrated in Figure.2, where each MeasID links one instance of MeasObjectNR with one instance of ReportConfigNR.

Meanwhile, according to below RAN1#116b agreement [6], RAN1 has agreed to reuse existing CSI framework for data collection of beam management.

Agreement

For network-sided AI/ML model for BM-Case1 and BM-Case2,

- support using existing CSI framework for configuration of Set A as the starting point

- support using existing CSI framework for configuration of Set B as the starting point

- Note: Purpose, such as above “For NW-sided model, for BM-Case1 and BM-Case2” and “Set A” and “Set B”, will not be specified in RAN 1 specifications

Thus, RAN2 should integrate existing L1 CSI framework into L3 measurement framework illustrated in Figure.2. The existing RRC structure of CSI framework is illustrated in Figure.3, where each csi-ResourceConfigId links one instance of csi-ResourceConfig with one instance of csi-ReportConfig.

Figure. 2 Illustration of existing RRC structure of L3 RRM and immediate MDT (note: some unrelated IEs, e.g., offsetMO, are omitted)

Figure. 3 Illustration of existing RRC structure of L1 CSI framework (note: some unrelated IEs, e.g., csi-SSB-ResourceSet, are omitted)

Based on analysis of existing RRC structure, our initial consideration on how to integrate L1 measurement logging into L3 measurement framework can be found in Figure.4 with below explanation:

- New L1 CSI resource configuration (e.g. list of NZP CSI-RS resource sets) and logging configuration (e.g. logging interval) are introduced within MeasObjectNR.

- New on-demand reporting configuration is introduced within ReportConfigNR to indicate at least reporting type (i.e. on-demand) and reporting quantity configuration (e.g. L1 RSPR and/or beam index).

Figure. 4 An example of how to integrate logged L1 measurement into L3 RRM framework (green modules are introduced for logged L1 measurement/reporting)

So, we propose:

Proposal 2: MeasConfig in RRCReconfiguration is reused to configure logged L1 measurements with below specification changes:

- Within MeasObjectNR: introduce L1 CSI resource configuration and logging configuration.

- Within ReportConfigNR: introduce on-demand reporting configuration to indicate at least reporting type (i.e. on-demand) and reporting quantity configuration (e.g. L1 RSPR and/or beam index).

Then, we address below two FFSs of RAN2#127 agreements [3]:

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

#### 2.1.2.2 FFS#1: if multiple configurations can be provided to the UE

During offline discussion in RAN2#127b, we think that there are two different understandings on what “multiple configurations” means:

- Understanding 1: it means multiple configurations of logged L1 measurements

  - For example, allow multiple MeasConfig in example of Figure 4.

- Understanding 2: it means multiple CSI-RS resource sets in one logged L1 measurement configuration

  - For example, allow multiple L1-ResourceConfig in one MeasConfig in example of Figure 4.

We prefer understanding 2 because we understand the intention is to allow different L1 CSI-RS resource sets follow different measurement / logging configurations (e.g. different L1 CSI resource sets are measured with different periodicities). This is a valid use case because datasets collected from different L1 CSI resource sets may be used to train different AI/ML models by the NW. Furthermore, please note that existing L1 CSI framework already supports this feature. As illustrated in Figure. 3, multiple NZP-CSI-RS-ResourceSet and multiple CSI-ReportConfig can be configured where one NZP-CSI-RS-ResourceSet can be associated with one CSI-ReportConfig via CSI-ResourceConfigId.

Observation 3: Existing L1 CSI framework already supports multiple CSI-RS resource set configurations: multiple NZP-CSI-RS-ResourceSet and multiple CSI-ReportConfig can be configured where one NZP-CSI-RS-ResourceSet can be associated with one CSI-ReportConfig via CSI-ResourceConfigId.

We think it is straight forward to reuse this signaling framework. As illustrated in Figure. 4, under MeasObjectNR. multiple L1 CSI resource sets and multiple logging configuration can be configured, where one L1 CSI resource set is associated with one logging configuration via CSI-ResourceConfigId.

Proposal 3a: Do not support multiple configurations of logged L1 measurements (e.g. multiple MeasConfig).

Proposal 3b: Similar to existing L1 CSI framework, multiple L1 CSI resource sets can be configured to be linked with different logging configurations (e.g. with different logging periodicities) in MeasObjectNR.

#### 2.1.2.3 FFS#2: if dynamic activation/deactivation is support

We think it is not reasonable to support dynamic activation/deactivation. Enhanced MDT is intended to support data collection for model training, and thereby multiple logged L1 measurement samples are reported via a single RRC message. Thus, it is a L3 procedure and only large data volume makes sense. If NW needs to reconfigure the data collection (e.g. change the logging interval), it can use RRC reconfiguration. We don’t see benefit to support dynamic activation / deactivation.

Proposal 4: Do not support dynamic activation/deactivation of logged L1 measurements.

### 2.1.3 Step 2: How UE logs L1 measurement

#### 2.1.3.1 FFS#1: Other measurement to log

In RAN2#127b [4], it was agreed to at least log L1-RSRP and/or beam ID in NW-sided model training, and we have an FFS on whether other data to be collected.

Agreements on NW side data collection

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

We think it is important to allow the NW to know when each sample of reported L1 measurement is measured. Thus, the UE can include the time stamp when the L1 measurement (e.g. L1-RSRP) is logged. Meanwhile, please note that logging time stamp was already supported in logged MDT:

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...

}

Proposal 5: Besides L1 RSRP and/or beam-IDs, the UE also collects logging time stamp in its AS buffer.

#### 2.1.2.2 FFS#2: FFS on the memory size

We need to determine the minimum AS layer memory size. In legacy NR, logged MDT and QoE measurements (including paused measurements and IDLE/INACTIVE measurements) had specified the minimum AS layer memory size, which are all 64kB according to TS 38.306 [7]:

Logged MDT (From Section 4.2.18 of TS 38.306) loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE state. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The minimum memory size of MDT logged measurements is 64KB. For eRedCap UE supporting this feature, the minimum memory size of MDT logged measurements is 16KB.; UE; No; No; No. QoE (from Section 6 of TS 38.306) AS layer memory size for QoE paused measurement reports; It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17.. AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE; It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports".

Observation 4: In legacy NR, logged MDT, QoE measurements in IDLE/INACTIVE state and QoE paused measurements had specified the minimum AS layer memory size, which are all 64KB according to TS 38.306.

Meanwhile, RAN1 Reply LS (R1-2310681) [8] provided typical data size for model training in beam management. According to the highlighted parts, we can estimate that the typical training data size stored in AS layer memory is 100 bits8 instances (Set B) + 500 bits4 instances (Set A) = 2800 bits=0.35KB.

Observation 5: Based on the typical training data size of beam management provided in RAN1 Reply LS (R1-2310681), the typical training data size stored in AS layer memory is 100 bits8 instances (Set B) + 500 bits4 instances (Set A) =0.35KB.

From RAN1 reply LS (R1-2310681) LCM purpose; UE-side/NW-side models; Data content; Typical data size (per data sample); Typical latency requirement; Notes. Training; UE-side, NW-side; L1-RSRPs and/or beam-IDs; See Note 1 for L1-RSRPs; Relaxed. ==omit table rows on “Inference” and “Monitoring” === Note 1: There is no agreement on the data size of L1-RSRPs for Set A or Set B, but the following typical data size is provided as guidance for RAN2 discussion. Based on existing L1-RSRP reporting methodology, i.e., 7 bits for the strongest beam and 4 bits for the remaining beams, for Set B = 16 as an example, the typical data size would be 67 (hence up to ~100 bits), and for Set A = 128 as an example, the typical data size would be 515 (hence up to ~500 bits) if all beams in Set A were to be collected. For BM Case 2, the data size L1-RSRPs for Set A and Set B represents the data size per predicted future time instance and per history measurement time instance, respectively. Payload size may not be fixed. Note 2: There are no agreements on the reporting type. Note 4: Feasibility and necessity of the monitoring schemes listed in the table are under discussion. Note 5: For BM Case 2, the typical value of the number of history measurement time instance used in evaluations is up to 8 and typical value of the number of predicted future time instance is 1~4.

Based on above analysis, we think it is sufficient to reuse the same minimum AS layer memory size as logged MDT and QoE. Thus, we propose:

Proposal 6: The minimum AS layer memory size for L1 measurement logging is same as existing logged MDT and QoE (i.e. 64KB).

#### 2.1.3.3 FFS#3: (upon low power) whether the UE stops autonomously or report to the network

We discuss the FFS in agreement of RAN2#127 [3]:

Agreements

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network .

We have concern on both solutions listed in the FFS:

- Alt-1: UE autonomous stopping: Please note that RAN2#127 has agreed it is up to UE implementation to determines power state [3], which means there is no specified requirement on low power state. Thus, we think the UE autonomous stopping seems to be too risky because the UE and NW may not be aligned on the understanding of “low power state”.

- Alt-2: UE reports to the Network: If this solution is adopted, it is quite likely that UAI is used to report “low power state”. However, we have concern that the NW will ignore the UE’s reporting without any followed action because Section 7.9 of TS 38.300 [9] has captured “it is up to gNB whether to accommodate the request in UAI”. It will make this feature useless.

To resolve above issues, we prefer to specify a solution in-between Alt-1 and Alt-2 which is similar to reactive temporary restriction reporting specified in Rel-18 MUSIM. In detail, the UE starts T348 upon sending UAI with temporary restriction request, and the UE may autonomously apply the requested temporary capability restriction if T348 expires without reception of NW reaction. The detailed RAN2 agreement and its specification in TS 38.331 [10] can be found below:

RAN2#123b agreement: We will introduce ‘wait timer’ for the reactive approach - The UE starts the timer when the UE requests a temporary restriction to the network if the timer is configured. We assume network configures the length for this timer. - Stop: if UE receives reconfiguration that does not exceed the capabilities that UE suggested via capability restriction report - Expiry: UE can apply the temporary UE capability restriction upon the timer expiry. From Section 7.1.1 of TS 38.331: Timer; Start; Stop; At expiry. T348; Upon transmission of MUSIM temporary restriction of musim-CapRestriction for serving cell(s) with capabilities restricted, release of SCell or release of SCG.; Upon reception of RRCReconfiguration message that does not exceed UE temporary capability restriction transmitted via musim-CapRestriction.; UE may apply the temporary UE capability restriction in accordance with the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction..

Observation 6: Rel-18 MUSIM had specified reactive temporary restriction reporting. Specifically, the UE starts T348 upon sending UAI with temporary capability restriction request, and it may autonomously apply the requested temporary restriction if T348 expires without reception of NW reaction.

We believe similar solution can be introduced to harmonize Alt-1 and Alt-2. Specifically, the UE may send UAI to notify the NW on its low power state and starts a wait timer. If the wait timer expires without reception of NW reaction, it may stop the data collection.

Proposal 7: On the low power state issue during data collection, introduce the following solution similar to reactive temporary restriction reporting of Rel-18 MUSIM:

- Upon low power state, the UE may send UAI to notify the NW.

- The UE starts a wait timer when sending the UAI. If the wait timer expires without reception of NW reaction, it may stop the data collection.

#### 2.1.3.4 FFS#4: Details of event based data collection/logging

RAN2#127 [3] has agreed to support both periodic and event triggered logging. And it is FFS on details of radio condition based event.

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

We understand that the intention of radio condition based event is to allow NW to collect different datasets for cell center case and cell edge case, respectively. As a consequence, if the radio condition based event is not introduced, the UE may mix the data in cell center and cell edge together, which may make NW can’t train separate AI/ML models. Furthermore, existing logged MDT has supported two events to trigger logging:

- Event L1: when serving cell L3 measurement < threshold (similar to A2)

EventType-r16 ::= CHOICE {

outOfCoverage     NULL,

eventL1           SEQUENCE {

l1-Threshold      MeasTriggerQuantity,

hysteresis        Hysteresis,

timeToTrigger     TimeToTrigger

},

...

}

eventType The value outOfCoverage indicates the UE to perform logging of measurements when the UE enters any cell selection state, and the value eventL1 indicates the UE to perform logging of measurements when the triggering condition (similar as event A2 as specified in 5.5.4.3) as configured in the event is met for the camping cell in camped normally state.

Event L2: when UE enters any cell selection.

We think it is straight forward to reuse Event L1 (similar to event A2) to collect datasets for cell edge case. Meanwhile, we think event similar to A1 (i.e. L3 serving cell measurement becomes better than absolute threshold) can also be introduced to collect datasets for cell center case. Thus, we propose:

Proposal 8: Support the following two radio condition based event triggered logging:

- Event X1: when L3 serving cell measurement becomes better than absolute threshold (similar to A1).

- Event X2: when L3 serving cell measurement becomes worse than absolute threshold (similar to A2).

During online discussion of RAN2#127b [3], there was proposal to introduce area based event trigger, e.g. the UE performs logging only when the UE is within the configured cell list. In our understanding, this is not event trigger. Instead, it is areaConfiguration specified in legacy management based logged MDT:

LoggedMeasurementConfiguration-r16-IEs ::=  SEQUENCE {

traceReference-r16                          TraceReference-r16,

traceRecordingSessionRef-r16                OCTET STRING (SIZE (2)),

tce-Id-r16                                  OCTET STRING (SIZE (1)),

absoluteTimeInfo-r16                        AbsoluteTimeInfo-r16,

areaConfiguration-r16                       AreaConfiguration-r16                    OPTIONAL,  --Need R

plmn-IdentityList-r16                       PLMN-IdentityList2-r16                   OPTIONAL,  --Need R

bt-NameList-r16                             SetupRelease {BT-NameList-r16}           OPTIONAL,  --Need M

wlan-NameList-r16                           SetupRelease {WLAN-NameList-r16}         OPTIONAL,  --Need M

sensor-NameList-r16                         SetupRelease {Sensor-NameList-r16}       OPTIONAL,  --Need M

loggingDuration-r16                         LoggingDuration-r16,

reportType                                  CHOICE {

periodical                                  LoggedPeriodicalReportConfig-r16,

eventTriggered                              LoggedEventTriggerConfig-r16,

...

},

lateNonCriticalExtension                    OCTET STRING                             OPTIONAL,

nonCriticalExtension                        LoggedMeasurementConfiguration-v1700-IEs OPTIONAL

}

areaConfiguration

Used to restrict the area in which the UE performs measurement logging to cells broadcasting any of the included cell identities, the included tracking area codes/ frequencies, the included PNI-NPN identities or the SNPN identities. If areaConfiguration-r17 is present, the UE shall ignore areaConfiguration-r16. The areaConfiguration-v1800 is a non-critical extension of areaConfiguration-r17. See NOTE 1.

From Section 5.5a.3.2 of TS 38.331:

Omit unrelated part

2>	else if the reportType is set to eventTriggered and eventType is set to eventL1:

3>	if the UE is in camped normally state on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

3>	if the UE is in camped normally state on an NR cell and if the registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

4>	if areaConfiguration is not included in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig; or

4>	if the current serving cell is part of the area indicated by cag-ConfigList of areaConfiguration in VarLogMeasConfig, or if the current camping cell is part of the area indicated by snpn-ConfigList of areaConfiguration in VarLogMeasConfig;

5>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met;

Meanwhile, please note that TS 38.331 has specified procedure text when both Event L1 and areaConfiguration are configured in Section 5.5a.3.2 [10]:

Observation 7: areaConfiguration was introduced in legacy logged MDT as one configuration rather than one event, and TS 38.331 specifeid the procedure text when both Event L1 and areaConfiguration are configured.

Since similar specification can be reused in logged L1 measurement, the specification impacts seem to be managable. Thus, we propose to reuse the same modeling of logged MDT.

Proposal 9: Following the same modeling in logged MDT, introduce area Configuration in logged L1 measurement as one configuration rather than one trigger event.

### 2.1.4 Step 3: Details of availability reporting

In RAN2#127b [4], it was agreed to only support on-demand reporting of logged measurement, and availability reporting is supported. But the details of availability reporting are FFS.

Agreements on NW side data collection

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

#### 2.1.4.1 FFS#1: Definition of availability indication

According to previous discussion, we understand that at least two types of availability indication are required:

- Indicate UE’s logged measurement is available for NW retrieval.

- Indicate UE’s internal status, including low power state, low memory state and AS buffer is full.

And their detailed format can be further discussed.

Proposal 10: Support at least the following two types of availability indication. FFS their detailed format.

- Type 1: indicate UE’s logged measurement is available for NW retrieval

- Type 2: indicate UE’s internal status, including low power state, low memory state and AS buffer is full.

#### 2.1.4.2 FFS#2: How data availability indication is sent to the network

In legacy logged MDT, the UE may include the availability indication in various RRC complete messages (i.e., RRCReconfigurationComplete / RRCSetupComplete / RRCResumeComplete / RRCReestablishmentComplete). Essentially, it means only NW triggered availability indication reporting is supported. It is sufficient for logged MDT because logged MDT measurement is performed when the UE is in RRC_IDLE / RRC_INACTIVE state and NW is aware when the UE enters RRC_CONNECTED state. However, logged L1 measurement is performed in RRC_CONNECTED state and NW may not be aware when the UE has available logged measurement.

Observation 8: In legacy logged MDT, only NW triggered availability indication reporting is supported via various RRC complete messages because it is performed when the UE is in RRC_IDLE / RRC_INACTIVE state and NW is aware when the UE enters RRC_CONNECTED state.

Thus, UE initiated availability reporting needs to be supported as well, and it is straight forward to reuse UAI.

Proposal 11: Support both NW triggered availability reporting and UE initiated availability reporting:

- NW triggered: reporting as response to NW request via various RRC complete messages.

- UE initiated: it is up to UE implementation when to report via UAI message.

### 2.1.5 Step 4: Reporting of logged L1 measurement

RAN2#127b further discussed which SRB is used to report logged L1 measurement, and down scoped to between new SRB and SRB4 [4]:

Agreements on NW side data collection

- Low priority SRB will be used.    FFS new SRB or use of SRB4

We prefer to introduce a new SRB due to below consideration

- Introducing a new SRB is useful to have differentiated handling between transfer of AI/ML dataset and transfer of QoE measurements.

  - If reusing SRB4 for AI/ML dataset transfer, the UE/NW can’t differentiate between the AI/ML dataset transfer and QoE measurement. As QoE measurements typically have large payload amount, it may block the transfer of AI/ML dataset. Furthermore, dataset transfer may have latency requirement as identified in training collaboration option 4 in RAN1#116 [11]. In this case, transfer of QoE measurements may cause latency of AI/ML dataset transfer and finally lead to failure of training collaboration.

Agreement

To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, study the following options:

- Option 1: Fully standardized reference model (structure + parameters)

- Option 2: Standardized dataset

- Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side

- Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side

- Option 5: Standardized model format + Reference model exchange between NW-side and UE-side

- Introducing a new SRB is more forward compatible because enhanced MDT should be a general framework for NW-sided data collection.

- Specification work of introducing a new SRB is manageable.

  - The spec work will be quite similar to SRB4. We don’t foresee significant work.

Observation 9: Dataset transfer may have latency requirement as identified in training collaboration option 4 in RAN1#116. If reusing SRB4 for AI/ML dataset transfer, transfer of QoE measurements may cause latency of AI/ML dataset transfer and finally lead to failure of training collaboration.

The new SRB should have lower priority than SRB1 and SRB2. Its priority between SRB4 can be further discussed.

Proposal 12: Introduce a new SRB to report logged L1 measurements with priority lower than SRB1/SRB2. FFS whether its priority is higher than SRB4.

### 2.1.6 How to handle logged measurement in HO or IDLE/INACTIVE state

This issue was raised during online discussion of RAN2#127b [4], but no conclusion was made. In our understanding, there are two different views when the UE enters IDLE/INACTIVE state or during HO but has logged L1 measurements in its AS buffer:

- Alt-1: The UE keeps the logged L1 measurements in its AS buffer. When it returns to CONNECTED state or completes HO, it notifies NW on the availability of stored measurements for NW to retrieve.

- Alt-2: The UE discards all the logged L1 measurements in its AS buffer.

We don’t think Alt-2 makes sense:

- Alt-2 implies risk of UE wasting its power for NW-side data collection but result in nothing. With such risk, the UE may not be willing to perform data collection for NW.

- The stored L1 measurements may still be useful to NW because the UE may enter CONNECTED state shortly.

- It will introduce extra spec work. Please note that Alt-1 is similar to existing logged MDT and early measurement, and thereby the existing spec changes can be reused.

Thus, we propose to adopt Alt-1:

Proposal 13: When the UE enters IDLE/INACTIVE state or during HO but has logged L1 measurements in its AS buffer, it keeps the logged measurements in its AS buffer. When it returns to CONNECTED state or completes HO, it notifies NW on the availability of stored measurements for NW to retrieve.

## 2.2 NW-sided data collection for AI/ML based positioning

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

  - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

First, please note that the updated Rel-19 WID [1] has clearly indicated RAN1 will not discuss any proposals on 2nd priority issues.

Following the guidance from the updated WID, we think RAN2 can also postpone the NW-side data collection discussion on case 2a/2b which are 2nd priority issues.

Proposal 14: Following guidance of updated WID, RAN2 postpone the discussion on data collection of AI/ML based positioning Case 2a)/2b) which are 2nd priority issues.

Then, for case 3a)/3b), Section 7.2.1.3.1 of TR 38.843 captured below conclusion [12]:

From Section 7.2.1.3.1 of TR 38.843

For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

Based on above conclusion, it seems to be straight forward for RAN2 to assume NPPRa will be used for exchange of dataset / measurement / label / inference outcome in case 3a/3b and RAN2 can just leave work to RAN1/RAN3. Thus, we propose:

Proposal 15: For case 3a)/3b) of AI/ML based positioning, RAN2 assume NPPRa is used to exchange dataset and inference outcome between gNB and LMF and RAN2 leave data collection design to RAN1/RAN3.

## 2.3 Collected data type of NW-sided data collection

NW-sided data collection includes NW-side model of AI/ML based management and case 2b)/3a)/3b) of AI/ML based positioning. Based on our discussion in Proposal 5, Proposal 14 and Proposal 15, we have the below observation:

- NW-side model of AI/ML based management: at least L1-RSRP, beam ID and logging time stamps are required data type. Whether other data to collect depends on RAN1.

- Positioning Case 2b): its discussion should be postponed.

- Positioning Case 3a)/3b): leave data collection design to RAN1/RAN3.

Thus, we don’t think RAN2 have immediate work on collected data type:

Proposal 16: On required data type for NW-sided data collection, RAN2 wait RAN1 conclusion on NW-side model of AI/ML based management and leave Positioning Case 3a)/3b) to RAN1/RAN3.

## 2.4 Collected data type of UE-sided data collection

UE-sided data collection includes UE-side model for AI/ML based beam management and positioning case 1/2a). As the UE-side model is deployed at UE side, the basic assumption is that UE-side model is trained by UE side, and thus the required data type corresponding to AI/ML model input is naturally known at UE side. Therefore, when UE uses UE-side AI/ML model for inference, the UE can generate the measurement for AI/ML model inference using the same type or format of the measurement as training phase by implementation. 3GPP should not restrict the data type used for input of AI/ML model training. Thus, we do not think it is necessary to specify the data type of AI/ML model input. 3GPP only need to specify model output for UE-side model, and reference signal (RS) used for model input measurement and its configuration.

Observation 10: As the UE-side model is deployed at UE side, UE-side model is trained by UE side, and thus the required data type corresponding to AI/ML model input is naturally known at UE side. 3GPP should not restrict the data type used for input of AI/ML model training.

Thus, we propose:

Proposal 17: On model training and inference for UE-sided model, RAN2 assume that 3GPP only specify model output, reference signal (RS) used for model input measurement and its configuration. The specific data used for model input and their intermediate processing is up to UE implementation.

Meanwhile, we agree that data type needs to be specified for performance monitoring, especially when the NW performs the monitoring. But it is RAN1 expertise and RAN1 is working on it for both AI/ML based management and AI/ML based positioning. Thus, to avoid potential duplicated work, we propose RAN2 to leave the work to RAN1.

Proposal 18: On performance monitoring for UE-sided model, RAN2 leaves to RAN1 on the required data type.

Finally, RAN2 should send LS to RAN1 on Proposal 17 and Proposal 18.

Proposal 19: RAN2 send LS to RAN1 to check any issue for assumption in Proposal 17 and Proposal 18.

# 3 Conclusion

In this contribution, we discuss remaining issues of NW-sided data collection. Our Observation are:

Observation 1: Difference between OAM centric and gNB centric data collection is which NW entity (OAM vs gNB) to trigger data collection and generate data collection configuration. Their AS procedure is same.

Observation 2: UE privacy requirement is preserved in OAM centric data collection due to UE consent mechanism in MDT, but it is not clear how to satisfy UE privacy requirement in gNB centric data collection.

Observation 3: Existing L1 CSI framework already supports multiple CSI-RS resource set configurations: multiple NZP-CSI-RS-ResourceSet and multiple CSI-ReportConfig can be configured where one NZP-CSI-RS-ResourceSet can be associated with one CSI-ReportConfig via CSI-ResourceConfigId.

Observation 4: In legacy NR, logged MDT, QoE measurements in IDLE/INACTIVE state and QoE paused measurements had specified the minimum AS layer memory size, which are all 64KB according to TS 38.306.

Observation 5: Based on the typical training data size of beam management provided in RAN1 Reply LS (R1-2310681), the typical training data size stored in AS layer memory is 100 bits8 instances (Set B) + 500 bits4 instances (Set A) =0.35KB.

Observation 6: Rel-18 MUSIM had specified reactive temporary restriction reporting. Specifically, the UE starts T348 upon sending UAI with temporary capability restriction request, and it may autonomously apply the requested temporary restriction if T348 expires without reception of NW reaction.

Observation 7: areaConfiguration was introduced in legacy logged MDT as one configuration rather than one event, and TS 38.331 specifeid the procedure text when both Event L1 and areaConfiguration are configured.

Observation 8: In legacy logged MDT, only NW triggered availability indication reporting is supported via various RRC complete messages because it is performed when the UE is in RRC_IDLE / RRC_INACTIVE state and NW is aware when the UE enters RRC_CONNECTED state.

Observation 9: Dataset transfer may have latency requirement as identified in training collaboration option 4 in RAN1#116. If reusing SRB4 for AI/ML dataset transfer, transfer of QoE measurements may cause latency of AI/ML dataset transfer and finally lead to failure of training collaboration.

Observation 10: As the UE-side model is deployed at UE side, UE-side model is trained by UE side, and thus the required data type corresponding to AI/ML model input is naturally known at UE side. 3GPP should not restrict the data type used for input of AI/ML model training.

Based on observations, our proposals can be found below:

Overall procedure of enhanced MDT

Proposal 1: To strive for a common measurement framework, similar UE consent mechanism in MDT is supported in gNB-centric data collection, i.e. UE consent is required before NW configures the UE gNB-centric data collection. FFS signaling details.

RRC framework of enhanced MDT

Proposal 2: MeasConfig in RRCReconfiguration is reused to configure logged L1 measurements with below specification changes:

- Within MeasObjectNR: introduce L1 CSI resource configuration and logging configuration.

- Within ReportConfigNR: introduce on-demand reporting configuration to indicate at least reporting type (i.e. on-demand) and reporting quantity configuration (e.g. L1 RSPR and/or beam index).

Proposal 3a: Do not support multiple configurations of logged L1 measurements (e.g. multiple MeasConfig).

Proposal 3b: Similar to existing L1 CSI framework, multiple L1 CSI resource sets can be configured to be linked with different logging configurations (e.g. with different logging periodicities) in MeasObjectNR.

Proposal 4: Do not support dynamic activation/deactivation of logged L1 measurements.

How the UE performs logging

Proposal 5: Besides L1 RSRP and/or beam-IDs, the UE also collects logging time stamp in its AS buffer.

Proposal 6: The minimum AS layer memory size for L1 measurement logging is same as existing logged MDT and QoE (i.e. 64KB).

Proposal 7: On the low power state issue during data collection, introduce the following solution similar to reactive temporary restriction reporting of Rel-18 MUSIM:

- Upon low power state, the UE may send UAI to notify the NW.

- The UE starts a wait timer when sending the UAI. If the wait timer expires without reception of NW reaction, it may stop the data collection.

Proposal 8: Support the following two radio condition based event triggered logging:

- Event X1: when L3 serving cell measurement becomes better than absolute threshold (similar to A1).

- Event X2: when L3 serving cell measurement becomes worse than absolute threshold (similar to A2).

Proposal 9: Following the same modeling in logged MDT, introduce area Configuration in logged L1 measurement as one configuration rather than one trigger event.

Details of availability reporting

Proposal 10: Support at least the following two types of availability indication. FFS their detailed format.

- Type 1: indicate UE’s logged measurement is available for NW retrieval

- Type 2: indicate UE’s internal status, including low power state, low memory state and AS buffer is full.

Proposal 11: Support both NW triggered availability reporting and UE initiated availability reporting:

- NW triggered: reporting as response to NW request via various RRC complete messages.

- UE initiated: it is up to UE implementation when to report via UAI message.

Reporting of logged L1 measurement

Proposal 12: Introduce a new SRB to report logged L1 measurements with priority lower than SRB1/SRB2. FFS whether its priority is higher than SRB4.

Proposal 13: When the UE enters IDLE/INACTIVE state or during HO but has logged L1 measurements in its AS buffer, it keeps the logged measurements in its AS buffer. When it returns to CONNECTED state or completes HO, it notifies NW on the availability of stored measurements for NW to retrieve.

NW-sided data collection for AI/ML based positioning

Proposal 14: Following guidance of updated WID, RAN2 postpone the discussion on data collection of AI/ML based positioning Case 2a)/2b) which are 2nd priority issues.

Proposal 15: For case 3a)/3b) of AI/ML based positioning, RAN2 assume NPPRa is used to exchange dataset and inference outcome between gNB and LMF and RAN2 leave data collection design to RAN1/RAN3.

Collected data type of NW-sided data collection

Proposal 16: On required data type for NW-sided data collection, RAN2 wait RAN1 conclusion on NW-side model of AI/ML based management and leave Positioning Case 3a)/3b) to RAN1/RAN3.

Collected data type of UE-sided data collection

Proposal 17: On model training and inference for UE-sided model, RAN2 assume that 3GPP only specify model output, reference signal (RS) used for model input measurement and its configuration. The specific data used for model input and their intermediate processing is up to UE implementation.

Proposal 18: On performance monitoring for UE-sided model, RAN2 leaves to RAN1 on the required data type.

Proposal 19: RAN2 send LS to RAN1 to check any issue for assumption in Proposal 17 and Proposal 18.

# 4 References

[1] RP-234039, New WID: New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#126, Chair Notes.

[3] RAN2#127, Chair Notes.

[4] RAN2#127b, Chair Notes.

[5] TS 32.422-v17.8.0, Trace control and configuration management, 2022-9.

[6] RAN1#116b, Chair Notes.

[7] TS 38.306-v18.0.0, User Equipment (UE) radio access capabilities, 2023-12.

[8] R1-2310681, Reply LS on Data Collection Requirements and Assumptions, RAN1.

[9] TS 38.300-v18.0.0, NR and NG-RAN Overall Description; Stage 2, 2023-12.

[10] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[11] RAN1#116, Chair Notes.

[12] TR 38.843-v2.0.1, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 2023-12.