# 3GPP TSG RAN WG2 Meeting #128      	                           R2-2409964
Orlando, USA, November 18-22, 2024

Agenda item:	8.2.3

Source:	Apple

Title:	Discussion on Ambient IoT Paging

WID/SID:	FS_Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# 1 Introduction

The following agreements have been reached during RAN2#125bis [1]:

- Legacy paging message for device will not be supported. - Legacy paging occasion and legacy DRX for the device is not supported. This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well). - RAN2 assumes that the device will not support tracking/RAN area update procedure. - For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message. FFS pending the details from SA2

And the following agreements have been reached during RAN2#126 [2]:

- RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. - AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. - From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

And the following agreements have been reached during RAN2#127[3]:

- Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices - wait for further RAN1 progress on indication of the start of access occasion.

And the following agreements have been reached during RAN2#127bis[4]:

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase. - Based on this information the device determines whether to skip sending the response to paging. - From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs. RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs. - If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA - For all use cases, device determines the random access type from the paging message. FFS whether it is explicitly or implicitly. FFS whether we will down selection for all RA types (2step, 3step, CFRA) or try to unify the design for 2step, 3step

In this paper, we discuss the remaining issues on A-IoT paging design.

# 2 Discussion

### 2.1 Paging round

For A-IoT paging design, one thing to consider is that the “success” of A-IoT paging is not guaranteed, and the performance of paging could be poor if the paging message is just transmitted one-time. This is due to the fact that the reachability of the A-IoT devices are directly affected by the energy harvesting status of the devices. Also, the success ratio for contention-based access is relatively low. Therefore, for each CN request for paging, the reader can initiate multiple paging message. First, it is important to clarify the “Paging round” concept which should be defined as the time duration between two consecutive A-IoT paging messages triggered for the same CN request. A paging round provides/configures slots for devices to access during this duration, so paging round is sort of equivalent to an “access round”. The next paging message will provide/configure “fresh” slots for the devices to access, and so on.

Proposal 1	“Paging round” is defined as the duration between two consecutive paging messages associated with the same CN request.

Then, regarding how to avoid duplicated response from devices, RAN2#127bis [4] has agreed to introduce some information in the paging message which is “short and simple”. For the sake of distinguishing transmissions of paging message associated to the same CN request, this information is needed in the paging message of each paging round, so that the devices at least can recognize the retransmission of the same paging request (to avoid duplicated response) with the same transaction identifier. There are some discussion on whether this information is a transaction ID, session ID or NDI-like, which is to be further determined in WI stage.

But there is one thing worth clarifying is that the subsequent message should still contain the device ID which is used to identify the paging recipients. This is because the transaction ID or session ID can only be recognized by the devices which has already received the initial paging message, but not by the devices which has failed to receive the initial paging message. Therefore, to effectively reaching the recipients which fails to receive the initial paging message, the device ID has to be included in every paging round, in addition to the transaction ID/session ID, which is used to suppress duplicate response.

Proposal 2	Device ID(s) and transaction ID(or session ID/NDI) are included in the paging message of every paging round.

Based on the agreements of CBRA (Contention-Based Random Access) procedure, the first two messages in CBRA are supposed to be used for contention resolution. Thus, the A-IoT device which has already responded to the same paging message transmitted earlier shall not try to respond again if the Msg1 & Msg 2 have been completed.

Proposal 3	A-IoT device has already completed Msg1/Msg2 exchange in a Paging procedure identified by transaction ID should suppress responding to paging message(s) containing the same transaction ID.

### 2.2 Paging with Multiple device IDs

RAN2#127bis [4] has agreed the following for including multiple device IDs in the paging message:

From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs.    RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

We think for the DL of A-IoT air interface, RAN2 intends to avoid having a segmentation scheme introduced. It is widely assumed that each device ID is 128-bits long itself. Based on RAN1 LS reply [5], the maximal TB size for both R2D and D2R is only 1000 bits. Considering the necessary A-IoT MAC layer overhead and upper layer overhead, a paging message can only include up to 7 device IDs. It is also worth noting that the paging message may also include the command together if “command-only” case is to be supported and that will further reduce the number of device IDs which can be accommodated in one R2D transmission. Also, support those extra device IDs, although just a few, means some added variety in R2D message formats. That will also result into more device complexity, as device has to prepare to decode any number of device ID fields in the R2D message.

Given that, it is questionable what is the real benefit to include 6 or 7 device IDs in a single paging message, instead of paging them one by one by the reader. It is quite uncommon that the inventory request from CN will only target 6 or 7 individual A-IoT devices. If the CN intends to reach a large number of homogenous A-IoT devices, then it’d better to include a single “group device ID” instead.

Based on the analysis above, we believe that “paging with multiple device IDs” is kind of optimization with very limited usage scenarios. It may help to reduce the # of DL transmissions but adding the device complexity. We sugest this optimization is not supported in Rel-19 work.

Proposal 4	“Paging message containing multiple device IDs” is not supported in Rel-19.

### 2.3 Reader ID

When CN generates a paging request for inventory, it may send the same CN request to multiple readers because it does not know the exact location(s) of the A-IoT devices. Then, it is quite probable that the device may be paged by multiple different readers, simultaneously or at different time. Although some may argue NW can coordinate the A-IoT readers to only trigger paging from different reader at different time, we think this may negatively impact the overall latency for a CN-initiated inventory procedure. Some sort of parallel paging in a wide area is desirable to ensure a timely competion of A-IoT transaction.

In the SA2 solution [6], “Reader ID” has been proposed to be used for both Topology 1 and Topology 2 to make device aware of whether it is handled by the same or different reader. Therefore, we think it is beneficial to introduce a “reader ID” to at least allow the device to differentiate those readers in the signaling process. The assignment/allocation of “reader ID” should be controlled by CN. This can be optionally included in the paging message.

Whether the device should respond to every request from different readers can be further discussed.

Proposal 5	“Reader ID” is optionally indicated in the A-IoT paging message to differentiate readers. FFS whether the device receiving multiple paging messages from different readers should only response to one of them or all of them.

### 2.4 Paging with Overlapping CN Request

One consequence of the allowing multiple paging rounds is that the paging procedure associated with a CN request would take a much longer time to complete, compared to a single-shot transmission of paging message in PRDCH channel. Therefore, whether the A-IoT reader can be involved in multiple overlapping CN-triggered paging procedures need to be discussed. If reader conducts multiple paging transactions at the same time, then an A-IoT device may be potentially paged by the same reader multiple times with different “paging records” (e.g. with different paging masks/filters), either in the same R2D transmission or in multiple subsequent transmissions. Then, a couple of complicated questions needs to be answered. For example, whether the device need to respond to each of those separate paging transactions? And if not, how to select/prioritize the paging message to respond. It is worth noting that solving those issues would add complexity to the device side. So, RAN2 should have some restrictions on CN/NW side to not cause such a scenario. For instance, the reader may not initiate a new paging procedure associated to a new CN request until the previous inventory procedure is completed.

Proposal 6	An A-IoT reader should not initiate paging for a new CN request which overlaps with the paging procedure triggered by a prior CN request.

### 2.5 D2R scheduling in Paging message

It has been agreed in RAN2#126 [2] that

AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

Regarding the FFS point above, we notice that RAN1 has a general agreement on D2R scheduling as below:

Scheduling information of PDRCH transmission is provided by a corresponding PRDCH.

Based on the above RAN1 agreement, we believe at least some scheduling information is to be provided by the R2D transmission. Given the options being discussed in RAN1 meeting [3], D2R transmission’s timing, in general, should be relative to the transmission timing of R2D message. Therefore, for D2R response scheduling, we need consider the following information:

- Resource type (exclusive or shared)

- Start time of (as relative to the R2D message)

- Time duration of the D2D transmission

- Frequency information (RBs, frequency offset) in PDRCH channel

The TB size of A-IoT is limited. It is worth nothing that the paging message itself may contain multiple identifiers as agreed in RAN2#126. Also, the length of a A-IoT device identifier can be as long as 128bit, and additional prefix/mask information may need to be included to identify a group of devices. Thus, there may not be enough room for a D2R transmission to contain too many R2D scheduling information in a DL MAC PDU. In particular, paging may trigger contention-based access (e.g., slotted aloha), which need multiple candidate D2R resources (e.g. access occasions) to be used by the intended paging recipients to choose from. So, we think it would be unreasonable to assume the scheduling of D2R transmissions are solely rely on the on-the-fly scheduling information in PRDCH transmission. Some of the scheduling information must be (pre-)configured instead of explicitly indicated in the paging message itself.

Proposal 7	RAN2 discuss which scheduling information should be (pre-)configured for D2R response scheduling instead of explicitly indicated in the paging message.

Regarding the resource type indication (exclusive vs. shared) for D2R scheduling in paging message, we think this can be indicated in an efficient manner by assuming each resource is shared by default. The reason for this design is that the contention-based RA is usually triggered by a paging message intends to reach a group of devices. As a result, more candidate “shared” resources need to be scheduled in the paging message. If there is a way to avoid spending precious information bits to indicate the resource type, it would be beneficial. Thus, it is proposed to assume the resource scheduled in paging message by default is “shared” unless explicitly indicated as a contention-free resource exclusively to be used by one intended A-IoT device.

Proposal 8	D2R resource indicated in Paging message is by default shared resource, unless explicitly indicated as contention-free resource.

Finally, the paging message is not the only R2D message to be transmitted by the reader in PRDCH channel. It is also possible that some other R2D messages can contain corresponding D2R scheduling information. For example, A-IoT Msg2 in RA procedure may contain the scheduling information for A-IoT Msg3. We think the scheduling design for D2R transmissions in A-IoT system should follow a unified solution applicable to all R2D messages. There is no need to design different scheduling schemes for A-IoT paging and other A-IoT R2D message(s).

Proposal 9	D2R scheduling mechanism in PRDCH is common for both Paging message and other DL R2D messages.

# 3 Conclusion

In this contribution, we discuss the paging issue for Ambient IoT, and have the following proposals:

Proposal 1	“Paging round” is defined as the duration between two consecutive paging messages associated with the same CN request.

Proposal 2	Device ID(s) and transaction ID(or session ID/NDI) are included in the paging message of every paging round.

Proposal 3	A-IoT device has already completed Msg1/Msg2 exchange in a Paging procedure identified by transaction ID should suppress responding to paging message(s) containing the same transaction ID.

Proposal 4	“Paging message containing multiple device IDs” is not supported in Rel-19.

Proposal 5	“Reader ID” is optionally indicated in the A-IoT paging message to differentiate readers. FFS whether the device receiving multiple paging messages from different readers should only response to one of them or all of them.

Proposal 6	An A-IoT reader should not initiate paging for a new CN request which overlaps with the paging procedure triggered by a prior CN request.

Proposal 7	RAN2 discuss which scheduling information should be (pre-)configured for D2R response scheduling instead of explicitly indicated in the paging message.

Proposal 8	D2R resource indicated in Paging message is by default shared resource, unless explicitly indicated as contention-free resource.

Proposal 9	D2R scheduling mechanism in PRDCH is common for both Paging message and other DL R2D messages.

# 4 References

[1] Chairman’s Note, RAN2#125bis

[2] Chairman’s Note, RAN2#126

[3] Chairman’s Note, RAN1#127

[4] Chairman’s Notes, RAN2#127bis

[5] R2-2409405	Reply LS to RAN2 on data block sizes for Ambient IoT (R1-2409250; contact: MediaTek)	RAN1 LS in	Rel-19	FS_AmbientIoT, FS_Ambient_IoT_solutions	To:RAN2	Cc:SA2

[6] S2-2411267, (P-CR) 23.700-13: KI#1, #2, #3, New Solution: Harmonized RAN-CN solution using AIOTF for Topology 2. 	Ericsson