# 3GPP TSG RAN WG2 Meeting #128      	                           R2-2409967
Orlando, USA, November 18-22, 2024

Agenda item:	8.13.2

Source:	Apple

Title:	Relay discovery and selection for Multi-hop UE-to-NW Relay

WID/SID:	NR_SL_relay_enh2– Release 19

Document for:	Discussion and Decision

# 1 Introduction

RAN2#127[1] has made the following agreements on relay discovery and selection:

- From RAN2 perspective, models A and B can both be supported. - Reuse existing AS discovery protocol to transmit discovery message for multi-hop U2N relaying. - Reuse SL-SRB4 to transmit discovery message for multi-hop U2N relaying. - Reuse existing resource pools defined for discovery message transmission and reception - Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs; FFS mode 1 for remote UE. - Configuration can be provided by SIB/dedicated message or pre-configured as in Rel-17/18. - If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18. - If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds). - FFS discovery conditions for the intermediate relay UEs.

In this paper, we discuss some controversial issues and FFS in relay discovery and selection aspects for Multi-hop UE-to-NW relay.

# 2 Discussion

### 2.1 Uu threshold for intermediate relay UE discovery message transmission

In 3GPP Rel-17, a UE-to-NW relay shall check both lower and upper Uu RSRP thresholds to decide whether it can be an in-coverage UE-to-NW relay UE. For the Rel-19 scenario, we think an intermediate relay UE, if it is in coverage, should also check the same thresholds to determine whether it can be the last relay UE first. Only if it cannot act as the last relay UE, it will consider itself a potential candidate of intermediate relay UE (assuming it can reach or discover another relay UE in coverage). Thus, before any Model A/B discovery message transmission from an intermediate relay UE, the UE itself needs to determine its role first.

Proposal 1 	An in-coverage relay UE shall always determine whether it can be the “Last relay UE” first and may become intermediate relay UE only if it cannot satisfy the conditions of last relay UE.

For example, in Model B discovery, the remote UE or another intermediate relay UE may send a discovery solicitation message, and if the receiving UE determine that it can be the last relay UE based on legacy Uu thresholds configured for U2N relay UE, then there is no need of any forwarding of relay discovery message. Otherwise, it then can forward the discovery message as an intermediate relay UE candidate.

Figure 1: Relationship between intermediate relay UE location region and Uu RSRP

Therefore, we think there is no need to introduce any new Uu thresholds for intermediate relay UE.

Proposal 2 	No need to have separate Uu threshold(s) for intermediate relay UE.

### 2.2 Resource allocation for discovery transmission

Regarding the FFS in the following agreement [1]:

Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs; FFS mode 1 for remote UE.

We think for remote UE in Multi-hop R19 scenario, it does not have a direct Uu link and it cannot receive DCI signal transmitted by gNB. Therefore, mode 1 resource allocation is anyway crippled this remote UE as not all mode 1 mechanisms are working. It is much simpler to not support mode 1 and only allow remote UE to use resource allocaton mode 2.

It is also worth clarifying that the agreement “Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs;” means for legacy R17/R18 relay which has to be the last relay UE. For intermediate relay UE which connects to the gNB via multiple hops, mode 1 support is also questionable. Hence, a much cleaner deisgn is to only have mode 2 resource allocation for both remote UE and intermediate relay UEs.

Proposal 3 	Only mode 2 RA is supported for remote UE and intermediate relay UE.

### 2.3 L2 ID

One of the principles held in Rel-17/Rel-18 Sidelink Relay design is that the relay discovery and relay communication using different L2 ID(s). We think this should also be held for the Rel-19 design.

Proposal 4 	Source Layer 2 ID used for relay discovery messages are different from the source layer 2 ID used for relay communication.

Then, regarding the discovery message transmitted by intermediate relay UE, there would be one associated Src L2 ID. However, there is nothing wrong for this UE also acting as a remote UE by itself. In that case, the relay UE can also choose a different L2 ID for the “remote UE” role . Therefore, except for the UE itself, it will appear like two different UE(s) in other node’s view for the sake of discovery. This is a legitimate scenario, and we do not see why it should be prohibited.

Proposal 5 	It is up to intermediate relay UE implementation that whether the same or multiple different L2 IDs are used for relay discovery message transmission, when it acts as different roles.

For relay discovery, it is also possible that the U2N relay discovery messages transmitted by different L2 IDs (even from the same physical UE), are responded by different relay UEs, as the upper layer discovery “User Info” are different. RSC codes may also be different. It is not clear how to restrict the intermediate relay UE can always only choose the same next-hop relay node, unless some detailed changes are added in SA2 and RAN2 spec to forbid this. This may result some specification impact even for legacy UE behaviour.

Anyway, this is not a RAN2 only issue, so we suggest checking with SA2 that whether the following scenario as shown in Figure 2 can be supported:

Figure 2 A double role “Relay + Remote” UE selecting different relay UEs

Proposal 6 	Check with SA2 whether the scenario that different relay UEs are selected by different roles is possible and whether it should be supported or excluded.

### 2.4 Cross-Path Scenarios

There are certain cross-path scenarios discussed in the post-127meeting email discussion [2], but there is no consensus whether they are to be considered in Rel-19 work. For example, they are two possible scenarios are shown in Figure 2 below:

Figure 3: Cross-path scenarios

In scenario (a), Remote UE has a single path Remote →relay_first→relay_last1→gNB1 to reach the NW, and Remote UE 2 has its respective single multi-hop path Remote 2→ relay_first→relay_last2→gNB2 to reach the NW. In scenario (b), similarly, there are two different paths between remote UE(s) and their respective gNBs, although sharing a common PC5 link between the two intermediate relay UEs.

It is a misunderstanding that the above scenarios are multi-path scenarios (e.g., there are two indirect path between remote UE ang gNB(s). As can be seen above, for each Uu DRB between remote UE and its corresponding gNB, (i.e., between the pair of Uu PDCP entities), there is only a single path. The multi-path concern is only meaningful is the PDCP entity is on the intermediate relay UE. But for end-to-end traffic transported by intermediate relay UEs, only SRAP entity is operating and there is no need to establish Uu PDCP entity in . Even in that case, the Uu PDCP entity in intermediate relay UE can still use just a single path to reach one of the preferred gNBs for its own traffic.

Proposal 7	Cross-path scenarios shall be supported as there are no multiple paths established/used between Uu PDCP entities.

### 2.5 Intermediate Relay UE reselection

Any PC5 link of intermediate relay UE may fail. If such a RLF is detected (for example, as shown in Figure 4) To consider the forward extensibility of relay discovery design, we consider the scenario with two hops as an example:

Figure 4: PC5 RLF detected between 2nd intermediate relay UE and last relay UE

There are two options can be considered:

Option 1: The intermediate relay UE performs relay reselection and find a proper last relay UE.

Option 2: The intermediate relay UE sends notification message to remote UE, and let remote UE to re-initiate Model B discovery.

In our understanding, both options are feasible, and an intermediate UE can even do both. Nonetheless, for the sake of fast recovery, it is desirable to support intermediate relay UE to immediately start relay reselection procedure to find an alternative path towards gNB.

Similarly, if a Uu RLF is detected, the last relay UE will inform the Uu RLF to the intermediate relay UE. In this case, the intermediate relay UE shall also trigger relay re-selection for fast recovery.

Proposal 8	Support intermediate relay UE to trigger relay reselection if PC5 RLF is detected or Uu RLF is informed.

# 3 Conclusion

In this contribution, we discuss the Relay discovery and selection for Multi-hop UE-to-NW Relay, and have the following proposals:

Proposal 1 	An in-coverage relay UE shall always determine whether it can be the “Last relay UE” first and may become intermediate relay UE only if it cannot satisfy the conditions of last relay UE.

Proposal 2 	No need to have separate Uu threshold(s) for intermediate relay UE.

Proposal 3 	Only mode 2 RA is supported for remote UE and intermediate relay UE.

Proposal 4 	Source Layer 2 ID used for relay discovery messages are different from the source layer 2 ID used for relay communication.

Proposal 5 	It is up to intermediate relay UE implementation that whether the same or multiple different L2 IDs are used for relay discovery message transmission, when it acts as different roles.

Proposal 6 	Check with SA2 whether the scenario that different relay UEs are selected by different roles is possible and whether it should be supported or excluded.

Proposal 7	Cross-path scenarios shall be supported as there are no multiple paths established/used between Uu PDCP entities.

Proposal 8	Support intermediate relay UE to trigger relay reselection if PC5 RLF is detected or Uu RLF is informed.

# 4 References

[1] Chairman’s notes, RAN2#127

[2] R2-24xxxxx, Report of [Post127][401][Relay] Multi-hop relay discovery and (re)selection (LG)