3GPP TSG-RAN WG2 Meeting #128	R2-2409995

# Orlando, USA,  November 18th - 22th 2024

Agenda Item:	8.11.2

Source:	Ericsson

Title:	SBFD RA aspects

Document for:	Discussion

# 1	Introduction

In the WID [1], the following objective regarding random access operation has been defined for RAN2

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier:

  - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

We discuss the RAN2 aspects for the above objective in this paper. In the below, we use the term SBFD RA operation to stand for random access operation in SBFD symbols unless otherwise stated. We use the term legacy RA operation to stand for random access operation during non SBFD symbols.

In this paper, a SBFD aware UE means a UE which is capable of operating in a cell configured with SBFD feature, i.e., the cell/the gNB transmits DL and receive UL simultaneously in SBFD slots and symbols within a carrier. The UE can be aware of SBFD configurations so that the UE knows which slots/symbols are SBFD capable, which are also referred to as SBFD slots/symbols. This doesn’t mean that the UE needs to support full duplex operation.

# 2	Discussion

## 2.1 Agreements

RAN2 has made the below agreements at RAN2#127bis:

- RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact.

- From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

- Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

- For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

- The following two RACH configuration options are considered for SBFD based random access:

    - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed.

    - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

- For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters.

- The RACH configuration for SBFD is transmitted via SIB1.

- FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

## 2.2	General RA configuration

In RAN1 #117 RAN1 agreed to support two options for RACH configuration, a single RACH configuration and an additional RACH configuration. It was also agreed that a UE is not required to support both configurations (cf. yellow highlighting), considering the scenarios for the different configurations are substantially different.

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

The above agreement does not say anything on whether a cell can both SBFD RACH configuration options (i.e., Option 1 with Alt 1-1 and Option 2). In our view, the use case for a cell supporting both SBFD RACH configuration options is non-existent, considering the highly different properties of the two configuration options. Whereas the single RACH configuration i.e., RACH configuration Option 1 with Alt 1-1 may provide larger RACH capacity and lower latency, the second SBFD RACH configuration i.e., RACH configuration Option 2 will provide increased coverage and range (and higher RACH capacity). Higher RACH capacity comes at the expense of increased overhead, in particular for requiring more ROs in already scarce UL resources, and adding even more ROs. In practise, a cell needs to provide both lower latency, and increased coverage and range, which is not reasonable.

In addition to the network aspects of supporting both RACH configuration options within a cell, made agreements and the remaining specification effort would also suffer. RAN1 has not had this use case in mind and has not at all discussed in during previous sessions. That results in that made agreements have not taken it into account. Additionally, agreements may turn out to be ambiguous in the sense that they have not at all considered, e.g., prioritization between SBFD ROs and only prioritization between SBFD ROs and legacy ROs.

Supporting both SBFD RACH configuration options in a cell is not justified since:

  - the use case is non-existing,

  - overhead will substantially increase,

  - it will complicate the remaining specification work, and

  - made agreements risk becoming ambiguous.

Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

Regardless of whether RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2 is applied,  it is beneficial to aim for a unified design/framework between RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2 to simplify specification complexity. In an example, we can avoid specifying different UE behaviours and procedures for different RACH configuration options.

To aim for a unified RACH design/framework between RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2.

Specifically for RACH configuration Option 2, it is worth to discuss how to implement the additional SBFD RACH configuration in relation to the legacy RACH configuration. In the current MAC spec, RACH configuration is not visible to the MAC layer. In other words, there is no UE procedure to select a specific RACH configuration. It is worth to keep this legacy modelling method when introducing additional RACH configuration for SBFD RA operation.

In the current MAC spec, RACH configuration is not visible to the MAC layer. In other words, there is no UE procedure to select a specific RACH configuration. It is worth to keep this legacy modelling method when introducing additional RACH configuration for SBFD RA operation.

Therefore, we suggest.

As in the legacy, for RACH configuration option 2, the additional SBFD RACH configuration is not visible in the MAC spec. In other words, the introduction of selection of RACH configuration should be avoided in the MAC spec.

## 2.3 SBFD RA Enabling

Even though a network supports SBFD feature, it may not be desirable for it to enable SBFD RA operation. SBFD RA operation requires additional RA resources (e.g., ROs) to be allocated in the cell. This may result in fewer resources left for non RA transmissions (e.g., data and control signalling transmissions) in the cell. On one hand, SBFD RA operation can be beneficial to improve RACH performances for some UEs. On the other hand, some other UEs may suffer from performance degradation due to fewer resources for data and control signalling transmissions. It would be preferred to have a good balance between SBFD RA operation and other non RA transmissions.

We propose to explicitly indicate whether SBFD RA operation is supported for a cell supporting SBFD feature.

Even if the network supports SBFD, the additional overhead associated with the introduction of more ROs should not be mandated.

Network can decide if SBFD RA operation is enabled in a cell supporting SBFD feature.

In addition, if RAN2 agrees to introduce enabling signalling for cell supporting SBFD feature, we think the cell specific signalling option should be sufficient. There is no motivation to introduce UE dedicated signalling option.

Support cell specific signalling for enabling SBFD RA operation.

## 2.4	RO Prioritization and trigger events

RAN1 has made the below agreements regarding RO selection for initial transmission and retransmissions.

Agreement For SBFD-aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs For SBFD aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure For SBFD-aware UEs and RACH configuration Option 2, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs For SBFD aware UEs and RACH configuration Option 2, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

In addition, RAN2 also discussed this issue in RAN2#127. However, no consensus was made. We think the issue would affect both RAN1 and RAN2. Given RAN1 has made further progress on the issue. RAN2 should also continue discussion, instead just waiting for outcome from RAN1 discussions.

Although RAN1 has agreed two RACH configuration options, i.e., Option 1 with Alt 1-1 and Option 2, we believe the issue is valid for both options, especially for RO selection. Given RAN1 has only discussed selection criteria for ROs, RAN2 can follow RAN1 and focus on RO selection.

Same as RAN1, RAN2 focus on discussion of prioritization/selection of ROs.

It would be beneficial from a MAC perspective to have a unified solution including both Option 1 and Option 2 for RO (re)selection.

SBFD RACH configuration can be provided by the gNB via either one single RACH configuration (i.e., the legacy RACH configuration with possible enhancement) or separate RACH configuration. In either of these options, a legacy UE can only see legacy ROs (which are in UL or flexible symbol by TDD-DL-UL common). A SBFD aware UE can see both legacy ROs (ROs in non SBFD symbols) and SBFD ROs (ROs in SBFD symbols, i.e., configured as DL by TDD-DL-UL common).

A legacy UE can only see legacy ROs (which are in UL or flexible symbol by TDD-DL-UL common).

A SBFD aware UE can see both legacy ROs (ROs in non SBFD symbols) and SBFD ROs (ROs in SBFD symbols, i.e., configured as DL or FL by TDD-DL-UL common).

For initial PRACH transmission attempt, we prefer Option 2 (as described in the above) versus Option 3. The UE can select legacy ROs or SBFD ROs based on certain conditions. One of the possible conditions would be RACH trigger event. SBFD operation is supposed to achieve merits for SBFD aware UE including latency reduction. Certain trigger events including beam failure recovery, handover, or RACH-based LTM cell switch may be sensitive to latency, it may be beneficial to only apply SBFD RACH access for those specific trigger events. In addition, allowing a UE to perform SBFD RA for a RACH trigger event which is delay insensitive, may cause congestion of SBFD RA resources, which would negatively affect RACH performance of other UEs with delay sensitive RACH trigger event. Therefore, we think it is reasonable to limit SBFD resources to delay sensitive RACH trigger events.

Allowing a UE to perform SBFD RA for a RACH trigger event which is delay insensitive, may cause congestion of SBFD RA resources, which would negatively affect RACH performance of other UEs with delay sensitive RACH trigger event.

In addition, we think such UE autonomous selection of RO type, can be only performed if there is no indication/signalling on RO type received from the network. Since SBFD RA is designed for either latency reduction or coverage extension purposes, the network has full knowledge to decide when to enable SBFD RA. Therefore, it would be necessary to support the network to indicate the RO type to SBFD aware UE. In this case, the UE autonomous selection of RO type can be only performed if there is no signalling/indication on RO type received from the network.

The network can decide one specific RO type between legacy-ROs and additional-ROs for an SBFD ware UE. FFS on signaling details.

SBFD aware UE can select one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations only if no indication on RO type is received from network.

RAN2 has agreed that

- Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

Based on the above agreement, it would mean that delay sensitive RACH trigger events are considered as conditions to trigger the UE to select one type of ROs between legacy-ROs and additional-Ros, for initial PRACH transmission attempt.

Delay sensitive RACH trigger events e.g., BFR, handover and RACH-based LTM cell switch are considered as conditions to trigger the SBFD aware UE to select one type of ROs between legacy-ROs and additional-ROs, for initial PRACH transmission attempt.

Further, RAN2 has made the below working assumption regarding RACH trigger events:

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Given the above reasons, we think it is reasonable to not confirm the above working assumption.

In contrast to the working assumption agreed in RAN2#127, for delay sensitive RACH trigger events e.g., BFR, handover and RACH-based LTE cell switch, SBFD aware UE selects SBFD RO for initial PRACH transmission attempt only if no indication on RO type is received from network.

In addition to the RACH trigger events, it is also beneficial to support another condition, i.e., RSRP threshold. It has been observed that UL transmissions during SBFD symbols may cause CLI to other UEs. In this case, limiting UL transmission during SBFD symbols for a UE close to cell border may be beneficial to mitigate CLI to other UEs. The network can provide an RSRP threshold to a SBFD aware UE. The UE applies SBFD RA only if the measured DL RSRP is above that threshold.

SBFD aware UE selects SBFD RO for initial PRACH transmission attempt when the measured RSRP is above a configured RSRP threshold and no indication on RO type is received from network.

An SBFD aware UE can use SBFD ROs and/or legacy ROs. It may be necessary to discuss if the UE can mix ROs of different RO types when performing RA operation. Given the UE may apply different power control parameters/settings for RACH operation in SBFD symbols and non SBFD symbols, it may be not preferred for the UE to mix ROs of different RO types when performing RA operation.

It may be also necessary for RAN2 to study whether the SBFD aware UE to use both SBFD ROs and legacy ROs at the same time especially when the UE has detected congestion or RA problem for the selected ROs. In an example, the UE first uses legacy ROs, since the triggered RA procedure is not delay sensitive. However, the UE detects RA failures/problem after a while, then there is risk that the UE cannot fulfil the latency requirement for the RA procedure, it would be helpful if the UE is allowed to fall back to SBFD ROs. It may be also possible that the UE initially selects SBFD ROs, however the UE later experiences RA problem due to interference e.g., CLI introduced by other UEs or gNBs, thus it may be beneficial to allow the UE to fall back to legacy ROs.

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempts (i.e., RACH problem is detected) in legacy RACH occasions, UE is allowed to switch to SBFD RACH occasions.

## 2.5	SBFD awareness early indication

Regarding SBFD awareness early indication, RAN2 has made the below agreement:

- From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

The remaining issue is whether a SBFD aware UE can indicate SBFD awareness to the gNB via Msg3 or Msg5. We think this is not needed either due to below reasons:

- this would require an indicator to be introduced in either MAC layer or RRC signaling, which would increase unnecessary design efforts for RAN2.

- With early indication via Msg3 or Msg5, the gNB may be only feasible to schedule SBFD resources to subsequent signalling transmissions. The achieved latency reduction would be marginal given that the UE will anyway provide its capability report to the gNB after the RACH procedure.

- The UE doesn’t choose SBFD RO for Msg1 transmission, meaning that the UE sees no need to apply SBFD RA (to exploit latency reduction or coverage extension). In this sense, it is unnecessary for the gNB to schedule SBFD resources to the UE before the gNB receives the UE  capability from the UE or CN.

Support SBFD awareness early indication via Msg3 or Msg5 would require RAN2 to introduce an indicator in either MAC layer or RRC signalling, which would increase unnecessary design efforts for RAN2.

With early indication via Msg3 or Msg5, the gNB may be only feasible to schedule SBFD resources to subsequent signalling transmissions. The achieved latency reduction would be marginal given that the gNB will anyway receive the UE capability from UE or CN after the RACH procedure.

If the UE sees no need to apply SBFD RA (to exploit latency reduction or coverage extension), the UE would not choose SBFD RO for Msg1 transmission.

Therefore, in our view, it is unnecessary to introduce early indication for SBFD awareness. Anyway, an SBFD aware UE can be identified by the gNB at an early stage when the UE uses SBFD RO, which is sufficient.

SBFD awareness early indication via Msg3 or Msg5 is not pursued for SBFD aware UE.

## 2.6	SBFD Indication for CFRA

RAN2 has agreed to strive for a common SBFD CBFA framework independent of RRC state.

RAN2 to strive for a common SBFD CBRA framework independent of RRC state.

In addition, it is also very natural to support contention free RA for SBFD.

To support CFRA SBFD operation.

In the legacy, contention free RA is configured by the gNB via RRC signaling (e.g., RRCReconfigurationwithSync) including a field rach-ConfigDedicated, which comprises dedicated PRACH resources, based on which, the UE can initiate a contention free RA.

As another example, dedicated PRACH resources can be also configured to the UE for beam failure recovery (e.g., candidateBeamRSList in BeamFailureRecoveryConfig indicates set of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated RA parameter) so that the UE an apply contention free RA for beam failure recovery.

As yet another example, the gNB can send a PDCCH order to the UE indicating a contention free RA to be triggered. The legacy PDCCH order cannot indicate the triggered RA is for SBFD or non SBFD.

For either of the above cases, a SBFD aware UE is not able to interpret whether the configured dedicated PRACH resources based on the legacy signaling, is for SBFD operation or for non SBFD operation.

Therefore, an indicator can be included in the dedicated RA configuration or a PDCCH order signaled to a UE, indicating the RA resource types for the contention free RA initiated by the UE according to the dedicated RA configuration or the PDCCH order. Alternatively, the gNB can provide separate dedicated RA configuration/resources to the UE for SBFD RA, compared to the legacy RA. This alternative is not preferred since the gNB can decide if the UE needs to apply SBFD RA based on the UE’s capability and the need. The gNB can further decide if the UE needs to apply a CFRA. In this case, it doesn’t make sense that the gNB provides dedicated RA configurations/resources for both SBFD RA and legacy RA to the UE.

An indicator is included in the dedicated RA configuration or a PDCCH order indicating the RA resource types, i.e., legacy RA resource, or SBFD RA resource.

Providing separate dedicated RA configurations/resources for SBFD RA and legacy RA to SBFD aware UE by the network is not pursued.

## 2.7	RA-RNTI collision

At RAN2#127bis, some companies have proposed to study the issue on RA-RNTI collision. They declare that:

For PRACH configuration Option 2 (i.e., an SBFD-aware UE is configured with a legacy and additional PRACH configurations), there is a possibility that the SBFD ROs configured by the additional configurations may overlap with the legacy ROs. This could happen in flexible symbols (as both legacy and SBFD ROs can be configured in flexible symbols). When this overlapping happens, there is a non-negligible possibility that at least s_id and t_id of the overlapping legacy RO and SBFD RO are the same (and of course, the ul-carrier_id is the same since we are talking about the same carrier). Now the tricky part is the f_id. If the legacy specs is kept as is, then at the point when there is such overlapping, there is a mismatch between the frequency indices being used by legacy UEs and the ones being used by SBFD UEs. This leads to the same RA-RNTI and confuses these UE when receiving the same RA-RNTI.

More specifically, the UE calculates the RA-RANTI when sending the Msg 1 of the RA procedure. According to TS38.321, the RA-RNTI is computed as

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where,

- s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

- t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80)

- f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8)

- ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier)

In our view, we think the issue is invalid due to the below reasons

- According to RAN1 agreements,

a RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default. A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. This is only supported for RACH configuration option 2 and only supported for the ROs configured by the additional RACH configuration. If network configures such RO as a valid RO, UE should treat the RO as an additional-RO in SBFD symbols, and the followings are assumed by network and UE:

- The same frequency resources are used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL transmit power is used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL spatial domain filter is used for both the SBFD segment and non-SBFD segment of the PRACH.

- UE doesn’t stop PRACH transmission in the transition period/gap (if any) between SBFD and non-SBFD symbols

- There are no phase coherency requirements on the UE between the SBFD segment and non-SBFD segment of the PRACH.

- Other assumptions are not precluded.

NOTE: For FR2, network may need to ensure that the additional-RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB.

For a SBFD RO across SBFD symbols and non-SBFD symbols, if the SBFD RO is overlapping with the legacy RO on a 2nd symbol or beyond, there is no issue, in such case, at least the s_id would be different, which would be sufficient to distinguish RA-RNTIs between the SBFD RO and the legacy RO which are overlapping.

Regarding whether a SBFD RO across SBFD symbols and non-SBFD symbols, may be overlapping with a legacy RO in the first symbol, we think it can be either avoided by NW configuration, or RA-RNTI collision can be avoided by allocating different preamble list to the legacy RACH configuration and additional RACH configuration.

- In addition, a configured RO starting from non SBFD symbols and ending in SBFD symbols, is invalid.

For a RO across SBFD symbols and non-SBFD symbols, the RO may be overlapping with a legacy RO on 2nd symbol or beyond, in such case, at least the s_id would be different, which would be sufficient to distinguish RA-RNTIs between the SBFD RO and the legacy RO which are overlapping.

A configured RO starting from non SBFD symbols and ending in SBFD symbols, is invalid.

All in all, we see no issue for RA-RNTI.

No need to update the RA-RNTI formula for SBFD RA configuration.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	Supporting both SBFD RACH configuration options in a cell is not justified since:

a.	the use case is non-existing,

b.	overhead will substantially increase,

c.	it will complicate the remaining specification work, and

d.	made agreements risk becoming ambiguous.

Observation 2	In the current MAC spec, RACH configuration is not visible to the MAC layer. In other words, there is no UE procedure to select a specific RACH configuration. It is worth to keep this legacy modelling method when introducing additional RACH configuration for SBFD RA operation.

Observation 3	Even if the network supports SBFD, the additional overhead associated with the introduction of more ROs should not be mandated.

Observation 4	Same as RAN1, RAN2 focus on discussion of prioritization/selection of ROs.

Observation 5	It would be beneficial from a MAC perspective to have a unified solution including both Option 1 and Option 2 for RO (re)selection.

Observation 6	A legacy UE can only see legacy ROs (which are in UL or flexible symbol by TDD-DL-UL common).

Observation 7	A SBFD aware UE can see both legacy ROs (ROs in non SBFD symbols) and SBFD ROs (ROs in SBFD symbols, i.e., configured as DL or FL by TDD-DL-UL common).

Observation 8	Allowing a UE to perform SBFD RA for a RACH trigger event which is delay insensitive, may cause congestion of SBFD RA resources, which would negatively affect RACH performance of other UEs with delay sensitive RACH trigger event.

Observation 9	Delay sensitive RACH trigger events e.g., BFR, handover and RACH-based LTM cell switch are considered as conditions to trigger the SBFD aware UE to select one type of ROs between legacy-ROs and additional-ROs, for initial PRACH transmission attempt.

Observation 10	Support SBFD awareness early indication via Msg3 or Msg5 would require RAN2 to introduce an indicator in either MAC layer or RRC signalling, which would increase unnecessary design efforts for RAN2.

Observation 11	With early indication via Msg3 or Msg5, the gNB may be only feasible to schedule SBFD resources to subsequent signalling transmissions. The achieved latency reduction would be marginal given that the gNB will anyway receive the UE capability from UE or CN after the RACH procedure.

Observation 12	If the UE sees no need to apply SBFD RA (to exploit latency reduction or coverage extension), the UE would not choose SBFD RO for Msg1 transmission.

Observation 13	For a RO across SBFD symbols and non-SBFD symbols, the RO may be overlapping with a legacy RO on 2nd symbol or beyond, in such case, at least the s_id would be different, which would be sufficient to distinguish RA-RNTIs between the SBFD RO and the legacy RO which are overlapping.

Observation 14	A configured RO starting from non SBFD symbols and ending in SBFD symbols, is invalid.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

Proposal 2	To aim for a unified RACH design/framework between RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2.

Proposal 3	As in the legacy, for RACH configuration option 2, the additional SBFD RACH configuration is not visible in the MAC spec. In other words, the introduction of selection of RACH configuration should be avoided in the MAC spec.

Proposal 4	Network can decide if SBFD RA operation is enabled in a cell supporting SBFD feature.

Proposal 5	Support cell specific signalling for enabling SBFD RA operation.

Proposal 6	The network can decide one specific RO type between legacy-ROs and additional-ROs for an SBFD ware UE. FFS on signaling details.

Proposal 7	SBFD aware UE can select one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations only if no indication on RO type is received from network.

Proposal 8	In contrast to the working assumption agreed in RAN2#127, for delay sensitive RACH trigger events e.g., BFR, handover and RACH-based LTE cell switch, SBFD aware UE selects SBFD RO for initial PRACH transmission attempt only if no indication on RO type is received from network.

Proposal 9	SBFD aware UE selects SBFD RO for initial PRACH transmission attempt when the measured RSRP is above a configured RSRP threshold and no indication on RO type is received from network.

Proposal 10	For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempts (i.e., RACH problem is detected) in legacy RACH occasions, UE is allowed to switch to SBFD RACH occasions.

Proposal 11	SBFD awareness early indication via Msg3 or Msg5 is not pursued for SBFD aware UE.

Proposal 12	To support CFRA SBFD operation.

Proposal 13	An indicator is included in the dedicated RA configuration or a PDCCH order indicating the RA resource types, i.e., legacy RA resource, or SBFD RA resource.

Proposal 14	Providing separate dedicated RA configurations/resources for SBFD RA and legacy RA to SBFD aware UE by the network is not pursued.

Proposal 15	No need to update the RA-RNTI formula for SBFD RA configuration.

# 4	References

- [1] 	RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei,

- Shanghai, China, June 17-20, 2024, RAN#104.