3GPP TSG-RAN WG2 Meeting #128	R2-2409996

# Orlando, USA,  November 18th - 22th 2024

Agenda Item:	8.11.3

Source:	Ericsson

Title:	Non-RA aspects for subband full duplex (SBFD) operation

Document for:	Discussion

# 1	Introduction

In this paper, we discuss other aspects/objective which are not directly related to RA operation, shown in the WI objective below.

- Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2]

In this paper, a SBFD aware UE means a UE which is capable of operating in a cell configured with SBFD feature, i.e., the cell/the gNB transmits DL and receive UL simultaneously in SBFD slots and symbols within a carrier. The UE can be aware of SBFD configurations so that the UE knows which slots/symbols are SBFD capable, which are also referred to as SBFD slots/symbols. This doesn’t mean that the UE needs to support full duplex operation.

# 2	Discussion

## 2.1	CSI-RS measurement

RAN1 has made the below agreements regarding CSI report.

Agreement

For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options.

- Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only.

  - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only.

    - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation.

- Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD

Based the above agreement, it is observed that a SBFD aware UE may support separate CSI-RS monitoring and CSI report for SBFD symbols and non SBFD symbols. This is based on the fact that the UE may experience different propagation characteristics between SBFD symbols and non SBFD symbols. In this case, due to distinct measurement results and propagation characteristics between SBFD symbols and non SBFD symbols, the UE may need to monitor beam failure instances during SBFD symbols and non SBFD symbols separately.  This is expected to be one issue which needs to be addressed.

According to RAN1 agreements, the gNB may configure separate CSI-RS (periodic/semi-persistent) report on SBFD and non-SBFD, which may affect the UE’s BF monitoring.

In legacy, the UE performs BF detection solely based on DL RS measurements. However, for SBFD operation, the UE performs UL transmissions in SBFD symbols using UL subband, in addition to UL transmissions in non SBFD symbols, i.e., UL symbols. Due to different UL transmission propagation characteristics for SBFD symbols and non SBFD symbols, the UE may experience transmission failure in SBFD symbols, but not in non-SBFD symbols, or vice versa. However, the legacy BFD and BFR procedures cannot reflect such UL-oriented failures.

The legacy BFD and BFR procedure is DL RS measurement oriented, which may not reflect UL failure caused by SBFD operation.

In addition, the beam failure monitoring may be also related to TCI framework (either the unified TCI framework or UL/DL TCI framework.

The discussion of potential impact of SBFD operation on BFD and BFR may depend on whether the unified TCI framework or UL/DL TCI framework is being used by the UE.

RAN2 to discuss the potential impact of SBFD operation on the BFR procedure.

## 2.2	Mobility aspects

From the above agreements as highlighted in clause 2.1, it is observed that a SBFD aware UE may support separate CSI-RS monitoring and measurement for SBFD symbols and non SBFD symbols. The UE may further use L1 CSI-RS measurement results to derive L3 measurements. In this case, several options are feasible for the UE to derive L3 measurement results:

Option 1: only consider measurement results in SBFD symbols

Option 2: only consider measurement results in non SBFD symbols

Option 3: consider measurement results in both SBFD symbols and non SBFD symbols

Application of different option may therefore affect HO procedure, i.e., triggering of HO events and selection of the target cell etc.

RAN2 to discuss the potential impact of SBFD operation on the HO procedure.

At RAN2#127bis, some companies have proposed that SBFD aware UE is provided with information of the SBFD configuration of neighbouring cells, in order to assist inter-cell CSI measurement. Based on the above analyses, we think this is a valid issue.

RAN2 to discuss the potential impact of SBFD operation on the inter cell CSI measurement.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	According to RAN1 agreements, the gNB may configure separate CSI-RS (periodic/semi-persistent) report on SBFD and non-SBFD, which may affect the UE’s BF monitoring.

Observation 2	The legacy BFD and BFR procedure is DL RS measurement oriented, which may not reflect UL failure caused by SBFD operation.

Observation 3	The discussion of potential impact of SBFD operation on BFD and BFR may depend on whether the unified TCI framework or UL/DL TCI framework is being used by the UE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss the potential impact of SBFD operation on the BFR procedure.

Proposal 2	RAN2 to discuss the potential impact of SBFD operation on the HO procedure.

Proposal 3	RAN2 to discuss the potential impact of SBFD operation on the inter cell CSI measurement.

# 4	References

- [1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Shanghai, China, June 17-20, 2024, RAN#104.