3GPP TSG-RAN WG2 Meeting #128	R2-2410012

Orlando, USA, Nov 18th – 22nd, 2024

# Agenda item:	8.6.2

# Source:	ETRI

# Title:	Further Discussion on inter -CU LTM

# Document for:	Discussion, Decision

- Introduction

In RAN2#128 meeting, RAN2 reached the following agreements to support inter-CU LTM [1]:

Agreements on inter-CU LTM - The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs). - For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18. - The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3. - Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline: - The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part. - FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations. - The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message. - Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed. - The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part. - The MN can request an SCG reference configuration from any of the involved SNs. - For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g., a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN. - The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells. - The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN. - In order to support subsequent inter-CU SCG LTM, the MN needs to transfer, during LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN. - Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. - Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM. - RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen: - Inter-MN LTM and intra-SN LTM - Inter-SN LTM and intra-MN LTM

While RAN2 have made a lot of progress, there are still some issues that need further discussion. In this paper, we discuss those issues that needs to support inter-CU LTM.

- Discussion

Update of candidate LTM cell configuration update

When discussing stage-2 signaling procedure for inter-CU LTM, the focus was mainly on the preparation of inter-CU LTM configuration, and the sharing or update of the LTM configuration was not discussed in detail. Unlike Rel-18 intra-CU LTM, signaling for updating candidate LTM cell configurations between network nodes needs to be defined in Rel-19 inter-CU LTM. Some papers about the need for sharing or updating the LTM configuration has been proposed and some of them was discussed and agreed. The operation where a source gNB-CU informs candidate gNB-CU(s) of the common CSI resource configuration can also be considered an example of LTM configuration update. A change in PDU sessions can be another use case for the LTM configuration update. A UE can keep the LTM configuration relatively long time due to the support of subsequent LTM and may add a new PDU session while the LTM configuration is still active. In such cases, it is necessary to verify whether the existing candidate cells can support the new PDU session, and based on the results, an update of the existing LTM configuration may be required.

While the detailed update procedure for the LTM configuration should be handled by RAN3, it is necessary to discuss, from the RAN2 perspective, when and how the LTM configuration should be updated. The results of this discussion can be informed to RAN3 for consideration when designing the signaling process. The followings can be discussed regarding the update of LTM configuration.

- Message for LTM configuration update: The HO REQUEST/ACKNOWLEDGE messages can be reused for updating the candidate LTM configuration, or new messages can be defined specifically for this procedure.

- Sharing of the updated LTM configuration among candidate cells: The timing for sharing the prepared LTM configuration among candidate cells has not yet been determined. One option is for the source gNB-CU to inform the target gNB-CU of the LTM configuration when a cell switch occurs. Alternatively, the source gNB-CU could share the LTM configuration with candidate cells when notifying them of the common CSI resource configuration. Similarly, the updated LTM configuration could either be shared with the target cell at the time of the next cell switch or distributed to all candidate cells immediately after the LTM configuration is updated.

Proposal 1: The source gNB can trigger the update of the candidate LTM cell configurations while the candidate LTM cell configuration is active.

Proposal 2: RAN2 is kindly asked to discuss when and how the updated LTM configuration is informed to candidate gNB-CU(s).

Security Key Update in non-DC cases

RAN2 has identified four potential solutions for security key updates during Inter-CU LTM and has shared them with SA3 for further security impacts analysis. SA3 has reviewed the proposed solutions and described their conclusions in the reply LS from SA3 [2] as follow:

The rekeying method preferred by most companies in SA WG3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch. This solution was preferred as it provides the NCC value using the protected RRC signalling and hence is considered to be secure. SA WG3 understands that, RAN WG2 is aiming to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures. If it is necessary for the RAN WGs to avoid RRC signalling between cell switches, then SA WG3 can analyse further methods like, - Security threat of the unprotected MAC CE to deliver the NCC value. - Sending NCC value per candidate cell via RRC message during LTM preparation phase.

The WID clearly states that RAN2 should specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches. Therefore, the solution that utilizes RRC signaling to configure a security context for the next inter-CU LTM cell switch does not meet the objectives outlined in the WID.

Observation 1: the solution that utilizes RRC signaling to configure a security context for the next inter-CU LTM cell switch does not meet the objectives outlined in the WID.

While we may await SA3’s conclusions on the alternative solutions, it would be beneficial for RAN2 to discuss the impact of each solution from RAN2's perspective, as these impacts have not yet been discussed within RAN2. Through these discussions, we could potentially narrow down the candidate solutions and provide SA3 with additional input based on RAN2’s conclusions.

The first option, where an NCC value or index is included in the cell switch command MAC CE, may have an impact on the cell switch decision procedure. Currently, the consensus is that the gNB-DU triggers the CSC MAC CE and then notifies the gNB-CU of the cell switch. However, the gNB-DU does not know whether the target cell is an intra-CU or inter-CU cell, nor does it have any security key information. If the first option is adopted, the gNB-DU would need to inform the gNB-CU of the cell switch before triggering the CSC MAC CE and only transmit the CSC MAC CE after receiving a response from the gNB-CU. Given the additional delay introduced by this cell switch decision procedure, the gNB-DU would either keep extra information on NCC value (or index) for each LTM candidate inter-cell, or it would need to make the cell switch decision earlier. Furthermore, this solution would not be feasible if support for fast recovery for inter-CU LTM or conditional inter-CU LTM were required in future, as the CSC MAC CE is not used in those cases.

Observation 2: the first option, which includes an NCC value or index in the cell switch command MAC CE, may affect the cell switch decision procedure. Furthermore, this solution will not be feasible if fast recovery for inter-CU LTM or conditional inter-CU LTM is to be supported.

In contrast to Option 1, where an NCC value is provided via the CSC MAC CE, Options 2 and 3 pre-configure the UE with a security context for subsequent inter-CU LTM cell switches. Although the specific procedures differ slightly, both options use RRC signaling to configure the security context. Since this RRC signaling is both integrity-protected and encrypted, Options 2 and 3 do not present any critical security issues. Additionally, once the security context for subsequent LTM cell switches is configured during the LTM preparation phase, no further RRC signaling is required between cell switches. These options would also be feasible if fast recovery for inter-CU LTM or conditional inter-CU LTM were required.

Observation 3: Option 2 & Option 3, which pre-configure a security context for subsequent LTM cell switches, does not require RRC signaling between cell switches and present no critical security issues.

Proposal 3: RAN2 is kindly asked to discuss the impact of each four options from RAN2 perspective, narrow down the candidate solutions based on these discussions, and inform SA3 of RANs’2 preferred option for further analysis and detailed procedure.

- Conclusions

In this paper, we discuss further issues on inter -CU LTM related issue. Following are the proposals made in this paper:

Proposal 1: The source gNB can trigger the update of the candidate LTM cell configurations while the candidate LTM cell configuration is active.

Proposal 2: RAN2 is kindly asked to discuss when and how the updated LTM configuration is informed to candidate gNB-CU(s).

Observation 1: the solution that utilizes RRC signaling to configure a security context for the next inter-CU LTM cell switch does not meet the objectives outlined in the WID.

Observation 2: the first option, which includes an NCC value or index in the cell switch command MAC CE, may affect the cell switch decision procedure. Furthermore, this solution will not be feasible if fast recovery for inter-CU LTM or conditional inter-CU LTM is to be supported.

Observation 3: Option 2 & Option 3, which pre-configure a security context for subsequent LTM cell switches, does not require RRC signaling between cell switches and present no critical security issues.

Proposal 3: RAN2 is kindly asked to discuss the impact of each four options from RAN2 perspective, narrow down the candidate solutions based on these discussions, and inform SA3 of RANs’2 preferred option for further analysis and detailed procedure.

- References

- Chair notes RAN2#127-bis;

- R2-2409534, “Reply LS on security handling for inter-CU LTM in non-DC cases”