3GPP TSG-RAN WG2 Meeting #128	                                   R2-2410038

Orlando, USA, Nov.18th – 22nd , 2024

Source:	TCL

Title:	Discussion on how to apply additional priority for delay-aware LCP

Agenda item:	8.7.4.1

Document for:	Discussion and decision

- Introduction

- According to the RAN2#127bis meeting[1], the following agreements on LCP enhancements for XR are reached:

- How/when to apply additional priority As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation - Remaining time threshold Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold. For now, we will use “LCH priority-adjusted data”

In this contribution, we aim to discuss how to apply additional priority for delay-aware LCP enhancement.

- Discussion

In the latest RAN2#126 and RAN2#127bis meetings, it was agreed that when a LCH has LCH priority-adjusted data, the UE will utilize additional LCH priority for that LCH during the Logical Channel Prioritization (LCP) procedure. This additional LCH priority applies to the LCH in both the 1st and 2nd rounds of the resource allocation procedure within LCP, provided that the LCH has LCH priority-adjusted data for transmission at the start of the first round. This contribution focuses on the pertinent detailed issues and the application of additional priority for delay-aware LCP.

  - Issue 1: Whether to adjust Priority for the 2nd Round to Ensure Fairness

As a baseline, the additional LCH priority is applied to an LCH in both the first and second rounds of resource allocation within LCP, whenever the LCH has LCH priority-adjusted data for transmission at the start of the first round (Option 1). However, some companies have expressed concerns about whether the priority should be adjusted for the second round to ensure fairness. In other words, during each round of resource allocation in LCP, the UE needs to assess whether the LCH contains LCH priority-adjusted data to determine whether to apply the additional LCH priority (Option 2). Below are the pros and cons of both options:

|  | Pros | Cons |
| --- | --- | --- |
| Option 1 | Lower LCP processing complexity and shorter latency for the UE | delay-aware LCP can’t be guaranteed in the 2nd round of LCP |
| Option 2 | Ensures fairness among LCHs and prioritizes data with urgent remaining delays for transmission | High LCP processing complexity and long latency for the UE |

Observation 1: Although Option 1 is favorable for UE implementation, it cannot guarantee that LCH priority-adjusted data will always be prioritized for transmission.

Considering fairness and the priority scheduling of urgent data with low latency, we believe that supporting Option 2 is more reasonable. Furthermore, since it is already supported that during the first round of resource allocation in LCP, the UE determines whether there is LCH priority-adjusted data, we think the UE is still capable of making such a determination again in the second round and then deciding on the priority of the LCH based on this information. The added complexity should be manageable by the UE.

Observation 2: The complexity of the UE determining whether there is LCH priority-adjusted data in both the first and second rounds of resource allocation within LCP should be within the UE's processing capability.

Proposal 1: It is suggested to adjust priority for the 2nd round to ensure LCH priority-adjusted data is prioritized for transmission.

  - Issue 2: Whether to Update the Priority Bit Rate (PBR) When Using Additional LCH Priority

Regarding how to apply additional priority for delay-aware LCP, another question arises: if an LCH uses additional priority during the first round of resource allocation, should the PBR be updated accordingly? According to TS38.321, when the network allocates a priority to the UE, it simultaneously allocates a PBR to prevent resource allocation starvation. The UE determines the total amount of resources allocated to the current LCH based on the PBR configured by the network. When an LCH contains LCH priority-adjusted data, i.e., data with a remaining delay less than the threshold configured by the network, the UE uses the additional priority as the new priority for that LCH in resource allocation. During the first round of resource allocation, continuing to use the legacy PBR may result in allocated resources that are insufficient to fully schedule the LCH priority-adjusted data. Therefore, it is necessary to use an updated PBR to ensure that the LCH priority-adjusted data is fully scheduled. For example, the network could configure an additional PBR when assigning additional priority to the LCH, or the UE could adjust the PBR based on the proportion of LCH priority-adjusted data within the LCH, as authorized by the network during resource allocation.

Proposal 2: To ensure that the LCH priority-adjusted data is fully scheduled, it is suggested that the UE update the PBR when using additional LCH priority, such as by using an additional PBR configured by the network or a PBR updated by the UE based on the total amount or proportion of LCH priority-adjusted data.

- Conclusion

In this contribution, we provided our observations and proposals on LCP enhancements mechanism for XR as follows:

Observation 1: Although Option 1 is favorable for UE implementation, it cannot guarantee that LCH priority-adjusted data will always be prioritized for transmission.

Observation 2: The complexity of the UE determining whether there is LCH priority-adjusted data in both the first and second rounds of resource allocation within LCP should be within the UE's processing capability.

Proposal 1: It is suggested to adjust priority for the 2nd round to ensure LCH priority-adjusted data is prioritized for transmission.

Proposal 2: To ensure that the LCH priority-adjusted data is fully scheduled, it is suggested that the UE update the PBR when using additional LCH priority, such as by using an additional PBR configured by the network or a PBR updated by the UE based on the total amount or proportion of LCH priority-adjusted data.

- References

- RAN2#127bis meeting agreements.