3GPP TSG-RAN WG2 #128	R2-2410065

Orlando, USA, Nov 18th – 22nd, 2024

Source:		NEC

Agenda item:		8.10.2	MRO enhancements for Rel-18 mobility features

Title:	MRO for Rel-18 mobility

Document for:		Discussion and decision

- Introduction

During last RAN2 meeting, RAN2 achieved the following way forward for the WI [1]

Agreements 1: Continue the work of MRO for LTM 1st priority. 2: Focus CHO with candidate SCGs (focus on agreed scenarios by RAN3) with 2nd priority 3: For Rel-18 leftovers, regarding SON for SDT, focus on RSRP and data volume in SON reports. Other enhancements (including the additional failure cause) shall be paused unless time permits. 4: Close the stage-2 work on MRO for MR-DC SCG failure. 5: Pause subsequent CPAC and wait if any for RAN3 LS. 6: Pause and wait if any for RAN3 LS for SON/MDT for Slicing and NTN. 7: Send an LS to RAN3 that RAN2 want to prioritize RAN3-based solutions on P5 and P6. 8: Pause and wait for RAN3 LS on MHI Enhancement for SCG Deactivation/Activation.

RAN2 also made the following agreements for MRO of LTM [1]

Agreements - The UE shall log cell IDs such as reestablishment cell ID, source, failed, reconnect cell ID, following the same principle as RLF, HOF and successful recovery, incl. the time between UE executing the LTM command and the failure. - If RA-based LTM failure happens the UE logs and reports RACH info in the RLF report. Additional information is TBD. - Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined). - Include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation.

Agreements - Unless RAN3 defines a NW-based solution: Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell. - UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

And made the following agreements for MRO of CHO with candidate SCG(s) [1]

Agreements - UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS. - Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

In this paper, we provide our further consideration on MRO for the CHO with candidate SCG(s).

- Discussion

2.1 LTM

RACH-less LTM is one key functionality of LTM cell switch to reduce mobility latency cause by random access procedure, and MRO should provide necessary information to the network such that the network can configure the UE to perform more RACH-less LTM cell switch procedure if the condition is allowed. During last meeting, it was agreed that unless RAN3 defines a NW-based solution, the UE logs and reports whether and how the UE got the TA value used for a failed LTM switch. And we think some additional information can be considered to be reported to facilitate the better triggering of RACH-less LTM.

Currently, whether the TA obtained by the source DU is valid is based on network implementation, the TA received by the UE may be already invalid, which leads to LTM failure. And even if the UE is configured TA measurement, TA value can be provided in LTM cell switch command, and the UE performs RACH-less based LTM based on the TA provided in the LTM cell switch command. It is useful if the UE could report the information of TA measurement to the network, and the network can analysis, based on it, if the LTM failure is cause by the improper TA value provided to the UE, or the if network can skip triggering of PDCCH order based early UL sync, if it proves that the UE-based TA measurement works well.

Proposal 1: Include the following information to be included in RLF-report and SHR:

- TA value measured by the UE

- Difference between the TA value received from the LTM cell switch command and TA value measured by the UE

Additionally, For RACH-less LTM, both dynamic grant and configured grant can be used. And the UE shall apply the TCI state indicated in the LTM cell switch command for dynamic grant reception or configured grant resource selection. To analysis whether the failure of RACH-less LTM is due to improper TCI state information, the related information can be reported, as this information is not part of UE Context and the network may not store this information.

Proposal 2: Include the following information to be included in RLF-report and SHR:

- TCI state information included in the LTM cell switch command

- Beam with the TCI state information in the LTM cell switch command

2.2 CHO with candidate SCG(s)

Currently, the UE only logs the ID of the target PCell in RLF-report and SHR. In case of CHO with candidate SCG(s), it is natural that the UE also report the ID of the target PSCell of the CHO with candidate SCG(s) procedure, such that the network has complete information of the target cells.

Proposal 3. UE logs the ID of target PSCell of the CHO with candidate SCG(s) procedure in RLF-report and SHR.

For execution condition of one candidate cell, at most two triggering conditions can be configuration. Currently, the UE can store information related to trigger condition(s) for candidate PCell in measurement results for candidate PCells of RLF-report, i.e., the first fulfilled trigger condition, and the time gap between the fulfilment of the two trigger conditions. In this way, the network can further optimize the two trigger conditions associated with the PCell. During last meeting, it was agreed that measurement results of PCells and PSCells is included in the RLF-report. For CHO with candidate SCG(s), it is beneficial that the UE also collect the information related to the two trigger conditions for the candidate PSCells, such that the network can optimize the trigger condition(s) of the candidate PSCells in case of CHO with candidate SCG(s).

Proposal 4. UE includes in the measurement result of one candidate PSCell in the RLF-report, information related to the triggering condition(s) for the candidate PSCell, which comprises

- the first fulfilled triggering condition for the candidate PSCell

- the time gap between the fulfilment of the two triggering conditions for the candidate PSCell

If both CHO execution condition and CPAC execution condition are fulfilled, the UE would initiate CHO with SCG(s) execution procedure, and perform random access towards the target PCell and target PSCell simultaneously. And if SCG failure happens first, the UE suspend the SCG transmission and initiate the SCGFailureInformation message transmission procedure. However, before transmission of the SCGFailureInformation including the SCG reconfigurationWithSync failure information, if CHO execution failure happens, RLF-report would be generated and the SCGFailureInformation message will not get a chance to be transmitted to the network. In order to optimize the PSCell configuration in this case, the SCG reconfigurationWithSync failure information should be included in the RLF-report.

Proposal 5. If CPAC execution failure happens before the CHO execution failure, UE logs and reports the SCG reconfigurationWithSync failure information in RLF-report.

# 3. Conclusion

In this paper, we discuss on further consideration on the MRO for CHO with candidate SCG(s), and have the following proposals:

MRO for LTM

Proposal 1: Include the following information to be included in RLF-report and SHR:

- TA value measured by the UE

- Difference between the TA value received from the LTM cell switch command and TA value measured by the UE

Proposal 2: Include the following information to be included in RLF-report and SHR:

- TCI state information included in the LTM cell switch command

- Beam with the TCI state information in the LTM cell switch command

MRO for CHO with candidate SCG(s)

Proposal 3. UE logs the ID of the target PSCell of the CHO with candidate SCG(s) procedure in RLF-report and SHR.

Proposal 4. UE includes in the measurement result of one candidate PSCell in the RLF-report, information related to the triggering condition(s) for the candidate PSCell, which comprises

- the first fulfilled triggering condition for the candidate PSCell

- the time gap between the fulfilment of the two triggering conditions for the candidate PSCell

Proposal 5. If CPAC execution failure happens before the CHO execution failure, UE logs and reports the SCG reconfigurationWithSync failure information in RLF-report.

# Reference

[1] Meeting report of RAN2 #127bis