3GPP TSG-RAN WG2 Meeting #128	                R2-2410069

Orlando, USA, Nov. 18th – Nov. 22nd, 2024

Agenda item:		8.1.3

Source:			Sharp

Title:		Discussion on NW-side Data Collection

Document for:	 	Discussion

- Introduction

During Rel-18, data collection for the training of NW-side models was discussed with the following agreements in TR 38.843 [1]. The following principles apply to all the currently identified used cases (i.e., the beam management (BM), the positioning, and the CSI compression and prediction use cases).

- A set of general data collection principles is expected to be considered for network-side model training. These include: - UE to support data logging - UE to report the collected data periodically, event-based, and on-demand - The UE memory, processing power, energy consumption, signalling overhead should be considered

In RAN2#127 meeting, the following agreements were made [2].

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support. - UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. - Measurements for data collection purposes and logging based can be controlled based on power state of the UE. It is up to UE implementation how the UE determines power state. FFS whether the UE stops autonomously or if it reports to the network. - FFS whether AS buffer event based reporting is supported. FFS if we send availability indication or full report if it is supported - FFS on event based data collection/logging - On-demand request from the network is supported. FFS details on signalling

In the previous RAN2#127bis meeting, the following agreements were made [3].

- Periodic logging is supported for training data collection procedure in R19 - Event-triggered data logging will be supported. At least radio condition based event triggered logging will be supported. FFS the details of radio condition based event. FFS if other events are supported. - Periodic reporting of logged data is not supported. - On-demand reporting of the logged measurements will be specified - UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. FFS of details of the message - The UE can indicate the availability of logged data to the network to assist network to trigger UEInformationRequest. FFS trigger/definition of availability indication. and FFS how data availability indication is sent to the network - Low priority SRB will be used. FFS new SRB or use of SRB4 - For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE. FFS if other data needs to be collected based on RAN1 progress

In our view, some issues are addressed for the BM used cases and can be further designed for the next step, while for the positioning use cases, it is still not clear how the NW-side data collection can be achieved. As a result, in this contribution, we would like to share our views on 1) Further details of the data collection in BM use cases and 2) Basic designs of the positioning use cases.

- Discussion

UE Capability for Data Collection

First, since the measurements are performed by the UE, the data collection for NW-side model needs both the consent and the capability of the UE. That is, whether the UE measures the SSB/CSI-RS (for BM use cases) and PRS (for positioning use cases) and reports the result for the purpose of data collection for NW-side data collection is based on the UE capability. The NW can obtain the measurement results for NW-side model only when the UE supports the function of data collection and is able to perform the function (e.g., has sufficient power and memory for data collection for NW-side model).

In our views, for BM use cases, the existing RRC messages UECapabilityEnquiry and UECapabilityInformation can be used to request/indicate the UE’s capabilities related to data collection for NW-side model, while the existing LPP messages ProvideCapabilities and RequestCapabilities can achieve the coordination. Specifically for the positioning use cases, the UE may not be capable of some UE-based positioning methods (e.g., DL TDoA) to acquire a ground truth label, but may be able to perform the measurement and collect the data (e.g., RSTD). In this case, the UE can provide such information in the corresponding IEs (e.g., DL-TDOA-ProvideCapabilities) to indicate the detailed capabilities. Alternatively, a new IE specifically for AI/ML can be considered. For example, the LMF and the UE can exchange the AI/ML-related capabilities via AIML-RequestCapabilities and AIML-ProvideCapabilities messages. In our views, we consider the new IE for AI/ML would be beneficial to incorporate all the capabilities related to different positioning methods.

Additionally, it was agreed to have a minimum AS layer memory size in the last meeting. Considering that the UE’s memory status configured for data collection may vary, we propose that the UE includes the memory size information in the RRC/LPP message providing the capability information. The memory size information can be an absolute value or can be an estimated value (e.g., the maximum number of instances that a UE can log).

Proposal 1: For BM use cases, the UECapabilityEnquiry and UECapabilityInformation messages are used to request and provide the UE capabilities for data collection for NW-side model.

Proposal 2: For Pos use cases, new IEs AIML-RequestCapabilities and AIML-ProvideCapabilities are used to request and provide the UE capabilities for data collection for NW-side model.

Proposal 3: The UE can include the AS layer memory size information in the RRC/LPP message providing the UE capability information. The AS layer memory size information could be quantized by the maximum number of instances to be logged.

Measurement Configuration

For BM use cases, the measurement configuration can take L1 CSI report framework as baseline. That is, the measurement is configured as a CSI-RS resource set with additional indication contained in the resource set configuration so that the UE can identify the resource set is for AI/ML purpose (regardless of inference, training, or monitoring). Additionally, the indication can be an associated ID to enable the network to collect datasets associated with multiple NW-side additional conditions.

Proposal 4: For BM use cases, the measurement is configured as a CSI-RS resource set with an associated ID.

Similarly, for positioning use cases, the measurement configuration can take the legacy ProvideAssistanceData message as a baseline. In our view, the LMF should be capable of flexibly switching among different NW-side models corresponding to different positioning methods (e.g., TDOA, AOD, Multi-RTT). As a result, LMF needs to collect data with different measurement quantities (e.g., RSTD, RSRP, RxTxTimeDiff, etc.) in different additional conditions, and the LMF needs to explicitly indicate the required measurement quantities in the measurement configuration. To allow the LMF to dynamically configure the measurement quantities, we propose to use a new type of ProvideAssistanceData message (e.g., AIML-ProvideAssistanceData) to configure the UE what to measure. Similar to the legacy ProvideAssistanceData message, the AIML-ProvideAssistanceData message contains PRS resource configurations; additionally, it also configures whether one or more of the measurement quantities (i.e., RSTD, RSRP, RTT) need to be measured by the UE. In the AIML-ProvideAssistanceData message, the associated ID (or the area ID) which identifies the NW-side additional conditions can also be provided.

Observation 1: LMF should be capable of flexibly switching between different NW-side models leveraging different positioning methods, so it may need to collect data with different measurement quantities in different additional conditions.

Proposal 5: For Pos use cases, a new type of ProvideAssistanceData message (e.g., AIML-ProvideAssistanceData) is used for the LMF to configure the measurements, where the configuration can include the required measurement quantity, the PRS resource set, and the associated ID (or area ID or Cell Global Identity).

Report Configuration

For BM use cases, there are two options for the report configuration.

        - L1-based report configuration (e.g., via the CSI-ReportConfig). This option is straightforward since the legacy CSI reporting works with the association between a CSI resource configuration and a CSI report configuration.

        - L3-based report configuration (e.g., via the ReportConfig). This option is also feasible since the legacy L3 report configuration allows the report of service cell beams by including the indication includeBeamMeasurements in the report configuration.

In our views, L3-based configuration is preferable since the latency requirement on data collection for NW-side model in training case is not stringent. Still, both the options can be considered and further discussion is needed.

The logging configuration can indicate when the UE starts and stops logging the measurement results, and it can be configured in the report configuration after the L1/L3-based report configuration is determined. Regarding the buffer status, a counter can be configured to limit the number of logged instances to avoid overflow problem. In addition, as agreed in the last meeting, the logging can be performed in an event-triggered manner; that is, a condition (e.g., the signal quality better/worse than a threshold) triggers the logging.

Observation 2: For BM use cases, both the legacy L1-based and L3-based report configurations are feasible for data collection for NW-side model

Proposal 6: For BM use cases, RAN2 discuss the report configuration to be L1-based or L3-based.

Proposal 7: For BM use cases, the report configuration further contains: 1) a counter limiting the number of logged instances, 2) a condition that triggers the UE to start logging.

As for the positioning use cases, it is necessary to address whether event-triggered logging is supported. In our view, since the goal of NW-side data collection is to facilitate the date collection for the NW, it would be pursued if the NW can configure which data to be collected, especially in a certain area. As a result, for pos use cases, we propose to support event-triggered logging.

Observation 3: It is up to NW to determine which data to be collected.

Proposal 8: For Pos use cases, RAN2 to support event-triggered logging.

Another question to address is whether periodic/event-triggered/on-demand reporting is supported. In our view, the same concept can be applied to the positioning use cases for the reporting behavior; namely, on-demand reporting is supported, while periodic and event-triggered reporting is not supported. Also, since the event-triggered logging is supported, the UE’s indication of data availability would be pursued.

Proposal 9: For Pos use cases, RAN2 to support on-demand reporting. Additionally, RAN2 to support the indication of data availability.

As for the report configuration for Pos use case, the legacy RequestLocationInformation message can be taken as a baseline. Considering some possible AI/ML-specific parameters that can be leveraged for the data collection, we propose a new type of RequestLocationInformation message (e.g., an AIML-RequestLocationInformation message) as the report configuration. The AIML-RequestLocationInformation message) as the report configuration can include the following information:

- A counter to limit the number of logged instances.

- A condition to trigger the UE to start logging.

- An indication whether the ground truth label obtained by the UE should be included in the report.

- The measurement quantities to be reported.

- A validity period to associate the measurement result with the ground truth.

For the above-mentioned information, the first and the second bullets are the same as in BM use cases for the same reason. Depending on the UE’s capability to obtain the ground truth label, the LMF can determine whether to include the indication to request that the UE reports the ground truth label along with the measurement results. Additionally, since multiple measurement quantities can be configured for the UE to measure, it would be flexible for the LMF to decide which measurement quantities are needed at some given time. In addition, since the time at which the UE obtains the measurement result (e.g., the RSTD) and the time at which the UE obtains the ground truth label (e.g., the location) may be different, the LMF can provide a validity period to the UE via the RequestLocationInformation message. The UE can then associate the measurement results with the ground truth label according to the validity period. For example, if the time difference between the measurement result and the ground truth label is shorter than the validity period, the UE associates the measurement result and the ground truth label as a single data. By doing so, the timestamps of the measurement result and the ground truth label are not needed to be reported.

Observation 4: For Pos use cases, the UE may obtain the ground truth label by itself.

Proposal 10: For Pos use cases, use a new type of RequestLocationInformation message (e.g., an AIML-RequestLocationInformation message) as the report configuration.

Proposal 11: For Pos use cases, the report configuration further contains: 1) a counter limiting the number of logged instances, 2) a condition that triggers the UE to start logging, 3) an indication whether to report the ground truth label, and 4) the measurement quantities to be reported.

Proposal 12: For Pos use cases, the LMF provides a validity period for the UE to associate the measurement result and the ground truth label via the AIML-RequestLocationInformation message. The UE can associate the measurement result and the ground truth label as a single data without reporting their timestamps if the difference of their timestamps is shorter than the validity period.

Enhancements of On-Demand Report

It is agreed that on-demand reporting is supported for NW-side data collection. Also, for BM case, UEInformationRequest/UEInformationResponse are agreed to be used, and the UE can indicate the availability of logged data to the network to trigger the on-demand request. Then, the next step is how the availability is indicated; i.e., which signalling is used and whether other information is included.

In our view, UAI is a candidate for the data availability indication. With the operation of prohibit timer, the UE can send a UAI indicating there is available data when the timer expires, to reduce the signalling overhead. In addition, the UE can also include the size of the buffered data in the UAI to inform the NW.

Proposal 13: For BM use cases, UAI is used for the indication of data availability for NW-side data collection. The UE can additionally include the size of the buffered data in the UAI.

Also, the UE can be configured to collect data under different conditions. In such cases, since it is up to the NW to determine which data to be collected, the NW can indicate the UE to only report the data associated with a specific condition. The indication can be included in the UEInformationRequest.

Proposal 14: For BM use cases, The NW can indicate the UE to only report the data associated with a specific condition via the UEInformationRequest message.

As for the pos use cases, based on the analysis in Section 2.3, it is desirable to have on-demand reporting and an indication of data availability. For the data availability indication, it can be achieved by adding the indication in the ProvideLocationInformation message. As for the on-demand request/report procedure, the messages RequestLocationInformation and ProvideLocationInformation can be reused.

Proposal 15: For Pos use cases, the following procedure can be considered for on-demand reporting:

- Step1: UE indicates the availability of data to the LMF via ProvideLocationInformation.

- Step2: LMF requests the logged data via RequestLocationInformation

- Step3: UE transfers the logged data to the LMF via ProvideLocationInformation

In case that on-demand measurement report is supported, a UE may face the problem that the logged measurement results increase upon every logging instance while the network does not request the measurement results. To prevent the UE from the overwhelming size of logged measurement results, a dropping rule is needed. In our view, although this can be up to UE’s implementation, an upper bound of logging time (like 48 hours in logged MDT) can be discussed.

Also, in current WID, the CSI-prediction use case is still under studying. Since the CSI-prediction use case can also be one-sided model, and the network can be the entity training and possessing the CSI-prediction models, NW-side data collection for the CSI-prediction use cases is also a valid issue. Since the frameworks of BM and CSI prediction share multiple similar properties (e.g., the same measurement/reporting structure), the NW-side data collection principles agreed for BM use cases can be applied to the CSI-prediction use cases as much as possible. Thus, considering the CSI-prediction is under studying, we propose to assume the NW-side data collection principles to be applied to CSI-prediction use cases.

Observation 5: For both BM and Pos use cases, a dropping method is needed to prevent the UE from the overwhelming size of logged measurement results.

Proposal 16: An upper bound of logging time (e.g., 48 hours in logged MDT case) is specified for the use case of data collection for NW-side model.

Proposal 17: RAN2 assumes the principles for NW-side data collection in BM use cases to be applicable in CSI-prediction use cases.

Conclusion

Based on the above, we have the following observations and proposals.

Observation 1: LMF should be capable of flexibly switching between different NW-side models leveraging different positioning methods, so it may need to collect data with different measurement quantities in different additional conditions.

Observation 2: For BM use cases, both the legacy L1-based and L3-based report configurations are feasible for data collection for NW-side model

Observation 3: It is up to NW to determine which data to be collected.

Observation 4: For Pos use cases, the UE may obtain the ground truth label by itself.

Observation 5: For both BM and Pos use cases, a dropping method is needed to prevent the UE from the overwhelming size of logged measurement results.

Proposal 1: For BM use cases, the UECapabilityEnquiry and UECapabilityInformation messages are used to request and provide the UE capabilities for data collection for NW-side model.

Proposal 2: For Pos use cases, new IEs AIML-RequestCapabilities and AIML-ProvideCapabilities are used to request and provide the UE capabilities for data collection for NW-side model.

Proposal 3: The UE can include the AS layer memory size information in the RRC/LPP message providing the UE capability information. The AS layer memory size information could be quantized by the maximum number of instances to be logged.

Proposal 4: For BM use cases, the measurement is configured as a CSI-RS resource set with an associated ID.

Proposal 5: For Pos use cases, a new type of ProvideAssistanceData message (e.g., AIML-ProvideAssistanceData) is used for the LMF to configure the measurements, where the configuration can include the required measurement quantity, the PRS resource set, and the associated ID (or area ID or Cell Global Identity).

Proposal 6: For BM use cases, RAN2 discuss the report configuration to be L1-based or L3-based.

Proposal 7: For BM use cases, the report configuration further contains: 1) a counter limiting the number of logged instances, 2) a condition that triggers the UE to start logging.

Proposal 8: For Pos use cases, RAN2 to support event-triggered logging.

Proposal 9: For Pos use cases, RAN2 to support on-demand reporting. Additionally, RAN2 to support the indication of data availability.

Proposal 10: For Pos use cases, use a new type of RequestLocationInformation message (e.g., an AIML-RequestLocationInformation message) as the report configuration.

Proposal 11: For Pos use cases, the report configuration further contains: 1) a counter limiting the number of logged instances, 2) a condition that triggers the UE to start logging, 3) an indication whether to report the ground truth label, and 4) the measurement quantities to be reported.

Proposal 12: For Pos use cases, the LMF provides a validity period for the UE to associate the measurement result and the ground truth label via the AIML-RequestLocationInformation message. The UE can associate the measurement result and the ground truth label as a single data without reporting their timestamps if the difference of their timestamps is shorter than the validity period.

Proposal 13: For BM use cases, UAI is used for the indication of data availability for NW-side data collection. The UE can additionally include the size of the buffered data in the UAI.

Proposal 14: For BM use cases, the NW can indicate the UE to only report the data associated with a specific condition via the UEInformationRequest message.

Proposal 15: For Pos use cases, the following procedure can be considered for on-demand reporting:

- Step1: UE indicates the availability of data to the LMF via ProvideLocationInformation.

- Step2: LMF requests the logged data via RequestLocationInformation

- Step3: UE transfers the logged data to the LMF via ProvideLocationInformation

Proposal 16: An upper bound of logging time (e.g., 48 hours in logged MDT case) is specified for the use case of data collection for NW-side model.

Proposal 17: RAN2 assumes the principles for NW-side data collection in BM use cases to be applicable in CSI-prediction use cases.

Reference

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, December 2023

- Report of 3GPP TSG RAN WG2 meeting#127, August 19-23, 2024

- Report of 3GPP TSG RAN WG2 meeting#127bis, October 14-18, 2024