3GPP TSG-RAN WG2 #128	R2-2410084

Orlando, USA, 18th – 22nd Nov 2024

Agenda item:	   8.3.5

Source:		Lenovo

Title:                             Other aspects of RRM measurement prediction

Document for:		Discussion and Decision

# Introduction

As instructed by the SID, the AI for mobility SI will study the RRM measurement prediction. In this paper, we try to analyse the scenarios of spatial and temporal domain predictions.

RP-234055 Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2]

# Discussion

The UE measures multiple beams (at least one) of a cell and the measurement results (power values) are averaged to derive the cell quality. In doing so, the UE is configured to consider a subset of the detected beams. Filtering takes place at two different levels: at the physical layer to derive beam quality and then at RRC level to derive cell quality from multiple beams. Cell quality from beam measurements is derived in the same way for the serving cell(s) and for the non-serving cell(s). Measurement reports may contain the measurement results of the X best beams if the UE is configured to do so by the gNB.

Figure 1: RRM measurement [38.300]

The high-level categorization of RRM measurement prediction is done in two ways, prediction in the spatial domain and prediction in the temporal domain. Considering the progress of AIML for air interface (R19) as a reference, the use case of beam management has been classified for spatial-domain beam prediction and temporal-domain beam prediction.

The following are selected as representative sub-use cases:

BM-Case1: Spatial-domain Downlink beam prediction

Predicting Set A of beams based on measurement results of Set B of beams

i): Set A and Set B are different (Set B is NOT a subset of Set A).

ii): Set B is a subset of Set A.

Note: Set A is for DL beam prediction. The codebook construction of Set A and Set B can be clarified by companies.

BM-Case2: Temporal Downlink beam prediction

Predicting Set A of beams based on the historic measurement results of Set B of beams

i): Set A and Set B are different (Set B is NOT a subset of Set A).

ii): Set B is a subset of Set A (Set A and Set B are not the same).

iii): Set A and Set B are the same.

Note: Set A is a set of beams which are predicted, and Set B is a set of beams whose measurements are taken as inputs of the AI/ML model.

The following agreements were made on the general aspects of RRM measurement prediction:

Agreements

- For cell level measurement prediction model, at least consider the following cases:

Case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

Case 2: To directly predict cell level results based on cell level results.

Case 3: To directly predict cell level results based on beam level results

- We will consider intra-frequency intra and inter-cell spatial domain measurement predictions, for beam and cell level measurements.

- For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level.   As baseline we will focus on pure temporal prediction.

- The following items can be considered as a baseline for the prediction accuracy of the cell-level measurement prediction：

Spatial-domain prediction: RSRP difference to the actual measurement

Temporal prediction: RSRP difference to the actual measurement

measurement reduction rate as one KPI

According to the agreements in the previous meetings, both spatial domain prediction and temporal domain prediction will be considered. At the same time, pure temporal prediction will be the baseline for measurement predictions. For temporal domain prediction, two cases: Case A and Case B, referring to the RAN1 AI/ML air interface TR, were agreed upon. It would be helpful to develop a common understanding of Case A and Case B of temporal prediction, defined as follows according to TR 38.843:

Case A: based on the number of measurements/RSs and prediction time.

Case B: based on a periodicity T of the required reference signals for measurements.

The AI for mobility SID suggests studying both UE-sided and NW-sided models for RRM measurement prediction. In either case, the RRM measurement prediction conducted by UE or NW can be beneficial for the NW in determining the mobility related configuration/decision (e.g., beam management, handover). For example, it can be used by the network to decide which beams to configure. Predictions at the NW-side can be used by the network to make handover decisions efficiently for example to reduce ping-pong effect.

Observation: The RRM measurement prediction conducted by UE or NW can be beneficial for the NW in determining the mobility-related decisions (e.g., beam management, handover).

UE-side RRM measurement prediction

As the RRM measurements are a function of UE, therefore, it would be logical to consider UE-sided measurement prediction as the starting point for this agenda item. NW-sided measurement prediction can be studied after some progress on UE-sided measurement prediction has been made. Besides, RRM measurement prediction using UE-sided model has more relevance with RAN2 specification, while NW-sided model has more RAN3/SA impacts.

Observation: RRM measurement prediction using UE-sided model is the baseline.

The two possible approaches that are being discussed in this study are beam-level and cell-level predictions. Beam-level measurement predictions can be used for the beam selection in case of mobility. During RACH initial access in a mobility scenario, the UE may select the “best beam” in the target cell based on the prediction. Thus, measurement prediction can help to reduce the number of measurements performed by the UE. Additionally, cell-level measurement prediction such as temporal domain prediction can enable the selection of a “best target cell” for a potential handover. Thus, improving the performance of handover by reducing the probability of a handover failure.

For UE-sided RRM measurement prediction, the UE must be configured by the network for predicting the measurements which may vary depending on the different cases of measurement prediction such as spatial/temporal domain prediction and their further classifications. Beam level measurements are generated directly from the L1 measurements while cell level measurements are derived from the L1 measurements using certain rules. From beam management in AI/ML for air interface: A set of beams i.e., set B, is used as an input to predict another set of beams i.e., set A. For temporal or spatial domain prediction, the network provides a configuration of set B beams for the UE to measure, which is used for inference for the UE-side model and thus set A is predicted. Another possible scenario can be when the network provides a configuration of set B and set A, when set B is not a subset of set A. For beam and cell level measurements, intra and inter-cell spatial domain measurement predictions, will be considered in this study. As cell-level measurements is the focus of this study, they can be either generated or predicted based on the beam-level or cell-level inputs. RAN2 agreement lists three possible cases for cell-level RRM measurement prediction:

Case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

Case 2: To directly predict cell level results based on cell level results.

Case 3: To directly predict cell level results based on beam level results

Taking these cases as the baseline, we assume Cell-1 to be the candidate for intra-cell RRM measurement predictions and Cell-1/Cell-2 as the candidate for inter-cell RRM measurement predictions. Table 1 represents the sub-cases highlighting input, intermediate output, and the final output, for the three cases agreed for RRM measurement prediction.

Observations:

Table 1: Cases of RRM measurement prediction

| Spatial domain prediction: Inter-cell (Cell1 and Cell2) |  |  |  |
| --- | --- | --- | --- |
|  | Case 1A | Case 2A | Case 3A |
| Input | beam-level (BL) results of Cell1 | cell-level (CL) results of Cell1 | beam-level (BL) results of Cell1 |
| Intermediate output | predicted beam-level (BL) results of Cell2 | cell-level (CL) results of Cell2 | cell-level (CL) results of Cell2 |
| Output | Predicted cell-level measurements of Cell2 |  |  |
| Temporal & Spatial domain prediction: Intra-cell (Cell1) |  |  |  |
|  | Case 1B | Case 2B | Case 3B |
| Input | beam-level (BL) results of Cell1 | cell-level (CL) results of Cell1 | beam-level (BL) results of Cell1 |
| Intermediate output | predicted beam-level (BL) results of Cell1 | cell-level (CL) results of Cell1 | cell-level (CL) results of Cell1 |
| Output | Predicted cell-level measurements of Cell1 |  |  |

The goal of temporal domain and spatial domain prediction can be derived from the final prediction output. RAN2 is suggested to discuss if temporal domain and spatial domain RRM measurement prediction can be applied at the same time.

Proposal 1: RAN2 should discuss the feasibility of the coexistence of temporal domain and spatial domain RRM measurement predictions.

The above table presents different cases of temporal and spatial domain prediction for inter/intra-cell scenarios. The assumption is that a configuration from the network is always required, which is used for or in the process of measurement inference. However, the measurement configuration may be different for each case and depends on whether the cell-level measurements are predicted based on predicted beam-level results, cell-level results, or beam-level results, of the same cell or a different cell. In some cases, it may be a configuration of a set of beams, as agreed in R19 AI/ML air interface for beam management. Based on the progress of LCM in R19 AI/ML air interface, the legacy measurement configuration framework can be used as a baseline, which can be extended further considering different cases.

Proposal 2: Legacy RRC measurement configuration framework is used for data collection and inference of RRM measurement prediction. Details are FFS.

Observation: The network configuration can differ for the different cases of spatial and temporal domain RRM measurement prediction.

Considering the baseline case of pure temporal domain RRM measurement predictions, in some cases, the network may provide the UE with more than one set of configurations, for example, different set A and set B configurations to be measured and/or predicted. UE may activate one of the configurations from the set upon being configured. The configuration for measurement prediction can also include further guiding instructions from the network for the UE to activate certain configurations.

Proposal 3: NW configures the UE with one or more configurations for RRM measurement prediction including instructions to activate a respective configuration.

Proposal 4: The initial state of configuration (activated/deactivated) is configurable by the network.

Proposal 5: Legacy RRC measurement reporting framework is used as a baseline for UE to report the prediction outcome to the NW. Possible reporting enhancements for RRM measurement prediction are left to the normative phase discussion.

In legacy, s-measureconfig is configured to UE which will control UE when to perform the measurement for non-serving cell. The purpose is for power saving since a UE located at the cell centre will not be handed over to a neighbour cell. RAN2 agreed to support intra-frequency intra and inter-cell spatial domain measurement predictions, for beam and cell level measurements. Similarly, when UE stays at centre of one cell, UE is not expected to predict Inter-cell measurement results. One option is that legacy s-measureconfig can be used to control when the UE should perform measurement prediction for non-serving cells. Alternatively, one separate threshold can be configured for the UE to determine when the measurement predictions should be performed for the non-serving cell. For example, if the quality of the serving cell is less than the configured threshold, the measurement prediction for non-serving cells can be triggered.

Proposal 6: UE may perform measurement prediction for non-serving cells if the channel quality of the serving cell is worse than a configured threshold. FFS the threshold is from legacy s-measureconfig IE or a separate threshold.

NW-side RRM measurement prediction

As described in the SID, the AIML model used for RRM measurement prediction can also be deployed at NW. compared to using a UE-sided model, using a NW-sided model for RRM measurement prediction may have the following advantages:

- NW has more powerful computing capability

- NW can easily collect enough training data from UEs under its coverage via legacy L1/L3 measurement reports. The complexity of data collection to train a UE-sided model can be avoided.

- NW can start the NW-sided AIML inference without additional prediction related configuration towards UE.

- For the sake of model performance monitoring, NW node can easily configure the UE to do necessary L1/L3 measurements and report, or even request the neighbour NW node to provide the necessary UE measurements after a handover execution.

From RAN2 point of view, how NW requests input to support RRM measurement prediction using NW-sided model would be the focus. As a starting point assumption, NW can request input from UE in one-shot (aperiodic), periodic, or event-triggered manner.

Proposal 7: NW requests input from UE in a one-shot (aperiodic), periodic, or event-triggered manner for RRM measurement prediction for NW-sided model.

The input from UE will of course include any legacy L1/L3 beam measurement results or legacy L3 cell measurement results. At the same time, we believe the L1 beam prediction result by a UE supporting the AI for air interface beam prediction use case could also be considered as possible input for further L3 beam/cell level result prediction at NW.

Proposal 8: The input from UE could be either legacy L1/L3 beam/cell measurements or L1 beam prediction results (if UE is capable of L1 beam prediction in the AI for air interface).

Cell-specific vs cluster-based RRM measurement prediction

There have been some discussions in the past meetings on cluster-based RRM measurement prediction, however, we think that RAN2 should discuss the interpreation and differences in cell-specific and cluster-based AI/ML models. In our understanding, a cell-specific AI/ML model is trained with inputs from a particular cell, and it can only be applied for inference (satisfying accuracy requirement) to that particular cell. Whereas cluster-based models are trained using inputs from different cells and can be applied for inference to one or more cells in the cluster. If the difference between the two is limited to the training phase, then there may not be a need to specify these as separate approaches.

Proposal 9: RAN2 should discuss the difference between cell-specific and cluster-based prediction approach.

# 3	Conclusion

Based on the discussion above, we request RAN2 to discuss and agree on the following proposals:

UE-sided RRM measurement prediction

Observation: The RRM measurement prediction conducted by UE or NW can be beneficial for the NW in determining the mobility-related decisions (e.g., beam management, handover).

Observation: RRM measurement prediction using UE-sided model is the baseline.

Proposal 1: RAN2 should discuss the feasibility of the coexistence of temporal domain and spatial domain RRM measurement predictions.

Proposal 2: Legacy RRC measurement configuration framework is used for data collection and inference of RRM measurement prediction. Details are FFS.

Observation: The network configuration can differ for the different cases of spatial and temporal domain RRM measurement prediction.

Proposal 3: NW configures the UE with one or more configurations for RRM measurement prediction including instructions to activate a respective configuration.

Proposal 4: The initial state of configuration (activated/deactivated) is configurable by the network.

Proposal 5: Legacy RRC measurement reporting framework is used as a baseline for UE to report the prediction outcome to the NW. Possible reporting enhancements for RRM measurement prediction are left to the normative phase discussion.

Proposal 6: UE may perform measurement prediction for non-serving cells if the channel quality of the serving cell is worse than a configured threshold. FFS the threshold is from legacy s-measureconfig IE or a separate threshold.

NW-sided RRM measurement prediction

Proposal 7: NW requests input from UE in a one-shot (aperiodic), periodic, or event-triggered manner for RRM measurement prediction for NW-sided model.

Proposal 8: The input from UE could be either legacy L1/L3 beam/cell measurements or L1 beam prediction results (if UE is capable of L1 beam prediction in the AI for air interface).

Cell-specific vs cluster-based RRM measurement prediction

Proposal 9: RAN2 should discuss the difference between cell-specific and cluster-based prediction approach.

# Reference

[1] RP-234055, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, RAN#102.

[2] TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)