3GPP TSG-RAN WG2 Meeting #128	R2-2410114

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:	8.6.3

Source: 	China Telecom

Title:	Discussion on L1 event triggered measurement reporting

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, the Rel-19 WI of Mobility enhancement was widely discussed and much progress has been made on measurement enhancements for LTM. Regarding the event-triggered L1 measurement reporting for LTM, the following agreements were reached.

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12.	NW can configure a dedicated SR configuration for MR MAC CE transmission.

In this contribution, we would like to share our opinions on further details of L1 event triggered measurement reporting for LTM.

- Discussion

Event evaluation and triggering

In previous RAN2 meeting, the issue on beam(s) of the serving cell and candidate cell used for event evaluation was discussed. For the event evaluation in L1 measurement reporting for serving cell, current beam (i.e. a beam corresponding to the indicated TCI state) is used, while for the candidate cell(s), any beam in candidate RS configuration can be used for LTM event evaluation. In last RAN2 meeting, the granularity of TTT and TTT operation on current beam changing were discussed but with no consensus reached.

Regarding the granularity of TTT, potential options include per beam, per the associated CSI-RS set or per cell. Since RAN2 has agreed to support beam-level event for event-triggered L1 measurement reporting, it is straightforward to have beam-level TTT for event evaluation. Besides, the intention to support TTT for event-triggered L1 measurement reporting is to avoid unnecessary L1 measurement reporting due to sudden changes of beam quality. TTT operation per beam can help to identify relatively stable beam(s) for measurement reporting. For per cell or per CSI-RS set associated TTT operation, it will be more complex for the UE to maintain the TTT timer and determine the beam(s) for measurement reporting. For example, during TTT, if the candidate beam that satisfy the entering condition is changed, whether to reset the TTT may need further discussion. When triggering the measurement report, which beam(s) shall be treated as satisfying the event may need further discussion as well. Thus, we prefer to have TTT operation per beam.

Proposal 1: For event evaluation for event-triggered L1 LTM measurement reporting, TTT operation is per beam.

Based on the assumption of per beam TTT operation, we can discuss TTT operation on current beam changing case by case, since different events apply to different beams.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

For Event LTM2, only beam of serving cell is related. It is already agreed that only current beam is considered for event evaluation for serving cell. Thus, there is only one TTT for current beam. When the current beam is changed, there is no need to reset the TTT if the new current beam still satisfies the entering/leaving condition of event LTM2. If the new current beam has better or worse quality than the former one and doesn’t satisfy the entering condition or leaving condition any more, the TTT could be stopped as normal TTT operation.

For Event LTM4, only beam of candidate cell is related. In this case, it is obvious that the change of current beam shall not influence the TTT operation of candidate beam.

For Event LTM3 and LTM5, there might be multiple TTTs for multiple candidate beams that satisfy the entering / leaving condition(s). The same logic could be followed as Event LTM2. After changing to a new current beam, for the candidate beam(s) that still meet the event conditions, TTT are not reset.

To have a unique TTT operation for different event configuration, we suggest that TTT is not reset on current beam changing.

Proposal 2: For event evaluation for event-triggered L1 LTM measurement reporting, TTT is not reset on current beam changing.

In legacy Layer 3 mobility, S-MeasureConfig is used to avoid unnecessary measurements on non-serving cells when the quality of the current serving cell is good enough. For event-triggered L1 measurement reporting for LTM, similar mechanism could be considered as well to avoid unnecessary measurements on candidate beams if the current beam of the serving cell is good enough.

Proposal 3: To avoid unnecessary measurements on candidate beams in LTM, S-Measure like mechanism could be considered for event-triggered L1 LTM measurement report.

Contents in event-triggered measurement report

In last RAN2 meeting, the contents in event-triggered measurement report were discussed. The baseline of contents for event-triggered L1 measurement include beam information, beam quantity and triggered event information. Regarding how to indicate beam information, the following potential solutions are listed as FFS.

- Option 1: SSBRI/CRI

- Option 2: LTM config ID + SSB/CSI-RS id

In Rel-18 LTM, CSI report procedure is used for reporting L1 measurements for LTM. SSBRI is used to indicate the beam information associated to L1-RSRP that reported within a single L1 measurement report instance. As specified in TS 38.331, the max number of SSBRI could be up to 512, which may need up to 9 bits. Considering MAC CE is byte aligned, if we have flexible length of SSBRI fields in a MAC CE, it will be hard to design the format of MR MAC CE. If fixed length of SSBRI fields is used, 2 bytes might be needed. For option 2, it is easier to design the MAC CE and more flexible for reporting.

Proposal 4: For indicating beam information for candidate cell in MR MAC CE, the LTM config ID and SSB/CSI-RS ID could be used.

In last RAN2 meeting, it was agreed that MR MAC CE can include up to N beams. Whether the beam should satisfy the event or not remains FFS. For Event LTM2, if only the beam satisfying the event is reported, then only the L1 measurement results of current beam is included. If no other beam satisfying other events at the same time, the network will not get additional information of other beam(s) from candidate cell(s) or serving cell. Thus, it is useful to report the L1 measurement result of other beams from candidate cell(s) or serving cell as well. For other events, if TTT operation is per beam, it is very likely that the related beams satisfy the conditions in different time. Similarly, if only reporting the beam satisfied the event is allowed, then the network may not get enough L1 measurement results in one MR MAC CE. Thus, we suggest allowing to report beam(s) not satisfying the event in MR MAC CE. Whether to allow it can be configured by the NW.

Proposal 5: For event-triggered L1 LTM measurement reporting, the beam(s) not satisfying the event cloud be reported in MR MAC CE, which is configured by the NW.

- Conclusion

In this contribution, we discuss L1 event triggered measurement reporting for Rel-19 LTM. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: For event evaluation for event-triggered L1 LTM measurement reporting, TTT operation is per beam.

Proposal 2: For event evaluation for event-triggered L1 LTM measurement reporting, TTT is not reset on current beam changing.

Proposal 3: To avoid unnecessary measurements on candidate beams in LTM, S-Measure like mechanism could be considered for event-triggered L1 LTM measurement report.

Proposal 4: For indicating beam information for candidate cell in MR MAC CE, the LTM config ID and SSB/CSI-RS ID could be used.

Proposal 5: For event-triggered L1 LTM measurement reporting, the beam(s) not satisfying the event cloud be reported in MR MAC CE, which is configured by the NW.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

RP-24xxxxx	draft Report of 3GPP TSG RAN WG1 meeting #116bis

3GPP TS 38.331 18.0.0: "NR; Radio Resource Control (RRC) protocol specification".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB

R2-24xxxx	Draft report of 3GPP TSG RAN WG2 meeting #127