3GPP TSG-RAN WG2 Meeting #128	R2-2410115

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:	8.6.4

Source: 	China Telecom

Title:	Discussion on conditional intra-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, the Rel-19 WI of Mobility enhancement was widely discussed and much progress has been made. Regarding condition intra-CU LTM, the following agreements were reached.

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

In this contribution, we would like to share our opinions on remaining issues on conditional Intra-CU LTM in non-DC case.

- Discussion

Stage-2 procedure for intra-CU conditional LTM

For stage-2 procedure of intra-CU conditional LTM, it is common understanding that it includes conditional LTM preparation, the early synchronization, conditional LTM evaluation and execution and conditional LTM completion. Based on the agreements made in last RAN2 meeting, we try to give a potential stage-2 signalling flow for intra-CU conditional LTM in non-DC case as illustrated in Fig.1.

Proposal 1: RAN2 considers the stage-2 signalling flow for intra-CU conditional LTM in non-DC case, as illustrated in Fig.1.

Fig.1 Stage-2 signalling flow for intra-CU conditional LTM in non-DC case

Regarding the early synchronization phase, it is agreed that PDCCH ordered early TA acquisition is supported for conditional LTM in last RAN2 meeting. How the NW determines candidate cells for early sync and when to trigger the procedure remain FFS. There are the following potential solutions discussed in last meeting.

- Option 1: UE needs to be configured with legacy L3 measurement reporting or LTM L1 measurement reporting if conditional LTM is configured.

- Option 2: UE requests NW to trigger PDCCH ordered early TA on certain candidate cells.

- Option 3: Up to NW implementation.

For Option 2, it will introduce extra interaction between the UE and the gNB before triggering the conditional LTM. If the channel quality is already bad, the procedure may fail, which may influence the execution of conditional LTM. For Option 1, it is workable but restricts the NW configuration, which is not recommended. Thus, Option 3 is enough.

Proposal 2: For conditional LTM, the triggering of PDCCH ordered early TA acquisition and the selection of candidate cells for early sync could be up to NW implementation.

UE evaluated conditions for triggering conditional LTM

In previous RAN2 meetings, the event-triggered L1 measurement reporting was widely discussed and much progress has been made, which can provide guidance to UE evaluated conditions for triggering conditional LTM.

In RAN2#126 meeting, the following beam-level events for L1 LTM measurement reporting are introduced.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

In last meeting, it is agreed that Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. The detailed definition of events for conditional LTM is not discussed.

For conditional LTM, it is common understanding that the triggering event shall be in beam-level as well. For conditional LTM cell switch, it is also very important to avoid “Ping Pong” or unnecessary cell switch, which may reduce the mobility performance and user experience. Therefore, not only the quality of the candidate beam but also the quality of the serving beam shall be considered when evaluating whether to trigger a conditional LTM. Similar to CHO, the following conditional events could be introduced for conditional LTM.

- CondEvent LTM3: Beam of conditional candidate cell becomes amount of offset better than beam of serving cell;

- CondEvent LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of conditional candidate cell becomes better than another absolute threshold2.

Proposal 3: RAN2 to consider the following beam-level conditional events in order to support conditional LTM in non-DC case.

- CondEvent LTM3: Beam of conditional candidate cell becomes amount of offset better than beam of serving cell;

- CondEvent LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of conditional candidate cell becomes better than another absolute threshold2.

For event-triggered L1 measurement reporting, the following agreements were reached on beam of the serving cell and candidate cell used for event evaluation.

Agreements on L1 MR event evaluation

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

Considering the conditional LTM is a kind of cell switch procedure, which is not similar to L1 measurement reporting, to follow the same principle on event evaluation may lead to more frequent cell switch. Thus, the serving beam and candidate beam(s) used for event evaluation in above conditional events might need further discussion. Potential options are listed as below.

- Option 1: all the configured beams of the serving cell or candidate cell;

- Option 2: a certain number of beams of the serving cell or candidate cell, e.g. the best X beams or the worst Y beams;

- Option 3: a single beam of the serving cell or candidate cell, e.g. the best beam, the worst beam or any beam.

For Option 1, it seems too strict to trigger conditional LTM only when all the configured candidate beams are good enough while the serving beams are extremely bad. It may take a long time to meet the trigger conditions, which may lead to cell switch failure or reduce the user’s experience. For Option 3, only a single beam is used for evaluating the trigger conditions, which may lead to more frequent or unnecessary conditional LTM cell switch. Option 2 could be a trade-off solution, which can reduce the evaluation time before triggering conditional LTM as well as guarantee the UE to select a target cell with good enough beams. Only when a certain number of beams of the serving cell or conditional candidate cell meet the trigger conditions of conditional events, the conditional LTM execution could be triggered.

Proposal 4: For event evaluation for conditional LTM, a certain number of beams of the serving cell or conditional candidate cell, e.g. the best X beams or the worst Y beams, could be used.

In CHO, the network can configure up to 2 condition events for a certain candidate cell. When configured, both of the conditional events have to be met simultaneously for triggering CHO. Since the motivation to support conditional LTM is not only to achieve high robustness but also short interruption, there seems no strong need to allow the configuration of up to 2 events for a certain candidate cell.

Proposal 5: For conditional LTM, there is no need to allow the configuration of up to 2 events for a certain candidate cell.

Intra-CU conditional LTM based on L3 measurement

In previous meeting, L3 measurement based LTM is discussed in both RAN2 and RAN3. The following agreements were reached in last RAN2 meeting and sent to RAN3 for further discussion.

RAN2 understand early synchronization is also applied to L3 measurement based LTM.

RAN2 also understand for L3 measurement based LTM, that the CU decides early synchronization according to the received L3 measured results, and indicates that decision with the corresponding beam information to a DU, then the DU will trigger early synchronization.

Based on the above discussion, whether to support L3 measurement based conditional LTM was discussed as well. The following FFS is related to L3 measurement based conditional LTM.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

For conditional LTM execution condition, if only L3 measurement is performed, the beam-level event will not be used. Then it is reasonable to reuse CHO conditions. But the detail configuration may need further discussion. For the generation of execution condition when L3 measurement is used, it is quite similar to CHO case so that the gNB-CU is responsible for the decision of execution conditions.

However, since the R18 LTM based on L3 measurement is still under discussion in RAN3, which may also influnce the L3 measurement based conditional LTM. We suggest RAN2 first focus on the discussion for L1 measurement based conditional LTM, and wait for more RAN3 progress for L3 measurement based conditional LTM.

Proposal 6: RAN2 focus on the discussion for L1 measurement based conditional LTM, and wait for more RAN3 progress for L3 measurement based conditional LTM.

- Conclusion

In this contribution, we discuss the remaining issues on conditional intra-CU LTM in non-DC case. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: RAN2 considers the stage-2 signalling flow for intra-CU conditional LTM in non-DC case, as illustrated in Fig.1.

Proposal 2: For conditional LTM, the triggering of PDCCH ordered early TA acquisition and the selection of candidate cells for early sync could be up to NW implementation.

Proposal 3: RAN2 to consider the following beam-level conditional events in order to support conditional LTM in non-DC case.

- CondEvent LTM3: Beam of conditional candidate cell becomes amount of offset better than beam of serving cell;

- CondEvent LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of conditional candidate cell becomes better than another absolute threshold2.

Proposal 4: For event evaluation for conditional LTM, a certain number of beams of the serving cell or conditional candidate cell, e.g. the best X beams or the worst Y beams, could be used.

Proposal 5: For conditional LTM, there is no need to allow the configuration of up to 2 events for a certain candidate cell.

Proposal 6: RAN2 focus on the discussion for L1 measurement based conditional LTM, and wait for more RAN3 progress for L3 measurement based conditional LTM.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

RP-24xxxxx	draft Report of 3GPP TSG RAN WG1 meeting #116bis

3GPP TS 38.331 18.0.0: "NR; Radio Resource Control (RRC) protocol specification".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB